| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N1024-v3 |
| --- | --- |

| Title: | Description of Core Experiment 4 (CE4): Inter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Haitao Yang Chun-Chi Chen | Email: | haitao.yang@huawei.com chunchic@qti.qualcomm.com |
| Source: | CE4 coordinators |  |  |

_____________________________

- Abstract

The goal of Core Experiment 4 (CE4) is to investigate various inter prediction techniques on top of VTM-5.0. It comprises three categories,

- Switchable interpolation filter,

- Affine motion compensation,

- Combination of LIC and affine MC.

CE4-1: Switchable interpolation filter

Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 1.1 | Additional half-pel AMVR mode only (for comparison purposes) | JVET-N0309 | A. Henkel (HHI) |  |
| 1.2 | 1.1 + one alternative switchable luma half-pel interpolation filter, selected half-pel filter is propagated as part of the spatial merging candidate | JVET-N0309 | A. Henkel (HHI) |  |
| 1.3 | 1.1 + two alternative switchable luma half-pel interpolation filters, selected half-pel filter is propagated as part of the spatial merging candidate | JVET-N0309 | A. Henkel (HHI) |  |
| 1.4 | 1.2 without propagation of selected half-pel filter in merge mode | JVET-N0309 | A. Henkel (HHI) |  |
| 1.5 | 1.3 without propagation of selected half-pel filter in merge mode | JVET-N0309 | A. Henkel (HHI) |  |
| 1.6 | 1.2 with DMVR disabled for CUs using an alternative half-pel interpolation filter | JVET-N0309 | A. Henkel (HHI) |  |
| 1.7 | 1.3 with DMVR disabled for CUs using an alternative half-pel interpolation filter | JVET-N0309 | A. Henkel (HHI) |  |

Detailed description of tests

- Test 1.1~1.3 in JVET-N0309 (HHI)

In JVET-N0309, a switched luma half-pel interpolation filter is proposed.  The switching of the half-pel luma interpolation filter is done depending on the motion vector accuracy.  In addition to the existing quarter-pel, full-pel, and 4-pel AMVR modes, a new half-pel accuracy AMVR mode is introduced.  Only in case of half-pel motion vector accuracy, an alternative half-pel luma interpolation filter can be selected.

Half-pel AMVR mode

An additional AMVR mode for non-affine non-merge inter-coded CUs is proposed which allows signaling of motion vector differences at half-pel accuracy.  The existing AMVR scheme of the current VVC draft is extended straightforward in the following way: Directly following the syntax element amvr_flag, if amvr_flag == 1, there is a new context-modeled binary syntax element hpel_amvr_flag which indicates usage of the new half-pel AMVR mode if hpel_amvr_flag == 1. Otherwise, i.e. if hpel_amvr_flag == 0, the selection between full-pel and 4-pel AMVR mode is indicated by the syntax element amvr_precision_flag as in the current VVC draft.

Alternative luma half-pel interpolation filters

In JVET-N0309, alternative luma half-pel interpolation filters are proposed. For a non-affine non-merge inter-coded CU which uses half-pel motion vector accuracy (i.e., the half-pel AMVR mode), an alternative luma half-pel interpolation filter is used. For test 1.2, one alternative luma half-pel interpolation filter is used. For test 1.3, a switching between two alternative half-pel interpolation filters is made based on the value of a new syntax element if_idx. The syntax element if_idx is only signaled in case of half-pel AMVR mode. In case of skip/merge mode using a spatial merging candidate, the information which interpolation filter is applied for the half-pel position is inherited from the neighbouring block.

- Test 1.4~1.5 in JVET-N0309 (HHI)

The tests 1.4 and 1.5 correspond to tests 1.2 and 1.3, with the distinction that in tests 1.4 and 1.5 the information which interpolation filter is applied for the half-pel position is not inherited from the neighbouring block in case of skip/merge mode using a spatial merging candidate.

- Test 1.6~1.7 in JVET-N0309 (HHI)

The tests 1.6 and 1.7 correspond to tests 1.2 and 1.3, with the distinction that in tests 1.6 and 1.7 the DMVR process is not applied to CUs which are encoded in spatial skip/merge mode and which have inherited the information from the neighbouring block that an alternative interpolation filter is applied for the half-pel position.

CE4-2: Affine motion compensation

Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 2.1a | Prediction refinement with optical flow. The subblock size for affine motion compensation is 4x4 for both uni- and bi-prediction. | JVET-N0236 | J. Luo (InterDigital) | T.-D. Chuang (MediaTek) |
| 2.1b | Prediction refinement with optical flow. The subblock size for affine motion compensation is 4x4 for uni-prediction and 8x8 for bi-prediction. | JVET-N0236 | J. Luo (InterDigital) | T.-D. Chuang (MediaTek) |
| 2.1c | Prediction refinement with optical flow. The subblock size for affine motion compensation is 4x4 for uni-prediction with and 8x8 for bi-prediction. 8-tap interpolation filter is used for 8x8 subblocks. | JVET-N0236 | J. Luo (InterDigital) | T.-D. Chuang (MediaTek) |
| 2.2a | Prediction refinement with optical flow. The subblock size for affine motion compensation is 4x4 for both uni- and bi-prediction. The delta MV for the refinement is calculated for each 2x2. The pixels on the extended borders are copied from the nearest integer pixel position in the reference picture using nearest rounding. | JVET-N0504 | H. Huang (Qualcomm) |  |
| 2.2b | Prediction refinement with optical flow. The subblock size for affine motion compensation is 4x4 for both uni- and bi-prediction. The delta MV for the refinement is calculated for each 2x2. The pixels on the extended borders are copied from the nearest integer pixel position in the reference picture using floor rounding. | JVET-N0504 | H. Huang (Qualcomm) |  |
| 2.3a | Interweaved prediction for luma component. | JVET-N0261 | K. Zhang (Bytedance) |  |
| 2.3b | Interweaved prediction for luma component with equal weighting values | JVET-N0261 | K. Zhang (Bytedance) |  |
| 2.3c | 2.3a + 2.1a PROF is applied on the two auxiliary prediction individually. | JVET-N0261 | K. Zhang (Bytedance) |  |
| 2.3d | 2.3b + 2.1a PROF is applied on the two auxiliary prediction individually. | JVET-N0261 | K. Zhang (Bytedance) |  |
| 2.4a | Simplifications of interweaved affine motion compensation in 2.3a, reusing bi-prediction processes. | JVET-N0399 | G. Li (Tencent) |  |
| 2.4b | Simplifications of interweaved affine motion compensation in 2.3b, reusing bi-prediction processes. | JVET-N0399 | G. Li (Tencent) |  |
| 2.5a | Phase-variant affine MC. Up to 9 rows and 4 columns of MVx/MVy are derived. | JVET-N0510 | T.-D. Chuang (MediaTek) |  |
| 2.5b | Phase-variant affine MC. Only 5 rows and 4 columns of MVx/MVy are derived. | JVET-N0510 | T.-D. Chuang (MediaTek) |  |
| 2.5c | Phase-variant affine MC. 2.5a with fallback mode disabled. | JVET-N0510 | T.-D. Chuang (MediaTek) |  |
| 2.5d | Phase-variant affine MC. The input reference samples of a 4x4 block is limited to be 9x9 square block. | JVET-N0510 | T.-D. Chuang (MediaTek) |  |
| 2.5e | 2.5a + 2.1a | JVET-N0510 | T.-D. Chuang (MediaTek) |  |

Complexity analysis

Basic operations per 4x4 subblock should be provided for complexity analysis. In addition, analysis of additional memory access and local buffer requirement along with a technique under test is requested, normalized to 4x4 subblocks as well. The capability of parallel processing

| Test# | Number of mults | Number of adds | Number of shifts | Additional memory access | Additional local buffer | Others |
| --- | --- | --- | --- | --- | --- | --- |
| VTM-5.0 |  |  |  |  |  |  |
| 2.1a |  |  |  |  |  |  |
| 2.1b |  |  |  |  |  |  |
| … |  |  |  |  |  |  |

Subjective evaluation

Following common test condition, sequence CatRobot1 encoded with RA configuration at QP 37 will be used for subjective evaluation.

Detailed description of tests

- Test 2.1: Prediction refinement with optical flow for affine mode (JVET-N0236, InterDigital)

To achieve a finer granularity of motion compensation, this contribution proposes a method to refine the sub-block based affine motion compensated prediction with optical flow. After the sub-block based affine motion compensation is performed, luma prediction sample is refined by adding a difference derived by the optical flow equation. The sub-block-based affine motion compensation is performed to generate sub-block prediction. The spatial gradients of the sub-block prediction are calculated at each sample location using a 3-tap filter [-1, 0, 1]. The sub-block prediction is extended by one pixel on each side for the gradient calculation. The pixels on the extended borders are copied from the nearest integer pixel position in the reference picture. The MV difference between pixel MV and the sub-block MV, as shown in the below figure. The luma prediction refinement is calculated by the optical flow equation based on the MV differences and gradients. Finally, the luma prediction refinement is added to the sub-block prediction.

3 sub-tests are planned in CE. In test a, affine subblock size is 4x4 in both uni- and bi-prediction. In test b, affine subblock size is 4x4 for uni-prediction and 8x8 for bi-prediction. Test c is test b except 8-tap interpolation filter is used for 8x8 sub-block.

Figure. Sub-block MV VSB and pixel Δv(i,j) (red arrow)

- Test 2.2: Simplified prediction refinement for affine motion compensation (JVET-N0504, Qualcomm)

Based on test 2.1, futher simplification is proposed.

In the proposed method, the prediction refienment method is combined with 2x2 subblock based affine motion compenstion to simplified the refinement process. The 2x2 subblock motion field is used for deriving the refinement offset. The spatial gradients of the sub-block prediction are calculated at each sample location using a 3-tap filter [-1, 0, 1]. The sub-block prediction is extended by one pixel on each side for the gradient calculation.

In test 2.2a, the gradient is calculated in the 4x4 subblock basis and without refering the prediction sample outside the 4x4. The pixels on the extended borders are copied from the nearest integer pixel position in the reference picture using nearest rounding.

In test 2.2b, the gradient calculation is aligned with that in BDOF. The pixels on the extended borders are copied from the nearest integer pixel position in the reference picture using floor rounding.

The proposed method is applied to both uni-prediction and bi-prediction, but it’s applied to luma only.

- Test 2.3: Interweaved Prediction for Affine Motion Compensation (JVET-N0261, Bytedance)

In this contribution, an interweaved prediction approach is proposed for affine motion compensation (AMC). With the interweaved prediction, a coding block is divided into sub-blocks with two different dividing patterns. Then two auxiliary predictions are generated by AMC with the two dividing patterns. The final prediction is calculated as a weighted-sum of the two auxiliary predictions.

In test 2.3a, interweaved prediction will be tested with the unequal weighting values as proposed in JVET-N0261.

In test 2.3b, interweaved prediction will be tested with equal weighting values (1/2, 1/2).

In test 2.3c, test 2.3a will be combined with test 2.1a.

In test 2.3d, test 2.3b will be combined with test 2.1a.

- Test 2.4: Simplifications of interweaved affine mode (JVET-N0399, Tencent)

In interweaved affine mode, the two predictions are both uni-predicted, and then a weighted averaging process is invoked to generate the final prediction. To keep higher interpolation accuracy and simple control logic, it is proposed to regard interweaved affine mode as a special bi-predicted mode such that the weighted averaging may reuse the current weighted bi-prediction process with interweaved affine weighting table as input.

Moreover, it is proposed to further simplify the weighting process: Always using 2:2 weighting for all pixels except for the shaded region (as shown in Fig. 1) which keeps using 4:0 weighting as only one prediction is available for such a region.

Figure. Interweaved prediction is not applied on the shaded regions.

- Test 2.5: Phase-variant affine subblock motion compensation (JVET-N0510, MediaTek)

In this CE test, the phase-variant affine subblock motion compensation (MC) is tested. The conventional two stage horizontal-vertical interpolation is applied. However, unlike the phase-invariant block-based MC that uses the same horizontal filter for all sample rows and the same vertical filter for all sample columns, different phases of filters can be applied to different sample rows and different sample columns in the affine subblock. The used interpolation filter sets in the proposed method shall be the same as those in VTM5.0, the 6-tap filter for affine MC. If the delta MVs of control point MVs are too small, the fallback mode, the original phase-invariant affine MC in VTM, is applied. The number of filtering operations for each affine subblock in the proposed method is also shall be the same as that in VTM5.0.

Four tests are conducted. In the first test, up to 11 rows and 4 columns of MVx/MVy are derived. In the second test, only part of rows and 4 columns of MVx/MVy are derived. In the third test, the fallback mode is disabled. In the fourth test, the input reference samples of a 4x4 block is limited to be a squared block, 9x9 for 6-tap affine motion compensation filter.

In test 2.5e, the motion compensation part uses the phase-variant affine subblock motion compensation. After the MC, the PROF is applied. The nearest integer reference samples are used in gradient calculation. The delta MV of each samples are calculated. The delta MV is the sample-based affine MV minus the MV derived by the phase-variant MC (same MVx for a row and same MVy for a column).

CE4-3: Combination of LIC and affine MC

Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 3.1a | Combination of LIC (as CE1-5.1b in JVET-N0021 and with 16x16 grid constraint) and affine MC | JVET-N0343 JVET-N0205 JVET-N0074 JVET-N0171 | V. Seregin (Qualcomm) K. Abe (Panasonic) C.-W. Hsu (MediaTek) X. Ma (Huawei) |  |
| 3.1b | Test 3.1a + encoder fast algorithm | JVET-N0343 JVET-N0205 JVET-N0074 JVET-N0171 | V. Seregin (Qualcomm) K. Abe (Panasonic) C.-W. Hsu (MediaTek) X. Ma (Huawei) |  |
| 3.2a | Test 3.1a + min/max simplification | JVET-N0716 | A. Filippov (Huawei) |  |
| 3.2b | Test 3.1b + min/max simplification | JVET-N0716 | A. Filippov (Huawei) |  |

Detailed description of tests

- Test 3.1 Combination of LIC and affine MC (JVET-N0343/N0205/N0074N0171/, Qualcomm, Panasonic, MediaTek, Huawei)

As shown in Figure, the reference samples in the top template are fetched by each sub-block MVs in top row and the reference samples in the left template are fetched by each sub-block MVs in left column. LIC is applied using the composite template.

Figure. Reference samples in proposed method

Proposed method is applied on the 16x16 level, which means determining LIC parameters on a 16x16 grid for larger CUs, and with following restrictions,

- Without using reconstructed intra coded, CIIP and IBC blocks.

- Disabling LIC over CTU boundaries is not necessary

- Disable for CIIP and IBC blocks

- Disable LIC for the blocks with less than 64 luma samples.

- No temporal inheritance of LIC flag

- No pruning based on LIC flag in merging candidate list generation

- Disable LIC for all subblock modes, including triangle mode and BDOF

- It is not applied to 128xN and Nx128 blocks

- It is not applied to bi-prediction

- Test 3.2 Tests 3.1 + min/max simplification (JVET-N0716, Huawei, HHI)

The LIC parameters (a;b) of (Equation 1) are derived through LMS method (Equation 2) with the reference and reconstructed samples of the L-shape around the current block. To avoid integer divisions, the number of samples should be a power of 2 so that the divisions are carried out with right shifting. The size of the L-shaped templates depends on bit-depth and cannot exceed sizeL = 2 x min(cuWidth, cuHeight).

predcorr(s) = a * pred(s) + b

Equation 1: the LIC parameters (a;b) allow correcting the prediction.

a=refs×curr-ref(s)×cur(r)Ncurr²-ref(s)×ref(s)N

b=cur(r)N-a×ref(s)N

Equation 2: the LIC parameters (a;b) are derived with LMS method.

In the perspective of LIC, contribution JVET-N0716 proposes to apply MinMax method using in VTM-4.0 to deriving LIC parameters a and b. Additional improvements proposed in JVET-N0716 on top of the MinMax methods relate to the following aspects:

- No template downsampling: since the MinMax method does not require that the number of processed template samples would be a power of 2, it can be skipped for both current and reference blocks.

- Template filtering: filtering can be applied to template to remove outliers using smoothing FIR filtering.

- Mean values: mean values ymean and xmean can be calculated by performing averaging separately for each side and both results are averaged with each other; then parameter b is calculated as follows:

b=ymean-a*xmean

- Slope value: not only for the smallest luma value A and largest luma value B but also for the second-smallest luma value A' and the second-largest luma value B' are searched as shown in Fig. 1 in JVET-N0716. Then, the trend line can be fitted between the average of the data points at A and A' (by using yA = (A + A') >>  1) and the average of the data points at B and B' (by using yB = (B + B') >> 1), which generally leads to a better data-fit:

a=yB-yAxB-xA

Test condition and evaluation criteria

Each testing item is required to be compared against VTM-5.0, evaluated using BD rates, encoding time, decoding time according to Common Test Conditions (CTC) in JVET-N1010. Simulation with RA and LDB configurations are mandatory.

Crosscheck report

Crosscheckers shall fill in the table below, and send it to CE coordinators 3 days before T4.

| Test# | Crosschecker | SW match CE description (Y/N) | Text match SW (Y/N) | Simulation results (Y/N) | EncT/DecT match (Y/N) |
| --- | --- | --- | --- | --- | --- |
| x.x |  |  |  |  |  |

Explanation of each item in the table

- SW match CE description: Y SW implementation match CE description, N mismatch.

- Text match SW: Y text match SW, N mismatch.

- Simulation results: Y substantial results (all but class A for RA, all but class B for LDB) obtained and match results from tester, N mismatch or no substantial results.

- EncT/DecT match: Y encoder and decoder run time match those from tester, N mismatch.

SW integration guideline

All tests are required to be implemented on top of VTM-5.0.

Timeline

| T1 | Apr. 17, 2019 | Finalize CE description (3 weeks after the meeting) |
| --- | --- | --- |
| T2 | May 13, 2019 | Deliver software packages and begin cross-checking (2 weeks after VTM-5.0 release) |
| T3 | Jun. 5, 2019 | Freeze software revision and provide full results for proponents Begin final crosschecking for cross-checkers |
| T4 | Jun. 18, 2019 | Upload contribution documents with specification text and complete test results (regular document deadline – 1 week) |

Participant list

| Company/ Organization | Participant Name | Email Address |
| --- | --- | --- |
| Technicolor | Antoine Robert Philippe Bordes Tangi Poirier Fabrice Le Léannec Franck Galpin Ya Chen | antoine.robert@technicolor.com philippe.bordes@technicolor.com tangi.poirier@technicolor.com fabrice.leleannec@technicolor.com franck.galpin@technicolor.com Ya.CHEN@technicolor.com |
| HHI | Martin Winken Robert Skupin Jens Brandenburg Anastasia Henkel | martin.winken@hhi.fraunhofer.de robert.skupin@hhi.fraunhofer.de jens.brandenburg@hhi.fraunhofer.de anastasia.henkel@hhi-extern.fraunhofer.de |
| Huawei | Huanbang Chen Haitao Yang Jianle Chen Sychev Maxim Solovyev Timofey Alexey Filippov Vasily Rufitskiy Semih Esenlik Alexander Karabutov | chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com Sychev.Maxim@huawei.com solovyev.timofey@huawei.com alexey.filippov@huawei.com vasily.rufitskiy@huawei.com semih.esenlik@huawei.com karabutov.alexander@huawei.com |
| LGE | Seung Hwan Kim Jaehyun Lim Naeri Park Hyeongmun Jang Jie Zhao Seethal Paluri | seunghwan3.kim@lge.com Jaehyun.lim@lge.com naeri.park@lge.com hm.jang@lge.com jie.zhao@lge.com seethal.paluri@lge.com |
| Tencent | Xiaozhong Xu Jing Ye Guichun Li Shan Liu Meng Xu Xiang Li | xiaozhongxu@tencent.com jingye@tencent.com guichunli@tenceNnt.com shanl@tencent.com mengxxu@tencent.com xlxiangli@tencent.com |
| USTC | Yue Li Li Li | lytt@mail.ustc.edu.cn lili90th@gmail.com |
| HiSilicon | Na Zhang Xu Chen Jianhua Zhang | zhangna25@huawei.com anderson.chen@hisilicon.com zhengjianhua@hisilicon.com |
| BBC | Saverio Blasi | saverio.blasi@bbc.co.uk |
| ETRI | Jungwon Kang Sung-Chang Lim Hahyun Lee | jungwon@etri.re.kr sclim@etri.re.kr hanilee@etri.re.kr |
| Panasonic | Kiyofumi Abe Tadamasa Toma Che-Wei Kuo Chong Soon Lim | abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com chewei.kuo@sg.panasonic.com chongsoon.lim@sg.panasonic.com |
| Fujitsu | Jianqing Zhu Kimihiko Kazui | zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Sony | Kenji Kondo | kenji.kondo@sony.com |
| Qualcomm | Wei-Jung Chien Vadim Seregin Chun-Chi Chen Yan Zhang Yu Han Han Huang Chao-Hsiung Hung Luong Pham Van Dmytro Rusanovskyy | wchien@qti.qualcomm.com vseregin@qti.qualcomm.com chunchic@qti.qualcomm.com yzh@qti.qualcomm.com yuhan@qti.qualcomm.com hanhuang@qti.qualcomm.com chaohsiu@qti.qualcomm.com lphamvan@qti.qualcomm.com dmytror@qti.qualcomm.com |
| Alibaba | Yu-Chen Sun | yuchen.s@alibaba-inc.com |
| Hikvision | Fangdong Chen | chenfangdong@hikvision.com |
| Sharp | Tomonori Hashimoto Takeshi Chujoh Tomohiro Ikai Kiran Misra Eiichi Sasaki Tianyang Zhou | tomonori.hashimoto@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp misrak@sharplabs.com eiichi.sasaki@sharp.co.jp zhou.tianyang@sharp.co.jp |
| Ericsson | Ruoyang Yu Julien Le Tanou | ruoyang.yu@ericsson.com julien.le.tanou@ericsson.com |
| Samsung Electronics | Min Woo Park Seungsoo Jeong Anish Tamse Kiho Choi Jie Chen Minsoo Park Yinji Piao | m.w.park@samsung.com ss00.jeong@samsung.com tamse.anish@samsung.com kiho14.choi@samsung.com jie1222.chen@samsung.com mss.park@samsung.com yj1003.piao@samsung.com |
| Nokia | Jani Lainema Alireza Aminlou Ramin Ghaznavi-Youvalari | jani.lainema@nokia.com alireza.aminlou@nokia.com ramin.ghaznavi-youvalari@nokia.com |
| InterDigital | Yuwen He Daniel Luo Hua Yang Wei Chen Saurav Bandyopadhyay | Yuwen.He@interdigital.com Daniel.Luo@InterDigital.com hua.yang@interdigtial.com wei.chen@interdigital.com Saurav.Bandyopadhyay@InterDigital.com |
| Arris | Krit Panusopone Yue Yu David Baylon Seungwook Hong Limin Wang | Krit.panusopone@arris.com Yue.yu@arris.com David.baylone@arris.com Seungwook.hong@arris.com Limin.wang@arris.com |
| MediaTek | Yu-Wen Huang Chih-Wei Hsu Ching-Yeh Chen Tzu-Der Chuang Shih-Ta Hsiang Yuling Hsiao Jenny Lai Yu-Chi Su Zhi-Yi Lin Chun-Chia Chen Yucheng Lin | yuwen.huang@mediatek.com cw.hsu@mediatek.com chingyeh.chen@mediatek.com Peter.Chuang@mediatek.com shih-ta.hsiang@mediatek.com Yuling.Hsiao@mediatek.com Jenny.Lai@mediatek.com Yu-Chi.Su@mediatek.com Zhi-Yi.Lin@mediatek.com chunchia.chen@mediatek.com Yucheng.Lin@mediatek.com |
| NHK | Shunsuke Iwamura | iwamura.s-gc@nhk.or.jp |
| Netflix | Andrey Norkin | anorkin@netflix.com |
| Orange | Gordon Clare | gordon.clare@orange.com |
| Broadcom | Minhua Zhou Pensong Chen Wade Wan | minhua.zhou@broadcom.com peisong.chen@broadcom.com wade.wan@broadcom.com |
| Canon | Jonathan Taquet | Jonathan.TAQUET@crf.canon.fr |
| PKU | Zhao Wang Suhong Wang Xuewei Meng | zhaowang@pku.edu.cn suhong.wang@pku.edu.cn xwmeng@pku.edu.cn |
| Chips&Media | Wook Je Jeong | nick.jeong@chipsnmedia.com |
| Aachen University | Johannes Sauer | sauer@ient.rwth-aachen.de |
| HIT | Yang Wang Xiaopeng Fan | wangyang.cs@hit.edu.cn fxp@hit.edu.cn |
| WILUS | Greg Ko | greg.ko@wilusgroup.com |
| ITRI | Ching-Chieh Lin | JackLin@itri.com |
| Bytedance | Kai Zhang Li Zhang | zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Kwai | Yi-Wen Chen Xianglin Wang | yiwenchen@kwai.com xianglinwang@kwai.com |
| HNU | Haechul Choi Heeji Han Daehyeok Gwon jaeryun Choe | choihc@hanbat.ac.kr sarhakor@naver.com skyeye0530@naver.com tjswo9395@naver.com |
| Dolby | Peng Yin | pyin@dolby.com |
| Intel | Grace Yu | grace.yu@intel.com |
| DJI | Xiaozhen Zheng | xiaozhen.zheng@dji.com |
| Alibaba | Yan Ye Jie Chen | yan.ye@alibaba-inc.com jiechen.cj@alibaba-inc.com |
| Digital Insights | Yongjo Ahn | yjahn@digitalinsights.co.kr |
| Foxconn | Yu-Ciao Yang Po-Han Lin Tsung-Hua Li | yuciaoyang@fginnov.com PohanLin@fginnov.com thli@fginnov.com |