| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N1025 |
| --- | --- |

| Title: | Description of Core Experiment 5 (CE5): Loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JVET |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen Andrey Norkin | Tel: Email: | chingyeh.chen@mediatek.com anorkin@netflix.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The goal of this Core Experiment (CE) is to improve and/or enhance loop filtering techniques in the test model (VTM). Techniques covered by this CE are related to improvements to deblocking filter and adaptive loop filter (ALF).

The deblocking filter techniques are evaluated according to subjective picture quality as well as BD-rate gains. These are organized in CE5-1, CE5-2, and CE5-3, which address aspects of the long deblocking filter and sub-pu improvements, filtering at 4x4 block boundaries, and sub-sample MV threshold for deblocking decisions. Participants in this activity are Ericsson, Huawei, MediaTek and LG.

In CE5-4 ALF subtest, signaling alternative luma filters sets and chroma filters at tile group level (i.e. they could be put in APS) is tested to evaluate the coding efficiency improvement. More than one luma filter set can be signaled at once per tile group with some syntax constraints between successive filter sets. Filters from different filter sets defined at tile group level can be mixed in a CTU specific set, with additional signaling in CTU data. Participant in this activity is Canon.

The software basis for this CE is VTM-5.0 [2]. For the test sequences, configurations and test conditions, in JVET-N1010 [1] for SDR sequences are used. For subjective viewing, additional results with sequences encoded with higher QPs may be required.

# Participants

| Nr. | Name | Company | Email |
| --- | --- | --- | --- |
| 1 | Patrice Onno | Canon | patrice.onno@crf.canon.fr |
| 2 | Kenneth Andersson | Ericsson | kenneth.r.andersson@ericsson.com |
| 3 | Anand Meher Kotra | Huawei | anand.meher.kotra@huawei.com |
| 4 | Hyeongmun Jang | LG | hm.jang@lge.com |
| 5 | Chia-ming Tsai | MediaTek | Chia-ming.Tsai@mediatek.com |

# Test conditions and evaluation criteria

The proposals should be encoded according to two test conditions, which correspond to two anchors.

Anchor-1 is VTM-5.0 according to the CTC [1].

Anchor-2 (for deblocking sub-tests CE5-1, CE5-2, CE5-3) is VTM-5.0 with ALF switched off (other conditions are the same as in Anchor 2).

The proposals in the CE shall be implemented based on VTM-5.0. The results for two test conditions shall be provided for CE5-1, CE5-2, and CE5-3: Test Conditions 1 use configuration of Anchor 1, Test Conditions 2 use configuration of Anchor 2. The results for LDP configuration are mandatory for this CE. For CE5-4, only the results for Anchor-1 shall be provided

Proposals will be compared with respect to subjective, objective quality, and complexity.

# Tool description

## Introduction

The main goal with this Core Experiment is to enhance the subjective quality of the Test Model and to ensure that CUs can be filtered in parallel.

Following aspects will be studied:

- Sub-pu deblocking and long filter modification

- Deblocking for 4x4 block boundaries

- Sub-sample MV difference threshold for deblocking decision

- ALF with alternative luma filters sets and chroma filters

Different technologies will be compared at a subjective viewing at the next meeting.

# CE5-1: Sub-PU deblocking and longer tap filter

Proposal in this sub-CE investigate changes to sub-PU deblocking and longer deblocking filter of VVC.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE5-1.1 | Anand Meher Kotra Anand.meher.kotra@huawei.com JVET-N0182 | Chia-Ming Tsai chia-ming.tsai@mediatek.com |

## Description of technologies in CE5-1

#### CE5-1.1 Modification of sub-pu deblocking and longer tap filter

This test proposes two modifications to the current deblocking filter design in VTM.

The first modification disables application of longer tap filter for implicit TU boundaries.

The second modification disables the filtering for first and last sub-pu edges for a block which uses sub-pu tools. This reportedly has an advantage that the longer tap filter can directly be applied to the coding unit edge without any further checks.

# CE5-2: Deblocking at 4x4 block boundaries

The proposals in this CE study application of deblocking filtering to the 4x4 boundaries. Common feature of proposals studied in this CE is filtering on the block boundaries coinciding with the 4x4 sample grid and making sure it does not introduce dependency of deblocking for an edge on a parallel edge preceding in a filtering order. Test CE5-2.2 investigates whether filtering any edges adjacent to the blocks of size 4 can be avoided.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE5-2.1 | Kenneth Andersson kenneth.r.andersson@ericsson.com Anand Meher Kotra Anand.meher.kotra@huawei.com Chia-Ming Tsai chia-ming.tsai@mediatek.com JVET-N0098 | Hyeongmun Jang hm.jang@lge.com |
| CE5-2.2 (CE5-2.1 + chroma) | Kenneth Andersson kenneth.r.andersson@ericsson.com Anand Meher Kotra Anand.meher.kotra@huawei.com Chia-Ming Tsai chia-ming.tsai@mediatek.com JVET-N0098 | Hyeongmun Jang hm.jang@lge.com |
| CE5-2.3 (without chroma) | Hyeongmun Jang hm.jang@lge.com JVET-N0463 | Kenneth Andersson kenneth.r.andersson@ericsson.com |
| CE5-2.4 (with chroma) | Hyeongmun Jang hm.jang@lge.com JVET-N0463 | Kenneth Andersson kenneth.r.andersson@ericsson.com |
| CE5-2.5 Chroma on 4x4 chroma grid | Hyeongmun Jang hm.jang@lge.com |  |

## Description of technologies in CE5-2

#### CE5-2.1 Deblocking for 4xN, Nx4, 8xN and Nx8 blocks JVET-N0098

In current VVC software large rectangular blocks of size Nx8 and Nx4 are not deblocked where N can be up to 64. This test applies deblocking on 4x4 grid and the number of samples to read and modify is limited to allow for parallel friendly processing. If at least one side of the block boundary has a length orthogonal to the block boundary equal to 4 only use the weak filter, and only one sample is filtered on each side of the block boundary in this case.

#### CE5-2.2 Deblocking for 4xN, Nx4, 8xN and Nx8 blocks JVET-N0098

This test would check the CE5-2.1 + chroma deblocking performed on the 4x4 chroma grid

#### CE5-2.3 Parallel deblocking filter JVET-N0463

Deblocking filter can be processed in parallel because it is not applied to 4x4 TU and PU boundaries. Within this CE, the performance of the parallel deblocking method described in JVET-D0044 is to be studied in terms of subjective quality and computational complexity. Chroma deblocking is performed in the same way as in the current deblocking.

#### CE5-2.4 Parallel deblocking filter JVET-N0463

This test would study the CE5-2.3 + chroma deblocking changes as in JVET-N0463.

#### CE5-2.5 Deblocking chroma on 4x4 chroma grid

Chroma deblocking is performed on the 4x4 chroma grid (current VTM chroma deblocking is done on the 8x8 chroma grid).

# CE5-3: Sub-sample MV threshold for deblocking decisions

Proposal in this sub-CE investigate effect of using a sub-sample MV threshold for deblocking decisions .

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE5-3.1 | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-N0359 | Chia-Ming Tsai chia-ming.tsai@mediatek.com |

## Description of technologies in CE5-3

#### CE5-3.1 MV difference threshold in deblocking decisions JVET-N0359

It has been shown in JVET-N0359 that block artifacts can occur between two blocks for a magnitude difference in horizontal or vertical motion vector component smaller than one luma sample.

The test will investigate the effect of different sub-sample thresholds in deblocking decisions.

# CE5-4: Adaptive loop filter

#### CE5-4 Alternative luma filter sets and alternative chroma filters for ALF JVET-N0243

In JVET-N0243, alternative luma filters sets were signaled at tile group level (i.e. they could be put in APS), so that more than one luma filter set can be signaled at once per tile group with some syntax reduction mechanism that enable to further merge filter indexes between successive filter sets. Filters from different filter sets defined at tile group level can be mixed in a CTU specific set, with additional signaling in CTU data.

In JVET-N0243, alternative chroma filters were also signaled at tile group level, so that more than one chroma filter can be signaled at once per tile group. One alternative chroma filter can be selected per chroma component and per CTU with additional signaling in CTU data.

In JVET-N0243, there were no pre-trained fixed filter sets, nor temporal filter sets, which are introduced in VTM5.0 with adoption of JVET-N0415. The following tests would allow clarifying gains of each sub-tool in JVET-N0243 and their potential overlap with adopted JVET-N0415.

Five tests will be performed.

- CE5-4.1 will provide results of mixing different luma filters at CTU level coming from the different filters signaled in the APS(s).

- CE5-4.2 will provide results when enabling the signaling of additional luma filter sets in APS.

- CE5-4.3 will provide combined results of CE5-4.1 and CE5-4.2.

- CE5-4.4 will provide results when enabling alternative chroma filters in VTM5.0 at CTU level.

- CE5-4.5 will provide results of the combination of tests CE5-4.1, CE5-4.2 and CE5-4.4.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE5-4.1 | VTM5.0 + usage of mixed luma filters at CTU level | Canon |  |
| CE5-4.2 | VTM5.0 + additional luma filter sets signalled in APS | Canon |  |
| CE5-4.3 | Combination of Test 5-4.1 and Test 5-4.2 | Canon |  |
| CE5-4.4 | VTM5.0 + alternative chroma filters + CTU chroma filter selection | Canon |  |
| CE5-4.5 | Combination of test 5-4.3 and 5-4.4 | Canon |  |

# Expected Gains

- Subjective picture quality improvement while preferably keeping rate-distortion performance.

- Coding efficiency improvement.

# Evaluation Criteria

1. Measure impact on Bitrate/PSNR. The BD-rate is calculated for CTC and for CTC with ALF switched off relative to Anchor-1 and Anchor-2 respectively. The results for LDP configurations are mandatory for both CTC and CTC with ALF switched off. For CE5-4, only the BD-rate is calculated for CTC.

2. Visual impact on output pictures of representative sequences. The subjective test is expected to be held to determine the visual impact. The subjective test will use 10 second versions of the sequences. The proponents shall provide the bitstreams and decoders for the sequences that are chosen for evaluation in the subjective test.

3. Complexity of the proposals evaluated in the CE shall be compared. In particular, the following aspects of the proposals shall be summarized for the deblocking filters (CE5-1, CE5-2, CE5-3):

- decoding/encoding time vs the anchor

- number of samples from the block boundary that can be modified by deblocking (may be per block size)

- number of samples from the block boundary that are needed for deblocking decisions (may be per block size)

- number of filtering operations per sample including decisions and filtering (such as additions, multiplications, comparisons)

- number of line buffers

- parallelization capabilities, such as size of a unit of a picture that can be processed in parallel to other units

- whether proposals increase the worst case complexity and if so, by how much

- data should be provided for both luma and/or chroma if they are modified

# Subjective test(s)

Expert viewing at the Gothenburg meeting will be performed for proposals in CE5-1, CE5-2, CE5-3.

The following test sequences and conditions will be used in the subjective tests. The sequence length is 10 seconds. This means that the versions of some sequences encoded for the subjective test are longer than those used in the CTC [1].

The subjective test sequences shall be provided by May 15, 2019 (uploaded to the JVET ftp folder /ce/JVET-N1031_Deblocking), coding configuration with ALF ON and ALF OFF shall be used. The sequences listed in this section shall also be available at the Geneva meeting from the proponents for their corresponding sub-CEs (CE5-1, CE5-2, CE5-3).

The anchors for the subjective viewing will use the ALF ON and ALF OFF configurations. Anchors in CE5-2 will be provided by A.M. Kotra, and anchors in CE5-1 and CE5-3 will be provided by [volunteer].

The following sequences and QP will be used for subjective evaluation in CE5-1 and CE5-3 category:

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | FoodMarket | 3840x2160p | 60fps | 600 | RA | 39, 34 |
| 02 | Campfire | 3840x2160p | 30fps | 300 | RA | 39, 34 |
| 03 | Kimono | 1920x1080p | 24fps | 240 | LDB | 34, 39 |
| 04 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 34, 39 |
| 05 | Red Kayak* | 1920x1080p | 30fps | 300 | RA | 34, 39 |

* First 300 frames of the RedKayak sequence shall be used.

The following sequences and QP will be used for subjective evaluation in CE5-2 category:

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Redkayak | 1920x1080p | 30fps | 300 | RA | 34, 39 |
| 02 | Cactus | 1920x1080p | 50fps | 500 | LDB | 34, 39 |
| 03 | FourPeople | 1280x720p | 60fps | 600 | LDP | 34, 39 |
| 04 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 34, 39 |

* First 300 frames of the RedKayak sequence shall be used.

## Subjective test description:

The test material at resolution lower than 4K will be expanded to full screen by the player and evaluated together with the 4K test material.

# Test conditions and data to be provided

The test conditions are according to CTC on VTM with ALF turned on and VTM with ALF turned off for CE5-1, CE5-2, CE5-3. In addition, the proponents of these sub-tests need to provide results for additional QP and additional test sequences chosen by the CE (see above). The sequences length for the subjective viewing is 10 seconds. The complexity of the proposals according to Section 7 should be reported by the proponents.

The test conditions are according the CTC on VTM for CE5-4.

Studying effect of the deblocking methods on 4:4:4 chroma format is planned.

# Time-line and Responsibilities

T1: 2019-April-17:					Final CE description uploaded

T2: VTM5.0 release + 2 weeks		Cross-checking begins, proponents provide software

T3: 2019-June-5: 						Final version of CE software, full test results must be provided at this time. Sequences for the subjective test are uploaded to the ftp.

# References

- “JVET common test conditions and software reference configurations”, JVET-N1010.

- “Algorithm description for Versatile Video Coding and Test Model 5 (VTM5)”, JVET-N1002.