| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N1027-v3 |
| --- | --- |

| Title: | Description of Core Experiment 7 (CE 7): Quantization and coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Heiko Schwarz Muhammed Coban | Email: | heiko.schwarz@hhi.fraunhofer.de mcoban@qti.qualcomm.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

The purpose of this core experiment is to explore the coding efficiency and complexity impact of proposed algorithms for quantization, transform coefficient/residual coding. This core experiment evaluates the impact of:

- Context modeling/selection/coding order and context-coded bin limits for syntax elements related to transform coefficients/residuals

The objectives of this CE include:

- Investigation of the impact of context selection and signaling schemes for transform coefficient/residual levels on the coding efficiency and complexity.

- Investigation of the impact of context-coded bin limits on coding efficiency and complexity.

# Participants List

| Affiliation | Person, Email address |
| --- | --- |
| Qualcomm | Muhammed Coban <mcoban@qti.qualcomm.com> Dmytro Rusanovskyy <dmytror@qti.qualcomm.com> Jie Dong <jiedong@qti.qualcomm.com> Ted Hsieh <thsieh@qti.qualcomm.com> Geert Van der Auwera <geertv@qti.qualcomm.com> Marta Karczewicz <martak@qti.qualcomm.com> |
| Fraunhofer HHI | Heiko Schwarz <heiko.schwarz@hhi.fraunhofer.de> Tung Nguyen <tung.nguyen@hhi.fraunhofer.de> Christian Helmrich <christian.helmrich@hhi.fraunhofer.de> Benjamin Bross <benjamin.bross@hhi.fraunhofer.de> |
| Samsung | Yinji Piao <yj1003.piao@samsung.com> Kiho Choi <kiho14.choi@samsung.com> |
| Huawei | Alexey Filippov <alexey.filippov@huawei.com> Jianle Chen <Jianle.chen@huawei.com> Yang Haitao <haitao.yang@huawei.com> Yin Zhao <yin.zhao@huawei.com> Sergey Ikonin <Sergey.Ikonin@huawei.com> Biao Wang <biao.wang@huawei.com> Vasily Rufitskiy <vasily.rufitskiy@huawei.com> Alexander Karabutov <karabutov.alexander@huawei.com> Sergey Ikonin <Sergey.Ikonin@huawei.com> |
| Orange | Pierrick Philippe <pierrick.philippe@orange.com> Felix Henry <felix.henry@orange.com> Gordon Clare <gordon.clare@orange.com> |
| MediaTek | Mohammed Sarwer <Mohammed.Sarwer@mediatek.com> Olena Chubach <Olena.Chubach@mediatek.com> Yu-Wen Huang <yuwen.huang@mediatek.com> Chih-Wei Hsu <cw.hsu@mediatek.com> Ching-Yeh Chen <chingyeh.chen@mediatek.com> T.-D. Chuang <peter.chuang@mediatek.com> Shih-Ta Hsiang <shih-ta.hsiang@mediatek.com> |
| Sharp | Eiichi Sasaki <eiichi.sasaki@sharp.co.jp> Takeshi Chujoh <chujoh.takeshi@sharp.co.jp> Tomohiro Ikai <ikai.tomohiro@sharp.co.jp> Kiran Misra <misrak@sharplabs.com> Andrew Segall <asegall@sharplabs.com> Frank Bossen <frank.bossen@me.com> |
| Apple | Alexis Tourapis <atourapis@apple.com> Jae Hoon Kim <i.jaehoon.kim@icloud.com> |
| KDDI | Kei Kawamura <ki-kawamura@kddi.com> Kyohei Unno <ky-unno@kddi-research.jp> Yoshitaka Kidani <yo-kidani@kddi.com> |
| Kwai | Yi-Wen Chen <yiwenchen@kwai.com> Xianglin Wang <xianglinwang@kwai.com> Xiaoyu Xiu <xiaoyuxiu@kwai.com> Tsung-Chuan Ma <tsung-chuanma@kwai.com> Jhuhong Jheng <jhuhong-jheng@kwai.com> |
| Bytedance | Li Zhang <lizhang.idm@bytedance.com> Hongbin Liu <liuhongbin.01@bytedance.com> |
| Nokia | Jani Lainema <jani.lainema@nokia.com> |
| Broadcom | Minhua Zhou <minhua.zhou@broadcom.com> Wade Wan <wade.wan@broadcom.com> Peisong Chen <peisong.chen@broadcom.com> |
| MediaKind | Julien Le Tanou <julien.letanou@mediakind.com> |
| Sony | Takeshi Tsukuba <Takeshi.Tsukuba@sony.com> Karl Sharman <karl.sharman@eu.sony.com> |
| InterDigital | Yuwen He <Yuwen.He@InterDigital.com> Rahul Vanam <Rahul.vanam@interdigital.com> |
| Canon | Chris Rosewarne <chris.rosewarne@cisra.canon.com.au> Jonathan Gan <jonathan.gan@cisra.canon.com.au> |
| BBC | Andre Seixas Dias <Andre.SeixasDias@bbc.co.uk> Saverio Blasi <saverio.blasi@bbc.co.uk> |
| Technicolor | Fabien Racape <fabien.racape@technicolor.com> Ya Chen <Ya.chen@technicolor.com> Edouard Francois <Edouard.francois@technicolor.com> Philippe de Lagrange <Philippe.delagrange@technicolor.com> Leleannec Fabrice <Fabrice.Leleannec@technicolor.com> |
| HiSilicon | Jianhua Zheng <zhengjianhua@hisilicon.com> Quanhe Yu <yuquanhe@hisilicon.com> |
| ETRI | Jungwon Kang <jungwon@etri.re.kr> Sung-Chang Lim <sclim@etri.re.kr> |
| Panasonic | Kiyofumi Abe <abe.kiyo@jp.panasonic.com> Yusuke Kato <kato.yusuke003@jp.panasonic.com> |
| LG | Jung-Ah Choi <jungah.choi@lge.com> Jin Heo <jin78.heo@lge.com> Sunmi Yoo <sunmi.yoo@lge.com> Jangwon Choi <jangwon84.choi@lge.com> Ling Li <l.li@lge.com> Seung Hwan Kim <seunghwan3.kim@lge.com> Mehdi Salehifar <mehdi.salehifar@lge.com> Moonmo Koo <moonmo.koo@lge.com> Jaehyun Lim <jaehyun.lim@lge.com> Jane Zhao <jie.zhao@lge.com> |
| Dolby | Sean McCarthy <smcca@dolby.com> Peng Yin <pyin@dolby.com> |
| Comcast | Dan Grois <grois@ieee.org> |
| Intel | Sabine Roessel <sabine.roessel@intel.com> |
| Tsinghua University | Benben Niu <nbb16@mails.tsinghua.edu.cn> |
| Tencent | Xin Zhao <xinzzhao@tencent.com> Xiang Li <xlxiangli@tencent.com> Cheung Auyeung <cauyeung@tencent.com> |
| WILUS | Dongcheol Kim <dongcheol.kim@wilusgroup.com> |
| Foxconn | Yao-Jen Chang <yaojen.chang.foxconn@gmail.com> |
| NHK | Shunsuke Iwamura <iwamura.s-gc@nhk.or.jp > |
| Tongji University | Tao Lin <lintao@tongji.edu.cn> |
| Alibaba | Jie Chen <jiechen.cj@alibaba-inc.com> Ru-Ling Liao <ruling.lrl@alibaba-inc.com> |

# Tools under Test

The following tools will be considered in this CE. Details are described in sec. 6.

|  | Proponent | Document | Tool description |
| --- | --- | --- | --- |
| 1 | Mediatek | JVET-N0091 | TB-level constraints on context-coded bins for coefficient coding |
| 2 | Qualcomm | JVET-N0106 | TU-level limits on context-coded bins for coefficient coding |
| 3 | HHI | JVET-N0282 | Joint chroma residual coding with multiple modes |
| 4 | Qualcomm | JVET-N0347 | Joint coding of chroma residuals |
| 5 | Mediatek | JVET-N0094 | Context modelling of transform skip mode |
| 6 | HHI | JVET-N0357 | Context modelling of sign for TS residual coding |
| 7 | Tencent | JVET-N0366 | Modified limitation on context coded bins for TS residual coding |
| 8 | Qualcomm | JVET-N0455 | Sign context modelling and level mapping for TS residual coding |
| 9 | Kwai | JVET-N0326 | Simplification of cbf coding |

# Test Description

## Software, reference configurations, and test conditions

All tools have to be integrated into the VTM-5 software (outcome of the Geneva meeting).  Results have to be reported for the VTM-5 software and the common test conditions (standard configuration files).

All tools are tested against the VTM-5 anchor (same set of tools, except the tool to be tested).

Results have to be provided for two ranges of QP values:

- Standard QPs (37, 32, 27, 22): AI, RA, and LD configurations as specified in CTC (all frames);

- Low QPs (17, 12, 7, 2): AI, RA, and LD configurations with 100 frames only (i.e., by using the encoder option “--FramesToBeEncoded=100”).

For category 2 tests (joint chroma coding), additional HDR results according to JVET-N1011 have to be provided. For HDR, only tests for the standard QP set (22, 27, 32, 37) are required (visual testing is not required).

For category 3 tests (transform skip residual codind), additional results for screen coding content sequences (class TGM with YUV 4:2:0 format) [10] have to be provided using class F CTC configuration. For category 3 tests, proponents are also encouraged to provide additional IBC off tests for screen content sequences (class F and TGM).

## Category 1 test bin to bit ratio measurements

Bin to bit ratio measurements for context coded as well as bypass coded bins of transform coefficient coding have to be provided for Category 1 tests using VTM-5.0 decoder SW with

#define RExt__DECODER_DEBUG_BIT_STATISTICS                1

setting. The following stats have to be collected (JVET-N0049).

- The peak context coded bin rate, bypass bin rate and bit-rate of a coded sequence

- The average context coded bin rate, bypass bin rate and bit-rate over a coded sequence.

- The average context coded bin rate, bypass bin rate and bit-rate consumed by the transform coefficients coding over a coded sequence.

JVET-N0049 SW’s maximum context coded bins for transform coefficient syntax elements as well as overall peak context coded bins rate feature will be integrated into VTM-5 decoder SW. The stats for the following syntax elements for transform coding have to be used for reporting transform coefficient coding stats.

- Context-coded bins: sig_coeff_flag, abs_level_gt1_flag, par_level_flag, abs_level_gt3_flag

- Bypass-coded bins: abs_remainder, dec_abs_level

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-rate). Proponents are encouraged to provide supplementary results. For Category 2 tests with HDR, proponents have to provide results for the HDR metrics specified in JVET-N1011.

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the reference configurations);

- Decoding time (compared to the reference configuration).

In addition, proponents are encouraged to provide the following (but not limited to) information for complexity and entropy throughput analysis:

- Number of context models

- Number of context-coded and bypass bins in average case and the worst case

- Table size of scanning paths

- Analysis of the frequency of context switches

- Analysis of the dependency between two consecutive bins

- Analysis of entropy parsing dependency (whether bits to bins conversion depends on complex reconstruction and/or pixel processing process)

- Analysis of entropy de-binarization dependency (whether bins to symbols conversion depends on complex reconstruction and/or pixel processing process)

## Test plan

The core experiment is split into the following tests:

Category 1: Transform coefficient coding

|  | Tester |  |  | Tool |  | Cross checker |
| --- | --- | --- | --- | --- | --- | --- |
| CE7-1.1 | T.-D. Chuang, M. Coban (Mediatek / Qualcomm) |  |  | TB-level constraints on context-coded bins for coefficient coding based on the non-zero out region size of TB. |  |  |
| CE7-1.2 | T.-D. Chuang, M. Coban (Mediatek / Qualcomm) | TB-level constraints on context-coded bins for coefficient coding based on the last coded subblock position (area covered in non-zero out region up to the last subblock position) of TB. |  |

Category 2: Joint chroma residual coding

| CE7-2.1 | Christian Helmrich (HHI) | Joint chroma residual coding with multiple modes: JVET-N0282 configuration 1 (three modes with single chroma residual) | Geert Van der Auwera (Qualcomm) |
| --- | --- | --- | --- |
| CE7-2.2 | Christian Helmrich (HHI / Qualcomm) | Joint chroma residual coding with multiple modes: JVET-N0282, configuration 2 (two modes with single chroma residual and one mode with Hadamard transform) | Jani Lainema (Nokia) |
| CE7-2.3 | Geert Van der Auwera (Qualcomm) | Joint coding of chroma residuals: JVET-N0347 | Jani Lainema (Nokia) |

Category 3: Transform skip residual coding

|  | Tester |  | Tool | Cross checker |
| --- | --- | --- | --- | --- |
| CE7-3.1 | M. G. Sarwer (Mediatek) |  | Context modelling of transform skip mode: VTM-5 TS residual coding with 2 neighbours |  |
| CE7-3.2 | M. G. Sarwer (Mediatek) | Context modelling of transform skip mode: VTM-5 TS residual coding with 3 neighbours |  |  |
| CE7-3.3 | Tung Nguyen (HHI) |  | Context modelling of sign for TS residual coding: Configuration as in JVET-N0357 | Muhammed Coban (Qualcomm) |
| CE7-3.4 | Tung Nguyen (HHI) | Context modelling of sign for TS residual coding: Alternative configuration using different parameters |  | Muhammed Coban (Qualcomm) |
| CE7-3.5 | Xin Zhao (Tencent) |  | Modified limitation on context coded bins for TS residual coding, parity flag signaled after gt2 flag and up to 8 contexts coded gt2 flags per coefficient | Yi-Wen Chen (Kwai) |
| CE7-3.6 | Xin Zhao (Tencent) | Modified limitation on context coded bins for TS residual coding, parity flag is bypass coded and signaled after gt2 flag, up to 8 contexts coded gt2 flags per coefficient |  | Yi-Wen Chen (Kwai) |
| CE7-3.7 | Muhammed Coban (Qualcomm) |  | Sign context for TS residual coding | Tung Nguyen (HHI) |
| CE7-3.8 | Muhammed Coban (Qualcomm) | Level mapping for TS residual coding |  | Tung Nguyen (HHI) |
| CE7-3.9 | Muhammed Coban (Qualcomm) | Bitplane coding for TS residual coding |  | Tung Nguyen (HHI) |
| CE7-3.10 | Muhammed Coban (Qualcomm) | Sign context and level mapping for TS residual coding |  | Tung Nguyen (HHI) |
| CE7-3.11 | Muhammed Coban (Qualcomm) | Sign context, level mapping, and bitplane coding for TS residual coding |  | Tung Nguyen (HHI) |

Category 4: cbf coding

| CE7-4.1 | Yi-Wen Chen (Kwai) | Two CABAC contexts could be utilized to coding the tu_cbf_luma and the context selection is derived based on trDepth | Guichun Li (Tencent) |
| --- | --- | --- | --- |

# Timeline

General timeline for core experiments (as agreed by JVET).

| T1 | 3 weeks after meeting | - Final CE description uploaded |
| --- | --- | --- |
| T2 | 2 weeks after VTM-5 release | - Proponents completed initial integration of tools (in individual branch) - Cross checkers can start initial study of tools / software (Note: Proponents can continue to modify the software until T3) |
| T3 | 3 weeks before next JVET meeting or T2 + 1 week (whichever is later) | - Proponents froze changes to software branches - Proponents tag software and announce on JVET reflector - The names of the cross checkers and the list of specific tests for each CE tool are announced on JVET reflector - Proponents provide full test results - Cross checkers start final cross checks |
| T4 | regular document deadline – 1 week | - CE contribution documents including specification text and complete test results are uploaded to the JVET document repository |

Test results and cross check results are reported to CE coordinators (for inclusion into CE report).

# Detailed Description of Test

## TB-level constraints on context-coded bins for coefficient coding (N0091/ N0106)

In these CE tests, use of transform block (TB) level constraints for maximum number of context-coded bins instead of coefficient subblock-level constraint are tested. Two tests are evaluated:

- In the first test, for a luma TB, the maximum allowed number of context-coded bin is equal to tb_width*tb_height*1.75. For a chroma TB, the maximum allowed number of context-coded bin is equal to tb_width*tb_height*1.25. The tb_width and tb_height is the with and height of TB after zero-out.

- In the second test, for a luma TB, the maximum allowed number of context-coded bin is equal to subblock_size*lastPosSubblock*1.75, where lastPosSubblock is the number of subblocks traversed in the scanning of the non-zeroed region of transforms until reaching the subblock with the last coefficient position. For a chroma TB, the maximum allowed number of context-coded bin is equal to subblock_size*lastPosSubblock*1.25.

## Joint chroma residual coding with multiple modes (N0282)

In these CE tests, extensions of the joint chroma residual coding suggested in JVET-M0305 and adopted based on JVET-N0054 are investigated. In contrast to M0305 and N0054, in which the addition of one single-channel joint chroma residual coding mode (given by Cr = −Cb) was suggested, in the extensions, three modes for joint chroma residual coding with different mixing factors are supported (further described below). The modes are additionally characterized by a sign used for deriving the second chroma residual, which is coded in the tile group header or, alternatively, for each CTU or CU/TU. The usage of a joint chroma coding mode is indicated by a TU-level flag tu_joint_chroma_residual_flag and the selected mode is implicitly indicated by the chroma coded block flags.

Specifically, this CE7 test proposes to extend the adopted single-channel joint coding of chrominance residuals from JVET-N0054 by

- introducing further joint-coding modes in addition to the adopted difference mode; the encoder can select and signal the applied joint-chroma coding mode in each TU,

- allowing the use of dual-channel transform-based joint-chroma coding modes (e. g., Hadamard-based, KLT-based) instead of only single-channel downmix-based modes,

- supporting the additional signaling of the sign parameter (i. e., signs of the joint-decoding weights, CSign in JVET-N0282) for the applied joint-chroma coding mode(s).

As in N0054, a tile-group-wise chroma QP offset specifically for use with the joint-chroma coding modes can be signaled. When joint-chroma coding is active in a TU, this chroma QP offset is added to the applied luma-derived chroma QP during quantization and decoding of that TU.

Two configurations are tested, which are described in the following subsections.

### Configuration 1: Three modes with single chroma residuals

The transform unit syntax is modified as follows (changes are highlighted in red):

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) |  |
| tu_joint_chroma_residual_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_joint_chroma_residual_flag[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] && !tu_joint_chroma_residual_flag[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |

The added syntax element tu_joint_chroma_residual_flag has the following meaning. A value equal to 0 indicates that no joint chroma coding is used (i.e., the chroma components are individually coded and reconstructed). If tu_joint_chroma_residual_flag is equal to 1, a single joint chroma residual is coded and reconstructed using the inverse transform. The reconstructed residual shall be noted by resJointC in the following. Depending on the chroma cbfs, the residual for the Cb and Cr component (denoted by resCb and resCr, respectively) are reconstructed as specified in the following table.

| tu_cbf_cb | tu_cbf_cr | reconstruction of Cb and Cr residuals | mode |
| --- | --- | --- | --- |
| 1 | 0 | resCb[ x ][ y ] = resJointC[ x ][ y ] resCr[ x ][ y ] = ( CSign * resJointC[ x ][ y ] ) >> 1 | 1 |
| 1 | 1 | resCb[ x ][ y ] = resJointC[ x ][ y ] resCr[ x ][ y ] = CSign * resJointC[ x ][ y ] | 2 |
| 0 | 1 | resCb[ x ][ y ] = ( CSign * resJointC[ x ][ y ] ) >> 1 resCr[ x ][ y ] = resJointC[ x ][ y ] | 3 |

The value CSign is a sign value (+1 or -1), which is specified in the tile group header, or alternatively, at CTU or CU/TU level.

### Configuration 2: Two modes with single chroma residuals and one mode with two chroma residuals

The transform unit syntax is modified as follows (changes are highlighted in red):

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) |  |
| tu_joint_chroma_residual_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |

The added syntax element tu_joint_chroma_residual_flag has the following meaning. A value equal to 0 indicates that no joint chroma coding is used (i.e., the chroma components are individually coded and reconstructed). If tu_joint_chroma_residual_flag is equal to 1, one or two joint chroma residuals are coded and reconstructed using the inverse transform (two if both cbfs are equal to 1, and one if only one cbf is equal to 1). The reconstructed residuals shall be noted by resJointC1 and resJointC2 in the following, where resJointC1 is the residual coded for cIdx = 1 and resJointC2 is the residual coded for cIdx = 2. Depending on the chroma cbfs, the residual for the Cb and Cr component (denoted by resCb and resCr, respectively) are reconstructed as specified in the following table.

| tu_cbf_cb | tu_cbf_cr | reconstruction of Cb and Cr residuals | mode |
| --- | --- | --- | --- |
| 1 | 0 | resCb[ x ][ y ] = resJointC1[ x ][ y ] resCr[ x ][ y ] = CSign * resJointC1[ x ][ y ] | 2 |
| 1 | 1 | resCb[ x ][ y ] = resJointC1[ x ][ y ] + resJointC2[ x ][ y ] resCr[ x ][ y ] = CSign * ( resJointC1[ x ][ y ] – resJointC2[ x ][ y ] ) | 4 |
| 0 | 1 | if( joint_chroma_mode_flag ) { resCb[ x ][ y ] = ( CSign * resJointC2[ x ][ y ] ) >> 1 resCr[ x ][ y ] = resJointC2[ x ][ y ] } else { resCb[ x ][ y ] = resJointC2[ x ][ y ] resCr[ x ][ y ] = ( CSign * resJointC2[ x ][ y ] ) >> 1 } | 3 1 |

The value CSign is a sign value (+1 or -1), which is specified in the tile group header, or alternatively, at CTU or CU/TU level. The reconstruction for the case tu_cbf_cb = 0 and tu_cbf_cr = 1 depends on the values of the binary syntax element joint_chroma_mode_flag, which is transmitted in the tile group header, or alternatively, on a CTU or CU/TU level.

Non-normative determination of joint chroma residuals in the encoder

In the following, an example of the non-normative procedure for generating the joint chroma residuals is given. Depending on the mode (listed in the tables above), the following applies:

- If mode is equal to 4 (Hadamard transform, two residuals are coded), the joint residuals can be determined according to

	resJointC1[ x ][ y ] = ( resCb[ x ][ y] + CSign * resCr[ x ][ y ] ) / 2
	resJointC2[ x ][ y ] = ( resCb[ x ][ y] – CSign * resCr[ x ][ y ] ) / 2

- Otherwise, if mode is equal to 2 (single residual with reconstruction Cb=C, Cr=CSign*C), the joint residual can be determined according to

	resJointC[ x ][ y ] = ( resCb[ x ][ y ] + CSign * resCr[ x ][ y ] ) / 2

- Otherwise, if mode is equal to 1 (single residual with reconstruction Cb=C, Cr=(CSign*C)/2), the joint residual can be determined according to

	resJointC[ x ][ y ] = ( 4 * resCb[ x ][ y ] + 2 * CSign * resCr[ x ][ y ] ) / 5

- Otherwise (mode is equal to 3, i.e., single residual with reconstruction Cr=C, Cb=(CSign*C)/2), the joint residual can be determined according to

	resJointC[ x ][ y ] = ( 4 * resCr[ x ][ y ] + 2 * CSign * resCb[ x ][ y ] ) / 5

In the above example, for the Hadamard transform case (mode 4), the inverse transform is used in the encoder. For the single joint residual cases (modes 1, 2, and 3), the joint residual is determined in a way that the downmix distortion (i.e., the distortion that is obtained by representing the Cb and Cr residuals by a single channel C, without considering quantization) is minimized.

The non-normative encoding process may be optimized as part of the CE investigation.

## Joint coding of chroma residuals (N0347)

The tested method that is summarized in Table 1 builds upon the joint chroma residual coding mode that is adopted in the 14th JVET meeting. The Cb and Cr residuals are processed in the rescaled chroma domain before transforms, quantization, entropy coding. At the decoder-side, in case the signalled joint chroma residual coding mode is enabled for the transform block, the reconstruction of a Cb transform block consists of adding the two decoded Cb and Cr residuals to the predicted Cb block, while the reconstruction of a Cr transform block consists of subtracting the decoded Cb residual and adding the decoded Cr residual to the predicted Cr block.

Table 1: Tested method

|  | Method |
| --- | --- |
| Cb residual encoder processing (with resCb = origCb – predCb) | resJointCb = (resCb – resCr) / 2 |
| Cr residual encoder processing (with resCr = origCr – predCr) | resJointCr = (resCb + resCr) / 2 |
| Cb reconstruction (recCb) | recCb = predCb + resJointCb + resJointCr |
| Cr reconstruction (recCr) | recCr = predCr – resJointCb + resJointCr |

Table 2: Signalling

residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) {

if( cIdx == 1 && tu_cbf_cb[ x0 ][ y0 ] ) {

tu_cb_cr_joint_residual[ x0 ][ y0 ]

}

…

## Context modelling of transform skip mode (N0094)

In this CE test, a context modelling method for coefficient coding of the transform skip mode is tested. The context models for coefficient coding is derived from two or three previously decoded neighboring coefficients located at the left, top, and top-left positions of the current to-be-decoded coefficient. A separate and additional context set is defined for sig_coeff_flag, par_level_flag, abs_level_gt1_flag, and abs_level_gt3_flag. The Rice parameter is derived also based on reduced number of neighbors.

The proposed context modelling is implemented on the top of VTM-5 TS residual coding.

The derivation process of the context index of the syntax element sig_coeff_flag is as follows:

ctxInc = min( locSumAbsPass1, 5 )

The derivation process of the context index of the syntax elements par_level_flag, abs_level_gt1_flag, and abs_level_gt3_flag are given below:

ctxInc = Min( locSumAbsPass1 − locNumSig, 4 )

## Context modelling of sign for TS residual coding (N0357)

The proposed approach employs three context models as follows. For each transform block using transform skip mode (TSM), the number of sign syntax elements equal to one and the total number of sign syntax element are tracked. The relative ratio r between the two is updated and evaluated resulting in three context models: the first context model is employed for the case r < a, the second context model is employed for the case r > b, and the third (and initial) context model is employed for the case where a ≤ r ≤ b, with a = 0.05 and b = 0.95, respectively.

Test:

- As proposed in N0357

- Alternative configuration using different number of thresholds & contexts

## Modified limitation on context coded bins for TS residual coding (N0366)

Two aspects are proposed regarding the coefficient coding applied for transform skip mode. The signaling of parity flag is relocated to be after gt2 flag and more context-coded bins are allowed for gt2 flag given the total budget of context coded bins.

Test:

- parity flag signaled after gt2 flag and up to 8 contexts coded gt2 flags per coefficient

- parity flag is bypass coded and signaled after gt2 flag, up to 8 contexts coded gt2 flags per coefficient

## Sign context modelling and level mapping for TS residual coding (N0455)

Sign context modelling using top and left neighbor sign information is proposed. If both neighboring coefficients are zero or both nonzero but with opposite signs, then context offset 0 is used, otherwise if both are non-negative and then context offset 1 is used, for all other cases context offset 2 is used. This can be summarized by the following table.

| (X0, X1) | Context offset |
| --- | --- |
| (0, 0), (+, -), (-, +) | 0 |
| (+, +), (+, 0), (0, +) | 1 |
| (-, -), (-, 0), (0, -) | 2 |

The absolute coefficient level, absCoeffLevel, is mapped to a modified level to be coded by the following method that depends on the values of quantized residual samples to the left and above of the current residual sample.  Let X0 denote the absolute coefficient level to the left of the current coefficient and let X1 denote the absolute coefficient level of above coefficient. For representing a coefficient with absolute coefficient level absCoeff, a mapped absCoeffMod is coded which is derived as follows:

pred = max(X0, X1);

if (absCoeff == pred)

{

absCoeffMod = 1;

}

else

{

absCoeffMod = (absCoeff < pred) ? absCoeff + 1 : absCoeff;

}

Syntax elements sig_coeff_flag, coeff_sign_flag, abs_level_gt1_flag, par_level_flag, that are coded interleaved residual sample by residual sample in the first pass followed by abs_level_gtX_flag bitplanes are converted to bitplane coding in sig_coeff_flag, coeff_sign_flag, abs_level_gt1_flag, abs_level_gtX_flag (X: 3,5,..) followed by  par_level_flag bitplane order.

Test:

- Sign context modelling

- Level mapping

- Bitplane coding

## Simplification of cbf coding (N0326)

In current VVC, the intra sub-partition flag, cbf of previous sub-partition and transform depth are utilized to select one out of four contexts for coding the cbf of a luma block. It is proposed to select the context based on only trDepth. The simplified cbf coding for luma is illustrated as below.

##### Derivation process of ctxInc for the syntax element tu_cbf_luma

There are Two CABAC contexts could be utilized to code the tu_cbf_luma and the procedure to determine the context index (ctxInc) is illustrated below.

ctxInc = trDepth = = 0  ?  1  :  0

# References

- T.-D. Chuang, S.-T. Hsiang, Z.-Y. Lin, C.-Y. Chen, Y.-W. Huang, S.-M Lei, “CE7-related: TB-level constraints on context-coded bins for coefficient coding,” JVET-N0091, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

- M. Coban, M. Karczewicz, “CE7-related: TU-level limits on context-coded bins for coefficient coding,” JVET-N0106, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

- C. Helmrich, C. Rudat, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand, “CE7-related: Joint chroma residual coding with multiple modes,” JVET-N0282, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

- G. Van der Auwera, A. K. Ramasubramonian, M. Coban, M. Karczewicz, “CE7-related: Joint coding of chroma residuals,” JVET-N0347, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

- M. G. Sarwer, O. Chubach, C.-W. HsuT, Y.-W. Huang, S.-M Lei, “CE8-related: Contex modelling of transform skip mode,” JVET-N0094, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

- B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand, “CE8-related: Context Modelling of Sign for TS Residual Coding,” JVET-N0357, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

- X. Zhao, X. Li, X. Xu, S. Liu, “CE8-related: Modified limitation on context coded bins for CE8-4.3a and CE8-5.1a,” JVET-N0366, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

- M. Karczewicz, M. Coban, “CE8-related: TU-level limits on context-coded bins for coefficient coding,” JVET-N0455, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

- Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang, “Non-CE7: Simplification of cbf coding,” JVET-N0326, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

- X. Xu. Y.-H. Chao, Y.-C Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-N2038, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.