| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N1029 |
| --- | --- |

| Title: | Description of Core Experiment 9 (CE9): Decoder Motion Vector Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Output document of JVET |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Semih Esenlik Xiaoyu Xiu | Tel: Email: | semih.esenlik@huawei.com, xiaoyuxiu@kwai.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

This document is a description of Core Experiment 9 (CE9) on decoder motion vector derivation. The scope of the tools that are to be studied in this CE include decoder motion vector refinement and bi-directional optical flow. In total 13 tests are planned to be performed in two subtests, 6 tests in BDOF early termination subtest and 4 tests in DMVR enabling condition subtest.

# List of participants

| Participants | Proponent | Cross-checker |
| --- | --- | --- |
| Semih Esenlik (semih.esenlik@huawei.com) Jianle Chen (Jianle.Chen@huawei.com) Haitao Yang (haitao.yang@huawei.com) Huanbang Chen (chenhuanbang@huawei.com) Xiang Ma (maxiang6@huawei.com) | X | X |
| Yu-Wen Huang (yuwen.huang@mediatek.com) Ching-Yeh Chen (chingyeh.chen@mediatek.com) Tzu-Der Chuang (peter.chuang@mediatek.com) Chih-Wei Hsu (cw.hsu@mediatek.com) Chen-Yen Lai (jenny.lai@mediatek.com) | X | X |
| Kenji Kondo (kenji.kondo@sony.com) Masaru Ikeda (Masaru.Ikeda@sony.com) | X | X |
| Andrew Segall (asegall@sharplabs.com) Takeshi Chujoh (chujoh.takeshi@sharp.co.jp) Tomohiro Ikai (ikai.tomohiro@sharp.co.jp) Eiichi Sasaki (eiichi.sasaki@sharp.co.jp) | X | X |
| Kyohei Unno (ky-unno@kddi-research.jp) Kei Kawamura (ki-kawamura@kddi.com) Yoshitaka Kidani (yo-kidani@kddi.com) | X | X |
| Sriram Sethuraman (sriram.sethuraman@ittiam.com) | X | X |
| Xiaoyu Xiu (xiaoyuxiu@kwai.com) Yi-Wen Chen (yiwenchen@kwai.com) Tsung-Chuan Ma (tsung-chuanma@kwai.com) Xianglin Wang (xianglinwang@kwai.com) | X | X |

# Test conditions and evaluation criteria

The VVC Test Model (VTM5.0) will be used as anchor. Proposals will be evaluated using BD rates, encoder run time, decoder time with Common Test Conditions (CTC) [1] as agreed at the Geneva meeting.

# CE9-1: BDOF early termination

## Description of the tests

### CE9-1.1: JVET-N0097

In VTM4.0, DMVR is adopted to refine motion vectors (MVs) at the decoder for a coding unit (CU) by minimizing the SAD between L0 and L1 predictors; BDOF refines the luma sample predictors with a two-level early termination process. The first early termination is at CU level, and the second early termination is at 4×4 block level. At each level, the SAD between L0 and L1 predictors in one unit, which is one CU in the first level or one 4×4 block in the second level, is calculated first. If the SAD is smaller than one threshold, then BDOF is not applied to the current unit. Otherwise, BDOF parameters are derived and used to generate the final luma sample predictors. When the CU is coded with regular merge/skip mode, one reference picture is before the current picture and the other reference picture is after the current picture in display order, the temporal distance between the current picture and one reference picture is equal to that between the current picture and the other reference picture, and BCW selects equal weights for L0 and L1 predictors, DMVR is applied first and followed by BDOF. In JVET-N0097, several methods are proposed to relieve the critical path of cascading DMVR and BDOF. In this CE test, two of them are tested. One is using DMVR cost to disable BDOF. And the other is further disabling the early termination in BDOF.

- Use DMVR cost to disable BDOF

- Use DMVR cost to disable BDOF and disable BDOF early termination

### CE9-1.2: JVET-N0148

In this experiment, an early termination for BDOF by using SAD value of DMVR without additional matching process is tested. Depending on the search result of motion vector refinement, if a matching cost of DMVR is greater than or equal to the threshold, BDOF apply, otherwise BDOF does not apply.

In this experiment, all early termination of BDOF is disabled.

### CE9-1.3: JVET-N0158

An alternative method of SAD based early termination for BDOF is tested. Details are as follows.

- SADs are not calculated in the BDOF stage for early termination.

- SADs calculated in the DMVR stage are reused for early termination.

- At a block not applied DMVR, no early terminations are applied.

- If the smallest SAD at integer distance search stage in DMVR is less than the threshold, BDOF is not applied to the sub-PU.

- The threshold is defined as 16 * W * (H >> 1). Here, W means width of the sub-PU, and H means height of the sub-PU.

### CE9-1.4: JVET-N0296

This test uses the lowest SAD cost computed or derived during DMVR at a sub-block level to disable BDOF for that sub-block, whenever both DMVR and BDOF are applied to the CU to which the sub-block belongs. Whenever DMVR is not applied, but BDOF is applied, the 16x16 early termination check present in VTM5 will be used. The test removes the second level early termination for BDOF at the 4x4 level for all cases. The threshold value used shall be the same threshold that is used at the 16x16 level in VTM5. The main purpose of the test is to enable clock-gating for BDOF in hardware using information available from the DMVR stage to achieve a lower average power consumption. In software implementations, it is aimed at preserving a large portion of the average decoding time reduction offered by the early termination checks employed in VTM5.

There will be 2 tests:

- Test-1: Using only the best SAD cost amongst the integer distance DMVR refinement positions to compare against a threshold, and disabling BDOF if the value is less than or equal to the threshold.

- Test-2: Using the derived SAD cost based on the parametric error surface function, when parametric function based refinement is present. Otherwise, the best SAD cost amongst the integer distance DMVR refinement positions is used.

### CE9-1.5: JVET-N0507

In this test, a reuse of DMVR cost is tested for the early termination in BDOF. In BDOF, SAD values are calculated to check the early termination. In DMVR process, a SAD cost is computed to find refinement motion vectors. In this test, the SAD cost of DMVR is reused for the early termination in BDOF. If the SAD cost of DMVR is smaller than a threshold, BDOF process is skipped. If the SAD cost of DMVR is larger than the threshold, BDOF is processed, and a 4x4 level early termination is disabled. In the current design, there are two thresholds for 1st and 2nd early terminations. In this test, the threshold of 4x4 level early termination is used. In an additional test, also it’s tested that SAD calculation of BDOF is completely removed.

## Test Specifications

| # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| CE9-1.1a | Identical to CE9-1.5a. Use integer-distance DMVR cost to disable BDOF with threshold equal to 4x4 level TH*32 (only keep BDOF early termination when DMVR is disabled) | Jenny Lai (MediaTek) |  |
| CE9-1.1b | Identical to CE9-1.2a and CE9-1.5b Use integer-distance DMVR cost to disable BDOF with threshold equal to 4x4 level TH*32 and disable BDOF early termination in all cases | Jenny Lai (MediaTek) |  |
| CE9-1.2a | Identical to CE9-1.1b and CE9-1.5b By using the minimum DMVR cost with a threshold, disable BDOF. All early termination of BDOF is disabled. | T. Chujoh (Sharp) |  |
| CE9-1.2b | A threshold value of center position is changed. Otherwise, same as CE9-1.2a. | T. Chujoh (Sharp) |  |
| CE9-1.3 | Difference from CE9-1.1b, CE9-1.2a, and CE9-1.5b is only threshold. BDOF early termination by using the min SAD in DMVR first SAD stage or in integer search stage | K. Unno (KDDI) |  |
| CE9-1.4a | Disabling application of BDOF based on best cost computed or derived in DMVR, when both DMVR and BDOF are applied. Use only best SAD cost obtained from integer distance refinement positions. When only BDOF is applied, VTM5 16x16 early termination is applied. Threshold used is same as that will be used in VTM5 for 16x16 4x4 early termination check is disabled for all cases | S. Sethuraman (Ittiam) |  |
| CE9-1.4b | Use a derived best cost from parametric error surface when applicable. Otherwise, same as (CE9-1.4a). | S. Sethuraman (Ittiam) |  |
| CE9-1.5a | Identical to CE9-1.1a. Use SAD cost of DMVR and threshold of 4x4 level early termination. | K. Kondo (Sony) |  |
| CE9-1.5b | Identical to CE9-1.1b and CE9-1.2a CE9-1.5a + remove the SAD calculation in BDOF | K. Kondo (Sony) |  |

## Details of Tests

|  | the SAD calculated in DMVR used to disable BDOF | Threshold | Early Termination of BDOF, when DMVR is enabled | Early Termination of BDOF, when DMVR is disabled |
| --- | --- | --- | --- | --- |
| CE9-1.1a | integer-distance SAD | BDOF 4x4Level TH | Disable | Subblock level, 4x4 level |
| CE9-1.1b | integer-distance SAD | BDOF 4x4Level TH | Disable | Disable |
| CE9-1.2a | integer-distance SAD | 8*W*(H>>1) on SAD of DMVR (4x4 level th of BDOF) | Disable | Disable |
| CE9-1.2b | integer-distance SAD | 4*W*(H>>1) for center position (sub block level th of BDOF), otherwise 8*W*(H>>1) | Disable | Disable |
| CE9-1.3 | integer-distance SAD | 16*W*(H>>1) | Disable | Disable |
| CE9-1.4a | integer-distance SAD | BDOF Subblock-level TH | Disable | Subblock level |
| CE9-1.4b | fractional-distance SAD; integer-distance SAD, if no fractional-distance SAD | BDOF Subblock-level TH | Disable | Subblock level |
| CE9-1.5a | integer-distance SAD | BDOF 4x4Level TH | Disable | Subblock level, 4x4 level |
| CE9-1.5b | integer-distance SAD | BDOF 4x4Level TH | Disable | Disable |

# CE9-2: DMVR/BDOF enabling conditions

## Description of the tests

### CE9-2.1: JVET-N0145

This test studies a harmonization of DMVR and MMVD. To reduce redundancy between DMVR and MMVD, DMVR is applied with MMVD when MMVD distance is larger than 2pel. Proposed method just changes condition to apply DMVR, hence there is no worst case increasement of decoder and DMVR process is also unchanged.

### CE9-2.2: JVET-N0327

In the current CIIP design, the DMVR and the BDOF are always enabled to generate the inter prediction samples when the current CIIP CU is bi-predicted. In this CE test, to reduce the computational complexity of the CIIP mode, the method of disabling the DMVR and BDOF for the CIIP mode is studied.

### CE9-2.3: JVET-N0395

## Test Specifications

| # | description | Tester | Crosschecker |
| --- | --- | --- | --- |
| CE9-2.1a | Harmonization of DMVR and MMVD with MMVD distance threshold (for reference). | E. Sasaki (Sharp) |  |
| CE9-2.1b | CE9-2.1.a + encoder speed up | E. Sasaki (Sharp) |  |
| CE9-2.2 | The CIIP with disabled DMVR and BDOF. | X. Xiu (Kwai) |  |
| CE9-2.3 |  |  |  |

# Time-line and Responsibilities

| T1 |  | Finalize CE description |
| --- | --- | --- |
| T2 |  | Deliver software packages and initial study by cross-checkers. Proponents can continue to modify the software until T3 |
| T3 |  | Software is finalized. Proponents provide full results and final crosschecking begins. |
| T4 |  | Contribution upload deadline. |

# References

[1]	“JVET common test conditions and software reference configurations for SDR”, Document: JVET-N1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring