| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N1030-v2 |
| --- | --- |

| Title: | Description of Core Experiment 10 (CE10): Neural Network based Filter for Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Yiming Li Shan Liu Kei Kawamura | Email: | userlym@whu.edu.cn shanl@tencent.com ki-kawamura@kddi.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The purpose of this Core Experiment (CE) is to investigate the coding tools related to neural network (NN) based loop filter on top of the Test Model. In this CE, the impact of the bit-depth of the parameters, the difference between in-loop and post processing filter, etc., will be investigated. In addition to the encoder and decoder run-times, the network information, e.g., network parameter numbers, number of layers, and convolution kernel size will also be evaluated. The list of tools and experiments to be performed are detailed in this document.

Introduction

The goal of Core Experiment 10 (CE10) is to investigate the neural network based loop filter techniques on top of VTM5.0. It comprises 2 categories,

- CE 10.1: Topics on top of sequence-dependent (two-pass) methods

- CE 10.2: Topics on top of sequence-independent (one-pass) methods

Each category will investigate the following problems:

Mandatory:

- Test [bit-depth] The impact of the bit-depth of the parameters. Some typical values of the bit depth include 4/6/8/16.

- Test [in-loop/post filtering] The difference between in-loop and post processing filter.

Optional:

- Investigating the performance overlap between ALF and NN filter in the loop filter chain

- The generalization capability of the NN filter when the test QP different from the training QP. (The difference should be larger than 5)

- Adding pre-defined filter set

- Network configuration with similar number of weights

- Impact of training set in sequence-dependent methods

- …/etc.

Participant list

| Affiliation | Participant Name | Email Address | Proponent | Cross-checker |
| --- | --- | --- | --- | --- |
| Hikvision | Jiabao Yao Chaoyi Lin Li Wang | yaojiabao@hikvision.com linchaoyi@hikvision.com wangli7@hikvision.com | X |  |
| Intel | Hujun Yin Rongzhen Yang | hujun.yin@intel.com rongzhen.yang@intel.com | X |  |
| KDDI | Kei Kawamura Yoshitaka Kidani Kyohei Unno | ki-kawamura@kddi.com yo-kidani@kddi.com ky-unno@kddi-research.jp | X | X |
| MediaTek | Yu-Wen Huang Chih-Wei Hsu Yu-Ling Hsiao Ching-Yeh Chen Tzu-Der Chuang Olena Chubach | yuwen.huang@mediatek.com cw.hsu@mediatek.com Yuling.Hsiao@mediatek.com chingyeh.chen@mediatek.com Peter.Chuang@mediatek.com Olena.Chubach@mediatek.com | X |  |
| Tencent | Shan Liu Byeongdoo Choi | shanl@tencent.com bdchoi@tencent.com | X |  |
| USTC | Yue Li Ning Yan Yuanying Dai Dong Liu | lytt@mail.ustc.edu.cn nyan@mail.ustc.edu.cn daiyy@mail.ustc.edu.cn dongeliu@ustc.edu.cn | X |  |
| Wuhan Univ. | Yingbin Wang Yiming Li Zhenzhong Chen | wyb@whu.edu.cn userlym@whu.edu.cn zzchen@whu.edu.cn | X |  |
| LGE | Hyeongmun Jang Junghak Nam | hm.jang@lge.com junghak.nam@lge.com |  |  |
| Kwai | Xianglin Wang Yiwen Chen Xiaoyu Xiu Tsung-Chuan Ma | xianglinwang@kuaishou.com yiwenchen@kwai.com xiaoyuxiu@kwai.com tsung-chuanma@kwai.com |  |  |
| Technicolor | Fabien Racapé Franck Galpin | Fabien.Racape@technicolor.com Franck.Galpin@technicolor.com |  |  |
| NWPU | Shuai Wan Mingze Wang | swan@nwpu.edu.cn roronoa@mail.nwpu.edu.cn | X |  |
| Xidian Univ. | Junyan Huo Yanzhuo Ma | jyhuo@mail.xidian.edu.cn yzma@mail.xidian.edu.cn | X |  |
| Huawei | Sergey Ikonin Seungwook Hong | Sergey.Ikonin@huawei.com seuhong@outlook.com |  |  |
| Sharplabs | Andrew Segall Kiran Misra | asegall@sharplabs.com misrak@sharplabs.com |  |  |
| Fujitus | Jianqing Zhu Kimihiko Kazui | zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |  |  |
| Qualcomm | Nan Hu Yan Zhang Yao-Jen Chang | nanh@qti.qualcomm.com yzh@qti.qualcomm.com yjchang@qti.qualcomm.com |  |  |
| Nokia | Francesco Cricri Caglar Aytekin Alireza Zare Yat Lam | francesco.cricri@nokia.com caglar.aytekin@nokia.com alireza.zare@nokia.com yat.lam@nokia.com |  |  |
| Bytedance | Li Zhang | lizhang.idm@bytedance.com |  |  |
| Ubilinx | Eric Chai Weimin Zeng | eric.chai@ublnx.com weimin.zeng@ublnx.com |  |  |
| Panasonic | Chong Soon Lim Han Boon Teo | chongsoon.lim@sg.panasonic.com hanboon.teo@sg.panasonic.com |  |  |
| ETRI | Seunghyun Cho | shcho@etri.re.kr |  |  |
| InterDigital | Philippe Hanhart Yuwen He Rahul Vanam Shilin Wu Hua Yang | Philippe.Hanhart@InterDIgital.com Yuwen.He@Interdigital.com Rahul.Vanam@Interdigital.com Shilin.Wu@Interdigital.com Hua.Yang@Interdigital.com |  |  |
| Samsung | Chirag Pujara Raj Narayana Gadde | chirag.p@samsung.com raj.gadde@samsung.com |  |  |
| Dolby | Peng Yin | pyin@dolby.com |  |  |
| Netflix | Andrey Norkin | anorkin@netflix.com |  |  |
| Sharp | Tomonori Hashimoto Tomohiro Ikai Eiichi Sasaki | tomonori.hashimoto@sharp.co.jp ikai.tomohiro@sharp.co.jp eiichi.sasaki@sharp.co.jp |  |  |

Description of tools

CE10-1 Topics on top of sequence-dependent methods

Technical descriptions

- JVET-N0110 CE13-1.1: Convolutional neural network loop filter [Y.-L. Hsiao, O. Chubach, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

The proposed CNNLF is applied after the adaptive loop filter (ALF). To reduce the signalling overhead of CNN parameters and CNNLF decoding time, a neural network of only three layers is used and shared by luma and chroma. As shown in Figure 1, the parameters of a layer, MxNxPxQ, are explained as follows.

- M: horizontal spatial filter size in one channel
- N: vertical spatial filter size in one channel
- P: number of input channels
- Q: number of filters in one layer

The operations in each layer include convolution, batch normalization, adding bias, and a rectified linear unit (ReLU) per filter. An exception is for the last layer, where no ReLU is applied.

Figure 1.  The 3-layer CNN shared by luma and chroma

The parameters of CNN are signalled in the I-picture of each RAS of roughly 1 sec. The parameters are trained using only pictures of temporal levels 0 and 1 in the RAS. To improve coding efficiency, when the encoder decides not to apply CNNLF for the I-picture, the CNN parameters are not signalled, and the remaining pictures in this RAS are all set as CNNLF-off. A multi-level on/off control is applied at picture, CTB, and 32x32 block levels for each color component.

- JVET-N0480 CE13-1.2: Adaptive convolutional neural network loop filter, ]H. Yin, R. Yang, X. Fang, S. Ma (Intel)]

The proposed ACNNLF is applied after the adaptive loop filter (ALF). The tested ACNNLF has the following properties:

- Two layer CNN, as shown in the Figure 2, the 1st layer includes convolution with ReLu, the 2nd layer includes convolution without ReLu;

- Dynamic number of ACNNLFs for luma and dynamic number of ACNNLFs for chroma adaptively trained with video data, the number of ACNNLFs for luma and chroma are signalled in APS (Adaptation Parameter Set);

- The encoder selects the best ACNNLF for each of luma and chroma in each CTB from the trained set of ACNNLFs, to perform encoding. The weights of the trained set of ACNNLFs are signaled in the APS and the index of selected ACNNLF is signaled in the coded stream in CTU level;

The decoder receives the ACNNLF weights and the CTU level selected ACNNLF and uses the selected ACNNLF to perform the filtering process for each luma and chroma block during frame reconstruction.

Figure 2. ACNNLF for luma

The parameters of ACNNLF are signaled in APS. In RA configuration, only the I-frame contains ACNNLF parameters of this RAS. The on/off control is applied at the APS for luma and chroma components, and the acnnlf_luma_frame_enable_flag and acnnlf_chroma_frame_enable_flag syntax elements are used to indicate ACNNLF enable/disable. CTB level information indicates which ACNNLF is selected or if there is no ACNNLF applied in the CTB.

- JVET-N0712 CE13-related: Adaptive CNN based in-loop filtering with boundary weights [K. Kawamura, Y. Kidani, S. Naito (KDDI)]

The weight is used for sum of residual signals by the CNN filter and the signals after deblocking filter. In the introduction, a benefit of an outside control of CNN model is mentioned.

The basic weight is derived from the distance from the boundary. We reuse the boundary definition and its strength derivation from the deblocking filter operation. When the boundary strength is larger than 0, weight is further modified by the block types like intra or inter coding. Figure 3 shows the actual weights for both intra and inter blocks. Thick and thin lines illustrates the block and pixel boundary.

Figure 3. Derived weights for intra (left) and inter (right) blocks when the BS is significant.

Summary of Tests

| Test # | Test Part | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- | --- |
| CE10-1.1a | bit-depth | 4-bit(int) weights | JVET-N0110 | Y.-L. Hsiao (Mediatek) |  |
| CE10-1.1b |  | 6-bit(int) weights |  |  |  |
| CE10-1.1c |  | 8-bit(int) weights |  |  |  |
| CE10-1.2a | in-loop/ post filter | CNNLF as in-loop filter |  | Y.-L. Hsiao (Mediatek) |  |
| CE10-1.2b |  | CNNLF as post filter |  |  |  |
| CE10-1.3a | adding pre-defined filter set | Apply pre-defined filter set by encoder |  | Y.-L. Hsiao (Mediatek) |  |
| CE10-1.4a | network configuration with similar number of filters | 3 layers, 3x3x6x8, 3x3x8x8, 3x3x8x6 |  | Y.-L. Hsiao (Mediatek) |  |
| CE10-1.4b |  | 4 layers, 3x3x6x8, 3x3x8x8, 3x3x8x8, 3x3x8x6 |  |  |  |
| CE10-1.5a | training set | temporal levels 0 and 1 |  | Y.-L. Hsiao (Mediatek) |  |
| CE10-1.5b |  | I-frame only |  |  |  |
| CE10-1.6a | bit-depth | 4-bit(int) weights | JVET-N0480 | H. Yin (Intel) |  |
| CE10-1.6b |  | 6-bit(int) weights |  |  |  |
| CE10-1.7a | in-loop/ post filter | NN Filter as in-loop filter |  | H. Yin (Intel) |  |
| CE10-1.7b |  | NN Filter as post filter |  |  |  |
| CE10-1.8a | w./wo. ALF in filter chain | DBF + SAO + ALF + NN filter |  | H. Yin (Intel) |  |
| CE10-1.8b |  | DBF + SAO + NN filter |  |  |  |
| CE10-1.9a | Data Scaling for NN Input / Output Channel by Normalized QP | Data Scaling for NN Input Channel by Normalized QP |  | H. Yin (Intel) |  |
| CE10-1.9b |  | Data Scaling for NN Residential Block Output Channel by Normalized QP |  |  |  |
| CE10-1.10a | bit-depth | 6-bit(int) weights | JVET-N0712 | K. Kawamura (KDDI) |  |
| CE10-1.10b |  | 14-bit(int) weights |  |  |  |
| CE10-1.11a | in-loop/ post filtering | CNNF as in loop filter |  | K. Kawamura (KDDI) |  |
| CE10-1.11b |  | CNNF as post filter |  |  |  |

CE10-2 Topics on top of sequence-independent methods

Technical descriptions

- JVET-N0133 CE13-related: Integrated in-loop filter based on CNN [S. Wan, M.-Z.Wang, H. Gong, C.-Y. Zou(NPU), Y.-Z. Ma, J.-Y. Huo(Xidian Univ.), Y.-F. Yu, Y. Liu(OPPO)]

The proposed CNNLF includes six inputs: three reconstructed components and three auxiliary inputs, i.e. one QPmap and two CUmaps for luma component and chroma component. The overview structure is shown in Figure 4. Due to different components have different partition information, the three components will be processed jointly and separately to fuse its own auxiliary information. The network is shared by luma and chroma so the three components will be filtered simultaneously.

Figure 4. Main structure of the proposed CNNLF

- JVET-N0169 CE13-2.1: Convolutional Neural Network Filter (CNNF) for Intra Frame [J.Yao, L.Wang (Hikvision)]

Since the proposed CNNF works as a loop filter for intra frames, there is no difference between in-loop and post processing. The test [in-loop/post filtering] related with JVET-N0169 is abandoned.

Our CNNF replace the DBK and SAO, working together with ALF as the loop filter. The architectures of the proposed CNNF composed by one concat layer, 7 convolution layers with each followed by a ReLU layer, one convolution layer and one summation layer. These layers are connected one by one and form a network, as shown in Figure 5. For a given network, the layer parameters are automatically learned from thousands of training data by minimizing a target loss function in the training process. Once finished, the parameters are fixed and is used as filter coefficients in CNNF. The proposed CNNF includes two inputs: reconstruction and quantization parameter (QP), which makes it possible to use a single set of parameters to adapt to reconstructions with different qualities. Both the two inputs are normalized for better convergence in training process.

Figure 5. CNNF network

The related CE test focus on the impact of the bit-depth of the parameters. Some typical values of the bit depth include 4/6/8/16 (Test [bit-depth]).

- JVET-N0254 CE13: Dense Residual Convolutional Neural Network based In-Loop Filter (CE13-2.2 and CE13-2.3) [Y. Wang, Z. Chen, Y. Li(Wuhan Univ.), L. Zhao, S. Liu, X. Li(Tencent)]

The network structure of DRNLF is shown in Figure 6, where N and M denotes the number of dense residual unit (DRU) and convolution kernel, respectively. Specifically, N is set to 4 and M is set to 32 for the tradeoff between computational efficiency and performance. Normalized QP map is concatenated with the reconstructed frame as the input to DRN. So the input has 4 channels in total.

Figure 6. Network structure of DRNLF.

The main body of the proposed network consists of series of DRU. The structure of DRU is shown in Figure 7. The DRU directly propagates its input to the subsequent unit through the shortcut. To further reduce the computational cost, 3x3 depth-wise separable convolutional (DSC) layer is applied in DRU.

Figure 7. Dense Residual Unit (DRU)

Finally, the output of the network has 3 channels, which corresponds to Y, Cb, Cr, respectively. The filter is applied for both intra and inter pictures. Additional flag is signaled for each CTUs to indicate the on/off of the DRNLF.

- JVET-N0513 CE13: Experimental results of CNN-based In-Loop Filter (USTC) [Y. Dai, D. Liu, N. Yan, F. Wu (USTC)]

The proposed CNN-based filter is located between DF and SAO. A flag which indicates whether CTU is filtered by NN filter is transmitted to decoder. Residue learning technique is integrated into network to accelerate training. To reduce memory usage, parameters of several layers are shared, which means we can achieve the performance of deeper network with fewer parameters. As shown in Figure 8, layers of the same color represent the same parameters.

Figure 8. The structure of the proposed network

- JVET-N0710 CE13-2.6/CE13-2.7: Evaluation results of CNN based in-loop filtering [K. Kawamura, Y. Kidani, S. Naito (KDDI)]

The filter structure is shown in Figure 9. This filter has four layers with 3x3 taps. The input of “sum” block is residual signal from the left signal and reconstructed signal just after deblocking filter. The output of “sum” block is filtered pixels. Actual output is weighted sum of after-filtered and before-filtered pixels based on the distance of edge.

Figure 9. The structure of proposed CNNF

Filter coefficients are pre-defined in both the encoder and the decoder so that additional side information is not required. A representation of filter coefficients is fixed point value (integer value). The filter is applied for both intra and inter pictures.

Summary of Tests

| Test # | Test Part | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- | --- |
| CE10-2.1a | bit-depth | 32-bit(float) weights | JVET-N0133 | Shuai Wan (NPU) | TBD (Hikvision) |
| CE10-2.1b |  | 8-bit(int) weights |  |  |  |
| CE10-2.2a | in-loop/ post filter | CNNF as in-loop filter |  |  |  |
| CE10-2.2b |  | CNNF as post filter |  |  |  |
| CE10-2.3a | generalization capability (QP) | CTC QP-5 |  |  |  |
| CE10-2.3b |  | CTC QP+5 |  |  |  |
| CE10-2.4a | bit-depth | 4-bit(int) weights | JVET-N0169 | J. Yao (Hikvision) |  |
| CE10-2.4b |  | 6-bit(int) weights |  |  |  |
| CE10-2.4c |  | 8-bit(int) weights |  |  |  |
| CE10-2.4d |  | 16-bit(int) weights |  |  |  |
| CE10-2.5a | bit-depth | 32-bit(float) weights | JVET-N0254 | Y. Wang (Wuhan Univ.) |  |
| CE10-2.5b |  | 8-bit(int) weights |  |  |  |
| CE10-2.6a | in-loop/ post filter | CNNF as in loop filter |  |  |  |
| CE10-2.6b |  | CNNF as post filter |  |  |  |
| CE10-2.7a | generalization capability (QP) | CTC QP-5 |  |  |  |
| CE10-2.7b |  | CTC QP+5 |  |  |  |
| CE10-2.8a | bit-depth | 4-bit(int) weights | JVET-N0513 | Y. Dai(USTC) H. Zhao(USTC) |  |
| CE10-2.8b |  | 6-bit(int) weights |  |  |  |
| CE10-2.8c |  | 8-bit(int) weights |  |  |  |
| CE10-2.8d |  | 16-bit(int) weights |  |  |  |
| CE10-2.9a | in-loop/ post filter | CNNF as in loop filter |  |  |  |
| CE10-2.9b |  | CNNF as post filter |  |  |  |
| CE10-2.10a | bit-depth | 8-bit(int) weights | JVET-N0710 | K. Kawamura (KDDI) |  |
| CE10-2.10b |  | 16-bit(int) weights |  |  |  |
| CE10-2.10a | in-loop/ post filtering | CNNF as in loop filter |  | K. Kawamura (KDDI) |  |
| CE10-2.10b |  | CNNF as post filter |  |  |  |

Test condition and test set definition

Test condition

The following test conditions shall be used for this CE:

Software

The Test Model VTM-5.0 shall be used for these experiments, as specified in JVET-N1010.

Test configuration

This CE shall use the following two test conditions:

- Common Test Conditions (CTC).

- CTC, but short test, which only need to test the first intra period.

| Purpose | CTC test (Full test) |  |  | First intra period test (Short version) |
| --- | --- | --- | --- | --- |
|  | All Intra | Random Access | Low delay | Random Access |
| anchor | M (For SubCE2) | M | O | M,S |
| Bit-depth investigation | N/A | N/A | N/A | M,S |
| In-loop/post filtering | N/A | M | O | N/A |
| Others(e.g., generalization capability) | N/A | N/A | N/A | O,S |

Note: M denotes mandatory test. O denotes the optional test. S denotes the short test. The full test should follow the CTC. The short test should also follow the CTC, except the number of frames, which only need to test the first intra period.

Network analysis

For the understanding of network, e.g., complexity, the following information shall be provided for each test planned for this CE: (just list for example. More information is specified in JVET-M1006)

- Total Conv. Layers: Total numbers of convolutional layers in the network structure.

- Total FC Layers: Total number of fully-connected layers in the network structure.

- Framework: The neural network framework in the inference stage. (e.g. Caffe, Tensorflow, Pytorch)

- Param. Num: Total numbers of the parameters in the neural network.

- Mem.P(MB): Memory usage of the total parameters. Mem=P∙B1024*1024, where P is the parameters number, and B indicates the byte for storing one parameter.

- Mem.T(MB): Temporary memory. It denotes the memory used to store the outputs (feature map) in each intermediate layer (forward pass).

- Parameter precision: float or integer? And the bit-depth.

Timeline

| T0 |  | Apr. 18, 2019 |  | - CE description is finalized and uploaded |  |
| --- | --- | --- | --- | --- | --- |
| T1 |  | Apr. 29, 2019 |  | - Release of VTM software |  |
| T2 |  | T1+2 weeks: May 13, 2019 |  | - Proponents provide initial SW of CE tools - Cross-checkers may start initial study of tools / software |  |
| T3 |  | June 5, 2019 (T2+1week, or 4 weeks before next JVET meeting, whichever is later) |  | - SW of each CE test is finalized - Cross-checkers start final cross checks |  |
| T4 |  | June 18, 2019 (regular document upload deadline – 1 week) |  | - Complete results of each CE test shall be provided |  |
| T5 |  | Jun 22, 2019 (Regular document deadline -3 days) |  | - Cross-check reports to be sent to CE coordinators |  |
| T6 | June 25, 2019 | - CE10 summary report is provided - regular document upload deadline |