| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0357-v1 |
| --- | --- |

| Title: | Non-CE6: Maximum TU size for chroma format 4:2:2 and 4:4:4 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {l.li, junghak.nam, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

- Abstract

The contribution proposes maximum TU size considering chroma formats such as 4:2:2 and 4:4:4. It is proposed to separate maximum TU size for chroma maxTbSizeC into two components: maxTbWidthC and maxTbHeightC so that VPDU is supported.

Problem Statement

In HEVC, maximum TU size for chroma is set to equal to maxTbSizeY/max(SubWidthC, SubHeightC). That is, for chroma format 4:2:2, the maxTbSizeC is half of maxTbSizeY.

Since VVC can support non-square TUs, it is desirable to have large chroma TUs instead of small ones. However, for chroma format 4:2:2, if the maxTbSizeC is set to equal to maxTbSizeY/min(SubWidthC, SubHeightC), VPDU is not supported.

Proposed Specification Text

8.4.5.1	General decoding process for intra blocks

…

The maximum transform block size maxTbWidth and maxTbHeight are derived as follows:

maxTbWidth = ( cIdx  = =  0 ) ? MaxTbSizeY : MaxTbSizeY / SubWidthC

maxTbHeight = ( cIdx  = =  0 ) ? MaxTbSizeY : MaxTbSizeY / SubHeightC

The luma sample location is derived as follows:

( xTbY, yTbY ) = ( cIdx  = =  0 ) ? ( xTb0, yTb0 ) : ( xTb0 * SubWidthC, yTb0 * SubHeightC )	(8-50)

The following applies:

- If IntraSubPartitionsSplitType is equal to NO_ISP_SPLIT and nTbW is greater than maxTbWidth or nTbH is greater than maxTbHeight, the following ordered steps apply.

- The variables newTbW and newTbH are derived as follows:

newTbW = ( nTbW  >  maxTbWidth ) ? ( nTbW / 2 ) : nTbW	(8-51)

newTbH = ( nTbH   >  maxTbHeight) ? ( nTbH / 2 ) :  nTbH	(8-52)

- The general decoding process for intra blocks as specified in this clause is invoked with the location ( xTb0, yTb0 ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- If nTbW is greater than maxTbWidth, the general decoding process for intra blocks as specified in this clause is invoked with the location ( xTb0, yTb0 ) set equal to ( xTb0 + newTbW, yTb0 ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- If nTbH is greater than maxTbHeight, the general decoding process for intra blocks as specified in this clause is invoked with the location ( xTb0, yTb0 ) set equal to ( xTb0, yTb0 + newTbH ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- If nTbW is greater than maxTbWidth and nTbH is greater than maxTbHeight, the general decoding process for intra blocks as specified in this clause is invoked with the location ( xTb0, yTb0 ) set equal to ( xTb0 + newTbW, yTb0 + newTbH ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering.

…

8.5.8	Decoding process for the residual signal of coding blocks coded in inter prediction mode

…

The maximum transform block size maxTbWidth and maxTbHeight are derived as follows:

maxTbWidth = ( cIdx  = =  0 ) ? MaxTbSizeY : MaxTbSizeY / SubWidthC

maxTbHeight = ( cIdx  = =  0 ) ? MaxTbSizeY : MaxTbSizeY / SubHeightC

The luma sample location is derived as follows:

( xTbY, yTbY ) = ( cIdx  = =  0 ) ? ( xTb0, yTb0 ) : ( xTb0 * SubWidthC, yTb0 * SubHeightC )

The following applies:

- If nTbW is greater than maxTbWidth or nTbH is greater than maxTbHeight, the following ordered steps apply.

- The variables newTbW and newTbH are derived as follows:

newTbW = ( nTbW  >  maxTbWidth) ? ( nTbW / 2 ) : nTbW	(8-871)

newTbH = ( nTbH   >  maxTbHeight) ? ( nTbH / 2 ) :  nTbH	(8-872)

- The decoding process process for the residual signal of coding units coded in inter prediction mode as specified in this clause is invoked with the location ( xTb0, yTb0 ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- When nTbW is greater than maxTbWidth, the decoding process process for the residual signal of coding units coded in inter prediction mode as specified in this clause is invoked with the location ( xTb0, yTb0 ) set equal to ( xTb0 + newTbW, yTb0 ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, and the variable cIdx as inputs, and the output is a modified reconstructed picture .

- When nTbH is greater than maxTbHeight, the decoding process process for the residual signal of coding units coded in inter prediction mode as specified in this clause is invoked with the location ( xTb0, yTb0 ) set equal to ( xTb0, yTb0 + newTbH ), the transform block width nTbW set equal to newTbW and the height nTbH set equal to newTbH, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- When nTbW is greater than maxTbWidth aInd nTbH is greater than maxTbHeight, the decoding process process for the residual signal of coding units coded in inter prediction mode as specified in this clause is invoked with the location ( xTb0, yTb0 ) set equal to ( xTb0 + newTbW, yTb0 + newTbH ), the transform block width nTbW set equal to newTbW and height nTbH set equal to newTbH, and the variable cIdx as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- Otherwise, the scaling and transformation process as specified in clause 8.7.2 is invoked with the luma location ( xTbY, yTbY ), the variable cIdx, the transform width nTbW and the transform height nTbH as inputs, and the output is an (nTbW)x(nTbH) array resSamples.

Summary

In this contribution proposes maximum TU width and height to support VPDU for chroma format 4:2:2 and 4:4:4 in VVC. It is recommended to adopt this proposal so that VVC could support various chroma formats.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).