| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0366-r1 |
| --- | --- |

| Title: | Non-CE4 : Simplifications on BCW index derivation process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | [LG Electronics Inc.] |  |  |

_____________________________

- Abstract

In affine merge mode, for inherited candidates, BCW index is inherited from spatial neighbor block which is coded with affine mode while for constructed candidates, BCW index is derived from indices of the each CPs. In this proposal, the number of comparisons on BCW index derivation process for affine constructed merge mode, which was adopted in the previous meeting is reduced as follows.

- CASE 1: The number of comparison is reduced to 1.

- CASE 2: The number of comparison is reduced to 0.

The experimental results for CASE1 reportedly show 0.00% (0.00%) BD-rate changes with 100% (100%) and 100% (98%) encoding and decoding run-time compared to VTM5.0 with bug-fix (Ticket #312) as anchor in RA(LB) configurations. And the experimental results for CASE2 reportedly show 0.01% (-0.03%) BD-rate changes with 100% (100%) and 100% (101%) encoding and decoding run-time compared to VTM5.0 with bug-fix (Ticket #312) as anchor in RA(LB) configurations.

Introduction

For the affine constructed merge candidate, BCW index (i.e. bcwIdxConst) is derived as Table 1. Here, bcwIdxCornerX indicates BCW index of each control point with X = 0.. 2 and bcwIdxGroupX is BCW group index, which is divided to three groups {0}, {1, 2, 3} and {4}, i.e., {(-1/4, 5/4)}, {(1/4, 3/4), (2/4, 2/4), (3/4, 1/4)} and {(5/4, -1/4)}. To derive the BCW index, LUT is used for grouping and 6 comparisons are required when 3 CPs are used in worst case.

Table 1. BCW index derivation process

if (bcwIdxCorner0 == bcwIdxCorner1 && bcwIdxGroup0 == bcwIdxGroup2) bcwIdxConst = bcwIdxCorner0 else if (bcwIdxCorner0 == bcwIdxCorner2 && bcwIdxGroup0 == bcwIdxGroup1) bcwIdxConst = bcwIdxCorner0 else if (bcwIdxCorner1 == bcwIdxCorner2 && bcwIdxGroup1 == bcwIdxGroup0) bcwIdxConst = bcwIdxCorner2 else bcwIdxConst = Default value

Proposed method

Affine constructed candidate list is generated as combinations of two or three CP candidates as follows. Here, CP0, CP1, CP2 and CP3 are derived from spatial/temporal neighboring candidates as depicted in Figure 1.

[CP0, CP1, CP2]

[CP0, CP1, CP3]

[CP0, CP2, CP3]

[CP1, CP2, CP3]

[CP0, CP1]

[CP0, CP2]

Figure 1. CP Candidates for constructed mode

But, as BCW index is set to default value for temporal candidate, comparison condition of the bcwIdxCorner2(which is the 3rd CP in each combination) becomes unnecessary to include CP3.

[CP0, CP1, CP2]

[CP0, CP1, CP3]

[CP0, CP2, CP3]

[CP1, CP2, CP3]

[CP0, CP1]

[CP0, CP2]

if (bcwIdxCorner0 == bcwIdxCorner1 && bcwIdxGroup0 == bcwIdxGroup2) bcwIdxConst = bcwIdxCorner0 else if (bcwIdxCorner0 == bcwIdxCorner2 && bcwIdxGroup0 == bcwIdxGroup1) bcwIdxConst = bcwIdxCorner0 else if (bcwIdxCorner1 == bcwIdxCorner2 && bcwIdxGroup1 == bcwIdxGroup0) bcwIdxConst = bcwIdxCorner2 else bcwIdxConst = Default value

Also, for the current BCW index derivation design on the non-low-delay condition, group comparison process can be further simplified because only three BCW index ((1/4, 3/4), (2/4, 2/4) and (3/4, 1/4)) are used and these indices are mapped into only one group.

if (bcwIdxCorner0 == bcwIdxCorner1 && bcwIdxGroup0 == bcwIdxGroup2) bcwIdxConst = bcwIdxCorner0 else if (bcwIdxCorner0 == bcwIdxCorner2 && bcwIdxGroup0 == bcwIdxGroup1) bcwIdxConst = bcwIdxCorner0 else if (bcwIdxCorner1 == bcwIdxCorner2 && bcwIdxGroup1 == bcwIdxGroup0) bcwIdxConst = bcwIdxCorner2 else bcwIdxConst = Default value

Therefore, it is proposed to simplify the BCW index derivation process by removing the unnecessary conditions. To simplify the BCW index derivation for affine constructed mode, two methods are tested.

CASE 1: The number of comparison is reduced to 1.

If BCW index of the first CP equal to BCW index of the second CP, BCW index (bcwIdxConst) of the current candidate is set to bcwIdxCorner0. Otherwise, bcwIdxConst is set to default value (i.e., equal weight).

if (bcwIdxCorner0 == bcwIdxCorner1) bcwIdxConst = bcwIdxCorner0 else bcwIdxConst = Default value

CASE 2 : The number of comparison is reduced to 0.

The bcwIdxConst is set to BCW index of the first CP in the each constructed candidate combination.

bcwIdxConst = bcwIdxCorner0

Experimental results

The proposed method has been implemented on top of the VTM5.0 with default setting and tested using JVET SDR common test condition [2]. The experimental results for CASE1 reportedly show 0.00% (0.00%) BD-rate changes with 100% (100%) and 100% (98%) encoding and decoding run-time compared to VTM5.0 with bug-fix (Ticket #312) as anchor in RA(LB) configurations. And the experimental results for CASE2 reportedly show 0.01% (-0.03%) BD-rate changes with 100% (100%) and 100% (101%) encoding and decoding run-time compared to VTM5.0 with bug-fix (Ticket #312) as anchor in RA(LB) configurations.

CASE 1: The number of comparison is reduced to 1.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 with bug-fix (Ticket#312) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.14% | 0.01% | 100% | 100% |
| Class A2 | 0.01% | -0.09% | -0.08% | 100% | 100% |
| Class B | 0.01% | 0.03% | 0.05% | 100% | 100% |
| Class C | -0.01% | -0.07% | 0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class D | 0.01% | 0.02% | 0.05% | 100% | 100% |
| Class F | 0.01% | 0.01% | 0.05% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 with bug-fix (Ticket#312) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.20% | -0.01% | 99% | 97% |
| Class C | 0.01% | 0.25% | -0.32% | 100% | 99% |
| Class E | 0.02% | 0.20% | 0.43% | 100% | 99% |
| Overall | 0.00% | 0.05% | 0.00% | 100% | 98% |
| Class D | -0.03% | 0.14% | 0.14% | 101% | 99% |
| Class F | -0.02% | -0.36% | -0.65% | 100% | 100% |

CASE 2: The number of comparison is reduced to 0.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 with bug-fix (Ticket#312) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.08% | 0.05% | 100% | 100% |
| Class A2 | 0.00% | 0.01% | -0.02% | 100% | 100% |
| Class B | 0.01% | 0.08% | -0.01% | 100% | 100% |
| Class C | 0.00% | -0.13% | -0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | -0.03% | 100% | 100% |
| Class D | 0.01% | 0.04% | 0.03% | 100% | 100% |
| Class F | 0.02% | -0.01% | 0.02% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 with bug-fix (Ticket#312) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.15% | -0.19% | 100% | 100% |
| Class C | -0.03% | 0.18% | -0.36% | 100% | 100% |
| Class E | -0.04% | 0.22% | 0.06% | 100% | 101% |
| Overall | -0.03% | 0.05% | -0.18% | 100% | 101% |
| Class D | -0.03% | 0.14% | 0.08% | 100% | 101% |
| Class F | 0.01% | -0.30% | -0.56% | 100% | 100% |

Conclusion

In this contribution, simplified BCW index derivation methods for affine constructed candidate are proposed. The experimental results reportedly show negligible changes in BD-rate and encoding/decoding run-time. It is recommended to consider this method in the next VTM.

Reference

- Jie Chen, et al., “Non-CE2: Gbi inheritance for constructed affine merge candidate”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N0481, 14th Meeting: Geneva, CH, 19–27 March 2019.

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).