| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0022-v2 |
| --- | --- |

| Title: | CE2: Summary Report on Gradual Decoding Refresh |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui Jean-Marc Thiesse Hendry Limin Wang Kei Kawamura | Tel: Email: | kazui.kimihiko@fujitsu.com jean-marc.thiesse@vitec.com fhendry@futurewei.com limin.2.wang@nokia.com ki-kawamura@kddi.com |
| Source: | CE2 coordinators |  |  |

_____________________________

- Abstract

A summary of Core Experiment 2 (CE2) on gradual decoding refresh (GDR) is reported. The goal of CE2 is to study GDR tools/methods, for potential inclusion into the VVC standard.

Seven tests were defined in CE2:

–	CE2-1: Virtual boundary-based method (4 tests)

–	CE2-2: Slice boundary-based method (2 tests) (withdrawn)

–	CE2-3: Wavefront-based method (1 test)

All tests are evaluated based on the common test conditions defined in JVET-O2022 [3]. All tests and crosscheck results are integrated in this report.

Test conditions and guidelines

CTC conditions for SDR [2] as agreed at the 15th JVET meeting are used for the evaluation of objective performance. The following constraints are further applied as specified in the CE2 document [3].

- Low-delay B configuration only

- Interval of GDR or IRAP picture (1.0 second)

- QP setting (no offset for IDR and B picture)

- The GOP size is 1. Each picture refers up to 4 immediately preceding pictures in decoding order

The anchor is the ‘VVC Test Model’ or VTM. The VTM version 6.0 software is used with ‘vtm’ configuration files to produce the VTM anchor. Tools are implemented into the VTM version 6.0 software. Proposals are primarily compared with respect to objective compression efficiency while bitrate fluctuation and “exact match” aspects are also assessed.

Source codes corresponding with the test descriptions contained in this document are uploaded to the following CE2 GitLab project:

https://vcgit.hhi.fraunhofer.de/jvet-o-ce2/VVCSoftware_VTM.git

Cross-check reporting

Each test is covered by at least one cross-checker. Within CE2 it was agreed that cross-checkers answer the following questions:

- Did the cross-checker study the software? If so, did the cross-checker find any inconsistencies between the test description, the provided final software, and the VVC spec text changes?

- Do the cross-checked results match the proponent results for the test conditions? Were additional results cross-checked?

- Additional comments that may be helpful to the group?

Timeline

The following table lists the timeline for the CE2 study planned in Geneva meeting.

Table 3-1 – CE2 timeline

| T1 | 2 August 2019 (3 weeks after the JVET meeting) | CE test description finalized |
| --- | --- | --- |
| T2 | 26 August 2019 (Test model software release + 2 weeks) | Initial software of CE tests released; initial study by cross-checkers begins |
| T3 | 10 September 2019 (3 weeks before the next JVET meeting) | Software of CE tests is finalized; final cross-check begins |
| T4 | 17 September 2019 (Regular document deadline -1 week) | CE contribution document upload, including specification text, complete test results |
|  | 21 September 2019 (Regular document deadline -3 days) | Cross-check reports to be sent to CE coordinators |
|  | 24 September 2019 (Regular document deadline) | CE summary report upload |

After T2, it was proposed to change T3 and T4 from 10 September and 17 September to 13 September and 20 September respectively, considering the delivery date of VTM-6.0 software.

As of 26 September, no complete test result was provided from the proponents, due to the delay of their software implementation.

Overview of CE2 test description document changes

No changes excluding the timeline were made to the CE2 test description document [3] after the T1 deadline (2 August 2019) had passed.

Defined tests in CE2

  - Summary

Table 5-1 – List of tested tools in CE2

| Test # | Short description | Doc. # | Cross-checker |
| --- | --- | --- | --- |
| CE2-1.a | Virtual boundary-based method - Vertical virtual boundary - Position accuracy: 8 samples - Encoder-side control on MV search range - Encoder-side control on intra prediction mode restriction - Virtual boundary position signaled in PPS header send at every picture - Filters disabled across virtual boundary - LMCS disabled / CE2-1.a.bis: virtual boundary for LMCS chroma residue scale computation. | JVET-O0100 JVET-O1048 (VITEC) | KDDI |
| CE2-1.b | Virtual boundary-based method, variation of CE2-1.a - Virtual boundary position signaled in slice header - Encoder-side control on intra prediction mode restriction - Filters disabled across virtual boundary - Encoder-side control on MV search range | JVET-O0100 JVET-O1048 (VITEC) | Nokia |
| CE2-1.c | Virtual boundary-based method, variation of CE2-1.b - Normative reference sample restriction at virtual boundary for intra prediction - Filters disabled across virtual boundary - Encoder-side control on MV search range | JVET-O0100 JVET-O1048 (VITEC) | Futurewei |
| CE2-1.d withdrawn | Virtual boundary-based method, variation of CE2-1.c - Normative reference sample restriction at virtual boundary for intra prediction - Filters disabled across virtual boundary - Treating virtual boundary as picture boundary for motion compensation | JVET-O0100 JVET-O1048 (VITEC) | Fujitsu |
| CE2-2.a withdrawn | Slice boundary-based method - Vertical slice boundary - Position accuracy: CTU width - Encoder-side control on MV search range | JVET-O0124 (Futurewei) |  |
| CE2-2.b withdrawn | Slice boundary-based method, variation of CE2-2.a - Treating slice boundary as picture boundary for motion compensation | JVET-O0124 (Futurewei) |  |
| CE2-3 | Wavefront-based method - MV restrictions and in-loop filtering disabled - Virtual boundary syntax is moved to slice header - Virtual boundary syntax signals the start address and the length of intra coded area on wavefront | JVET-O0976 JVET-O0979 (Nokia) | VITEC |

Detailed comparison of tested tools

Table 5-2 – Refreshing scheme

| Test # | Shape of refreshed area | Signalling of area | Precision of boundary position | Spatial direction of refresh |
| --- | --- | --- | --- | --- |
| CE2-1.a | Vertically striped | PPS | 8 luma samples | Left to right |
| CE2-1.b | Vertically striped | Slice | 8 luma samples | Left to right |
| CE2-1.c | Vertically striped | Slice | 8 luma samples | Left to right |
| CE2-3 | Diagonally striped | Virtual boundary | GDR block size dependent upon picture size and refresh cycle | Upper left to lower right |

Table 5-3 – Method for avoiding contamination of sample from dirty area to clean area

| Test # | In intra prediction | In inter prediction | In MV prediction | In in-loop filtering |
| --- | --- | --- | --- | --- |
| CE2-1.a | Encoder-side mode restriction | Encoder-side MV restriction | Constrained by limiting the reference frame buffer when recycling (encoder-side restriction) | Disabled at virtual boundary (PPS level) |
| CE2-1.b | Encoder-side mode restriction | Encoder-side MV restriction | Constrained by limiting the reference frame buffer when recycling (encoder-side restriction) | Disabled at virtual boundary (Slice level) |
| CE2-1.c | Avoid the use of reference samples across the virtual boundary (normative) | Encoder-side MV restriction | Constrained by limiting the reference frame buffer when recycling (encoder-side restriction) | Disabled at virtual boundary (Slice level) |
| CE2-3 | Not necessary due to its boundary shape. | Encoder-side MV restriction | TMVP is not used if a CU is inside the clean region. | Disabled at virtual boundary (Slice level) |

Table 5-4 – Other aspects

| Test # | Additional normative changes | Decoding unit |
| --- | --- | --- |
| CE2-1.a | Only on CE2-1.a.bis: Picture reconstruction with luma dependent chroma residual scaling process for chroma samples in LMCS – prevent use of pixels across the virtual boundaries | If DU operation is desired, each CTU column can be coded as one tile. One tile one slice. |
| CE2-1.b | Picture reconstruction with luma dependent chroma residual scaling process for chroma samples in LMCS – prevent use of pixels across the virtual boundaries | If DU operation is desired, each CTU column can be coded as one tile. One tile one slice. |
| CE2-1.c | Picture reconstruction with luma dependent chroma residual scaling process for chroma samples in LMCS – prevent use of pixels across the virtual boundaries | If DU operation is desired, each CTU column can be coded as one tile. One tile one slice. |
| CE2-3 | Signalling of the length of intra coded area on wavefront | If DU operation is desired, each CTU column can be coded as one tile. One tile one slice. |

Table 5-5 – Other aspects

| Test # | Encoding condition | Cross-checker’s comment |
| --- | --- | --- |
| CE2-1.a | Disabling of LMCS at PPS level. |  |
| CE2-1.b |  | For class F, HashME is disabled. |
| CE2-1.c |  |  |
| CE2-3 | ALF, LMCS and local dual tree are disabled. | ALF, LMCS and local dual tree are disabled. For class F, IBC, HashME and BDPCM are disabled |

Test results

BD-Rate and runtime

Table 6-1 – BD-rate and runtime

|  | Low-delay B Main10 – Over VTM-6.0 with CE2 condition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Tester |  |  |  |  | Cross-checker |  |  |  |  |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE2-1.a | Class-C 6.34% | Class-C 5.27% | Class-C 5.61% | Class-C 100% | Class-C 105% |  |  |  |  |  |
| CE2-1.a.bis | Class-C 5.88% | Class-C 6.98% | Class-C 7.58% | Class-C 102% | Class-C 104% | Class-C 5.88% | Class-C 6.98% | Class-C 7.58% |  |  |
| CE2-1.b | 6.54% | 8.86% | 11.14% | 107% | 102% | 6.54% | 8.86% | 11.14% |  |  |
| CE2-1.c | 5.92% | 8.46% | 10.6% | 108% | 102% |  |  |  |  |  |
| CE2-3 | Class-C 9.69% | Class-C 10.88% | Class-C 11.43% | Class-C 152% |  | Class-C 9.69% | Class-C 10.88% | Class-C 11.43% |  |  |

Bitrate fluctuation

Table 6-2 – Bitrate fluctuation

| Test # | Tester | Cross-checker |
| --- | --- | --- |
| CE2-1.a | Fully smooth with fluctuation only due to content | Fully smooth with fluctuation only due to content |
| CE2-1.b | Fully smooth with fluctuation only due to content | Fully smooth with fluctuation only due to content |
| CE2-1.c | Fully smooth with fluctuation only due to content |  |
| CE2-3 | Fully smooth with fluctuation only due to content | Fully smooth with fluctuation only due to content. |

Exact match at recovery point picture

Table 6-3 – Exact match at recovery point picture

| Test # | Tester | Cross-checker |
| --- | --- | --- |
| CE2-1.a |  |  |
| CE2-1.b | 1st GRA, matched for at least 75 of Class C D E and F sequences. Some very minor leaks can occur for less than 25% of the decoded sequences. However we strongly believe that the correction of these minor leaks would not impact the RD performances. Anyway, if it would impact the RD performances, it would be in favor of 2-1.b rather than 2-1.c, while we advocate for 2-1.c. | 1st GRA, matched for a majority of Class C D E and F sequences. Small leaks for: BQMall Qp37 in Class C RaceHorseC Qp22, 27, and 32 in Class C BQSquare Qp 27 in Class D BlowingBubble Qp22 in Class D RaceHorseD Qp 27 in Class D FourPeople Qp 22, 32 and 37 in Class E Johnny Qp 32 in Class E Kristin Qp 22, 27 and 37 in Class E SlideEditing Qp 27 in Class F |
| CE2-1.c | 1st GRA, matched for Class C D E and F |  |
| CE2-3 | 1st GRA, matched for a majority of Class C D E and F sequences. Small leaks for: BasketBallDrill QP 32 and 37 Class C BqMall QP 22 and 27 Class C FourPeople QP 22 Class E Johnny QP 37 Class E KristenandSara QP 27 and 32 Class E SlideShow QP 32 Class F | 1st GRA, matched for a majority of Class C D E and F sequences. Small leaks for: BasketBallDrill QP 32 and 37 Class C BqMall QP 22 and 27 Class C FourPeople QP 22 Class E Johnny QP 37 Class E KristenandSara QP 27 and 32 Class E SlideShow QP 32 Class F |

References

Documents at previous meetings

- B. Bross, J. Chen, S. Liu “Versatile Video Coding (Draft 6)”, Document of Joint Video Experts Team, JVET-O2001-vE, July 2019

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010, March 2019

- K. Kazui, J-M. Thiesse, Hendry, K. Kawamura, “Description of Core Experiment 2 (CE2): Gradual decoding refresh”, Document of Joint Video Experts Team, JVET-O2022, July 2019

- Reference software VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- D. Gommelet, J.-M. Thiesse, D. Nicholson “CE11: Gradual Random Access (GRA) using encoder and normative restrictions (Tests CE11-2.1.a, CE11-2.1.b and CE11-2.2)”, Document of Joint Video Experts Team, JVET-O0100, July 2019

- Hendry, Y-K. Wang, J. Chen, S. Hong, “CE11/AHG14: Test 3.1 - an approach for support of GRA/GDR”, Document of Joint Video Experts Team, JVET-O0124, July 2019

- L. Wang, M. M. Hannuksela, “CE11-related: Wavefront-based GRA and Related Syntax”, Document of Joint Video Experts Team, JVET-O0976, July 2019

- L. Wang, “CE11-related: Wavefront-based GRA Method”, Document of Joint Video Experts Team, JVET-O0979, July 2019

- D. Gommelet, J.-M. Thiesse, D. Nicholson “Non-CE11: Non-CIP normative Gradual Random Access”, Document of Joint Video Experts Team, JVET-O1048, July 2019

CE documents

- D. Gommelet, J-M. Thiesse, D. Nicholson, “CE2: Gradual Decoding Refresh (GDR) using encoder and normative restrictions (Tests 2.1.a, 2.1.b and 2.1.c)” Document of Joint Video Experts Team, JVET-P0193, October 2019

- L. Wang, S. Hong, K. Panusopone, “C CE2-3: Wavefront-Based GRA” Document of Joint Video Experts Team, JVET-P0112, October 2019