| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0023-v2 |
| --- | --- |

| Title: | CE3: Summary Report on Intra Prediction and Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Ling Li Alexey Filippov | Email: | geertv@qti.qualcomm.com aurali@tencent.com Alexey.Filippov@huawei.com |
| Source: | CE3 coordinators |  |  |

_____________________________

# Abstract

This is the summary report of the third Core Experiment (CE3) [1]. The goal of CE3 is to study intra prediction tools, including mode coding, for potential inclusion into the VVC standard.

A total of 6 tests are defined on the matrix intra prediction (MIP) topic:

- CE3-1: MIP down-sampling process (2 tests)

- CE3-2: On rounding shift of MIP (2 tests)

- CE3-3: Up-sampling with fixed order in MIP (2 tests)

This document summarizes the objective results (BD-rates, runtimes), cross-check reports, and related input contributions.

# Test conditions and guidelines

CTC conditions as agreed at the 15th JVET meeting (Gothenburg, SE) for SDR are used for the evaluation of objective performance (document JVET-N1010 remains valid). The anchor is the ‘VVC Test Model’ or VTM. Both the ‘All Intra’ and ‘Random Access’ test configurations are used for all sub-CEs and providing the results for each of these configurations is mandatory. The VTM version 6.0 software is used with ‘vtm’ configuration files to produce the VTM anchor. Tools are implemented into the VTM version 6.0 software. Proposals are primarily compared with respect to objective compression efficiency while complexity aspects are also assessed.

Source codes corresponding with the test descriptions contained in this document are uploaded to the following CE3 GitLab project:

https://vcgit.hhi.fraunhofer.de/jvet-o-ce3/VVCSoftware_VTM.git

# Cross-check reporting

Each test is covered by at least one cross-checker and the cross-check reports are included within the respective sub-test sections. Within CE3 it was agreed that cross-checkers answer the following questions:

- Did the cross-checker study the software? If so, did the cross-checker find any inconsistencies between the test description, the provided final software, and the VVC spec text changes?

- Do the cross-checked results match the proponent results for the test conditions? Were additional results cross-checked?

- Additional comments that may be helpful to the group?

This report summarizes the answers in tables reporting whether the objective test results match in one table column and all other comments in a second table column. The spreadsheets with cross-checked results are attached to this summary report.

The deadline for providing the cross-check reports to the CE coordinators was 21 September 2019. The following cross-check reports were received late or were missing spreadsheets with cross-checked results:

| Test # | Date received | Comment |
| --- | --- | --- |
| CE3-1 | 27 Sept. 2019 | Result excels missing for second cross-check of CE3-1.1 and CE3-1.2 |

# Overview of CE3 test description document changes

The following changes were made to the CE3 test description document [1] after the T1 deadline (2 August 2019) had passed. Besides changes to contact persons and assignment of cross-checkers, the T2 deadline for release of initial software was updated due to somewhat delayed release of VTM6.0.

# CE3-1 on ‘MIP down-sampling process’

## Defined tests [1]

| Test # | Short description | Doc. # |
| --- | --- | --- |
| 3-1.1 | MIP downsampling by averaging two samples from the original boundary | JVET-P0064 (Ericsson) |
| 3-1.2 | MIP downsampling by N-tap filtering N samples from the original boundary |  |

### CE3-1.1

MIP downsapling process derives reduced boundary bdryred for matrix multiplication by averaging two samples from the original boundary.

Figure 1. An example of bdryred derivation for a 32 × 16 block.

### CE3-1.2

In CE3-1.2, four different M-tap low-pass boundary filters are designed depending on the boundary size.

- For boundary size is equal to 8, M = 2, the filter coefficients to use are [1 1] / 2

- For boundary size is equal to 16, M = 3, the filter coefficients to use are [1 2 1] / 4

- For boundary size is equal to 32, M = 5, the filter coefficients to use are [1 2 2 2 1] / 8

- For boundary size is equal to 64, M = 9, the filter coefficients to use are [1 2 2 2 2 2 2 2 1] / 16

Figure 2. An example of bdryred derivation for a 32 × 8 block.

Figure 3. An example of bdryred derivation for a 64 × 16 block.

## CE3-1: Test results

|  | All Intra - Over VTM-6.0 |  |  |  |  | Random Access - Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3-1.1 | 0.02% | -0.02% | -0.01% | 100% | 100% | 0.01% | 0.07% | 0.04% | 100% | 100% |
| 3-1.2 | 0.03% | 0.01% | 0.03% | 100% | 100% | 0.01% | 0.02% | -0.02% | 100% | 100% |

## CE3-1: Cross-check reports

| Test# | Cross-checker | Results match (y/n) | Cross-checker comments |
| --- | --- | --- | --- |
| 3-1.1 | Jonathan Pfaff (HHI) | y | The crosschecker studied the software and did not find any inconsistencies between the description and the software. The results match. |
| 3-1.2 |  | y |  |
| 3-1.1 | Alexey Filippov (Huawei) | y | The results provided by the proponents match the obtained results. There are no significant inconsistencies with the CE3 description. |
| 3-1.2 | Vasily Rufitskiy (Huawei) | y | The results provided by the proponents match the obtained results. There are no significant inconsistencies with the CE3 description. |

## CE3-1: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-P0401 | 3-1 | CE3-related: MIP downsampling process with re-trained MIP matrix |

# CE3-2 on ‘Rounding shift of MIP’

## Defined tests [1]

| Test # | Short description | Doc. # |
| --- | --- | --- |
| 3-2.1 | The table of weight shift (sW) of MIP is removed. The variable (sW) is fixed value. It doesn’t depend on both MipSizeId and modeId. | JVET-P0056 (Sony, Xidian Univ.) |
| 3-2.2 | The table of weight shift (sW) of MIP is simplified. The variable (sW) depends on only MipSizeId. It doesn’t depend on modeId. |  |

### CE3-2.1

In this method, a variable (sW), the number of shifts to compute a variable predMip is fixed to 5. The equations are changed as below. Also, the weighting parameters (mWeight) and the offset factor (fO) are modified because the number of shifts was changed from the original design.

oW = ( 1 << 4( sW − 1 ) ) −( (fO * (i = 0inSize - 1p i )+1) >> 1)

predMip[ x ][ y ] = ( ( ( i = 0inSize - 1mWeight i  y*incH*predC+x*incW *p i  ) + 
	oW ) >> 5sW ) + pTemp[ 0 ]

Memory storage reduction: 102 bits (= 3 bits * 34 elements)

### CE3-2.2

In this method, the variable (sW) depends on MipSizeId. the sW is set to 6 when MipSizeId is equal to 1. Otherwise, it is set to 5. Thus, the equations are changed as below.

oW = ( 1 << ((MipSizeId==1)? 5:4) ) −( (fO * (i = 0inSize - 1p i )+1) >> 1)

predMip[ x ][ y ] = ( ( ( i = 0inSize - 1mWeight i  y*incH*predC+x*incW *p i  ) + 
	oW ) >>  ((MipSizeId==1)? 6:5)sW ) + pTemp[ 0 ]

Memory storage reduction: 102 bits (=3 bits * 34 elements); additional conditions

## CE3-2: Test results

|  | Random Access - Over VTM-6.0 |  |  |  |  | Low Delay B - Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3-2.1 | 0.03% | -0.01% | 0.02% | 100% | 100% | 0.02% | 0.05% | 0.01% | 101% | 99% |
| 3-2.2 | 0.03% | 0.01% | 0.00% | 100% | 100% | 0.02% | 0.08% | 0.01% | 100% | 100% |

## CE3-2: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 3-2.1 | Jonathan Pfaff (HHI) | y | The crosschecker studied the software and did not find any inconsistencies between the the description and the software. The results match. As an additional comment, the crosschecker would like to add that the unified shift 5 was obtained by dividing all integral weights from the spec by two if the shift of the MIP mode was 6 and by four if the shift of the MIP mode was 7. |
| 3-2.2 |  | y |  |

## CE3-2: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |

# CE3-3 on ‘Up-sampling with a fixed order in MIP’

## Defined tests [1]

| Test # | Short description | Doc. # |
| --- | --- | --- |
| 3-3.1 | Up-sampling is performed firstly in the vertical direction and secondly in the horizontal direction | JVET-P0054 (Bytedance) |
| 3-3.2 | Up-sampling is performed firstly in the horizontal direction and secondly in the vertical direction |  |

## CE3-3: Test results

| Test # | All Intra - Over VTM-6.0 |  |  |  |  | Random Access - Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3-3.1 | 0.00% | 0.02% | -0.01% | 99% | 100% | -0.01% | 0.04% | 0.01% | 100% | 100% |
| 3-3.2 | 0.00% | -0.01% | 0.01% | 100% | 100% | 0.00% | 0.11% | 0.02% | 100% | 100% |

## CE3-3: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 3-3.1 | Chris Rosewarne (Canon) | y | Matching BD-rate result and negligible runtime impact. As per CE3 test conditions, AI and RA were tested. The source code for each subtest was studied and found to match the proposal description. VVC spec changes in the related contribution (JVET-P0054) were also studied and found to be matching. |
| 3-3.2 |  | y |  |

## CE3-3: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |

# References

- G. Van der Auwera, L. Li, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” 15th JVET Meeting, Gothenburg, SE, July 2019, JVET-O2023