| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0058-v2 |
| --- | --- |

| Title: | CE8-2.1: Transform Skip for Chroma with limiting maximum number of context-coded bin in TS residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This document reports results of CE8-2.1, which introduces transform skip for chroma and applies TS residual coding to chroma-transform-skipped block with limiting the maximum number of context-coded bins to 1.75 per coefficient.

Simulations results show that:

For common QP condition,

- When DualITree turned off, average BD-rate differences (Y, U, V) are (0.34%, -1.44%, -1.32%) for AI, (-0.16%, -2.54%, -2.28%) for RA, and (-0.07%, -2.51%, -2.46%) for LB, respectively.

- When DualITree turned on, average BD-rate differences (Y, U, V) are (-0.21%, -0.64%, -0.82%) for AI, (-0.10%, -1.54%, -1.52%) for RA, and (0.03%, -1.55%, -1.91%) for LB, respectively.

For low QP condition (QP=2, 7, 12, 17),

- When DualITree turned off, average BD-rate differences (Y, U, V) are (-2.15%, -4.02%, -4.04%) for AI, (-2.46%, -7.57%, -7.82%) for RA, and (-2.59%, -5.85%, -5.68%) for LB, respectively.

- When DualITree turned on, average BD-rate differences (Y, U, V) are (0.02%, -3.05%, -2.98%) for AI, (-1.17%, -7.03%, -7.24%) for RA, and (-1.90%, -5.92%, -5.85%) for LB, respectively.

In v2, high-level tool flag for chroma transform skip is additionally proposed as option, and additional results for YUV420/ClassF&ClassSCC are provided.

Additional results show that:

For common QP condition,

- For ClassF, average BD-rate differences (Y, U, V) are (0.18%, 0.34%, -0.10%) for AI, (0.24%, -0.06%, -0.10%) for RA, and (0.16%, -0.01%, 0.57%) for LB, respectively.

- For ClassSCC, average BD-rate differences (Y, U, V) are (0.03%, -1.57%, -1.94%) for AI, (0.12%, -1.25%, -1.51%) for RA, and (0.12%, -0.79%, -1.10%) for LB, respectively.

For low QP condition (QP=2, 7, 12, 17),

- For ClassF, average BD-rate differences (Y, U, V) are (0.05%, -1.43%, -1.19%) for AI, (0.16%, -3.11%, -2.96%) for RA, and (-040%, -2.36%, -1.96%) for LB, respectively.

- For ClassSCC, average BD-rate differences (Y, U, V) are (-3.41%, -5.67%, -5.95%) for AI, (-2.00%, -5.17%, -5.22%) for RA, and (-1.56%, -3.72%, -3.79%) for LB, respectively.

Introduction

In the last meeting, chroma transform skip was studied in CE8 [1]. It showed additional gains for SCC YUV420/YUV444 sequences at low QP condition. However, there was a concern on the maximum number of context coded bins in the TS residual coding.

From the meeting notes JVET-O_Notes_dB, “Concern is raised that the maximum number of context coded bins is larger in the TS residual entropy coding method, such that the overall number of context coded bins would further increase if it was extended to chroma. It would be desirable to have the same limit of maximum number of context coded bins (1.75 per coefficient) for both the conventional and the TS residual method.”

Therefore, chroma TS with limiting the maximum number of context-coded bins in TS residual coding to 1.75 per coefficient was further studied in CE8.

Note that JVET-O1169 [2] proposed limiting the number of context coded bins in TS residual coding to 1.75 per coefficient.

Proposal

CE8-2.1 applies following modifications into VVC WD/VTM-6.0:

- Signal transform type (transform_skip_flag and mts_idx) per component with minimum changes:

  - Additional single context model for chroma TS

  - No MTS for chroma

  - No changes of both context model and binarization for MTS signaling

- Interaction with TS residual coding

  - TS residual coding is straightforwardly used for chroma TS as well as luma TS case.

  - Limiting the maximum number of context-coded bin in TS residual coding to 1.75 per coefficient, which is the same number as in regular residual coding.

Summary of changes relative to JVET-O2001-vE are as below. Detail changes are found in JVET-P0058_DraftText.docx.

transform_skip_flag[ x0 ][ y0 ][ cIdx ] specifies whether a transform is applied to the luma associated transform block with cIdx or not. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. transform_skip_flag[ x0 ][ y0 ] equal to 1 specifies that no transform is applied to the luma associated transform block. transform_skip_flag[ x0 ][ y0 ] equal to 0 specifies that the decision whether transform is applied to the luma associated transform block with cIdx or not depends on other syntax elements.

When transform_skip_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If BdcpmFlag[ x0 ][ x0 ] is equal to 1 and cIdx is equal to 0, transform_skip_flag[ x0 ][ y0 ] is inferred to be equal to 1.

- Otherwise if (BdcpmFlag[ x0 ][ x0 ] is equal to 0 and cIdx is equal to 0), transform_skip_flag[ x0 ][ y0 ][ cIdx ] is inferred to be equal to 0.

- Otherwise ( cIdx is not equal to 0), transform_skip_flag[ x0 ][ y0 ][ cIdx ] is inferred to be equal to 0.

tu_mts_idx[ x0 ][ y0 ][ cIdx ] specifies which transform kernels are applied to the residual samples along the horizontal and vertical direction of the associated luma transform block with cIdx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture.

When tu_mts_idx[ x0 ][ y0 ][ cIdx ] is not present, it is inferred to be equal to 0.

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize ) |  |
| transform_skip_flag[ x0 ][ y0 ] |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) && ( !transform_skip_flag[ x0 ][ y0 ] ) ) |  |
| tu_mts_idx[ x0 ][ y0 ] |  |
| } |  |
| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| transform_mode( x0, y0, tbWidth, tbHeight, 0) |  |
| if(!transform_skip_flag[ x0 ][ y0 ][ 0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ){ |  |
| transform_mode( x0, y0, wC, wH, 1) |  |
| if( !transform_skip_flag[ x0 ][ y0 ][ 1 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC), Log2( hC), 1) |  |
| } |  |
| if( tu_cbf_cr[ x0 ][ y0 ] && !( tu_cbf_cb[ x0 ][ y0 ] && tu_joint_cbcr_residual_flag[ x0 ][ y0 ] )) { |  |
| transform_mode( x0, y0, wC, hC, 2) |  |
| if( !transfrom_skip_flag[ x0 ][ y0 ][ 2 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC), 2) |  |
| } |  |
| } |  |

| transform_mode( x0, y0, tbWidth, tbHeight, cIdx ) { | Descriptor |
| --- | --- |
| if (sps_transform_skip_enabled_flag && (tbWidth <= MaxTsSize) && (tbHeight <= MaxTsSize) && !(BdpcmFlag[ x0 ][ y0 ] && cIdx==0) && (IntraSubPartitionsSplit[ x0 ][ y0 ] == ISP_NO_SPLIT ¦¦ cIdx!=0) && ( !cu_sbt_flag ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if ( (cIdx==0) && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) && ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) && !trasnfrom_skip_flag[x0][y0][cIdx] ) |  |
| mts_idx [ x0 ][ y0 ][ cIdx ] | ae(v) |
| } |  |

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| log2SbSize = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| numSbCoeff = 1 << ( log2SbSize << 1 ) |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − 2 * log2SbSize ) ) − 1 |  |
| inferSbCbf = 1 |  |
| MaxCcbs = 2 * ( ( 1 << log2TbWidth ) * ( 1<< log2TbHeight ) ) * 7 >> 2 |  |
| … |  |
| } |  |

Option: High-level tool flag for Chroma Transform Skip

In order to provide a easy way to turn the chroma TS on/off, it is proposed to signal high-level tool flag for chroma transform skip (sps_transform_skip_chroma_enabled_flag) in SPS if sps_transform_skip_enabled_flag is equal to 1.

Suggested syntax changes relative to DraftText in JVET-P0058 are as below:

Semantics

sps_transform_skip_enabled_flag equa to 1 specifies that transform_skip_flag for luma (cIdx==0) may be present in the transform unit syntax. sps_transform_skip_enabled_flag equal to 0 specifies that transform_skip_flag for luma is not present in the transform unit syntax

sps_transform_skip_chroma_enabled_flag equa to 1 specifies that transform_skip_flag for chroma (cIdx!=0) may be present in the transform unit syntax. sps_transform_skip_chroma_enabled_flag equal to 0 specifies that transform_skip_flag for chroma is not present in the transform unit syntax.

When not present, the value of sps_transform_skip_chroma_enabled_flag is inferred to be equal to 0.

| seq_parameter_set_rbsp(){ |  |
| --- | --- |
| … |  |
| if( sps_transform_skip_enabled_flag ) { |  |
| sps_transform_skip_chroma_enabled_flag | u(1) |
| sps_bdpcm_enabled_flag | u(1) |
| } |  |

| transform_mode( x0, y0, tbWidth, tbHeight, cIdx ) { | Descriptor |
| --- | --- |
| if (sps_transform_skip_enabled_flag (( cIdx==0 && sps_transform_skip_enabled_flag) ¦¦ (cIdx!=0 && sps_transform_skip_chroma_enabled_flag)) && (tbWidth <= MaxTsSize) && (tbHeight <= MaxTsSize) && !(BdpcmFlag[ x0 ][ y0 ] && cIdx==0) && (IntraSubPartitionsSplit[ x0 ][ y0 ] == ISP_NO_SPLIT ¦¦ cIdx!=0) && ( !cu_sbt_flag ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if ( (cIdx==0) && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) && ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) && !trasnfrom_skip_flag[x0][y0][cIdx] ) |  |
| mts_idx [ x0 ][ y0 ][ cIdx ] | ae(v) |
| } |  |

Simulation Results

The proposed chroma TS has been implemented on top of VTM-6.0 and following simulations were conducted under CE8 test condition [3]:

- Test1:	VTM-6.0 DualITreeOff vs CE8-2.1 DualITreeOff under common QP condition

- Test2: VTM-6.0 DualITreeOff vs CE8-2.1 DualITreeOff under low QP condition (QP=2,7,12,17)

- Test3: VTM-6.0 DualITreeOn vs CE8-2.1 DualITreeOn under common QP condition

- Test4: VTM-6.0 DualITreeOn vs CE8-2.1 DualITreeOn under low QP condition (QP=2,7,12,17)

Table 3-1 to 3-4 summarizes results of each Test1, Test2, Test3 and Test4, respectively.

Note that palette bugfixes for ticket#512 and ticket#521 were applied to both anchor and test for simulations at QP=2/RA&LB.

Table 3-1: Results of CE8-2.1 DualITreeOff under common QP condition (Test1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.46% | -1.58% | -1.69% | 115% | 100% |
| TGM 720p | 0.82% | -1.36% | -0.73% | 111% | 101% |
| Animation | 0.23% | -0.36% | 0.06% | 109% | 100% |
| Mixed content | -0.15% | -3.69% | -4.03% | 111% | 100% |
| Camera-Captured content | 0.02% | 0.45% | 0.22% | 108% | 100% |
| Overall | 0.34% | -1.44% | -1.32% | 112% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.36% | -3.13% | -3.37% | 111% | 100% |
| TGM 720p | 0.22% | -2.38% | -1.51% | 107% | 100% |
| Animation | 0.20% | -1.21% | -0.43% | 107% | 100% |
| Mixed content | -0.91% | -5.18% | -5.38% | 106% | 100% |
| Camera-Captured content | 0.04% | 0.30% | 0.24% | 104% | 100% |
| Overall | -0.16% | -2.54% | -2.28% | 107% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.14% | -3.01% | -3.17% | 108% | 100% |
| TGM 720p | -0.01% | -2.80% | -2.00% | 105% | 100% |
| Animation | 0.26% | -1.22% | -0.81% | 105% | 99% |
| Mixed content | -0.81% | -4.39% | -5.56% | 103% | 101% |
| Camera-Captured content | 0.03% | -0.09% | 0.19% | 103% | 100% |
| Overall | -0.07% | -2.51% | -2.46% | 105% | 100% |

Table 3-2: Results of CE8-2.1 DualITreeOff under low QP condition (Test2)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -4.48% | -5.32% | -5.71% | 120% | 101% |
| TGM 720p | -1.45% | -3.16% | -3.07% | 115% | 101% |
| Animation | -0.20% | -3.12% | -2.01% | 111% | 101% |
| Mixed content | -3.38% | -6.97% | -7.83% | 117% | 99% |
| Camera-Captured content | 0.03% | -0.02% | -0.01% | 107% | 99% |
| Overall | -2.15% | -4.02% | -4.04% | 115% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -6.03% | -8.10% | -8.63% | 114% | 98% |
| TGM 720p | -1.15% | -6.29% | -6.77% | 109% | 99% |
| Animation | -2.47% | -8.32% | -7.30% | 110% | 98% |
| Mixed content | -1.07% | -12.81% | -13.63% | 108% | 94% |
| Camera-Captured content | -0.01% | -0.10% | -0.39% | 105% | 99% |
| Overall | -2.46% | -7.57% | -7.82% | 110% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -5.32% | -7.58% | -7.80% | 109% | 98% |
| TGM 720p | -0.99% | -4.40% | -3.72% | 107% | 98% |
| Animation | -3.50% | -6.65% | -6.04% | 108% | 100% |
| Mixed content | -1.89% | -8.54% | -8.70% | 106% | 101% |
| Camera-Captured content | -0.04% | -0.05% | -0.25% | 103% | 100% |
| Overall | -2.59% | -5.85% | -5.68% | 107% | 99% |

Table 3-3: Results of CE8-2.1 DualITreeOn under common QP condition (Test3)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.74% | -0.75% | -1.51% | 109% | 100% |
| TGM 720p | 0.24% | -0.68% | -0.54% | 105% | 99% |
| Animation | -0.11% | -0.79% | -0.32% | 105% | 100% |
| Mixed content | -0.37% | -0.88% | -1.46% | 105% | 100% |
| Camera-Captured content | 0.08% | 0.26% | 0.19% | 107% | 102% |
| Overall | -0.21% | -0.64% | -0.82% | 106% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.45% | -2.36% | -2.53% | 110% | 102% |
| TGM 720p | 0.07% | -1.25% | -1.29% | 105% | 100% |
| Animation | 0.17% | -1.46% | -0.64% | 105% | 99% |
| Mixed content | -0.20% | -2.14% | -2.48% | 104% | 99% |
| Camera-Captured content | 0.00% | 0.25% | 0.15% | 103% | 101% |
| Overall | -0.10% | -1.54% | -1.52% | 106% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.13% | -3.03% | -3.00% | 107% | 98% |
| TGM 720p | 0.20% | -1.22% | -2.02% | 104% | 100% |
| Animation | 0.29% | -1.29% | -0.89% | 102% | 95% |
| Mixed content | -0.23% | -1.26% | -2.60% | 102% | 91% |
| Camera-Captured content | 0.03% | -0.09% | -0.02% | 102% | 100% |
| Overall | 0.03% | -1.55% | -1.91% | 104% | 97% |

Table 3-4: Results of CE8-2.1 DualITreeOn under low QP condition (Test4)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.12% | -3.00% | -3.10% | 115% | 103% |
| TGM 720p | -0.07% | -2.50% | -2.49% | 112% | 102% |
| Animation | -0.24% | -3.68% | -2.48% | 108% | 101% |
| Mixed content | 0.59% | -5.25% | -5.98% | 112% | 102% |
| Camera-Captured content | 0.03% | -0.04% | 0.00% | 106% | 101% |
| Overall | 0.02% | -3.05% | -2.98% | 111% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -4.07% | -6.57% | -6.85% | 112% | 97% |
| TGM 720p | 0.38% | -4.95% | -5.21% | 107% | 100% |
| Animation | -2.39% | -8.29% | -7.37% | 110% | 98% |
| Mixed content | 1.06% | -13.79% | -14.87% | 108% | 97% |
| Camera-Captured content | -0.04% | -0.08% | -0.47% | 105% | 101% |
| Overall | -1.17% | -7.03% | -7.24% | 109% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -4.21% | -6.50% | -6.65% | 107% | 98% |
| TGM 720p | -0.65% | -4.68% | -3.96% | 106% | 101% |
| Animation | -2.97% | -6.77% | -6.26% | 110% | 100% |
| Mixed content | -0.65% | -9.86% | -10.64% | 105% | 96% |
| Camera-Captured content | -0.03% | -0.06% | -0.21% | 102% | 99% |
| Overall | -1.90% | -5.92% | -5.85% | 106% | 99% |

Additional simulation Results

Following simulations for ClassF and ClassSCC were conducted under common test condition [4]:

- Test5:	VTM-6.0 vs CE8-2.1 under common QP condition

- Test6: VTM-6.0 vs CE8-2.1 under low QP condition (QP=2,7,12,17)

Table 4-1 to 4-2 summarizes results of each Test5, and Test6, respectively.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F | 0.18% | 0.34% | -0.10% | 103% | 100% |
| Class SCC | 0.03% | -1.57% | -1.94% | 104% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F | 0.24% | -0.06% | -0.10% | 104% | 101% |
| Class SCC | 0.12% | -1.25% | -1.51% | 105% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F | 0.16% | -0.01% | 0.57% | 104% | 99% |
| Class SCC | 0.12% | -0.79% | -1.10% | 110% | 103% |

Table 4-2: Results of CE8-2.1 under low QP condition (Test6)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F | 0.05% | -1.43% | -1.19% | 106% | 100% |
| Class SCC | -3.41% | -5.67% | -5.95% | 112% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F | 0.16% | -3.11% | -2.96% | 108% | 101% |
| Class SCC | -2.00% | -5.17% | -5.22% | 109% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F | -0.40% | -2.36% | -1.96% | 110% | 103% |
| Class SCC | -1.56% | -3.72% | -3.79% | 111% | 100% |

Conclusion

It is recommended to adopt proposed changes into the next VTM/VVC WD and enable chroma transform skip for YUV444 sequences.

Reference

[1] T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki, “CE8-3.2: Chroma Transform Skip,” JVET-O0081, July 2019

[2] M. Coban, M. Karczewicz, “CE8(7)-related: Alignment of number of context coded bins per coefficients for TS residual coding and transform coefficient coding,” JVET-O1169, July. 2019

[3] X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-O2028, July 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).