| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0064-v1 |
| --- | --- |

| Title: | CE3-1: MIP downsampling process – align positions of reduced boundary samples with positions of reduced prediction samples |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Z. Zhang, K. Andersson, D. Saffar, R. Sjöberg, J. Ström, P. Wennersten, R. Yu | Tel: Email: | +46761499428 zhi.a.zhang@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In the current VVC, the samples of the MIP reduced boundary (MIP INPUT) are derived from the average of N boundary samples. This gives filtered boundary samples centered in-between the two boundary samples in the middle. That position is closer to the first of the two samples of reduced prediction (MIP OUTPUT). This means that the sample of MIP INPUT is biased towards the first, third, fifth, etc., sample of MIP OUTPUT.

This contribution reports the test results of two tests CE3-1.1 MIP downsampling by averaging two samples from the original boundary (the same as JVET-O0161 aspect two) and CE3-1.2 MIP downsampling by filtering M samples from the original boundary. In both two proposed methods the positions of the reduced boundary samples align with the positions of the reduced prediction samples.

CE3-1.1 has a luma BDR impact for AI/RA of 0.02%/0.01% compared to VTM-6.0 and decoding time impact for AI/RA of 100%/100% compared to VTM-6.0.

CE3-1.2 has a luma BDR impact for AI/RA of 0.03%/0.01% compared to VTM-6.0 and decoding time impact for AI/RA of 100%/100 compared to VTM-6.0.

# Introduction

In the current VVC, the MIP downsampling process derives reduced boundary (MIP INPUT) by averaging of N samples from left and top boundary. Depending on the dimension of the current block, N can be equal to 2, 4, 8 or 16.

One problem of the current MIP downsampling process is the mis-alignment between reduced boundary (MIP INPUT) and reduced prediction (MIP OUTPUT). The samples of MIP INPUT are derived from averaging of N boundary samples. This gives filtered boundary samples centered in-between the two boundary samples in the middle. That position is closer to the first of two samples of MIP OUTPUT. This means that the sample of MIP INPUT is biased towards first, third, fifth, etc., sample of MIP OUTPUT. Figure 1 shows an example of the mis-alignment between MIP INPUT and MIP OUTPUT.

Fig 1. An example of mis-alignment between MIP INPUT and MIP OUTPUT for a 32 × 32 block.

# Proposed method

In CE3-1, we propose two method for MIP downsampling process to derive MIP reduced boundary.

## CE3-1.1 MIP downsampling by averaging two samples from the original boundary (same as JVET-O0161 aspect two)

MIP downsapling process derives reduced boundary bdryred for matrix multiplication by averaging two samples from the original boundary. The samples to be used for averaging are aligned with samples of the reduced prediction predred in linear interpolation direction. Figure 2 is an example of bdryred derivation for a 32 × 16 block.

Fig 2. An example of bdryred derivation for a 32 × 16 block.

Proposed VVC draft text change for CE3-1.1:

8.4.5.2.2 MIP boundary sample downsampling process

Inputs to this process are:

- a variable nTbS specifying the transform block size,

- reference samples refS[ x ] with x = 0..nTbS − 1,

- a variable boundarySize specifying the downsampled boundary size.

Outputs of this process are the reduced boundary samples redS[ x ] with x = 0..boundarySize − 1 and upsampling boundary samples upsBdryS[ x ] with x = 0..upsBdrySize − 1.

The reduced boundary samples redS[ x ] with x = 0..boundarySize − 1 are derived as follows:

- If boundarySize is less than nTbs, the following applies:

bDwn = nTbs / boundarySize		(8-71)

redS[ x ] = ( i = 0bDwn - 1refS x*bDwn+i  + ( 1 << ( Log2( bDwn ) − 1 ) )) >> Log2( bDwn )	(8-72)

redS[ x ] = ( refS[ x * bDwn + (bDwn >> 1) −  1 ] + refS[ x * bDwn + bDwn −  1 ] + 1 ) >> 1	(8-72)

- Otherwise (boundarySize is equal to nTbs), redS[ x ] is set equal to refS[ x ].

## CE3-1.2 MIP downsampling by filtering M samples from the original boundary

In CE3-1.2, four different M-tap low-pass boundary filters are designed depending on the boundary size.

- For boundary size is equal to 8, M = 2, the filter coefficients to use are [1 1] / 2

- For boundary size is equal to 16, M = 3, the filter coefficients to use are [1 2 1] / 4

- For boundary size is equal to 32, M = 5, the filter coefficients to use are [1 2 2 2 1] / 8

- For boundary size is equal to 64, M = 9, the filter coefficients to use are [1 2 2 2 2 2 2 2 1] / 16

The M samples to be used in the MIP downsampling process are denoted as P0, P1…PM-1, where, the positions of P0 and PM-1 are aligned with the sample positions of the MIP OUTPUT in linear interpolation direction and P1 … PM-2 are samples positioned between P0 and PM-1. Figure 3 shows an example of the M-tap downsampling filter for a 32 × 8 block.

Fig 3. An example of bdryred derivation for a 32 × 8 block.

Figure 4 shows an example of the M-tap downsampling filter for a 64 × 16 block.

Fig 4. An example of bdryred derivation for a 64 × 16 block.

Proposed VVC draft text change for CE3-1.2:

8.4.5.2.2 MIP boundary sample downsampling process

Inputs to this process are:

- a variable nTbS specifying the transform block size,

- reference samples refS[ x ] with x = 0..nTbS − 1,

- a variable boundarySize specifying the downsampled boundary size.

Outputs of this process are the reduced boundary samples redS[ x ] with x = 0..boundarySize − 1 and upsampling boundary samples upsBdryS[ x ] with x = 0..upsBdrySize − 1.

The reduced boundary samples redS[ x ] with x = 0..boundarySize − 1 are derived as follows:

- If boundarySize is less than nTbs, the following applies:

bDwn = nTbs / boundarySize		(8-71)

redS[ x ] = ( i = 0bDwn - 1refS x*bDwn+i  + ( 1 << ( Log2( bDwn ) − 1 ) )) >> Log2( bDwn )	(8-72)

redS[ x ] = ( refS[ x * bDwn + (bDwn >> 1) −  1 ] + refS[ x * bDwn + bDwn −  1 ] + i = 1bDwn - 22* refS x*bDwn+i   +  ( 1 << ( Log2( bDwn ) − 1 ) ) ) >> Log2( bDwn )	(8-72)

- Otherwise (boundarySize is equal to nTbs), redS[ x ] is set equal to refS[ x ].

# Simulation results

## Results when CE3-1.1 downsampling method is applied

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,04% | 0,02% | 0,02% | 100% | 99% |
| Class A2 | 0,03% | 0,05% | -0,01% | 99% | 100% |
| Class B | 0,01% | -0,01% | -0,07% | 101% | 102% |
| Class C | 0,02% | -0,09% | 0,05% | 101% | 100% |
| Class E | 0,02% | -0,06% | -0,02% | 100% | 101% |
| Overall | 0,02% | -0,02% | -0,01% | 100% | 100% |
| Class D | 0,02% | -0,04% | -0,05% | 101% | 99% |
| Class F | -0,02% | -0,08% | -0,11% | 100% | 98% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,04% | -0,04% | 0,12% | 101% | 99% |
| Class A2 | 0,03% | 0,09% | 0,11% | 100% | 99% |
| Class B | -0,01% | 0,05% | -0,04% | 100% | 100% |
| Class C | 0,01% | 0,17% | 0,03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0,01% | 0,07% | 0,04% | 100% | 100% |
| Class D | -0,05% | -0,49% | 0,09% | 100% | 101% |
| Class F | -0,02% | -0,12% | -0,03% | 100% | 101% |

## Results when CE3-1.2 downsampling method is applied

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,05% | 0,09% | 0,04% | 100% | 102% |
| Class A2 | 0,04% | 0,09% | 0,01% | 100% | 101% |
| Class B | 0,02% | -0,02% | -0,01% | 101% | 100% |
| Class C | 0,02% | -0,09% | 0,09% | 100% | 99% |
| Class E | 0,01% | 0,01% | 0,01% | 100% | 100% |
| Overall | 0,03% | 0,01% | 0,03% | 100% | 100% |
| Class D | -0,01% | 0,14% | -0,03% | 99% | 101% |
| Class F | 0,00% | 0,01% | 0,04% | 100% | 102% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,02% | -0,15% | 0,01% | 100% | 99% |
| Class A2 | 0,02% | 0,10% | -0,05% | 100% | 99% |
| Class B | 0,00% | 0,09% | 0,05% | 100% | 102% |
| Class C | 0,00% | 0,00% | -0,10% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0,01% | 0,02% | -0,02% | 100% | 100% |
| Class D | -0,03% | -0,29% | 0,01% | 100% | 100% |
| Class F | -0,01% | -0,02% | -0,05% | 100% | 99% |

# Conclusion

CE3-1.1 aligns the positions of the reduced boundary samples with the positions of the reduced prediction samples and simplifies the MIP downsampling process. The downsampling process derives one MIP INPUT sample by averaging 2 boundary samples compared to the current VVC which derives one MIP INPUT sample by averaging up to 16 boundary samples. The coding efficiency impact is AI (0.02%) and RA (0.01%).

Since the proposed method uses a new downsampling process to derive reduced boundary, we believe the BDR penalty due to inconsistent downsampling process between matrix training and VTM simulation.

# References

[1]. Versatile Video Coding (Draft 6)

[2]. Non-CE3: Simplification on MIP boundary sample downsampling process

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).