| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0073_v2 |
| --- | --- |

| Title: | CE5-3.1 Combination of bilateral filter and SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jacob Ström, Per Wennersten, Jack Enhorn, Du Liu, Kenneth Andersson, Lukasz Litwic, Davood Saffar, Christopher Hollmann, Ruoyang Yu, Rickard Sjöberg Torshamnsgatan 23 164 83 Stockholm Sweden | Email: | {jacob.strom, per.wennersten, jack.enhorn, du.liu, kenneth.r.andersson, lukasz.litwic, davood.saffar, christopher.hollman, ruoyang.yu, rickard.sjoberg}@ ericsson.com |
| Source: | Ericsson Research |  |  |

_____________________________

# Abstract

This document presents the results of the core experiment CE5-3.1 on the bilateral filter in combination with SAO. Test1 covers the case when SAO is turned on. Test2 covers the case when SAO is turned off. Test3 covers the performance when RDO optimization of the bilateral filter is disabled in the encoder. Test4 covers the case when the filter is implemented as a post filter. All tests 1 through 4 are also tested with ALF turned off. Reported BDR figures for test1 are -0.40% (AI), -0.44% (RA), -0.32% (LD), -0.39% (LDP), encoder run times are reported as 106% (AI), 103% (RA), 102% (LD), 103% (LDP) and decoder run times are reported as 104% (AI), 102% (RA), 102% (LD), 102% (LDP). Reported BDR figures for test2 are -0.41% (AI), -0.46% (RA), -0.30% (LD), -0.41% (LDP), encoder run times are reported as 109% (AI), 103% (RA), 104% (LDP) and decoder run times are reported as 107% (AI), 102% (RA), 105% (LD), 102% (LDP). Reported BDR figures for test3 are -0.03% (AI), -0.04% (RA), -0.07% (LD), -0.09% (LDP), encoder run times are reported as 101% (AI), 100% (RA), 103% (LD), 101% (LDP) and decoder run times are reported as 105% (AI), 102% (RA), 105% (LD) 101% (LDP). Reported BDR figures for test4 are 0.60% (AI), 0.23% (RA), 0.23% (LD), 0.28% (LDP), and encoder run times are equivalent to anchor (since it is a post filter).

# Introduction

At the meeting in Gothenburg, a loop-filter based bilateral filter was proposed in contribution JVET-O0548, and it was decided that it should be tested in a CE:

“To be tested with similar conditions, i.e. also as potential replacement of SAO, and also without testing the filter in RDO stage, and also tested as a post filter.”

In CE5-3, this corresponds to test1 (similar conditions as JVET-O0548), test2 (replacing SAO), test3 (avoiding filtering in the RDO) and test4 (as a post filter).

During the presentation, one of the major criticisms from the floor was the fact that the filter could get losses unless included in the RDO stage. Another point that came up regarded the padding at the image border, where it was suggested that a simple extension would be preferable to the mirroring done in JVET-O0548. Therefore, in this contribution, we follow JVET-O0548 with three differences. The first is to be able to signal half-strength of the filter. This improves BD-rate performance for the case when the filter is not part of the RDO and is thus used in test 3. Another difference is that it is possible to change the way the QP selects the filter tables. Instead of using QP, the filter now uses QP+bilateral_filter_qp_offset. This functionality is not used in the CE-test, bilateral_filter_qp_offset=0 for all tests. The third difference is that the padding at the image border now is a simple extension, as suggested from the floor.

# Description of method

The proposed filter is carried out in the sample adaptive offset (SAO) loop-filter stage, as shown in Figure 1. Both the proposed bilateral filter (BIF) and SAO are using samples from deblocking as input. Each filter creates an offset, and these are added to the input sample and then clipped, before proceeding to ALF.

Fig. 1: Both the proposed filter (BIF) and SAO uses samples from the deblocking stage as input. Both create an offset, and these are added to the input sample and clipped.

In detail, the output sample IOUT is obtained as

IOUT=clip3IC+ΔIBIF+ΔISAO,                 (Eq. 1)

where IC is the input sample from deblocking, ΔIBIF is the offset from the bilateral filter and ΔISAO is the offset from SAO.

## Calculating the bilateral filter offset

The offset ΔIBIF for a bilateral filter is calculated using the surrounding 8-neighbors. As is shown in Figure 2, this means that the same sample values are used as for SAO.

Figure 2: The bilateral filter and the SAO filters access the same 3×3 pixels

At the picture border, where samples are unavailable, the bilateral filter uses extension (sample repetition) to fill in unavailable samples. For virtual boundaries, the behavior is the same as for SAO, i.e., no filtering occurs.

The samples surrounding the center sample IC are denoted according to Figure 3, where A, B, L and R stands for above, below, left and right and where NW, NE, SW, SE stands for north-west etc.

| INW | IA | INE |
| --- | --- | --- |
| IL | IC | IR |
| ISW | IB | ISE |

Figure 3: Naming convention for samples surrounding the center sample, IC.

Each surrounding sample IA, IR etc will contribute with a corresponding modifier value μΔIA, μΔIR, etc. These are calculated the following way: Starting with the contribution from the sample to the right, IR, we calculate the difference

ΔIR=IR-IC+4≫3,         (Eq. 2)

where |⋅| denotes absolute value. For data that is not 10-bit, we instead use ΔIR= IR-IC+2n-6≫n-7, where n = 8 for 8-bit data etc. The resulting value is now clipped so that it is smaller than 16:

sIR=min15,ΔIR.          (Eq. 3)

The modifier value is now calculated as

μΔIR=LUTROWsIR,  if IR-IC≥0,-LUTROWsIR           otherwise        (Eq. 4)

where LUTROW[ ] is an array of 16 values determined by the value of qpb = QP + bilateral_filter_qp_offset:

If qpb ≤ 22, then LUT(x) = (0, 4, 4, 4, 3, 2, 1 ,2, 1, 1, 1, 1, 0, 1, 1, -1), otherwise

if qpb ≤ 27, then LUT(x) = (0, 8, 11, 11, 7, 5, 5, 4 , 5, 4, 4, 2, 2 , 2, 2, -2), otherwise

if qpb ≤ 32, then LUT(x) = (0, 9, 16, 19, 22, 22, 20, 15, 12, 12, 11, 9, 9, 7, 8, -3), otherwise

if qpb ≤ 37, then LUT(x) = (0, 12, 21, 28, 33, 36, 40, 40, 40, 36, 29, 22, 19,17, 15, -3), otherwise

LUT(x) = (0, 17, 23, 33, 37, 41, 44, 44, 45, 44, 42, 27, 22, 17, 15, -3).

The modifier values for μΔIL, μΔIA and  μΔIB are calculated from IL, IA and IB in the same way. For diagonal samples, the calculation also follows Equations 2 and 3, but uses a value shifted by 1. Using the diagonal sample ISE as an example, we get

μΔISE=LUTROWsISE≫1,  if ISE-IC≥0,-(LUTROWsISE≫1)           otherwise        (Eq. 5)

and the other diagonal samples are calculated likewise. The modifier values are summed together

msum= μΔIA+μΔIB+μΔIL+μΔIR+μΔINW+μΔINE+μΔISW+μΔISE.        (Eq. 6)

Note that μΔIR equals -μΔIAfor the previous sample. Likewise, μΔIA equals -μΔIBfor the sample above, and similar symmetries can be found also for the diagonal modifier values. This means that in a hardware implementation, it is sufficient to calculate four values μΔIR, μΔIB, μΔISW and μΔISE, and the remaining four values can be obtained from previously calculated values.

The msum value is now multiplied either by c = 1, 2 or 3, which can be done using a single adder and logical AND gates in the following way:

cv=k1&msum≪1+k2&msum,        (Eq. 7)

where & denotes logical and and k1 is the most significant bit of the multiplier c and k2 is the least significant bit. The value to multiply with is obtained using the minimum block dimension D=min⁡(width, height) as shown in Table 1:

| Block type | D≤4 | 4<D<16 | D≥16 |
| --- | --- | --- | --- |
| Intra | 3 | 2 | 1 |
| Inter | 2 | 2 | 1 |

Table 1: Obtaining the c parameter from the minimum size D=min(width,height) of the block.

Finally, the bilateral filter offset ΔIBIF is calculated. For full strength filtering, we use

ΔIBIF=cv+16≫5,          Eq. 8

whereas for half-strength filtering, we instead use

ΔIBIF=cv+32≫6.        (Eq. 9)

A general formula for n-bit data is to use

radd=214-n-bilateral_filter_strengthrshift=15-n-bilateal_filter_strength ΔIBIF=cv+radd≫rshift,           (Eq. 10)

where bilateral_filter_strength can be 0 or 1 and is signalled in the pps.

# Hardware complexity

In this section, we count the hardware complexity needed to decode a sample. As is usual in core experiments in JVET, we calculate the number of adds, shifts and checks, where a check is a min- max- or abs operation. Table 2 shows the number of such operations for calculating the modifier value of a sample below or to the right. Rounding additions are calculated separately in brackets, since these can be implemented with a carry bit in the nearby addition or check. No adder is needed to negate the LUT value, since it will be used in an addition later and it is possible to invert all the bits and set the carry to 1.

| Operation | add | shift | check | LUT |
| --- | --- | --- | --- | --- |
| ΔIR=IR-IC+4≫3 | 1 (1) | 1 | 1 |  |
| sIR=min15, ΔIR |  |  | 1 |  |
| μΔIR=LUTROWsIR, if IR-IC≥0,-LUTROWsIR otherwise |  |  |  | 1 |
| in total | 1 (1) | 1 | 2 | 1 |

Table 2: Operations per modifier value.

Table 2 is also valid for diagonal samples, except that one more shift is needed. Table 3 describes the total number of operations per sample. In total 13 additions and 5 rounding additions are needed per sample, as well as 8 shifts, 8 checks and 4 look-ups.

| Variable calculated | add | shift | check |
| --- | --- | --- | --- |
| μΔIR and μΔIB | 2 (2) | 2 | 4 |
| μΔISW and μΔISE | 2 (2) | 4 | 4 |
| msum= μΔIA+μΔIB+μΔIL+μΔIR+μΔINW+μΔINE+μΔISW+μΔISE | 7 |  |  |
| cv=k1&msum≪1+k2&msum | 1 | 1 |  |
| ΔIBIF=cv+16≫5 | (1) | 1 |  |
| IOUT=clip3IC+ΔIBIF+ΔISAO | 1 |  |  |
| in total | 13 (5) | 8 | 8 |

Table 3: Operations per sample

The widths of the additions are reported in Table 4, where the addition being investigated is marked by ⊕ and ⊖. There is one 11-bit adder, four 10-bit adders, one 9-bit adder, one 8-bit adder, three 7-bit adders and three 6- bit adders.

| Operation | bit width | number |
| --- | --- | --- |
| ΔIR=IR⊖IC+4≫3 | 10 | 4 |
| msum= μΔIA+μΔIB+μΔIL+μΔIR+μΔINW⊕μΔINE+μΔISW⊕μΔISE | 6 | 2 |
| msum= μΔIA⊕μΔIB+μΔIL⊕μΔIR+μΔINW+μΔINE⊕μΔISW+μΔISE | 7 | 3 |
| msum= μΔIA+μΔIB⊕μΔIL+μΔIR+μΔINW+μΔINE+μΔISW+μΔISE | 8 | 1 |
| msum= μΔIA+μΔIB+μΔIL+μΔIR⊕μΔINW+μΔINE+μΔISW+μΔISE | 9 | 1 |
| cv=k1&msum≪1⊕k2&msum | 11 | 1 |
| IOUT=clip3IC+ΔIBIF⊕ΔISAO | 6 | 1 |

The LUT values can be stored in 6 bits, which means that 16×6×5 = 480 bits or 60 bytes are needed. Normally 7 bits would be required for signed values, but the sign bit can be uniquely recovered by ANDing the two next most significant bits, and thus does not need to be stored.

# Results

Figure 4 shows an example from the test sequence Cactus, RA configuration, qp 32, with ALF on.

(a) anchor                                        (b) CE5-3.1.1 (RDO)                          (c) CE5-3.1.3 (no RDO)                        (d) CE5-3.1.4 (post filter)

Figure 4: An example from Cactus, RA qp 32 with ALF turned on. Notice how the artifacts between the R and the L are effectively removed using both the RDO version (a) and the non-RDO version (b) of the filter.

As can be seen in the figure, the artifacts, especially pronounced between R and L, are strongly attenuated in both the RDO- and no-RDO version of the proposal. In comparison, the post filter is not as good at removing the artifacts.

## Results test 1

In this section we present the results from test 1:

- Test 1: Filtering in combination with SAO. This will be the base-line implementation similar to test3 in JVET-O0548 but implemented in VTM-6.0. Hence it will be allowed to apply the filtering on the reconstructed block in the RDO (mode selection part) of the encoder. It will also include the possibility to turn off the filter at the CTU level.

All intra:

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | 0.27% | 0.22% | 105% | 105% |
| Class A2 | -0.39% | 0.18% | 0.12% | 108% | 103% |
| Class B | -0.34% | 0.24% | 0.23% | 108% | 105% |
| Class C | -0.53% | 0.07% | 0.23% | 104% | 102% |
| Class E | -0.44% | 0.26% | 0.16% | 103% | 103% |
| Overall | -0.40% | 0.20% | 0.20% | 106% | 104% |
| Class D | -0.47% | 0.08% | 0.06% | 104% | 102% |
| Class F | -0.55% | -0.03% | -0.07% | 106% | 104% |

Random access:

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | 0.11% | 0.00% | 102% | 107% |
| Class A2 | -0.42% | 0.13% | 0.21% | 102% | 99% |
| Class B | -0.43% | 0.43% | 0.28% | 104% | 102% |
| Class C | -0.53% | 0.38% | 0.35% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.44% | 0.29% | 0.23% | 103% | 102% |
| Class D | -0.61% | 0.12% | 0.26% | 103% | 101% |
| Class F | -0.51% | 0.05% | 0.06% | 104% | 101% |

Low-delay B:

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.19% | 0.62% | 0.46% | 103% | 102% |
| Class C | -0.50% | 0.63% | 0.68% | 101% | 102% |
| Class E | -0.28% | 0.81% | 1.15% | 102% | 102% |
| Overall | -0.32% | 0.67% | 0.71% | 102% | 102% |
| Class D | -0.75% | 0.93% | 1.15% | 103% | 100% |
| Class F | -0.33% | 0.65% | 0.86% | 103% | 100% |

Low-delay P:

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.32% | 0.71% | 0.89% | 104% | 102% |
| Class C | -0.52% | 0.56% | 0.32% | 102% | 101% |
| Class E | -0.31% | 0.16% | 0.76% | 102% | 102% |
| Overall | -0.39% | 0.52% | 0.67% | 103% | 102% |
| Class D | -0.71% | 0.76% | 1.16% | 103% | 104% |
| Class F | -0.54% | 0.44% | 0.30% | 101% | 104% |

## Results test1 without ALF

This is the same as test1 except that both test and anchor has ALF turned off.

All intra:

|  | Over VTM-6.0 with ALF turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.48% | 0.25% | 0.30% | 106% | 108% |
| Class A2 | -0.55% | 0.22% | 0.14% | 106% | 105% |
| Class B | -0.48% | 0.27% | 0.23% | 107% | 104% |
| Class C | -0.70% | 0.13% | 0.25% | 108% | 101% |
| Class E | -0.59% | 0.26% | 0.17% | 104% | 104% |
| Overall | -0.56% | 0.23% | 0.22% | 106% | 104% |
| Class D | -0.60% | 0.17% | 0.23% | 109% | 104% |
| Class F | -0.86% | -0.02% | -0.06% | 104% | 106% |

Random access:

|  | Over VTM-6.0 with ALF turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.57% | 0.17% | 0.07% | 102% | 106% |
| Class A2 | -0.52% | 0.16% | 0.10% | 102% | 102% |
| Class B | -0.68% | 0.20% | 0.23% | 104% | 103% |
| Class C | -0.62% | 0.23% | 0.08% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.61% | 0.20% | 0.13% | 103% | 103% |
| Class D | -0.69% | 0.07% | 0.28% | 102% | 101% |
| Class F | -0.76% | 0.14% | 0.16% | 99% | 103% |

Low-delay B:

|  | Over VTM-6.0 with ALF turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.48% | 0.80% | 0.65% | 104% | 102% |
| Class C | -0.72% | 0.71% | 0.69% | 105% | 102% |
| Class E | 0.08% | 0.49% | 0.98% | 104% | 104% |
| Overall | -0.42% | 0.69% | 0.74% | 105% | 103% |
| Class D | -0.93% | 1.31% | 0.99% | 106% | 101% |
| Class F | -0.59% | 0.92% | 0.04% | 102% | 102% |

Low-delay P:

|  | Over VTM-6.0 with ALF turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.63% | 0.80% | 0.79% | 108% | 102% |
| Class C | -0.89% | 0.62% | 0.47% | 106% | 102% |
| Class E | -0.04% | 0.49% | 0.72% | 106% | 101% |
| Overall | -0.57% | 0.66% | 0.66% | 107% | 102% |
| Class D | -1.30% | 1.15% | 0.91% | 109% | 101% |
| Class F | -0.85% | 0.56% | 0.03% | 105% | 101% |

## Results test2

This section presents the test result of test 2, which is described as follows in the CE-description:

- Test 2: Filtering without SAO. In this implementation, SAO is turned off. Otherwise it is similar to test 1.

All intra:

|  | Over VTM-6.0 with SAO turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | 0.28% | 0.25% | 105% | 110% |
| Class A2 | -0.40% | 0.23% | 0.15% | 108% | 109% |
| Class B | -0.34% | 0.27% | 0.26% | 109% | 108% |
| Class C | -0.55% | 0.11% | 0.29% | 111% | 105% |
| Class E | -0.45% | 0.31% | 0.22% | 111% | 106% |
| Overall | -0.41% | 0.24% | 0.24% | 109% | 107% |
| Class D | -0.49% | 0.02% | -0.01% | 113% | 106% |
| Class F | -0.61% | -0.03% | -0.11% | 107% | 110% |

Random access:

|  | Over VTM-6.0 with SAO turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.37% | 0.21% | 0.06% | 102% | 106% |
| Class A2 | -0.52% | 0.16% | 0.15% | 103% | 102% |
| Class B | -0.43% | 0.33% | 0.22% | 104% | 101% |
| Class C | -0.51% | 0.36% | 0.32% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.46% | 0.28% | 0.20% | 103% | 102% |
| Class D | -0.58% | 0.16% | 0.18% | 104% | 99% |
| Class F | -0.61% | 0.13% | 0.06% | 104% | 103% |

Low-delay B:

|  | Over VTM-6.0 with SAO turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.23% | 0.63% | 0.48% | 104% | 106% |
| Class C | -0.45% | 0.62% | 0.59% | 103% | 103% |
| Class E | -0.21% | -0.06% | -0.02% | 102% | 105% |
| Overall | -0.30% | 0.45% | 0.39% | 103% | 105% |
| Class D | -0.69% | 1.31% | 1.34% | 104% | 102% |
| Class F | -0.45% | 0.11% | -0.14% | 101% | 106% |

Low-delay P:

|  | Over VTM-6.0 with SAO turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.33% | 0.66% | 0.39% | 105% | 104% |
| Class C | -0.49% | 0.45% | 0.49% | 104% | 101% |
| Class E | -0.44% | 0.27% | 1.50% | 103% | 101% |
| Overall | -0.41% | 0.49% | 0.70% | 104% | 102% |
| Class D | -0.70% | 1.15% | 0.95% | 107% | 100% |
| Class F | -0.54% | 0.65% | 0.54% | 102% | 105% |

## Results test2 without ALF

This is the same as test2 except that both test and anchor has ALF turned off.

All intra:

|  | Over VTM-6.0 with SAO and ALF turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.62% | 0.26% | 0.33% | 103% | 113% |
| Class A2 | -0.86% | 0.27% | 0.18% | 103% | 113% |
| Class B | -0.69% | 0.31% | 0.27% | 105% | 109% |
| Class C | -0.96% | 0.14% | 0.28% | 105% | 105% |
| Class E | -0.87% | 0.33% | 0.24% | 108% | 108% |
| Overall | -0.80% | 0.26% | 0.26% | 105% | 109% |
| Class D | -0.80% | 0.21% | 0.25% | 106% | 104% |
| Class F | -1.18% | -0.01% | -0.07% | 102% | 111% |

Random access:

|  | Over VTM-6.0 with SAO and ALF turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.83% | 0.18% | 0.04% | 101% | 109% |
| Class A2 | -1.39% | 0.12% | 0.01% | 102% | 106% |
| Class B | -1.31% | 0.29% | 0.22% | 104% | 104% |
| Class C | -0.99% | 0.21% | 0.15% | 104% | 103% |
| Class E |  |  |  |  |  |
| Overall | -1.15% | 0.21% | 0.13% | 103% | 105% |
| Class D | -0.76% | 0.03% | 0.15% | 103% | 101% |
| Class F | -1.31% | 0.21% | 0.10% | 100% | 105% |

Low-delay B:

|  | Over VTM-6.0 with SAO and ALF turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.04% | 1.08% | 1.17% | 105% | 106% |
| Class C | -1.41% | 0.64% | 0.70% | 104% | 102% |
| Class E | -0.74% | 0.48% | 1.34% | 100% | 102% |
| Overall | -1.09% | 0.78% | 1.06% | 103% | 104% |
| Class D | -1.21% | 1.22% | 1.03% | 103% | 102% |
| Class F | -1.63% | -0.06% | 0.08% | 99% | 106% |

Low-delay P: Decoder run times corrected.

|  | Over VTM-6.0 with SAO and ALF turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.48% | 0.79% | 1.61% | 106% | 105% |
| Class C | -2.36% | 0.62% | 0.64% | 104% | 101% |
| Class E | -1.83% | 0.15% | 0.32% | 103% | 105% |
| Overall | -2.28% | 0.58% | 0.96% | 105% | 104% |
| Class D | -2.18% | 1.24% | 1.37% | 105% | 103% |
| Class F | -1.90% | 0.63% | -0.44% | 102% | 104% |

## Results test3

Here we present the results of test 3 which is described the following way in the CE-description:

- Test 3: Filtering in combination with SAO without RDO. In this implementation, it will not be allowed to apply filtering on the reconstructed block in the RDO (mode selection part) of the encoder. Otherwise it is similar to test 1.

All intra:

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | -0.01% | 99% | 107% |
| Class A2 | -0.01% | -0.03% | -0.02% | 101% | 103% |
| Class B | -0.01% | -0.02% | -0.03% | 102% | 107% |
| Class C | -0.08% | -0.05% | -0.05% | 101% | 102% |
| Class E | -0.07% | -0.02% | -0.02% | 102% | 105% |
| Overall | -0.03% | -0.02% | -0.03% | 101% | 105% |
| Class D | -0.09% | -0.09% | -0.10% | 98% | 101% |
| Class F | -0.12% | -0.04% | -0.03% | 102% | 107% |

Random access:

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | -0.02% | 100% | 103% |
| Class A2 | 0.07% | 0.04% | 0.00% | 99% | 102% |
| Class B | -0.04% | 0.07% | -0.03% | 101% | 103% |
| Class C | -0.15% | -0.06% | -0.15% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.02% | -0.05% | 100% | 102% |
| Class D | -0.24% | -0.01% | 0.00% | 100% | 100% |
| Class F | -0.14% | -0.05% | -0.10% | 101% | 101% |

Low-delay B:

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.04% | -0.09% | 103% | 105% |
| Class C | -0.16% | -0.02% | -0.08% | 103% | 108% |
| Class E | 0.00% | 0.53% | 0.61% | 103% | 103% |
| Overall | -0.07% | 0.14% | 0.09% | 103% | 105% |
| Class D | -0.26% | 0.17% | 0.86% | 104% | 99% |
| Class F | -0.01% | 0.21% | 0.45% | 102% | 105% |

Low-delay P:

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.09% | 0.02% | -0.10% | 101% | 102% |
| Class C | -0.11% | -0.10% | 0.03% | 101% | 101% |
| Class E | -0.07% | -0.47% | -0.07% | 101% | 100% |
| Overall | -0.09% | -0.14% | -0.05% | 101% | 101% |
| Class D | -0.21% | -0.17% | -0.23% | 100% | 102% |
| Class F | -0.08% | 0.17% | 0.09% | 102% | 101% |

## Results test3 without ALF

This is the same as test3 except that both test and anchor has ALF turned off.

All intra:

|  | Over VTM-6.0 with ALF turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.00% | 0.02% | 103% | 106% |
| Class A2 | -0.10% | -0.02% | -0.02% | 104% | 106% |
| Class B | -0.10% | -0.02% | -0.02% | 101% | 106% |
| Class C | -0.20% | -0.03% | -0.04% | 98% | 102% |
| Class E | -0.18% | -0.03% | -0.03% | 96% | 105% |
| Overall | -0.13% | -0.02% | -0.02% | 100% | 105% |
| Class D | -0.18% | -0.03% | -0.04% | 102% | 106% |
| Class F | -0.37% | -0.03% | -0.03% | 102% | 106% |

Random access:

|  | Over VTM-6.0 with ALF turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.02% | -0.02% | 100% | 102% |
| Class A2 | 0.09% | -0.15% | -0.07% | 99% | 101% |
| Class B | -0.21% | -0.21% | -0.20% | 100% | 102% |
| Class C | -0.25% | -0.23% | -0.23% | 99% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.16% | -0.14% | 99% | 102% |
| Class D | -0.29% | -0.08% | -0.05% | 99% | 100% |
| Class F | -0.35% | -0.06% | -0.09% | 92% | 102% |

Low-delay B:

|  | Over VTM-6.0 with ALF turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.19% | 0.00% | -0.13% | 102% | 106% |
| Class C | -0.35% | -0.09% | -0.14% | 104% | 103% |
| Class E | 0.09% | 0.17% | 0.01% | 102% | 101% |
| Overall | -0.17% | 0.01% | -0.10% | 103% | 104% |
| Class D | -0.51% | 0.49% | -0.19% | 103% | 104% |
| Class F | -0.21% | 0.04% | 0.32% | 105% | 107% |

Low-delay P:

|  | Over VTM-6.0 with ALF turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.35% | -0.21% | -0.23% | 103% | 103% |
| Class C | -0.39% | -0.15% | -0.21% | 102% | 102% |
| Class E | -0.07% | 0.30% | 0.08% | 104% | 101% |
| Overall | -0.29% | -0.06% | -0.15% | 103% | 102% |
| Class D | -0.66% | -0.20% | -0.28% | 103% | 101% |
| Class F | -0.43% | 0.20% | 0.03% | 102% | 104% |

## Results test 4

Here we present the results for test 4, which is described in the following way in the CE-description:

- Test 4: Filtering as a post filter. In this implementation, the filtering will be applied outside the coding loop, as if it was used as a post filter. Hence the sample input to the filter should be the same as the output yuv samples of the decoded anchor. Apart from the sample input, the filter may use information available to the decoder, such as QP and/or transform size. Only parameters available in the decoder anchor bit streams are allowed.

All intra:

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.37% | 0.00% | 0.00% | 100% | 111% |
| Class A2 | 0.43% | 0.00% | 0.00% | 100% | 108% |
| Class B | 0.62% | 0.00% | 0.00% | 100% | 111% |
| Class C | 0.72% | 0.00% | 0.00% | 100% | 108% |
| Class E | 0.83% | 0.00% | 0.00% | 100% | 109% |
| Overall | 0.60% | 0.00% | 0.00% | 100% | 109% |
| Class D | 0.46% | 0.00% | 0.00% | 100% | 105% |
| Class F | 0.33% | 0.00% | 0.00% | 100% | 111% |

Random access:

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.20% | 0.00% | 0.00% | 100% | 108% |
| Class A2 | 0.35% | 0.00% | 0.00% | 100% | 105% |
| Class B | 0.27% | 0.00% | 0.00% | 100% | 105% |
| Class C | 0.12% | 0.00% | 0.00% | 100% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.23% | 0.00% | 0.00% | 100% | 106% |
| Class D | 0.05% | 0.00% | 0.00% | 100% | 105% |
| Class F | 0.01% | 0.00% | 0.00% | 100% | 104% |

Low-delay B:

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.37% | 0.00% | 0.00% | 100% | 107% |
| Class C | 0.18% | 0.00% | 0.00% | 100% | 107% |
| Class E | 0.06% | 0.00% | 0.00% | 100% | 106% |
| Overall | 0.23% | 0.00% | 0.00% | 100% | 107% |
| Class D | 0.09% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.01% | 0.00% | 0.00% | 100% | 114% |

Low-delay P:

|  | Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.44% | 0.00% | 0.00% | 100% | 105% |
| Class C | 0.24% | 0.00% | 0.00% | 100% | 105% |
| Class E | 0.08% | 0.00% | 0.00% | 100% | 108% |
| Overall | 0.28% | 0.00% | 0.00% | 100% | 106% |
| Class D | 0.13% | 0.00% | 0.00% | 100% | 105% |
| Class F | 0.02% | 0.00% | 0.00% | 100% | 106% |

## Results test4 without ALF

This is the same as test3 except that both test and anchor has ALF turned off.

All intra:

|  | Over VTM-6.0 with ALF turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.00% | 0.00% | 100% | 114% |
| Class A2 | 0.01% | 0.00% | 0.00% | 100% | 110% |
| Class B | 0.17% | 0.00% | 0.00% | 100% | 107% |
| Class C | 0.05% | 0.00% | 0.00% | 100% | 106% |
| Class E | 0.17% | 0.00% | 0.00% | 100% | 109% |
| Overall | 0.08% | 0.00% | 0.00% | 100% | 109% |
| Class D | 0.06% | 0.00% | 0.00% | 100% | 103% |
| Class F | -0.49% | 0.00% | 0.00% | 100% | 107% |

Random access:

|  | Over VTM-6.0 with ALF turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.00% | 100% | 109% |
| Class A2 | 0.14% | 0.00% | 0.00% | 100% | 109% |
| Class B | 0.03% | 0.00% | 0.00% | 100% | 107% |
| Class C | -0.07% | 0.00% | 0.00% | 100% | 107% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.00% | 0.00% | 100% | 107% |
| Class D | -0.11% | 0.00% | 0.00% | 100% | 110% |
| Class F | -0.21% | 0.00% | 0.00% | 100% | 113% |

Low-delay B:

|  | Over VTM-6.0 with ALF turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | 0.00% | 0.00% | 100% | 109% |
| Class C | -0.10% | 0.00% | 0.00% | 100% | 108% |
| Class E | 0.01% | 0.00% | 0.00% | 100% | 104% |
| Overall | 0.01% | 0.00% | 0.00% | 100% | 107% |
| Class D | -0.19% | 0.00% | 0.00% | 100% | 103% |
| Class F | -0.28% | 0.00% | 0.00% | 100% | 112% |

Low-delay P:

|  | Over VTM-6.0 with ALF turned off |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | 0.00% | 0.00% | 100% | 106% |
| Class C | -0.10% | 0.00% | 0.00% | 100% | 107% |
| Class E | -0.01% | 0.00% | 0.00% | 100% | 108% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 107% |
| Class D | -0.20% | 0.00% | 0.00% | 100% | 103% |
| Class F | -0.33% | 0.00% | 0.00% | 100% | 110% |

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).