| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0086-v2 |
| --- | --- |

| Title: | AHG16/Non-CE5: Deblocking boundary strength fix for Affine and TPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra Semih Esenlik Biao Wang Han Gao Elena Alshina | Tel: Email: | anand.meher.kotra@huawei.com |
| Source: | Huawei Technologies Dusseldorf GmbH |  |  |

_____________________________

# Abstract

This proposal addresses the deblocking design inconsistency of VTM-6.0 for the Affine and TPM (triangle prediction mode) deblocking which has been raised by Minhua in the AHG16 email reflector. Design inconsistency mainly stems from the fact that the deblocking boundary strength derivation process does not explicitly differentiate between transform unit edges and sub-prediction unit edges (Affine) and therefore for the case when an internal transform edge which is not a sub-prediction unit edge that has cbf (code block flag) value as 0 for both the blocks adjacent to the edge, the desired behavior is to not apply deblocking, but in the current design in few corner cases, the “motion information” of the adjacent blocks are checked and depending on it the deblocking filter might be applied.

The current proposal tries to address this problem by proposing a simplex fix in the boundary strength derivation process by disabling the application of “motion information” related boundary strength rules for the internal sub-pu edges which do not overlap on the internal 8 x 8 sample grid and for the internal edges which belong to a coding unit which is coded using the “TPM”.

The objective results, over VTM6.0 Anchor for CTC configuration are as follows

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | 0.00% | 0.00% | 0.00% | 100% | 101% |
| RA | 0.00% | 0.00% | 0.00% | 100% | 101% |
| LDB | -0.01% | 0.10% | -0.03% | 100% | 101% |
| LDP | 0.00% | 0.02% | 0.05% | 100% | 100% |

# Introduction

Figure 1 below shows the design inconsistency for deblocking in VTM-6.0 [1] (as mentioned by Minhua in the AHG16 email).

In the example shown in Fig.1, the CU size is 16x32 which is coded using Affine mode. The left hand part of the figure shows a 4x4 sample grid of the CU, for each of the 4x4 sub-blocks the motion information is stored. Therefore, there are three inner sub-block boundaries marked 1, 2, 3 (see the left hand part of Fig. 1).

The CU uses quad SBT (sub-block transform), therefore, sub-block boundary 1 is a TU boundary (see center part of Fig. 1). In cases where there are no non-zero coefficients in the current component (4x4 sub-blocks on the left and right hand side of edge 1) but the sub-block motion vector (MV) difference is large enough along the sub-block boundaries, deblocking is performed. This may lead to over smoothing at edge 1 and may deteriorate the quality of the picture while also increasing the processing complexity.

Deblocking is currently only applied to sub-blocks of an affine-coded PU on an 8x8 basis (see right hand part of Fig. 1, edge 2). Therefore, boundary 2 is also deblocked because it overlaps with a PU edge. According to the current de-blocking decision process, sub-block boundary 1 is marked as an edge because it aligns with a TU edge. Since it was already marked as an edge, the motion information differences are also checked, and may lead to deblocking in case there are motion information differences. As mentioned above, this may lead to over smoothing.

Although sub-block boundary 3 may have absolutely the same conditions as sub-block boundary 1, i.e. no non-zero coefficients along the edge, and MV difference may be large enough to trigger deblocking, deblocking is not applied for the edge 3, because edge 3 does not overlap with a TU edge (which is in contrast to edge 1).

Figure 1. Deblocking design inconsistency for Affine mode when SBT is used

A similar problem may exist when Triangle prediction mode (TPM) is used for a coding unit. As shown in Fig. 2, here the transform unit size is less than the coding unit size. Therefore the coding unit is divided into 4 TU’s TU1, TU2, TU3, TU4. In the case when the transform units have no non-zero coefficients, the deblocking will still be applied for the transform unit edges (area marked by the dashed circle at the center) whenever the motion vector information of the blocks adjacent to the edges (marked by the circle) is different. This may be result in over-smoothing, therefore resulting in a poor visual quality.

Figure2.  TU edges marked in red (at the center, see dashed circle) are deblocked even when there are no non-zero coefficients on both sides of the edge because MV’s may be different across the diagonal.

Therefore as shown in Fig. 1 and Fig. 2, the current deblocking design in some cases decides to filter few internal edges and in some cases not to filter.

# Proposed Solution

As shown in Fig. 3, the boundary strength derivation is modified by disabling the application of “motion information” related rules for the internal sub-pu edges which do not overlap on the implicit 8 x 8 sample grid of the given coding unit and for the internal edges which belong to a coding unit which is coded using the “TPM”.

Figure3.  Boundary strength derivation fix for Affine and TPM deblocking

# Simulation Results

The test is implemented on top of VTM-6.0 [1] and the simulation is performed under common test conditions as described in JVET-N1010 [2].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 105% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class A2 | 0.01% | 0.02% | 0.02% | 100% | 100% |
| Class B | 0.00% | 0.00% | -0.02% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class F | 0.00% | 0.00% | 0.01% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.08% | 0.05% | 100% | 101% |
| Class C | -0.02% | 0.00% | -0.11% | 100% | 100% |
| Class E | 0.01% | 0.26% | -0.05% | 100% | 100% |
| Overall | -0.01% | 0.10% | -0.03% | 100% | 101% |
| Class D | -0.02% | -0.01% | -0.08% | 100% | 101% |
| Class F | -0.02% | 0.23% | 0.25% | 100% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.06% | 0.03% | 100% | 100% |
| Class C | 0.01% | -0.04% | 0.12% | 100% | 100% |
| Class E | 0.00% | 0.05% | -0.02% | 100% | 100% |
| Overall | 0.00% | 0.02% | 0.05% | 100% | 100% |
| Class D | 0.02% | -0.01% | 0.04% | 100% | 101% |
| Class F | -0.03% | -0.25% | -0.22% | 100% | 101% |

# Conclusion

The current contribution proposes a simple boundary strength fix to resolve the design issue identified for the Affine and TPM deblocking (Issue was raised by Minhua in the AHG16 email). The suggested changes do not result in any BD-Rate change or subjective quality under CTC conditions. The specification text changes are attached. The software patch on top of VTM-6.0 is also attached. It is suggested to adopt the suggested boundary strength fix to resolve the deblocking design issue.

# References

- Versatile Video Coding (Draft 6), Document JVET-O2001-vE, B. Bross, J. Chen, S. Liu

- “JVET common test conditions and software reference configurations”, JVET-N1010

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).