| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0088 |
| --- | --- |

| Title: | CE1-1: RPR downsampling filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Andrew Segall | Email: | samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

# Abstract

This contribution reports the results of CE1 tests 1.1, 1.2 and 1.3.  These tests apply different downsampling filters in the interpolation process when the reference picture is of larger spatial resolution than the current picture. The tests evaluate the performance of the proposed filters using the common test condition defined for reference picture resampling (RPR). Luma BD-rate reductions of 2.3%, 4.6% and 9.8% are reported for PSNR1 at 1.5x, 2x and 4x scaling ratios, respectively. Luma BD-rate reductions of 1.9%, 3.0% and 4.6% are reported for PSNR2 at 1.5x, 2x and 4x scaling ratios, respectively. In version two (v2) of the contribution, the complete set of results for all 7 tested configurations was included, including a fix for Class B were the initial results were generated with the wrong number of frames for both anchor and test. Two new tables were also added to the contribution in section 3.4 and the proposed specification text was included relative to JVET-O2001-vE with all previous change marks accepted.

# Introduction

Support for reference picture resampling (RPR) was added to the VVC draft at the 15th JVET meeting. With RPR it is possible to change the coded width and height within a coded video sequence and predict from a reference picture of different resolution than the current picture. This is realized with a one-step approach where reference position for each sample in a block is calculated, taking into account the scaling factor between the pictures. In the method for RPR that was adopted into the VVC Committee Draft, the existing interpolation filter was used regardless of if downsampling or upsampling was performed. In JVET-O0240, it was proposed to use different filters for downsampling in order to provide better inter-prediction when the resolution is reduced.

# CE Test motivation and description

Three different downsampling filters were tested in CE1-1: Lanczos2, Lanczos1 and Lanczos2.67. The tested filters are design for scaling factors of 2x, 4x, and 1.5x, respectively but were also tested for scaling factors for which they were not designed as shown in table 1.

| Filter \ Scaling factor | 1.5x | 2x | 4x |
| --- | --- | --- | --- |
| Lanczos2 (test 1.1) | x | x | x |
| Lanczos1 (test 1.2) | - | x | x |
| Laczos2.67 (test 1.3) | x | x | - |

Table 1. Tested combinations of scaling factors and filters.

# Experimental Results

The CE tests were implemented in VTM6.1 [2] with the patch for CE1 applied and evaluated for the Low Delay B of the CTC [3] with the modifications defined in CE1 description.  A summary of the results is provided below.  Results are presented for both PSNR1 and PSNR2 as described in the RPR CTC [3]. The summary in sections 3.1, 3.2 and 3.3 only includes results for using downsampling filters applied at the scaling ratio the filters were designed for. Section 3.4 provides a summary table of applying the filters at other scaling ratios. The full set of  results can be found in the attached Excel files.

## CE1 Test 1.1 (Lanczos2)

|  |  |  | Low Delay B PSNR 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE1 anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2,37% | -1,49% | -1,64% | 101% | 102% |
| Class C | -5,63% | -4,02% | -3,76% | 98% | 98% |
| Class E | -6,34% | -2,96% | -2,12% | 98% | 97% |
| Overall | -4,45% | -2,70% | -2,47% | 100% | 100% |
| Class D | -5,13% | -3,20% | -3,18% | 99% | 100% |
| Class F |  |  |  |  |  |

|  |  |  | Low Delay B PSNR 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE1 anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1,55% | -1,05% | -1,14% | 101% | 102% |
| Class C | -3,50% | -2,32% | -2,26% | 98% | 98% |
| Class E | -4,30% | -2,31% | -1,96% | 98% | 97% |
| Overall | -2,89% | -1,79% | -1,72% | 100% | 100% |
| Class D | -2,84% | -2,49% | -2,38% | 99% | 100% |
| Class F |  |  |  |  |  |

## CE1 Test 1.2 (Lanczos1)

|  |  |  | Low Delay B PSNR 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE1 anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7,84% | -6,74% | -8,14% | 101% | 102% |
| Class C | -7,40% | -10,29% | -9,59% | 97% | 95% |
| Class E | -14,96% | -11,46% | -10,29% | 98% | 98% |
| Overall | -9,47% | -9,10% | -9,16% | 99% | 99% |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

|  |  |  | Low Delay B PSNR 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE1 anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3,95% | -4,17% | -4,74% | 101% | 102% |
| Class C | -3,50% | -4,53% | -3,97% | 97% | 95% |
| Class E | -7,02% | -5,95% | -5,83% | 98% | 98% |
| Overall | -4,57% | -4,74% | -4,76% | 99% | 99% |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

## CE1 Test 1.3 (Laczos2.67)

|  |  |  | Low Delay B PSNR 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE1 anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1,02% | -0,46% | -0,75% | 99% | 100% |
| Class C | -3,28% | -2,06% | -2,05% | 100% | 103% |
| Class E | -2,82% | -1,40% | -0,26% | 101% | 103% |
| Overall | -2,23% | -1,23% | -1,06% | 100% | 102% |
| Class D | -3,46% | -1,85% | -1,24% | 100% | 102% |
| Class F |  |  |  |  |  |

|  |  |  | Low Delay B PSNR 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE1 anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,77% | -0,59% | -0,92% | 99% | 100% |
| Class C | -2,57% | -2,06% | -1,97% | 100% | 103% |
| Class E | -2,67% | -2,98% | -1,21% | 101% | 103% |
| Overall | -1,85% | -1,68% | -1,34% | 100% | 102% |
| Class D | -2,35% | -1,67% | -1,29% | 100% | 102% |
| Class F |  |  |  |  |  |

## Different combinations of scaling factors and filters

CE1-1 included tests for applying the proposed filters also for scaling ratios that the filters were not designed for. A summary of the results is available here. The full set of results are available in the attached excel sheets.

BD-rate luma calculated with PSNR1:

| Filter \ Scaling factor | 1.5x | 2x | 4x |
| --- | --- | --- | --- |
| Lanczos2 (test 1.1) | 1,12% | -4,45% | -3,48% |
| Lanczos1 (test 1.2) | - | 2,97% | -9,47% |
| Laczos2.67 (test 1.3) | -2,23% | -2,80% | - |

BD-rate luma calculated with PSNR2:

| Filter \ Scaling factor | 1.5x | 2x | 4x |
| --- | --- | --- | --- |
| Lanczos2 (test 1.1) | 0,97% | -2,89% | -1,63% |
| Lanczos1 (test 1.2) | - | 2,18% | -4,57% |
| Laczos2.67 (test 1.3) | -1,85% | -1,77% | - |

# Conclusion

This contribution reports the performance of the downsampling filters for RPR as studied in the Core Experiment process.  The filters have the same precision and filter length as the existing interpolation filters in VVC. It is proposed that the derivation process for which filters to use is made based on the scaling factor, individually for horizontal and vertical filtering. Since the scaling factor relative a reference picture is constant for all blocks of the current picture, this derivation process can be performed once per picture. The results presented in this contribution show a significant compression benefit for using the proposed filters.

It is recommended to adopt the proposed RPR filters into the VVC draft specification.

# References

[1] J. Samuelsson, S. Deshpande, A. Segall " Adaptive Resolution Change (ARC) with downsampling," JVET-O0240, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 15th Meeting: Gothenburg, SE, 3–12 July 2019

[2] VVC reference software (VTM) version 6.1, available at: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, "JVET common test conditions and software reference configurations for SDR video," JVET-N1010, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 14th Meeting: Geneva, CH, 19–27 Mar. 2019

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).