| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0092-v1 |
| --- | --- |

| Title: | AHG13: Encoder speed-up for SMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang | Tel: Email: | chenhuanbang@huawei.com haitao.yang@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

In this contribution, two encoder modifications are proposed to speed up the motion estimation of symmetric motion vector difference (SMVD) mode of the VTM6.0 encoder, including 1) limiting the number of staring motion vector checking to 5 for SMVD; 2) aligning the distortion calculation of SMVD to regular BI prediction. Compared to VTM6.0 anchor, it is claimed that the proposed encoder speed-up saves 6% encoding run time without performance change.

With the proposed modifications, the tool off test results of SMVD reportedly show on average 0.26% BD rate with 97% encoding time.

Problem Statement

In the AHG13 report JVET-P0013 [1], the tool off test results of SMVD reportedly show on average 0.27% BD rate with 88% encoding time over VTM6.0. The encoding time of SMVD is abnormal and is caused by the adoption of JVET-O0592.

JVET-O0592 was adopted to enhance the motion estimation efficiency for regular inter mode, SMVD mode and affine mode in last meeting. One MV candidate list is maintained at encoder which contains at most the MV candidates of 15 previous CUs according to the coding order. The MV candidates in the list will be rounded to the integer precision and used as the additional candidates to select the optimal starting MV for the integer motion search and the initial MV candidates for the symmetrical motion vector difference (SMVD) mode. After this modification, at most 18 candidates will be checked for selecting the initial MV of SMVD mode, which increase the encoding time of SMVD significantly.

In addition, in current VTM6.0, the distortion calculation for SMVD is as follow:

- Perform motion compensation to get prediction signal of List 0 (Pred0)

- Perform motion compensation to get prediction signal of List 1 (Pred1)

- Perform weighted average to get BI prediction signal (PredBi)

- Calculate SATD between prediction signal and original signal (Org).

While for regular BI prediction, the distortion calculation is as follow:

- Perform motion compensation to get prediction signal of List 0 (Pred0)

- Calculate reference signal Ref by subtracting Pred0 from original signal (Ref = (w0+w1)/w1 * Org – w0/w1 * Pred0)

- Perform motion compensation to get prediction signal of List 1 (Pred1)

- Calculate SATD between prediction Pred1 and Ref

The design for SMVD has two drawbacks. Firstly, lack of SIMD optimization is for non-equal weighted in Step 3. Secondly, Step 3 should be applied once Pred0 or Pred1 is changed. While in the distortion calculation of each initial MV, MV0 is fixed and MV1 is modified according to different combination of MVP0 and MVP1. There is up to 4 combination of MVP0 and MVP1. Therefore, step 3 should be applied 4 times for each initial MV.

Proposal

To reduce the running time of SMVD, the following non-normative changes are proposed:

- Firstly, the maximum number of initial MV candidates is set to 5;

- The distortion calculation for SMVD is modified as the regular BI prediction search.

Experiment results

The proposed methods were implemented based on the VTM-6.0 reference software [2] and tested using the common test condition (CTC) as specified in [3]. Table 1 shows the overall BD-rate performance of the proposed encoder modifications for RA configuration, compared to VTM-6.0 anchor. Table 2 shows the tool off test results of SMVD, compared to the proposed encoder modifications.

Table 1 Coding performance over VTM-6.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.05% | 93% | 101% |
| Class A2 | 0.03% | 0.01% | 0.09% | 94% | 100% |
| Class B | 0.00% | 0.02% | 0.02% | 94% | 100% |
| Class C | -0.01% | 0.06% | 0.03% | 96% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.04% | 94% | 101% |
| Class D | 0.00% | -0.01% | 0.10% | 97% | 101% |
| Class F | -0.02% | -0.02% | 0.01% | 96% | 100% |

Table 2 Coding performance of SMVD tool off test over the proposal

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0+Speed up |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.24% | 0.25% | 0.28% | 96% | 100% |
| Class A2 | 0.33% | 0.32% | 0.29% | 97% | 101% |
| Class B | 0.29% | 0.28% | 0.24% | 97% | 101% |
| Class C | 0.20% | 0.08% | 0.16% | 97% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.26% | 0.23% | 0.23% | 97% | 100% |
| Class D | 0.13% | 0.07% | 0.10% | 98% | 99% |
| Class F | 0.26% | 0.28% | 0.22% | 98% | 101% |

Reference

- W.-J. Chien, J. Boyce, Y.-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, R.-L. Liao, S. Liu, D. Luo, “JVET AHG report: Tool reporting procedure (AHG13)”, JVET-P0013, the 16th JVET meeting, October 2019, Geneva, CH.

- VTM-6.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH.

Patent rights declaration(s)

Huawei Technologies Co., Ltd does not have any current or pending patent rights relating to the technology described in this contribution.