| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0094-v2 |
| --- | --- |

| Title: | AHG16/Non-CE4: Fix on overflow issue in PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang | Tel: Email: | chenhuanbang@huawei.com haitao.yang@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

In current VTM6.0, the refined prediction samples of PROF can go beyond 16-bit, which leads to using multipliers beyond 16-bit in the weighted prediction. It is claimed that both hardware and software implementation would be more complicated by the overflow issue. It is proposed to clip the refinement offset of PROF to [-8192, 8191] to ensure the refined prediction samples is within 16-bit. Simulation results reportedly show that the luma BD-rate is 0.00% and 0.00% in RA and LB cases, respectively, when compared to VTM-6.0.

Introduction

In VTM-6.0, extreme input values for affine motion compensation lead interpolated values to be in the range 2*a*b*255/64 = –15076 to (a2 + b2)*255/64 = 31396, where a is the sum of negative interpolation filter coefficients and equal to –22, and b is the sum of the positive ones and equal to 86.

However, after the applying of PROF, the refined prediction samples of PROF can go beyond 16-bit, which leads to using multipliers beyond 16-bit in the weighted prediction. Using multipliers beyond 16-bit adds cost to the weighted prediction part, it is not implementation friendly, especially for software implementations where 32-bit multipliers would be needed.

It is proposed to clip the refinement offset of PROF to [–8192, 8191] to solve the overflow issue. After the applying of PROF with clipping offset, the refined value will be the range –15076–8192 to 31396+8191, and the range size 62856 is within 16-bit range. The range can be shifted to singed 16-bit range by first adding a negative offset to the interpolated values before bi-prediction and then adding back a positive offset to the final result [3].

Experiment results

The proposed methods were implemented based on the VTM-6.0 reference software [1] and tested using the common test condition (CTC) as specified in [2]. Table 1 shows the overall BD-rate performance of the proposed encoder improvements for RA and LDB configurations, respectively, compared to VTM-6.0 anchors.

Table 1 Coding performance over VTM-6.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.01% | 0.01% | 0.01% | 100% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.01% | -0.01% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.07% | 0.20% | 0.00% | 100% | 101% |

Conclusions

It is proposed to clip the refinement offset of PROF to [-8192, 8191] to solve the overflow issue.

Reference

- VTM-6.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH.

- K. Ugur, A. Alshin, E. Alshina, F. Bossen, W.-J. Han, J.-H. Park, and J. Lainema, “Motion Compensated Prediction and Interpolation Filter Design in H.265/HEVC”, IEEE Journal of Selected Topic In Signal Processing, Vol. 7, No. 6, December, 2013

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Spec changes

#### 8.5.6.4 Prediction refinement with optical flow process

Inputs to this process are:

- two variables sbWidth and sbHeight specifying the width and the height of the current subblock,

- one (sbWidth + borderExtension) * (sbHeight + borderExtension) prediction sample array predSamples,

- one (sbWidth * sbHeight) motion vector difference array diffMv.

Output of this process is the (sbWidth)x(sbHeight) array pbSamples of prediction sample values.

Variable shift1 is set equal to Max( 6, BitDepthY − 6 ).

Variable dILimit is set equal to 1 << Max( 13, BitDepthY + 2 ).

For x =0..sbWidth − 1, y =0..sbHeight − 1, the following ordered steps apply:

- The variables gradientH[ x ][ y ] and gradientV[ x ][ y ] are derived as follows:

gradientH[ x ][ y ]  =  ( predSamples[ x + 2 ][ y ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 )	(8-796)

gradientV[ x ][ y ]  =  ( predSamples[ x ][ y + 2 ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 )	(8-797)

- The variable dI is derived as follows:

dI = gradientH[ x ][ y ] * diffMv[ x ][ y ][ 0 ] + gradientV[ x ][ y ] * diffMv[ x ][ y ][ 1 ]	(8-798)

- Prediction sample value at location ( x, y ) in the subblock is derived as follows:

pbSamples[ x ][ y ] = predSamples[ x + 1 ][ y + 1 ] + Clip3( -dILimit, dILimit − 1, ( ( dI + 1 ) >> 1 ) ) 	(8-799)