| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0111-v2 |
| --- | --- |

| Title: | CE3-Related: Chroma intra prediction mode mapping for 4:2:2 format |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Biao Wang, Semih Esenlik, Han Gao, Anand Meher Kotra, Elena Alshina | Tel: Email: | biao.wang@huawei.com |
| Source: | Huawei Technologies Dusseldorf GmbH |  |  |

_____________________________

# Abstract

This contribution proposes a bug fix on chroma intra mode mapping for 4:2:2 chroma format in the current VVC draft specification. Compared to luma component, the chroma samples are subsampled by half in horizontal direction under 4:2:2 chroma format. As a result, the chroma intra mode needs to be mapped to a new one according to the halved distance in horizontal direction. In current VVC draft specification, some of the mapped angular modes are not accurate. This contribution fixes these inaccurate mapped modes. Compared to VTM6.0, simulation results shows the bug-fixed version achieves consistent coding gain.

In version 2, presentation slides are uploaded.

# Background

In current VVC specification [1], the chroma intra angular mapping for 4:2:2 chroma format is defined with Table 8-3, as shown below.

Table 8-3 – Specification of the 4:2:2 mapping process from chroma intra prediction mode X to mode Y when chroma_format_idc is equal to 2

| mode X | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mode Y | 0 | 1 | 61 | 62 | 63 | 64 | 65 | 66 | 2 | 3 | 4 | 6 | 8 | 10 | 12 | 13 | 14 | 16 |
| mode X | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| mode Y | 18 | 20 | 22 | 23 | 24 | 26 | 28 | 30 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| mode X | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |
| mode Y | 42 | 43 | 44 | 44 | 44 | 45 | 46 | 46 | 46 | 47 | 48 | 48 | 48 | 49 | 50 | 51 | 52 | 52 |
| mode X | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 |  |  |  |  |  |
| mode Y | 52 | 53 | 54 | 54 | 54 | 55 | 56 | 56 | 56 | 57 | 58 | 59 | 60 |  |  |  |  |  |

The mapping for 4:2:2 chroma format is necessary because the block aspect ratio is changed, while for 4:2:0 format, the chroma component is subsampled at both horizontal and vertical direction, therefore block aspect ratio is not changed and no mapping is required. The block ratio change for 4:2:2 chroma format is shown in Figure 1.

1.A: original block aspect ratio             1.B: block aspect ratio change in chroma 4:2:2

Figure 1.A shows a block with 1:1 (width: height) block aspect ratio for the luma component. Because  subsampling only occur to the horizontal direction under 4:2:2 chroma format, the block aspect ratio is changed to 1:2 for chroma component. The mapping rationale is explained by an example of intra mode 8.

For angular mode 8, its corresponding intraPredAngle value is 16 according to Table 8-8 in VVC specification draft 6 [1].

Table 8-8 – Specification of intraPredAngle

| predModeIntra | −14 | −13 | −12 | −11 | −10 | −9 | −8 | −7 | −6 | −5 | −4 | −3 | −2 | −1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | 512 | 341 | 256 | 171 | 128 | 102 | 86 | 73 | 64 | 57 | 51 | 45 | 39 | 35 | 32 | 29 | 26 |
| predModeIntra | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| intraPredAngle | 23 | 20 | 18 | 16 | 14 | 12 | 10 | 8 | 6 | 4 | 3 | 2 | 1 | 0 | −1 | −2 | −3 |
| predModeIntra | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
| intraPredAngle | −4 | −6 | −8 | −10 | −12 | −14 | −16 | −18 | −20 | −23 | −26 | −29 | −32 | −29 | −26 | −23 | −20 |
| predModeIntra | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 |
| intraPredAngle | −18 | −16 | −14 | −12 | −10 | −8 | −6 | −4 | −3 | −2 | −1 | 0 | 1 | 2 | 3 | 4 | 6 |
| predModeIntra | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 |
| intraPredAngle | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 23 | 26 | 29 | 32 | 35 | 39 | 45 | 51 | 57 | 64 |
| predModeIntra | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 80 |  |  |  |  |  |  |  |  |  |
| intraPredAngle | 73 | 86 | 102 | 128 | 171 | 256 | 341 | 512 |  |  |  |  |  |  |  |  |  |

The value intraPredAngle represents a distance “width/2” as shown in Figure 1.A. Its corresponding angle is shown as “α” in Figure 1.A and can be defined as follows:

α = arctan(16/32) = arctan(1/2)

Its corresponding position where its angle arrow and left boundary crosses is ½ width below the position of mode 18, which locates at the middle of the left boundary. The length of the width in Figure 1.A is defined as half of the width of the current block. The corresponding position of mode 8, however, would be mapped to another angle since the width of horizontal is halved in 4:2:2 chroma format. Now its corresponding angle is arctan ((width/2)/(width/2)) = arctan (1) = 45 degree, which corresponds to the angle of mode 2 as shown in Figure 1.B.

# Problem and the proposed solution

It is found some of the mapped mode in Table 8-7 table are inaccurate. The following Table 1 shows how the correct mapped value are derived for input modes 34 to 50.

Table 1: intra prediction mode mapping derivation for 4:2:2 chroma format with input modes 34 to 50

|  | without subsampling |  | with 4:2:2 format |  | mapping |  |
| --- | --- | --- | --- | --- | --- | --- |
| input mode | intraPredAngle | angle | intraPredAngle/2 | angle | closest Angle | mapped mode |
| 34 | -32 | 45.00000 | -16 | 26.56505 | 26.56505 | 40 |
| 35 | -29 | 42.18444 | -14.5 | 24.37647 | 23.62938 | 41 |
| 36 | -26 | 39.09386 | -13 | 22.10945 | 23.62938 | 41 |
| 37 | -23 | 35.70669 | -11.5 | 19.76717 | 20.55605 | 42 |
| 38 | -20 | 32.00538 | -10 | 17.35402 | 17.35402 | 43 |
| 39 | -18 | 29.35775 | -9 | 15.70864 | 17.35402 | 43 |
| 40 | -16 | 26.56505 | -8 | 14.03624 | 14.03624 | 44 |
| 41 | -14 | 23.62938 | -7 | 12.33909 | 14.03624 | 44 |
| 42 | -12 | 20.55605 | -6 | 10.61966 | 10.61966 | 45 |
| 43 | -10 | 17.35402 | -5 | 8.88066 | 10.61966 | 45 |
| 44 | -8 | 14.03624 | -4 | 7.12502 | 7.12502 | 46 |
| 45 | -6 | 10.61966 | -3 | 5.35583 | 5.35583 | 47 |
| 46 | -4 | 7.12502 | -2 | 3.57633 | 3.57633 | 48 |
| 47 | -3 | 5.35583 | -1.5 | 2.68378 | 3.57633 | 48 |
| 48 | -2 | 3.57633 | -1 | 1.78991 | 1.78991 | 49 |
| 49 | -1 | 1.78991 | -0.5 | 0.89517 | 1.78991 | 49 |
| 50 | 0 | 0 | 0 | 0 | 50 | 50 |

With the subsampling of 4:2:2 chroma format, for modes 34 to 50 their corresponding value of intraPredAngle is halved, and a new angle with the value of intraPredAngle/2 can be derived. A mapped mode can be derived by searching the closest angle of the newly derived angle. The mapping for other modes could also be derived using a similar method. It is noted that for input modes from 2 to 33, their intraPredAngle is increased by 2x instead of being halved.

# Working draft modification

In the end, a corrected mapping Table 8-3 is derived as follows, with correct mapped modes highlighted:

Table 8-3: an example of a combination of the proposed embodiments

| mode X | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mode Y | 0 | 1 | 61 | 62 | 63 | 64 | 65 | 66 | 2 | 3 | 5 | 6 | 8 | 10 | 12 | 13 | 14 | 16 |
| mode X | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| mode Y | 18 | 20 | 22 | 23 | 24 | 26 | 28 | 30 | 31 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| mode X | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |
| mode Y | 41 | 42 | 43 | 43 | 44 | 44 | 45 | 45 | 46 | 47 | 48 | 48 | 49 | 49 | 50 | 51 | 51 | 52 |
| mode X | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 |  |  |  |  |  |
| mode Y | 52 | 53 | 54 | 55 | 55 | 56 | 56 | 57 | 57 | 58 | 59 | 59 | 60 |  |  |  |  |  |

# Results

The proposed correction is implemented on top of VTM-6.0. The simulations are conducted with 4:2:2 test sequences. Table 3 and Table 4 show the coding performance of the proposed correction for all intra and random access configurations, respectively. Compared to VTM-6.0, it can be observed that   consistent coding gain was achieved for all of the test sequences, with no runtime increase.

Table 3 BD-rate performance over VTM-6.0 under all intra configuration

|  | Y | U | V | EncTime | DecTime |
| --- | --- | --- | --- | --- | --- |
| Traffic | -0.04% | -0.06% | -0.02% | 101% | 100% |
| Kimono | -0.05% | 0.04% | -0.17% | 101% | 101% |
| Seeking | -0.01% | -0.01% | 0.02% | 100% | 100% |
| EBUHorse | -0.02% | -0.04% | 0.03% | 100% | 100% |
| EBUGraphics | -0.06% | 0.02% | 0.00% | 99% | 100% |
| EBUWaterRocksClose | -0.01% | 0.00% | 0.01% | 99% | 99% |
| EBUKidSoccer | 0.00% | -0.04% | 0.03% | 100% | 100% |
| All | -0.03% | -0.01% | -0.01% | 100% | 100% |

Table 4 BD-rate performance over VTM-6.0 under random access configuration

|  | Y | U | V | EncTime | DecTime |
| --- | --- | --- | --- | --- | --- |
| Traffic | 0.00% | -0.05% | 0.15% | 100% | 100% |
| Kimono | 0.00% | -0.02% | 0.00% | 100% | 100% |
| Seeking | -0.01% | -0.06% | -0.02% | 100% | 100% |
| EBUHorse | -0.02% | -0.09% | 0.05% | 100% | 100% |
| EBUGraphics | -0.01% | -0.07% | -0.07% | 100% | 100% |
| EBUWaterRocksClose | -0.02% | 0.07% | -0.01% | 100% | 100% |
| EBUKidSoccer | -0.01% | 0.07% | 0.19% | 100% | 100% |
| All | -0.01% | -0.02% | 0.04% | 100% | 100% |

# Conclusion

This contribution proposes a bug fix for chroma intra prediction mode mapping under 4:2:2 chroma format, using the theoretically correct values. Simulation results show that the proposed correction brings coding gain consistently with no runtime change. It is suggested to include the proposed correction in the next draft of the VVC specification.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “Versatile Video Coding (Draft 6)”, JVET-O2001-vE, B. Bross, J. Chen, and S. Liu