| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0131 |
| --- | --- |

| Title: | AHG12: On output subpicture sets |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Ahmed Hamza 9276 Scranton Road Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to indicate Output Sub-Picture Sets (OSPS) for sub-picture extraction and reposition across multiple layers. The proposal can be summarized as follows.

- Output Sub-picture Sets (OSPS) syntax elements are proposed for normative sub-bitstream extraction, reposition and conformance points for sub-pictures and sets.

- Sub-bitstream extraction process update to support sub-picture extraction and reposition based on OSPS.

# Introduction

Output subpicture sets (OSPS) was proposed in JVET-N0826, O0141, O0555 and O0183 to specify normative extraction and conformance points, and to be carried in SPS or VPS.

This contribution proposes a sub-picture-based design to support extraction and merging of sub-pictures for output. SPS can carry OSPS signaling if output subpicture is constrained to a single layer, otherwise, VPS may carry OSPS for scalability or layered subpicture.

Figure 1 is a multiple layer sub-picture example, where layer  0 is a low-resolution picture consisting of 6 sub-pictures, and layer 1 is a high-resolution picture consisting of 5 sub-pictures, and layer 2 is a sub-picture corresponding to sub-picture #2 at layer 0 and #8 at layer 1.

Figure 1 Example of a layer-based sub-picture structure

Output sub-picture set (OSPS) is set of sub-pictures accompanied by profile, tier, level and HRD information for means of extraction of a bitstream that is created from an input bitstream by operation of the sub-bitstream extraction and reposition process with the input bitstream, a target nuh_layer_id value lIdTarget, a target highest TemporalId value, and a target sub-picture list including the identifier, location and size of the sub-pictures as inputs. In Figure 1 example, an output picture can be formed from merged sub-pictures from layer 0 and layer 1, or from merged sub-picture from layer 0 and layer 1.

## Proposed changes of video parameter set

Table 1 is the proposed output subpicture sets syntax based on the proposal JVET_O0141. A few updates are highlighted in yellow.

Table 1 Proposed Video Parameter Set syntax

| output_sub_pic_sets( ) { | Descriptor |
| --- | --- |
| num_osps_format_sets_minus1 | ue(v) |
| for( i = 0; i <= num_osps_format_sets _minus1 ; i++ ){ |  |
| osps_width_in_luma_samples[ i ] | ue(v) |
| osps_height_in_luma_samples[ i ] | ue(v) |
| profile_tier_level(sps_max_sub_layers_minus1)[ i ] |  |
| } |  |
| num_osps_minus1 | ue(v) |
| for( i = 0; i <= num_osps_minus1 ; i++ ) { |  |
| num_subpics_in_osps_minus1[ i ] | ue(v) |
| osps_format_set_idx[ i ] | ue(v) |
| for( j = 0; j <= num_subpics_in_osps_minus1[ i ] ; j++ ) { |  |
| if( vps_max_layers_minus1 != 0) |  |
| subpic_layer_id_in_osps[ i ][ j ] | u(7) |
| num_subpic_slices_in_osps_minus1[ i ][ j ] |  |
| for (k = 0; k <= num_subpic_slices_in_osps_minus1[ i ][ j ]; k++) { |  |
| subpic_slice_id _in_osps[ i ][ j ][ k ] | ue(v) |
| } |  |
| subpic_x_offset_in_osps[ i ][ j ] | ue(v) |
| subpic_y_offset_in_osps[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

The semantic of new proposed syntax elements is defined as follows.

osps_present_flag equal to 1 specifies that output sub-picture sets are present. osps_present_flag equal to 0 specifies that no output sub-picture sets are present.

num_osps_format_sets specifies the number of output sub-picture set format sets present. The value of num_osps_format_sets shall be in the range of 1 to 512. When not present, num_osps_format_sets is inferred to be equal to 0.

osps_width_in_luma_samples[ i ] specifies the width in luma samples of the i-th OSPS format.

osps_height_in_luma_samples[ i ] specifies the height in luma samples of the i-th OSPS format.

profile_tier_level()[ i ] specifies the profile, tier and level of the i-th OSPS format.

num_osps_minus1 plus 1 specifies the number of OSPS present. The value of num_output_sub_pic_sets shall be in the range of 1 to 512. When not present, num_osps_minus1 is inferred to be equal to 0.

num_subpics_in_osps_minus1[ i ] plus 1 specifies the number of sub-pictures in the i-th OSPS present. The value of num_sub_pics_in_osps[ i ] shall be in the range of 1 to 512. When not present, num_sub_pics_in_osps[ i ] is inferred to be equal to 0.

osps_format_set_idx[ i ] specifies the index of osps format list.

sub_pic_layer_id_in_osps[ i ][ j ] specifies the layer identifier lIdTarget of j-th subpicture of i-th OSPS.

num_subpic_slices_in_osps_minus1[ i ][ j ] specifies the number of slices of j-th subpicture in i-th OSPS.

subpic_slice_id _in_osps[ i ][ j ][ k ] specifies the slice ID of k-th slice of j-th subpicture in i-th OSPS.

sub_pic_x_offset_in_osps[ i ][ j ] specifies the horizontal offset, in units of luma samples, of the top-left corner luma sample of the j-th sub-picture belonging to the i-th OSPS relative to the top-left corner luma sample of output picture in the CVS. When not present, the value of sub_pic_x_offset_in_osps[ i ][ j ] is inferred to be equal to 0. sub_pic_x_offset_in_osps[ i ][ j ] shall be an integer multiple of CtbSizeY.

sub_pic_y_offset_in_osps[ i ][ j ] specifies the vertical offset, in units of luma samples, of the top-left corner luma sample of the j-th sub-picture belonging to the i-th OSPS relative to the top-left corner luma sample of each picture in the CVS. When not present, the value of sub_pic_y_offset_in_osps[ i ][ j ] is inferred to be equal to 0. sub_pic_y_offset_in_osps[ i ][ j ] shall be an integer multiple of CtbSizeY.

## Proposed sub-bitstream extraction and reposition process for clause 10

Inputs to this process are a bitstream, and a target output sub-picture set subPicSetTarget.

Output of this process is a sub-bitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that is the output of the process specified in this clause with the bitstream, every lIdTarget associated with subPicSetTarget specified in the active VPS equal to any value in the range of 0 to 126, inclusive, and every tIdTarget associated with subPicSetTarget specified in the active VPS equal to any value in the range of 0 to 6, inclusive, and sIdtarget equal to sub_pic_id associated with subPicSetTarget specified in the active VPS as inputs, and that satisfies the following condition shall be a conforming bitstream:

–	The output sub-bitstream contains at least one VCL NAL unit with sub_pic_id equal to sIdTarget, TemporalId equal to tIdTarget and nuh_layer_id equal to lIdTarget.

NOTE – A conforming bitstream contains one or more coded slice NAL units with TemporalId equal to 0.

The extracted sub-bitstream is derived as follows:

–	Remove all NAL units with TemporalId greater than tIdTarget.

–	Remove all NAL units with nuh_layer_id not equal to lIdTarget.

The reposition bitstream is derived as follows:

–	Merge the NAL units of extracted sub-bitstreams in the order as specified in subPicSetTarget.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).