| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0134 |
| --- | --- |

| Title: | AHG17: On long-term reference picture signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Ma Haitao Yang | Tel: Email: | maxiang6@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

It is proposed to modify the semantic of long_term_ref_pics_flag in SPS, considering each inter-layer reference picture is marked as "used for long-term reference" in the decoding process. It is also proposed to allow updating the long-term reference picture position in the corresponding reference picture list in the slice header, without re-sending the syntax structures ref_pic_list_struct( listIdx, rplsIdx ).

Introduction

Propose 1: semantic modification of long_term_ref_pics_flag

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | u(4) |
| long_term_ref_pics_flag | u(1) |
| inter_layer_ref_pics_present_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| … |  |

The current semantics of long_term_ref_pics_flag and inter_layer_ref_pics_present_flag are as follows:

long_term_ref_pics_flag equal to 0 specifies that no LTRP is used for inter prediction of any coded picture in the CVS. long_term_ref_pics_flag equal to 1 specifies that LTRPs may be used for inter prediction of one or more coded pictures in the CVS.

inter_layer_ref_pics_present_flag equal to 0 specifies that no ILRP is used for inter prediction of any coded picture in the CVS. inter_layer_ref_pics_flag equal to 1 specifies that ILRPs may be used for inter prediction of one or more coded pictures in the CVS. When sps_video_parameter_set_id is equal to 0, the value of inter_layer_ref_pics_present_flag is inferred to be equal to 0..

When there is an ILRP, the referred picture also is marked as "used for long-term reference", according to the reference picture marking process as follows:

8.3.3 Decoding process for reference picture marking

This process is invoked once per picture, after decoding of a slice header and the decoding process for reference picture list construction for the slice as specified in clause 8.3.2, but prior to the decoding of the slice data. This process may result in one or more reference pictures in the DPB being marked as "unused for reference" or "used for long-term reference".

A decoded picture in the DPB can be marked as "unused for reference", "used for short-term reference" or "used for long-term reference", but only one among these three at any given moment during the operation of the decoding process. Assigning one of these markings to a picture implicitly removes another of these markings when applicable. When a picture is referred to as being marked as "used for reference", this collectively refers to the picture being marked as "used for short-term reference" or "used for long-term reference" (but not both).

STRPs and ILRPs are identified by their nuh_layer_id and PicOrderCntVal values. LTRPs are identified by their nuh_layer_id values and the Log2( MaxLtPicOrderCntLsb ) LSBs of their PicOrderCntVal values.

If the current picture is a CLVSS picture, all reference pictures currently in the DPB (if any) with the same nuh_layer_id as the current picture are marked as "unused for reference".

Otherwise, the following applies:

- For each LTRP entry in RefPicList[ 0 ] or RefPicList[ 1 ], when the referred picture is an STRP with the same nuh_layer_id as the current picture, the picture is marked as "used for long-term reference".

- Each reference picture with the same nuh_layer_id as the current picture in the DPB that is not referred to by any entry in RefPicList[ 0 ] or RefPicList[ 1 ] is marked as "unused for reference".

- For each ILRP entry in RefPicList[ 0 ] or RefPicList[ 1 ], the referred picture is marked as "used for long-term reference".

So, when inter_layer_ref_pics_present_flag equal to1, there may be an ILRP which is used in decoding process, and it is marked as "used for long-term reference". In this case, there is an LTRP is used in decoding process, even the long_term_ref_pics_flag equal to 0. So there is an inconsistent with the semantic of long_term_ref_pics_flag.

Propose 2: enabling updating LTRP position in slice header

In current VVC draft specification, the LTRP information is signaled in reference picture list syntax structure or slice header.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_sps_flag[ i ] | u(1) |
| if( ref_pic_list_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && |  |
| ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| slice_poc_lsb_lt[ i ][ j ] | u(v) |
| delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( delta_poc_msb_present_flag[ i ][ j ] ) |  |
| delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| … |  |

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( long_term_ref_pics_flag ) |  |
| ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( inter_layer_ref_pics_present_flag ) |  |
| inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( !inter_layer_ref_pics_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| if( long_term_ref_pics_flag ) |  |
| st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| if( AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] > 0 ) |  |
| strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } else if( !ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] ) |  |
| rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ j++ ] | u(v) |
| } else |  |
| ilrp_idc[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| } |  |
| } |  |

So, the long-term reference picture POC LSB can be signaled either in RPL or slice header. But the position only can be updated via RPL.

When the LTRP POC need to be updated, it can be achieved by signaling the POC information only in slice header, without signaling a whole reference picture list syntax structure. But when only LTRP position in the associated RPL need to be updated, a whole reference picture list syntax structure still need to be signaled, which will increase the bits cost.

Fig.1 Sequence with scene cut

For example, as Fig.1 shown, there is a sequences with two scenes, and the two scenes appear alternatively. In the start stage of the sequence, two LTRP (LtrpA, and LtrpB) are constructed and kept in DPB for reference for the later picture coding. When coding a picture belong to sceneA, LtrpA should be used for reference, and it need to be put in the front position in the reference picture list, and LtrpB should be in a later position, to save reference picture index coding bits for the coding blocks. While when coding a picture belong to sceneB, LtrpB should be used for reference, with a front position in the reference picture list, and LtrpA should be put in a later position. Here note that the POC information of LtrpA and LtrpB are not changed, only the position need to be updated. But current VVC draft specification must re-sending a whole reference picture list structure, since it cannot update the LTRP position only.

Proposal text

Propose 1: semantic modification of long_term_ref_pics_flag

Since long_term_ref_pics_flag only used to control parsing of ltrp_in_slice_header_flag and st_ref_pic_flag, the semantic is modified as follows:

long_term_ref_pics_flag equal to 1, specifies that ltrp_in_slice_header_flag, and st_ref_pic_flag are present in the syntax structures ref_pic_list_struct( listIdx, rplsIdx ). long_term_ref_pics_flag equal to 0 specifies that these syntax elements are not present in the syntax structures ref_pic_list_struct( listIdx, rplsIdx ). equal to 0 specifies that no LTRP is used for inter prediction of any coded picture in the CVS. long_term_ref_pics_flag equal to 1 specifies that LTRPs may be used for inter prediction of one or more coded pictures in the CVS.

Alternatively, the sematic can be modified as follows:

long_term_ref_pics_flag equal to 0 specifies that no LTRP is used for inter prediction of any coded picture in the CVS. long_term_ref_pics_flag equal to 1 specifies that LTRPs may be used for inter prediction of one or more coded pictures in the CVS. Here the LTRP excluding ILRP.

Propose 2: enabling updating LTRP position in slice header

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| long_term_ref_pics_flag | u(1) |
| if( long_term_ref_pics_flag ) |  |
| sps_ltrp_pos_override_flag | u(1) |
| inter_layer_ref_pics_present_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| rpl1_same_as_rpl0_flag | u(1) |
| … |  |

sps_ltrp_pos_override_flag equal to 1, specifies that ltrp_pos_override_flag[ i ] are present in slice header. sps_ltrp_pos_override_flag equal to 0 specifies that ltrp_pos_override_flag[ i ] are not present in slice header. When not prenent, the value of sps_ltrp_pos_override_flag is infreed to be equal to 0.

| slice_header( ) { |  |
| --- | --- |
| … |  |
| if( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( sps_ltrp_pos_override_flag ) |  |
| ltrp_pos_override_flag[ i ] | u(1) |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_sps_flag[ i ] | u(1) |
| if( ref_pic_list_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && |  |
| ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| slice_poc_lsb_lt[ i ][ j ] | u(v) |
| delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( delta_poc_msb_present_flag[ i ][ j ] ) |  |
| delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| if( ltrp_pos_override_flag[ i ] ) |  |
| ltrp_pos_in_ref_pic_list[ i ][ j ] | u(v) |
| } |  |
| } |  |
| … |  |

ltrp_pos_override_flag[ i ] equal to 1, specifies there maybe a LTRP position need to be overrided in the contructured reference picture list RefPicList[ i ]. ltrp_pos_override_flag[ i ] equal to 0, specifies there is no LTRP position need to be overrided in the contructured reference picture list RefPicList[ i ]. When not prenent, the value of ltrp_pos_override_flag[ i ] is infreed to be equal to 0.

ltrp_pos_in_ref_pic_list[ i ][ j ] specials the j-th LTRP position in the associated constructed reference picture list RefPicList[ i ]. The value of ltrp_pos_in_ref_pic_list[ i ][ j ] shall be in the range of 0 to num_ref_entries[ i ][ rplsIdx ] − 1, inclusive. For any two non-negative integer values of m and n, when m is dfferent from n, the value of ltrp_pos_in_ref_pic_list[ i ][m ] shall not be the same as ltrp_pos_in_ref_pic_list[ i ][ n ].

for( i = 0; i < 2; i++ ) {
		for( j = 0, k = 0, pocBase = PicOrderCntVal; j < num_ref_entries[ i ][ RplsIdx[ i ] ]; j++) {
			if( !inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
				if( st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
					RefPicPocList[ i ][ j ] = pocBase − DeltaPocValSt[ i ][ RplsIdx[ i ] ][ j ]
					if( there is a reference picture picA in the DPB with the same nuh_layer_id as the current picture
								and 	PicOrderCntVal equal to RefPicPocList[ i ][ j ] )
						RefPicList[ i ][ j ] = picA
					else
						RefPicList[ i ][ j ] = "no reference picture"				(8-5)
					pocBase = RefPicPocList[ i ][ j ]
				} else {
					if( !delta_poc_msb_cycle_lt[ i ][ k ] ) {
						if( there is a reference picA in the DPB with the same nuh_layer_id as the current picture and
									PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ) equal to PocLsbLt[ i ][ k ] )
							RefPicList[ i ][ j ] = picA
						else
							RefPicList[ i ][ j ] = "no reference picture"
						RefPicLtPocList[ i ][ j ] = PocLsbLt[ i ][ k ]
					} else {
						if( there is a reference picA in the DPB with the same nuh_layer_id as the current picture and
									PicOrderCntVal equal to 	FullPocLt[ i ][ k ] )
							RefPicList[ i ][ j ] = picA
						else
							RefPicList[ i ][ j ] = "no reference picture"
						RefPicLtPocList[ i ][ j ] = FullPocLt[ i ][ k ]
				}
					k++
			}
			} else {
				layerIdx = 	DirectDependentLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ ilrp_idc[ i ][ RplsIdx ][ j ] ]
				refPicLayerId = vps_layer_id[ layerIdx ]
				if( there is a reference picture picA in the DPB with nuh_layer_id equal to refPicLayerId and
							the same PicOrderCntVal as the current picture )
					RefPicList[ i ][ j ] = picA
				else
					RefPicList[ i ][ j ] = "no reference picture"
		}
	}
}

for( i = 0; i < 2; i++ ) {
		if( ltrp_pos_override_flag[ i ]) {
			for( j = 0, t = 0; j < num_ref_entries[ i ][ RplsIdx[ i ] ]; j++) {
				if( !inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] && !st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ]) 				{
					TempRefPicList[ i ][ ltrp_pos_in_ref_pic_list[ i ][ t ] ] = RefPicList[ i ][ j ]
					TempRefPicLtPocList[ i ][  ltrp_pos_in_ref_pic_list[ i ][ t ] ] = RefPicLtPocList[ i ][ j ]
					LtRefPicFlag[ i ][ RplsIdx[ i ] ][ ltrp_pos_in_ref_pic_list[ i ][ t ] ] = 1
					StRefPicFlag[ i ][ RplsIdx[ i ] ][ ltrp_pos_in_ref_pic_list[ i ][ t ] ] = 0
					t++
				}
			}
			for (j = 0, t = 0; j < num_ref_entries[ i ][ RplsIdx[ i ] ]; j++) {
				if(!LtRefPicFlag[ i ][ RplsIdx[ i ] ][ j ])
					TempNoLtRefPicPos[ t ] = j
					t++
				}
			for( j = 0, t = 0; j < num_ref_entries[ i ][ RplsIdx[ i ] ]; j++) {
				if( inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] | | st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
					TempRefPicList[ i ][ TempNoLtRefPicPos[ t ] ] = RefPicList[ i ][ j ]
						if(st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ]) {
							TempRefPicPocList[ i ][ TempNoLtRefPicPos[ t ] ] = RefPicPocList[ i ][ j ]
							StRefPicFlag[ i ][ RplsIdx[ i ] ][ TempNoLtRefPicPos[ t ] ] = 1
					} 
				t++
				}
			}
		RefPicList[ i ] = TempRefPicList[ i ]
		RefPicPocList[ i ] = TempRefPicPocList[ i ]
		RefPicLtPocList[ i ] = TempRefPicLtPocList[ i ]
		} else {
			StRefPicFlag = st_ref_pic_flag
		}
}

      - General decoding process for generating unavailable reference pictures

This process is invoked once per coded picture when the current picture is a CRA picture with NoIncorrectPicOutputFlag equal to 1 or a GDR picture with NoIncorrectPicOutputFlag equal to 1.

When this process is invoked, the following applies:

- For each 	RefPicList[ i ][ j ], with i in the range of 0 to 1, inclusive, and j in the range of 0 to num_ref_entries[ i ][ RplsIdx[ i ] ] − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause 8.3.4.2 and the following applies:

- The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id of the current picture.

- If st_ref_pic_flag StRefPicFlag[ i ][ RplsIdx[ i ] ][ j ] is equal to 1, the value of PicOrderCntVal for the generated picture is set equal to RefPicPocList[ i ][ j ] and the generated picture is marked as "used for short-term reference".

- Otherwise (st_ref_pic_flag StRefPicFlag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0), the value of PicOrderCntVal for the generated picture is set equal to RefPicLtPocList[ i ][ j ], the value of slice_pic_order_cnt_lsb for the generated picture is inferred to be equal to ( RefPicLtPocList[ i ][ j ] & ( MaxPicOrderCntLsb – 1 ) ), and the generated picture is marked as "used for long-term reference".

- The value of PicOutputFlag for the generated reference picture is set equal to 0.

- RefPicList[ i ][ j ] is set to be the generated reference picture.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).