| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, SE, 1–11 October 2019 | Document: JVET-P0136-v1 |
| --- | --- |

| Title: | Non-CE3: MIP Simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Haixin Wang, Yu Sun Yanzhuo Ma, Shuai Wan, Fuzheng Yang Yuanfang Yu Yang Liu | Email: | jyhuo@mail.xidian.edu.cn hxwang_2@stu.xidian.edu.cn ysun_xd@stu.xidian.edu.cn yzma@mail.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

Proposed is to simplify the 2D fO table with 34 elements to a 1D table with 3 values. The experimental results are as below:

For AI configuration: 0.00%, -0.01%, and -0.01%, with 100% EncT, 100% DecT;

For RA configuration: 0.00 %, 0.00%, and -0.02%, with 100% EncT, 100% DecT.

Introduction

In VTM6.0, matrix-based intra prediction (MIP) is applied to derive the intra predicted samples of luma component as：

predMip[ x ][ y ] = ( ( ( i = 0inSize - 1mWeight i  y*incH*predC+x*incW *p i  ) + ( 1 << ( sW − 1 ) ) − fO * (i = 0inSize - 1p i ) )  >> sW ) + pTemp[ 0 ]

Input value p[i] is calculated by:

- If MipSizeId[ xTbCmp ][ yTbCmp ] is equal to 2, the following applies:

p[ i ] = pTemp[ i + 1 ] − pTemp[ 0 ]

- Otherwise (MipSizeId[ xTbCmp ][ yTbCmp ] is less than 2), the following applies:

p[ 0 ] = pTemp[ 0 ] − ( 1 << ( BitDepthY − 1 ) )
p[ i ] = pTemp[ i ] − pTemp[ 0 ]	for x = 1.. inSize − 1

wherein fO is determined using Table 1 as below:

Table 1 –table of fO in VVC CD[1]

|  | modeId |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MipSizeId | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| 0 | 34 | 19 | 7 | 32 | 27 | 24 | 21 | 13 | 24 | 15 | 27 | 20 | 16 | 7 | 20 | 23 | 21 | 24 |
| 1 | 17 | 20 | 11 | 21 | 17 | 11 | 23 | 10 | 21 | 11 |  |  |  |  |  |  |  |  |
| 2 | 8 | 46 | 16 | 10 | 13 | 11 |  |  |  |  |  |  |  |  |  |  |  |  |

Proposed Methods

When MipSizeId is equal to 0 or 1, it is proposed to change the calculation of p[0] from

p[ 0 ] = pTemp[ 0 ] − ( 1 << ( BitDepthY − 1 ) )
to

p[ 0 ] = ( 1 << ( BitDepthY − 1 ) ) − pTemp[ 0 ]

Mathematically, sW, fO, and mWeight are also changed correspondingly to make the calculation of predMip[ x ][ y ] with the new p[0] identical to the one in VVC CD. Specifically, Table 2 is a new table for determine fO:

Table 2 – table of fO

|  | modeId |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MipSizeId | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| 0 | 34 | 19 | 7 | 32 | 27 | 24 | 21 | 13 | 24 | 15 | 27 | 20 | 16 | 7 | 20 | 23 | 21 | 24 |
| 1 | 17 | 20 | 11 | 21 | 17 | 11 | 23 | 10 | 21 | 11 |  |  |  |  |  |  |  |  |
| 2 | 8 | 46 | 16 | 10 | 13 | 11 |  |  |  |  |  |  |  |  |  |  |  |  |

It is observed from Table 2 that the dynamic range of fO in Table 2 is smaller than that in Table 1. Thus, the fO values in each row in Table 2 can be approximately represented by the largest fO value of each row without introducing much error. As shown in Table 3, 2D fO table with 34 elements is simplified to a 1D table with 3 values.

Table 3 – simplified table of fO

| MipSizeId | offset factor |
| --- | --- |
| 0 | 34 |
| 1 | 23 |
| 2 | 46 |

Experimental results

The experiments were performed based on VTM 6.0 software [2] under JVET common test condition [3]. Table 4 presents a summary of the results.

Table 4. Results of the proposed method over VTM6.0

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | 0.05% | 100% | 100% |
| Class A2 | 0.01% | 0.04% | -0.03% | 101% | 101% |
| Class B | 0.00% | 0.01% | -0.03% | 100% | 100% |
| Class C | 0.01% | -0.15% | 0.01% | 100% | 100% |
| Class E | 0.00% | 0.04% | -0.03% | 99% | 101% |
| Overall | 0.00% | -0.01% | -0.01% | 100% | 100% |
| Class D | -0.01% | -0.07% | -0.12% | 100% | 100% |
| Class F | 0.01% | -0.01% | -0.02% | 100% | 101% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.11% | 0.04% | 100% | 100% |
| Class A2 | 0.01% | 0.08% | 0.03% | 100% | 100% |
| Class B | -0.01% | 0.09% | -0.09% | 100% | 100% |
| Class C | -0.01% | -0.09% | -0.02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.02% | 100% | 100% |
| Class D | 0.01% | -0.21% | -0.27% | 100% | 100% |
| Class F | -0.03% | 0.10% | -0.10% | 101% | 101% |

Conclusions

In this contribution, the 2D fO table with 34 values is simplified as a 1D table with 3 values, without loss in overall coding efficiency. It is recommended to adopt the simplification into VVC CD.

References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019.

[2] https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-6.0

[3] F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-O2010, Gothenburg, July 2019.

Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).