| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, SE, 1–11 October 2019 | Document: JVET-P0137-v1 |
| --- | --- |

| Title: | Non-CE4: On coding of merge triangle index |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Haixin Wang, Yanzhuo Ma, Shuai Wan, Fuzheng Yang Yuanfang Yu Yang Liu | Email: | jyhuo@mail.xidian.edu.cn hxwang_2@stu.xidian.edu.cn yzma@mail.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

Proposed is a method to apply bypass decoding to all the bins of merge_triangle_idx0 and merge_triangle_idx1. Experimental results are as below:

For RA configuration: 0.01%, 0.06%, and 0.03%, with 100% EncT, 100% DecT;

For LB configuration: -0.05%, 0.26%, and 0.03%, with 100% EncT, 100% DecT.

Introduction

With TPM mode, a CU is split into two triangle-shaped partitions, i.e. diagonal split or anti-diagonal split. A flag (merge_triangle_split_dir) is signalled to indicate the triangle partition type, and two merge indices (merge_triangle_idx0 and merge_triangle_idx1) are signalled.

merge_triangle_idx0 and merge_triangle_idx1 are coded as follows: the first bin is coded with a context model and the other bins are bypass bins. The first bin shares the context model with the first bin of merge_idx.

Proposed Method

It is proposed to code all the bins of merge_triangle_idx0 and merge_triangle_idx1 as bypass bins. Based on VVC CD [1], modified text is as follow:

| Table 9-17 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| merge_idx[ ][ ] | 0 | bypass | bypass | bypass | bypass | na |
| merge_triangle_split_dir[ ][ ] | bypass | na | na | na | na | na |
| merge_triangle_idx0[ ][ ] | 0 bypass | bypass | bypass | bypass | bypass | na |
| merge_triangle_idx1[ ][ ] | 0 bypass | bypass | bypass | bypass | na | na |

Results

The experiments were performed based on VTM 6.0 software [2] under JVET common test condition [3]. Table below presents a summary of the results.

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.06% | 0.00% | 100% | 100% |
| Class A2 | 0.03% | 0.11% | -0.02% | 100% | 100% |
| Class B | 0.00% | 0.11% | 0.02% | 100% | 100% |
| Class C | 0.00% | 0.06% | 0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.06% | 0.03% | 100% | 100% |
| Class D | -0.06% | -0.07% | -0.02% | 100% | 100% |
| Class F | -0.02% | 0.00% | 0.00% | 101% | 100% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.08% | 0.02% | 99% | 100% |
| Class C | -0.06% | 0.16% | -0.03% | 100% | 100% |
| Class E | -0.11% | 0.97% | 0.59% | 99% | 99% |
| Overall | -0.05% | 0.26% | 0.15% | 100% | 100% |
| Class D | 0.01% | 0.33% | 0.44% | 100% | 99% |
| Class F | 0.04% | 0.00% | 0.56% | 100% | 99% |

Conclusion

In this contribution, all bins of the merge_triangle_idx0 and merge_triangle_idx1 flag are bypass coded, which avoids sharing context with the merge_idx. It is recommended to adopt the method into VVC CD.

References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019.

[2] https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-6.0

[3] F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-O2010, Gothenburg, July 2019.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).