| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0140-v2 |
| --- | --- |

| Title: | AHG17/AHG12: Some comments on the VVC text |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Chih-Wei Hsu, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The comments and suggested changes are summarized as:

- A subpicture is a rectangular region of a one or more rectangular slices within a picture. When subpics_present_flag is equal to 1, the value of rect_slice_flag shall be equal to 1.

- max_subpics_minus1 is changed to max_subpics_minus2 and added in the VPS. Adding num_subpics_minus2 in the SPS.

- Remove signalling num_tiles_in_pic_minus1 for the uniform tile spacing in the PPS.

- Add i > 0 for brick_idx_delta_sign_flag[ i ]. Correct the derivation of the bottom right brick index of the i-th slice.

# Introduction

The relevant elements of the SPS and the PPS in the current VVC draft [1] for discussion in this document are copied as below.

      - 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| output_flag_present_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| if( uniform_tile_spacing_flag && brick_splitting_present_flag ) |  |
| num_tiles_in_pic_minus1 | ue(v) |
| for( i = 0; brick_splitting_present_flag && i <= num_tiles_in_pic_minus1 + 1; i++ ) { |  |
| if( RowHeight[ i ] > 1 ) |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| if( RowHeight[ i ] > 2 ) |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus2[ i ] | ue(v) |
| for( j = 0; j <= num_brick_rows_minus2[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| bottom_right_brick_idx_length_minus1 | ue(v) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| brick_idx_delta_sign_flag[ i ] | u(1) |
| } |  |
| } |  |
| loop_filter_across_bricks_enabled_flag | u(1) |
| if( loop_filter_across_bricks_enabled_flag ) |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( rect_slice_flag ) { |  |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| slice_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Proposal

In the current VVC draft, it defines that a subpicture is a rectangular region of one or more slices within a picture. Some comments are bulleted as below.

- This contribution suggests adding a constraint on it that a subpicture is a rectangular region of a one or more rectangular slices within a picture. Although technically the raster scan slices may form a subpicture, it is not desirable, e.g. the picture width such as 360 video may be large enough that practically no subpicture structures may make them useful for the use cases. Separating the raster scan slices from the subpicture structures makes sense to the proponents and it would be more straightforward from the syntax point of view.

- When subpictures are present in a picture, the number of the subpictures are greater than or equal to 2, thus max_subpics_minus1 can be changed to max_subpics_minus2.

- When the picture sizes are added in the PPS, signalling num_tiles_in_pic_minus1 for the uniform tile spacing is no longer needed in the PPS.

- For the first slice, brick_idx_delta_sign_flag[ 0 ] is positive. The derivation of the bottom right brick index of the i-th slice is corrected.

The proposed text changes are as follows (highlighted in yellow).

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus12 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

subpics_present_flag equal to 1 indicates that subpicture parameters are present in the present in the SPS RBSP syntax. When subpics_present_flag is equal to 1, the value of rect_slice_flag shall be equal to 1. subpics_present_flag equal to 0 indicates that subpicture parameters are not present in the present in the SPS RBSP syntax.

NOTE 2 – When a bitstream is the result of a sub-bitstream extraction process and contains only a subset of the subpictures of the input bitstream to the sub-bitstream extraction process, it might be required to set the value of subpics_present_flag equal to 1 in the RBSP of the SPSs.

max_subpics_minus2 plus 2 specifies the maximum number of subpictures that may be present in the CVS. max_subpics_minus2 shall be in the range of 0 to 254. The value of 255 is reserved for future use by ITU-T | ISO/IEC.

The corresponding semantics for rect_slice_flag in the PPS may be modified as:

rect_slice_flag equal to 0 specifies that bricks within each slice are in raster scan order and the slice information is not signalled in PPS. rect_slice_flag equal to 1 specifies that bricks within each slice cover a rectangular region of the picture and the slice information is signalled in the PPS. When subpics_present_flag is equal to 1, the value of rect_slice_flag shall be equal to 1. When brick_splitting_present_flag is equal to 1, the value of rect_slice_flag shall be equal to 1. When not present, rect_slice_flag is inferred to be equal to 1.

It is also suggested that the maximum number of subpictures may be specified in the VPS instead and it is present as the number of subpictures in the SPS.

| Video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(6) |
| vps_max_subpics_minus2 | u(8) |
| if( vps_max_layers_minus1 > 0 ) |  |
| vps_all_independent_layers_flag | u(1) |
| … |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

vps_max_subpics_minus2 plus 2 specifies the maximum allowed number of subpictures in each CVS referring to the VPS. vps_max_subpics_minus2 shall be in the range of 0 to 254. The value of 255 is reserved for future use by ITU-T | ISO/IEC.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| nummax_subpics_minus12 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| … |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

num_subpics_minus2 plus 2 specifies the number of subpictures that may be present in the CVS. num_subpics_minus2 shall be in the range of 0 to 254. The value of 255 is reserved for future use by ITU-T | ISO/IEC.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| … |  |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| if( uniform_tile_spacing_flag && brick_splitting_present_flag ) |  |
| num_tiles_in_pic_minus1 | ue(v) |
| … |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag && !single_brick_per_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| bottom_right_brick_idx_length_minus1 | ue(v) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| bottom_right_brick_idx_delta[ i ] | u(v) |
| if( i > 0 ) |  |
| brick_idx_delta_sign_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |
| if( rect_slice_flag ) { |  |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| slice_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The derivation of the bottom right brick index of the i-th slice should be

for( j = 0; i  = =  0  &&  j < NumBricksInPic; j++ )
	BricksToSliceMap[ j ] = −1
NumBricksInSlice[ i ] = 0
BottomRightBrickIdx[ i ] = ( i  = =  0 ) ? bottom_right_brick_idx_delta[ i ] : 
      BottomRightBrickIdx[ i−1 ] + ( brick_idx_delta_sign_flag[ i ] ? 
          bottom_right_brick_idx_delta[ i ] : −bottom_right_brick_idx_delta[ i ])
for( j = BottomRightBrickIdx[ i ]; j  >=  0; j− − ) {
  if( BrickColBd[ j ]  <=  BrickColBd[ BottomRightBrickIdx[ i ] ]  &&	                (7-43)
		 BrickRowBd[ j ]  <=  BrickRowBd[ BottomRightBrickIdx[ i ] ]  &&						 BricksToSliceMap[ j ]  = =  −1 ) {
	 TopLeftBrickIdx[ i ] = j
     NumBricksInSlice[ i ]++
	 BricksToSliceMap[ j ] = i
  }
}

# Conclusions

The comments and suggested changes are summarized as:

- A subpicture is a rectangular region of a one or more rectangular slices within a picture. When subpics_present_flag is equal to 1, the value of rect_slice_flag shall be equal to 1.

- max_subpics_minus1 is changed to max_subpics_minus2 and added in the VPS.  Adding num_subpics_minus2 in the SPS.

- Remove signalling num_tiles_in_pic_minus1 for the uniform tile spacing in the PPS.

- Add i > 0 for brick_idx_delta_sign_flag[ i ]. Correct the derivation of the bottom right brick index of the i-th slice.

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).