| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0153-v2 |
| --- | --- |

| Title: | CE4-related: Overlapped block optical flow |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Ling Hsiao, Yu-Cheng Lin, Chun-Chia Chen, Chih-Wei Hsu, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yuling.hsiao, yucheng.lin, chunchia.chen, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to extend the prediction refinement with optical flow process to the above and left boundaries of inter coding blocks. The proposed method, overlapped block optical flow (OBOF), reuses the same processes in bi-directional optical flow (BDOF) and prediction refinement with optical flow (PROF) and only changes the sample-level motion vector (MV) offset to be the MV differences between the current block and neighbouring blocks. Results are summarized as follows.

VTM6.0-RA: {Y-BD-rate = -0.09%, EncT = 107%, DecT = 108%}
VTM6.0-LB: {Y-BD-rate = -0.31%, EncT = 108%, DecT = 111%}

# Introduction

In JVET-O0581 [1], block boundary prediction refinement with optical flow (BBPROF) for decoder side motion vector refinement (DMVR) was proposed. It uses the differences between the refined MVs in the current subblock and four neighboring subblocks to calculate the prediction refinement for the current subblock by optical flow process. Following the concept in JVET-O0581, it is proposed to apply BBPROF to above and left boundaries of inter coding blocks. It can be taken as an extension of existing optical flow tools, such as prediction refinement with optical flow (PROF) that only applies to affine coding units (CUs) and bi-directional optical flow (BDOF) that only applies to bi-prediction CUs with one reference picture before the current picture and the other reference picture after the current picture. With the proposed method, it is expected to reduce motion discontinuities among inter coding block boundaries so as to improve coding efficiency without introducing extra memory bandwidth of reference samples as required in overlapped block motion compensation (OBMC). Details are described in the following sections.

# Proposed method

The proposed method is called overlapped block optical flow (OBOF), which is applied to inter prediction units (PUs) to refine the block boundary samples with optical flow. One example is shown in Figure 1, the MV difference used by the optical flow are calculated from the current 4x4 subblock and the above (or left) 4x4 subblock. OBOF is only applied to the upper half and left half parts of the current PU. As PROF and BDOF, only luma prediction samples are refined by OBOF.

Figure 1. MV difference calculations between the current 4x4 subblock and neighboring 4x4 subblocks

More details of OBOF are described in the following steps:

Step 1: To generate sample prediction, Ii, j, the motion compensation (MC) of one PU is performed with one prediction sample padding on each side of the MC block for the gradient calculation. The MC and padding process of OBOF are the same as those of BDOF.

Step 2: The spatial gradients gx(i,j) and gy(i,j) of the prediction samples are calculated using a 3-tap filter [-1, 0, 1], which is the same as PROF and BDOF.

gxi, j=Ii+1, j-Ii-1, j

gyi, j=Ii, j+1-Ii, j-1

Step 3: The MV difference, MVdiff, between the current 4x4 subblock and the upper or left 4x4 subblock is calculated. It is used to derive a sample-level MV offset, Δv(i,j), in which “i” means horizontal sample coordinate and “j” means vertical sample coordinate, respectively, and (i, j) is (0,0) for the left-top sample in current PU.

MVAdiffi=MVnei(i,-1)-MVcurMVLdiff(j)=MVnei(-1,j)-MVcur

Δvai,j=wnj*MVAdiff(i), if j<32Δvli,j=wni*MVLdiffjΔvai,j=0, otherwise

Δvli,j=wni*MVLdiffj, if i<32Δvli,j=0, otherwise

Δvi,j=Δvai,j+Δvli,j

If the neighbouring MV is not available, the MV difference is set to zero. If the MV difference is larger than a threshold, the MV difference is forced to zero. The wn(j) and wn(i) are weighting factors determined by coordinates, and n is adaptively selected according to the PU height and PU width as shown in Equation 1 and Equation 2. Take wn(j) as example, if PU height is equal to 8 luma samples and PU width is equal to 8 luma samples, it will use w4[] to represent wn(j), but if PU height is equal to 4 luma samples and PU width is equal to 8 luma samples, it will use w2[] to represent wn(j). The same idea is applied to wn(i). The precision and range of sample-level MV offset are aligned with BDOF, which are 1/64-sample precision and in the range from -32 to 31.

Equation 1. Relation between n and PU height and width

n=min⁡(32,minPU height, PU width≫1)

Equation 2. Weights for MV difference

wni=n-in, if i<nwni=0, otherwise

Step 4: The luma prediction refinement is calculated based on the basic optical flow equation (as in BDOF and PROF) and added to the sub-block prediction Ii, j.

ΔI(i,j)= gxi,j*Δvx(i,j)+gyi,j*Δvy(i,j)

I'(i,j)= Ii,j+ΔI(i,j)

OBOF is applied to PUs which are translational uni-prediction, translational bi-prediction with bi-prediction with CU level weights index, translational bi-prediction with equal weight, triangle prediction mode, all 8x8 subblocks in subblock-based temporal merging candidate, and bi-prediction with only BDOF enabled.

As described above, the proposed OBOF is basically the same as BDOF or PROF except the sample-based MV offset calculation method. Therefore, OBOF can be regarded as one extension of the existing optical flow tools in VVC, such as BDOF and PROF. For hardware implementation, OBOF can be easily achieved by reusing most parts of BDOF and PROF designs, such as boundary padding, gradient calculation and sample refinement calculation. Since optical flow is supported by the hardware, OBOF can be used as a good candidate for coding gain improvement when BDOF is not applicable, e.g., low delay cases, or an alternative coding tool with less complexity compared with BDOF.

# Experiment results

The proposed OBOF has been implemented on top of VTM6.0. The common test conditions [2] are used for evaluation. Table 1 summarizes the results of the proposed OBOF.

Table 1. Results of the proposed OBOF

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.01% | -0.07% | 106% | 109% |
| Class A2 | -0.03% | 0.02% | -0.06% | 106% | 109% |
| Class B | -0.10% | 0.07% | -0.02% | 108% | 108% |
| Class C | -0.15% | -0.18% | -0.20% | 107% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.02% | -0.09% | 107% | 108% |
| Class D | -0.20% | -0.05% | -0.21% | 107% | 108% |
| Class F | -0.03% | -0.05% | -0.05% | 106% | 115% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.19% | -0.32% | -0.27% | 108% | 108% |
| Class C | -0.24% | -0.41% | -0.03% | 108% | 108% |
| Class E | -0.60% | 0.16% | -0.61% | 110% | 118% |
| Overall | -0.31% | -0.23% | -0.27% | 108% | 111% |
| Class D | -0.38% | -0.01% | 0.13% | 108% | 108% |
| Class F | 0.07% | 0.01% | 0.42% | 107% | 115% |

# Conclusion

In this contribution, an extension of existing optical flow tools, such as BDOF and PROF, is proposed. The prediction refinement for the above and left boundary of all inter PUs is applied by optical flow process to reduce the boundary discontinuity between two PUs.

# References

[1] W. Chen and Y. He, “Non-CE9: Block boundary prediction refinement with optical flow for DMVR,” Document of Joint Video Experts Team, JVET-O0581.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).