| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0171-v2 |
| --- | --- |

| Title: | AHG12: Modification for subpicture |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input document to JVET |  |  |
| Purpose: |  | Proposal |  |  |
| Author(s) or Contact(s): |  | Li Jingya Abe Kiyofumi Virginie Drugeon | Tel: Email: | +65 6552 5423 jingya.li@sg.panasonic.com |
| Source: |  | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes following modifications for subpictures:

  - To set minimum size of grid to minmum of CTU size.

  - As an alternative to a, to remove subpicture grid and and signal subpicture using slices.

  - As and alternative to b, to remove subpicture grid and signal subpicture using bricks.

  - As an alternative to b, to remove subpicture grid and signal subpicture using tiles.

# Introduction

At the Gothenburg meeting, subpictures were adopted (O01143) with the intent to support motion-constrained tile sets (MCTS) features. Instead of signaling MCTS using SEI messages in HEVC, subpictures are signaled in SPS RBSP to “have a more clearly specified conforming behaviors for systems and aplications” (JVET-O0141). Subpictures are defined as “An rectangular region of one or more slices within a picture”. Position and size of a subpictures are defined using grid which is defined as a multiple of 4. (JVET-O0555)

## To set minimum size of grid to minmum of CTU size.

1.1.1

In this proposed method, subpicture grid is to set in units of CTU size. By doing this, undesired behavior could be avoided that user may unintentionally create parital CTU across subpicture boundary by setting grid size below CTU size. The worst case storage for subpicture indices could also be reduced. Note that grid elements at right and bottom picture boundaries may not be a multiple of CTU size in this method.

1.1.2

A variation of this method is to signal position of top left CTU and width and height of each subpicture in units of CtbSizeY.

Example of Syntax change is shown in Table 1:

Table 1: syntax change of 1.1.2

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| log2_ctu_size_minus5 | u(2) |
| if( subpics_present_flag ) { |  |
| num_subpics_minus1 | u(8) |
| for( i = 0; i <= num_subpics_minus1; i++ ) { |  |
| subpic_ctu_addr_TL_x[i] | u(v) |
| subpic_ctu_addr_TL_y[i] | u(v) |
| subpic_width_minus1[i] | u(v) |
| subpic_height_minus1[i] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| log2_ctu_size_minus5 | u(2) |
| … |  |
| } |  |

## To signal subpicture using slices

A more straightforward approach is to define subpicture with its subset slices in PPS RBSP. In this method, subpicture indices are assigned to slices in a loop over all slices in the picture. Advantages are listed as below:

- Subpicture is defined as a group of slices, however, the syntax of subpictures are defined using a group of grid elements. This detachment is to keep the independency of SPS and PPS but creates potential interop problem that it is hard to check whether a subpicture contains incompleted slices. And an undesired user behavior could creates unconnected or nonrectangular subpictures. By defining subpicture syntax in PPS using slices, those problems could be easily eliminated.

- As the basic unit of subpicture grid element is 4x4, it takes up to 2.21MB to store subpicture indices for all grid elements for an 8K sequence. Even if size of subpicture grid is refined as in units of CTU or greater, it is still contrained by the value of least common divisor of all subpicture sizes therefore unnecessary signalling can’t avoided. By using the alternative approach that signalling subpicture indices in a slice loop, storage cost of subpicture indices could be reduced to minimum.

The second modification is to disallow raster scan slices when subpictures are present. Only rectangular type slices are enabled which makes it much easier to locate subpictures. An additional constraint is that when subpic_present_flag is true, number of subpictures shall be at least 2. An illustration of syntax change is given in shown in Table.

Table 2: syntax change of 1.2

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_subpics_present_flag | u(1) |
| if(sps_subpics_present_flag ) { |  |
| num_subpics_minus2 | u(8) |
| for( i = 0; i <= num_subpics_minus2+1; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_subpics_present_flag | u(1) |
| if( !single_brick_per_slice_flag&& !pps_subpics_present_flag) |  |
| rect_slice_flag | u(1) |
| … |  |
| if( rect_slice_flag ) { |  |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| if (pps_subpics_present_flag) { |  |
| for( i = 0; i <= num_slices_in_pic_minus1; i++){ |  |
| subpic_idx[i] | u(v) |
| } |  |
| } |  |
| } |  |
| … |  |

Figure 1: syntax change to signal subpictures using slices

## To signal subpicture using bricks

As an alternative to 1.2, another method of subpicture reprensentation is to use bricks. Using bricks helps to reduce the computational load to locate subpictures. It is also noted that by using bricks to signal subpictures, raster scan slices could be allowed when subpictures are present. An illustration of syntax change is given in table 3.

Table 3: syntax change of 1.3

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_subpics_present_flag | u(1) |
| if( sps_subpics_present_flag ) { |  |
| num_subpics_minus1 | u(8) |
| for( i = 0; i <= num_subpics_minus1; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |

| pic_parameter_set_rbsp( ) { |  | Descriptor |  |
| --- | --- | --- | --- |
| … |  |  |  |
| pps_subpics_present_flag |  | u(1) |  |
| if (pps_subpics_present_flag) { |  |  |  |
| for( i = 0; i < NumBricksInPic; i++){ |  |  |  |
| subpic_idx[i] |  | u(v) |  |
| } |  |  |  |
| } |  |  |  |
| … |  |  |  |

Figure 2: syntax change to signal subpictures using bricks

## To signal subpicture using tiles

The concept of subpictures is originated from motion-constrained tile sets (MCTS) [1] as an example shown in Figure 1, therefore it might be more straightforward to define subpictures using tiles since tiles are more close to subpictures in concept, as slices are also related to packaging. Drawback is that redundant signaling might be needed since the size of tile must be reset to be dependent on the smallest sized tile of MCTS source to support flexible subpicture sizes. However, it seems not a big issue from the very limited use cases of MCTS.

The definition of subpicture is therefore modified as:

A subpicture contains one or more complete slices and one or more complete tiles that collectively cover a rectangular region of a picture. An illustration of syntax change is given in Table 4.

Note that this proposed method disallows subpicture to be a subset of tiles. Using tiles rather than slices or bricks is also more implementation friendly to locate subpicture since tile are much easier to locate than others.

Figure 1: example of MCTS for view-port dependent application in HEVC

Table 4: syntax change of 1.4

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_subpics_present_flag | u(1) |
| if( sps_subpics_present_flag ) { |  |
| num_subpics_minus1 | u(8) |
| for( i = 0; i <= num_subpics_minus1; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |

| pic_parameter_set_rbsp( ) { |  | Descriptor |  |
| --- | --- | --- | --- |
| … |  |  |  |
| pps_subpics_present_flag |  | u(1) |  |
| if (pps_subpics_present_flag) { |  |  |  |
| for( i = 0; i <= num_tiles_in_pic_minus1; i++){ |  |  |  |
| subpic_idx[i] |  | u(v) |  |
| } |  |  |  |
| } |  |  |  |
| … |  |  |  |

Figure 3: syntax change to signal subpictures using tiles

# Additional information

Note that all proposed methods above simplifies specification text.

For 1.2-1.4, introducing pps_subpics_present_flag aims to keep the parsing independecy of SPS and PPS RBSP as suggested in JVET-O0236 adopted last meeting. Note that if such dependency is not required. (as current dependency seems already exists for the additional constraints that  pic_width_in_luma_samples and pic_height_in_luma_samples values depend on subpics_present_flag). pps_subpics_present_flag could be removed.

It might not be desirable to have a subpicture cross tile boundary (e.g., for subjective reasons and cleaner design); therefore for section 1.1-1.3, additional constraint for subpicture is added as below:

At least one of following conditions must be satisfied for a subpicture to be valid:

- The subpicture is completed contained by a tile.

- The subpicture contains complete tile(s).

# Reference

[1] R. Skupin, Y. Sanchez, C. Hellge, and T. Schierl. 2016. TileBased HEVC Video for Head Mounted Displays. In2016 IEEEInternational Symposium on Multimedia (ISM). 399–400.DOI:http://dx.doi.org/10.1109/ISM.2016.008

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).