| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0172-v1 |
| --- | --- |

| Title: | AHG16/Non-CE4: Prediction sample value clipping for PROF |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input document to JVET |  |  |
| Purpose: |  | Proposal |  |  |
| Author(s) or Contact(s): |  | Li Jingya Lim Chong Soon | Tel: Email: | +65 6552 5423 jingya.li@sg.panasonic.com |
| Source: |  | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes 2 different approaches that clip prediction sample value of PROF to 16 bit before sample weighting process:

  - Clip PROF refinement value to 14 bit so that prediction sample value before weighted prediction is within 16 bit.

  - Directly clip prediction sample value before weighted prediction to 16 bit.

Coding impact of test a and b are as below (with VTM6.0 as anchor):

Test a													RA:	   0.00%, 0.00%, 0.00%

LDB:     0.00%, 0.00%, 0.00%

Test b													RA:	   0.00%, 0.00%, 0.00%

LDB:     0.00%, 0.00%, 0.00%

# Introduction

In current VTM6.0, Prediction sample values of PROF before weighted prediction (indicated as src0 and src1 in equation [1]-[3]) are in 18 bit range, it is proposed to reduce it to 16 bit so that bitwidth of multiplications of weighting process is reduced. It is noted that there is almost no coding impact by adding this restriction.

An illustration of PROF process in VTM6.0 is given in equation [1]-[3]

pBSample[i,j]=Clip(0, 1023, (w0*src0i,j+w1*src1i,j+65600)>>7)		[1]

Wherein,

src0i, j= I0i,j+((gx0i,j*Δvx0i,j+gy0i,j*Δvy0i,j+1)≫1)	[2]

src1i, j= I1i,j+((gx1i,j*Δvx1(i,j)+gy1i,j*Δvy1(i,j))+1)≫1)	[3]

The first approach is to clip PROF refinement value to 14 bit so that prediction sample value before weighted prediction is within 16 bit ( which was implemented in JVET-O0070). More precisely, grad*dMV is clipped to [-8192, 8191] as shown in equation [4]-[5].

src0i, j= I0i,j+Clip( -213, 213-1,  ((gx0i,j*Δvx0i,j+gy0i,j*Δvy0i,j+1)≫1)        [4]

src1i, j= I1i,j+ Clip(-213, 213-1,  ((gx1i,j*Δvx1(i,j)+gy1i,j*Δvy1(i,j))+1)≫1)      [5]

The second approach is to directly clip src0 and src1 to 16 bits as shown in equation [6]-[7].

src0i, j= Clip( -215, 215-1,  I0i,j+((gx0i,j*Δvx0i,j+gy0i,j*Δvy0i,j+1)≫1)        [6]

src1i, j=  Clip(-215, 215-1,  I1i,j+((gx1i,j*Δvx1(i,j)+gy1i,j*Δvy1(i,j))+1)≫1)       [7]

Detailed bit range calculation is given in section 5.

# Simulation results

Simulation results for both approaches are as below:

Table 1: Results for test a that clips PROF refinement value to 14 bit

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class F | 0.01% | 0.01% | 0.01% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.01% | -0.01% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | 0.07% | 0.20% | 0.00% | 101% | 98% |

Table 2: Results for test b that clips prediction sample value before weighted prediction to 16 bit

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 98% |

# Conclusion

This contribution proposes 2 approaches to clip prediction sample values of PROF before weighted prediction to 16 bits so that bitwidth of multiplications of weighting process could be reduced. It is observed that there is no coding impact for both approaches.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Additonal information

Calculation of bit range of is given as below:

Prediction sample value after imterpolation is in the range of [-23314, 23296]

- Max value after hilterHor is (1023*86 – 32768) >> 2 = 13802

- Min value after filterHor is (1023*(-22)-32768)>>2 = -13819

- Max value after filterVer is (13802*86+(-13819)*(-22))>>6 = -23296

- Max value after filterVer is 13802*(-22)+(-13819)*86)>>6 = -23314

- Wherein 86 is max sum of filter coefficeints and -22 is min sum of filter coefficients

PROF refinement value is Grad*dMV is in range of [-46656, 45927]

- The gradient values can be as large as (23296)>>6-(-2314)>>6 = 729

- dMV is inrange of [-64, 63]

- Interpolation filter of affine is attached as below:

| 0 | 0 | 0 | 64 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | -3 | 63 | 4 | -2 | 1 | 0 |
| 0 | 1 | -5 | 62 | 8 | -3 | 1 | 0 |
| 0 | 2 | -8 | 60 | 13 | -4 | 1 | 0 |
| 0 | 3 | -10 | 58 | 17 | -5 | 1 | 0 |
| 0 | 3 | -11 | 52 | 26 | -8 | 2 | 0 |
| 0 | 2 | -9 | 47 | 31 | -10 | 3 | 0 |
| 0 | 3 | -11 | 45 | 34 | -10 | 3 | 0 |
| 0 | 3 | -11 | 40 | 40 | -11 | 3 | 0 |
| 0 | 3 | -10 | 34 | 45 | -11 | 3 | 0 |
| 0 | 3 | -10 | 31 | 47 | -9 | 2 | 0 |
| 0 | 2 | -8 | 26 | 52 | -11 | 3 | 0 |
| 0 | 1 | -5 | 17 | 58 | -10 | 3 | 0 |
| 0 | 1 | -4 | 13 | 60 | -8 | 2 | 0 |
| 0 | 1 | -3 | 8 | 62 | -5 | 1 | 0 |
| 0 | 1 | -2 | 4 | 63 | -3 | 1 | 0 |