| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P**-v1 |
| --- | --- |

| Title: | AHG16/Non-CE5: Cross component ALF simplification |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input document to JVET |  |  |
| Purpose: |  | Proposal |  |  |
| Author(s) or Contact(s): |  | Li Jingya Lim Chong Soon | Tel: Email: | +65 6552 5423 jingya.li@sg.panasonic.com |
| Source: |  | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes following simplification to cross-component ALF (CCALF):

  - Reduce number of signaled CCALF coefficients. Number of coefficents equals to 6, 7, 8 are tested separately.

  - Add a clipping process to refinement value of CCALF to reduce on-chip memory and also allows more flexibility to control subjective quality.

Coding impact of a is as below(with VTM6.0 as anchor):

Test a1 (CE5-2.1+ 6 coefficients without gain shift): 		All Intra:  0.10%, -6.60%, -4.83%

RA:	   0.16%, -9.00%, -8.22%

LDB:     0.13%, -11.98%, -8.24%

Test a2 (CE5-2.1+ 7 coefficients without gain shift): 		All Intra:  0.10%, -6.84%, -5.18%

RA:	   0.**%, 0.**%, 0.**%

LDB:     0.14%, -13.72%, -9.80%

Test a3 (CE5-2.1+ 8 coefficients without gain shift): 		All Intra:  0.11%, -7.13%, -5.42%

RA:	   0.16%, -9.75%, -9.40%

LDB:     0.16%, -13.90%, -10.05%

Coding impact of b is as below(with VTM6.0 as anchor):

Test b (CE5-2.1+ with clipping process): 				All Intra:  0.13%, -7.47%, -5.92%

RA:	   0.**%, 0.**%, 0.**%

LDB:     0.**%, 0.**%, 0.**%

Additional information(with VTM6.0 as anchor):

Test a1 (CE5-2.1+ 6 coefficients with gain shift): 		All Intra:  -1.02%, 1.92%, 3.89%

RA:	   0.**%, 0.**%, 0.**%

LDB:     0.**%, 0.**%, 0.**%

Test a2 (CE5-2.1+ 7 coefficients with gain shift): 		All Intra:  -1.02%, 1.68%, 3.51%

RA:	   -1.17%, 1.42%, 1.79%

LDB:     0.**%, 0.**%, 0.**%

Test a3 (CE5-2.1+ 8 coefficients with gain shift): 		All Intra:  -1.01%, 1.32%, 3.24%

RA:	   -1.16%, 1.31%, 1.49%

LDB:     0.**%, 0.**%, 0.**%

# Introduction

## Reduce CCALF’s number of multipltication:

AHG16 has following comments on CCALF (JVET-O0636):

The multiplication overhead of the CCALF being tested in the CE is about 50% of the current luma and chroma ALF design, while the expected gain is about 1% (on the top of existing ~5% ALF gain in RA).  It would be desirable to study whether a better trade-off exists outside the CE.

ALF has approximately 4.65% coding gain (RA) over VTM6.0, compared to ALF, CCALF increases complexity (counting number of multiplications) by 42% with approximately 1.16% gain (with gain shift from chroma to luma, RA), the increase of coding gain is not linear with the increase of complexity. It is preferred to reduce number of multiplication overhead of CCALF.

Number of coefficients = 6, 7, 8 are tested separately (filter shapes are shown in Figure 1). It is observed that most coding gain of CCALF is retained with a significant reduction of complexity:

- number of coefficients = 14(CE5-2, JVET-P0080, no gain shift), with 0.16%, -10.69%, -10.17% coding impact on RA

- number of coefficients = 6, reduce number of multiplication to 43%, with 0.16%, -9.00%, -8.22% coding impact on RA

- number of coefficients = 7, reduce number of multiplication to 50%, with **%, **%, **% coding impact on RA

- number of coefficients = 8, reduce number of multiplication to 57%, with 0.16%, -9.75%, -9.40% coding impact on RA

Figure 1: proposed shape of CCALF coefficients

## Reduce CCALF’s on-chip buffer:

The second change is to add a clipping process to refinement value (as indicated as ‘R’) of CCALF, wherein the clipping value is signaled in PPS with default clipping range as [-16, 15] (the default clipping range is also the maximum allowed range). By adding a clipping process, the on-chip buffer needed to store R (for parallel processing) is reduced from 16 bits per sample (16,384 bytes per CTU) to 5 bits per sample (5120 bytes per CTU). An  additional benefit is to allow more flexibility for subjective control wherein in some cases user might prefer a lower impact of CCALF on chroma sample values.

Illustration of CCALF refinement:

R=(n=118coeffn*pBSampleY[n]n+64)≫7	[1]

pBSampleC = pBSampleC + R;  					[2]

it is proposed to change equation [2] to [3]

pBSampleC = pBSampleC + Clip(R);  				[3]

Detailed calculation of complexity and on-chip buffer is fiven in section 4.

# Simulation results

Simulation results with and without luma chroma gain shift are as below.

Table 1: (test a1) Results for number of coefficients = 6 without gain shift

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -2.80% | -2.76% | 98% | 103% |
| Class A2 | 0.13% | -7.17% | -8.05% | 98% | 102% |
| Class B | 0.07% | -9.61% | -7.01% | 98% | 98% |
| Class C | 0.08% | -3.20% | -2.77% | 100% | 98% |
| Class E | 0.13% | -9.35% | -2.79% | 98% | 98% |
| Overall | 0.10% | -6.60% | -4.83% | 98% | 99% |
| Class D | 0.07% | -2.72% | -2.09% | 106% | 102% |
| Class F | 0.14% | -5.81% | -6.21% | 100% | 99% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -4.37% | -3.59% | 100% | 107% |
| Class A2 | 0.24% | -9.11% | -14.21% | 101% | 107% |
| Class B | 0.18% | -15.93% | -11.52% | 101% | 104% |
| Class C | 0.09% | -3.74% | -3.07% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | -9.00% | -8.22% | 101% | 103% |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | 76% | 76% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.17% | -17.58% | -14.83% | 96% | 98% |
| Class C | 0.09% | -3.68% | -3.16% | 96% | 100% |
| Class E | 0.14% | -13.71% | -4.03% | 98% | 97% |
| Overall | 0.13% | -11.98% | -8.24% | 96% | 99% |
| Class D | 0.13% | -5.65% | -3.80% | 97% | 100% |
| Class F | 0.04% | -6.96% | -4.99% | 94% | 92% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 2: (test a2) Results for number of coefficients = 7 without gain shift

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -2.96% | -3.14% | 100% | 104% |
| Class A2 | 0.13% | -7.52% | -8.30% | 98% | 104% |
| Class B | 0.08% | -9.81% | -7.56% | 100% | 101% |
| Class C | 0.08% | -3.29% | -2.83% | 100% | 99% |
| Class E | 0.14% | -9.80% | -3.26% | 100% | 102% |
| Overall | 0.10% | -6.84% | -5.18% | 100% | 102% |
| Class D | 0.08% | -2.72% | -2.07% | 100% | 97% |
| Class F | 0.15% | -5.98% | -6.21% | 100% | 99% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.21% | -16.74% | -12.99% | 100% | 103% |
| Class C | 0.09% | -4.15% | -3.37% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.17% | -19.04% | -17.39% | 98% | 105% |
| Class C | 0.09% | -5.47% | -4.26% | 97% | 101% |
| Class E | 0.14% | -15.85% | -4.55% | 99% | 102% |
| Overall | 0.14% | -13.72% | -9.80% | 98% | 103% |
| Class D | 0.13% | -6.25% | -3.99% | 96% | 99% |
| Class F | 0.27% | -7.73% | -5.98% | 99% | 103% |

Table 3: (test a3) Results for number of coefficients = 8 without gain shift

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | -3.09% | -3.43% | 110% | 110% |
| Class A2 | 0.13% | -7.76% | -8.40% | 98% | 101% |
| Class B | 0.08% | -10.18% | -7.75% | 100% | 98% |
| Class C | 0.09% | -3.59% | -3.34% | 108% | 102% |
| Class E | 0.15% | -10.22% | -3.30% | 97% | 94% |
| Overall | 0.11% | -7.13% | -5.42% | 102% | 100% |
| Class D | 0.08% | -2.77% | -2.63% | 98% | 94% |
| Class F | 0.15% | -5.99% | -6.31% | 102% | 101% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -4.87% | -4.73% | 100% | 106% |
| Class A2 | 0.24% | -10.08% | -14.94% | 100% | 106% |
| Class B | 0.19% | -16.81% | -13.46% | 100% | 103% |
| Class C | 0.09% | -4.33% | -3.67% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | -9.75% | -9.40% | 100% | 103% |
| Class D | #VALUE! | #VALUE! | #VALUE! | 99% | 95% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.19% | -19.33% | -17.45% | 101% | 109% |
| Class C | 0.14% | -5.44% | -4.45% | 95% | 99% |
| Class E | 0.15% | -16.13% | -5.20% | 98% | 100% |
| Overall | 0.16% | -13.90% | -10.05% | 98% | 103% |
| Class D | 0.11% | -6.38% | -3.90% | 95% | 98% |
| Class F | 0.21% | -7.91% | -6.68% | 98% | 104% |

Table 4: (test b) Results for applying clipping

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.13% | -4.18% | -3.90% | 100% | 101% |
| Class E | 0.19% | -10.03% | -3.45% | 102% | 104% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | 0.17% | -5.73% | -5.97% | 101% | 102% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.08% | -5.01% | -4.25% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.12% | -8.30% | -6.67% | 101% | 102% |
| Class E | 0.02% | -16.23% | -6.51% | 104% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

This contribution proposes 2 modifications for CCALF (1) reduce number of multiplications of CCALF from 42% of ALF to 18%, 21%, 24% respectively by using 6, 7, or 8 coefficients. It is observed that most of CCALF gain is retained. (2) a clipping process is added to refinement value of CCALF to reduce on chip memory. There is negligible impact on coding performance.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Additional information

Complexity Analysis:

For a 128x128 CTU, number of multiplication for ALF is

13*128*128+7*2*64*64 = 212992+57344= 270336

CCALF (JVET-O0636) has 14 signaled coefficients, number of multiplfication for CALF is:

2*14*64*64 = 114688 (42% increase compared to ALF)

Complexity of CCALF with 6, 7, 8 signaled coefficients is as below:

2*6*64*64 = 49152 (18% increase compared to ALF)

2*7*64*64 = 57344 (21% increase compared to ALF)

2*8*64*64 = 65536 (24% increase compared to ALF)

On Chip buffer analysis:

Coeff[n] is 8 bits in range of [-128, 127]

pBSampleY is in range of [0, 1023],

R is a 16 bit value in range of [(18*(-128)*1023+64)>>7, (18*127*1023+64)>>7] = [-18414, 18270],

For one CTU, it takes 2*16*64*64/8 = 16384 bytes.

If R is clipped to be within [-16, 15]

For one CTU, it takes 2*5*64*64/8 = 5120 bytes.

ALF itself needs (10*128*128+2*10*64*64)/8 = 30720 bytes to store luma and chroma sample values

(test a1) Results for number of coefficients = 6 with gain shift

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | -0.77% | -0.56% | 2.73% |  |  |
| Class C | -0.81% | 5.03% | 5.80% |  |  |
| Class E | -0.52% | -1.81% | 5.49% |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -0.81% | 5.56% | 6.29% |  |  |
| Class F | -0.77% | -0.16% | -0.85% |  |  |
| TGM | #VALUE! | #VALUE! | #VALUE! |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class C | -0.80% | 7.25% | 8.07% |  |  |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  |
| TGM | #VALUE! | #VALUE! | #VALUE! |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

(test a2) Results for number of coefficients = 7 with gain shift

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.21% | 5.96% | 4.82% |  |  |
| Class A2 | -2.03% | 1.11% | -0.02% |  |  |
| Class B | -0.77% | -0.76% | 2.12% |  |  |
| Class C | -0.81% | 4.95% | 5.73% |  |  |
| Class E | -0.51% | -2.31% | 5.04% |  |  |
| Overall | -1.02% | 1.68% | 3.51% |  |  |
| Class D | -0.80% | 5.60% | 6.29% |  |  |
| Class F | -0.76% | -0.37% | -0.98% |  |  |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | -0.53% | -5.29% | -0.97% |  |  |
| Class C | -0.82% | 6.68% | 7.70% |  |  |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

(test a3) Results for number of coefficients = 8 with gain shift

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.21% | 5.82% | 4.54% |  |  |
| Class A2 | -2.03% | 0.82% | -0.14% |  |  |
| Class B | -0.76% | -1.27% | 1.90% |  |  |
| Class C | -0.80% | 4.63% | 5.17% |  |  |
| Class E | -0.50% | -2.80% | 4.99% |  |  |
| Overall | -1.01% | 1.32% | 3.24% |  |  |
| Class D | -0.79% | 5.46% | 5.66% |  |  |
| Class F | -0.75% | -0.44% | -1.09% |  |  |
| TGM | #VALUE! | #VALUE! | #VALUE! |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.84% | 6.05% | 4.89% |  |  |
| Class A2 | -2.03% | 0.65% | -5.13% |  |  |
| Class B | -0.54% | -5.37% | -1.39% |  |  |
| Class C | -0.78% | 6.62% | 7.49% |  |  |
| Class E |  |  |  |  |  |
| Overall | -1.16% | 1.31% | 1.49% |  |  |
| Class D | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  |
| TGM | #VALUE! | #VALUE! | #VALUE! |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |