| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0174-v2 |
| --- | --- |

| Title: | Non-CE4: Modifications of TPM and GEO |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Input document to JVET |  |  |
| Purpose: |  | Proposal |  |  |
| Author(s) or Contact(s): |  | Li Jingya Lim Chong Soon | Tel: Email: | +65 6552 5423 jingya.li@sg.panasonic.com |
| Source: |  | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes following change for GEO and TPM:

  - Unify blending mask and MV storage mask generation process of TPM and GEO to avoid duplicate logics

  - Adaptive number of valid modes for GEO to make hardware validation more feasible

Coding impact of change a is as below(with VTM6.0 as anchor):	RA:	   -0.29%, -0.32%, -0.41%

LDB:     -0.62%, -0.64%, -0.59%

# Introduction

## Unify blending mask and MV storage mask generation process of TPM with GEO

Geometrical partitoning for inter blocks (JVET-O0489) was proposed in Gothenburg meeting that allows up to 140 modes (a simplified version uses 64 modes in CE4-1.2) to support different partitioning angles and distances.

The blending and MV storage mask generation process is different between GEO and TPM:

- For GEO, mask values are determined by calculating distance (ρ) from partitioning line of each sample in the GEO block. An example is shown in Figure 1. The partitioning line of a GEO mode block is defined by a GEO mode index indicating angle (α) index and distance from the center point of the block (distance idx = 0 indicates the partitioning line goes through block center). For a sample located at (x, y), the distance (ρ) is calculated as ρ=D1-D2, wherein D2 is a derived value using information of the partitioning line, and D2 is determined using the dot product of vector [y, x] and (cos(α), sin(α)). And the mask value is derived using the distance and a look up table.

- For TPM, mask values are determined directly by simply comparing x and y coordinate values without calculating distance from partitioning line.

Figure 1: An illustration of distance calculation process of GEO

Due to the different complexity level and number of modes, the hardware implementation cost for GEO and TPM are different.

- For TPM, there are limited number of modes (25 PU sizes * 2 partitions). The total cost to store all blending and MV storage masks costs 60 KB (Detailed calculation is given in section 5). If it is considered as hardware burden to store 60KB data, the mask can still be computed on-the-fly with little computation cost since the derivation process of masks are very simple.

- For GEO, number of modes are (19 PU sizes * 140 modes) much larger. The total cost to store all masks is 4,131 KB. A simplified version (64 modes) still costs 1,889 KB (Detailed calculation is given in section 5), which is pretty high for some hardware design. It is preferred to derive mask values on-the-fly.

From analysis above, it might be a better design to compute both TPM and GEO masks on-the-fly to save storage cost and unify the process to avoid duplicate logic for mask calculation.

Following approach is proposed to unify GEO and TPM mask generation process:

- GEO is enabled with 64 valid modes (CE4-1.2) and independent motion mask calculation process (CE 4-1.14).

- Align blending and MV storage masks generation process of TPM to GEO.

GEO uses a look up table (Table 1) to map the angle index (0 to 31) to a Dis[idx] value that indicates scaled value of cosine and sine of the angle (α). To align TPM’s blending and MV storage mask generation process with GEO, a second look up table (Table 2) is used to map an index (0 to 17) to scaled value of cosine and sine of corresponding angle (α), wherein the index is determined by width and height ratio of the block. The distance index of TPM is always set to 0. and the blending and MV storage masks calculation of TPM reuses that of GEO.

Table 1 - Look-up table Dis for derivation of geometric partitioning distance for geometric partitioning mode.

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 8 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dis[idx] | 64 | 63 | 59 | 53 | 45 | 36 | 0 | -36 | -45 | -53 | -59 | -63 |
| idx | 16 | 17 | 18 | 19 | 20 | 21 | 24 | 27 | 28 | 29 | 30 | 31 |
| Dis[idx] | -64 | -63 | -59 | -53 | -45 | -36 | 0 | 36 | 45 | 53 | 59 | 63 |

Table 2 - Look-up table Dis for derivation of geometric partitioning distance for triangle partitioning mode.

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| dis[idx] | -45 | -29 | -16 | -8 | -4 | -57 | -62 | -64 | -64 |
| idx | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| dis[idx] | 45 | 57 | 62 | 64 | 64 | 29 | 16 | 8 | 4 |

## Adaptive number of valid modes for GEO

GEO might create difficulty for hardware validation process (even for the simplified version of 64 modes), therefore it is proposed to allow using only a selection of GEO modes instead of full sets to make hardware validation easier and it also benefits low cost encoder by reducing number of modes. An illustration of syntax change is given in Table 1.

Table 3: Syntax change of 1.2

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if(sps_geo_enabled_flag){ |  |
| is_geo_mode_signaled | u(1) |
| if(is_geo_mode_signaled){ |  |
| num_geo_mode_minus_1 | u(8) |
| for(i = 0; i <= num_geo_mode_minus_1; i++){ |  |
| geo_mode_idx | u(8) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

is_geo_mode_signaled specifies if geo modes are explicitly signaled or not, if is_geo_mode_signaled is false, default 64 modes will be used.

num_geo_mode_minus_1 specified number of modes signaled, it is required that it shall be in the range of [1, 63].

geo_mode_idx specified mode index that is used. it is required that it shall be in the range of [1, 63].

Note that the block level signaling of geo mode (merge_geo_idx) is of coding length of Ceil( Log2(num_geo_mode_minus_1+1) ) bits. A geo mode index is indicated by geo_mode_idx[ merge_geo_idx ].

# Simulation results

Simulation results for the unification process is as below. Additional information of changes that using 64 valid modes (same as CE4-1.2) and independent motion mask calculation process (same as CE 4-1.14) is given in section 5. Note that encoding and decoding time is not accurate.

Table 4: Unification of blending and MV storage mask process of GEO and TPM

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.12% | -0.24% | 104% | 100% |
| Class A2 | -0.18% | -0.11% | -0.14% | 103% | 96% |
| Class B | -0.19% | -0.08% | -0.20% | 103% | 99% |
| Class C | -0.64% | -0.93% | -1.00% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.32% | -0.41% | 102% | 99% |
| Class D | -0.46% | -0.67% | -0.66% | 100% | 100% |
| Class F | -0.27% | -0.45% | -0.40% | 86% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.33% | -0.32% | -0.54% | 103% | 95% |
| Class C | -0.75% | -1.19% | -0.96% | 101% | 98% |
| Class E | -0.91% | -0.45% | -0.19% | 105% | 96% |
| Overall | -0.62% | -0.64% | -0.59% | 103% | 97% |
| Class D | -0.73% | -1.28% | -0.36% | 101% | 101% |
| Class F | -0.36% | -0.46% | 0.02% | 98% | 99% |

# Conclusion

This contributions proposes two approaches: (1) Unify the blending and MV storage mask calculation process to avoid duplicate of logics with -0.29% luma coding impact on RA and -0.62% luma coding impact on LDB. (2) Use adaptive number of valid GEO mode for the benefit of hardware validation and low cost encoder.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Additional information

Storage cost for TPM

| width | height | Luma blending mask bits | chroma blending mask bits | MV storage mask bits |  |
| --- | --- | --- | --- | --- | --- |
| 4 | 16 | 256 | 64 | 8 |  |
| 16 | 4 | 256 | 64 | 8 |  |
| 4 | 32 | 512 | 128 | 16 |  |
| 32 | 4 | 512 | 128 | 16 |  |
| 4 | 64 | 1024 | 256 | 32 |  |
| 64 | 4 | 1024 | 256 | 32 |  |
| 8 | 8 | 256 | 64 | 8 |  |
| 8 | 16 | 512 | 128 | 16 |  |
| 16 | 8 | 512 | 128 | 16 |  |
| 8 | 32 | 1024 | 256 | 32 |  |
| 32 | 8 | 1024 | 256 | 32 |  |
| 8 | 64 | 2048 | 512 | 64 |  |
| 64 | 8 | 2048 | 512 | 64 |  |
| 16 | 16 | 1024 | 256 | 32 |  |
| 16 | 32 | 2048 | 512 | 64 |  |
| 32 | 16 | 2048 | 512 | 64 |  |
| 16 | 64 | 4096 | 1024 | 128 |  |
| 64 | 16 | 4096 | 1024 | 128 |  |
| 32 | 32 | 4096 | 1024 | 128 |  |
| 32 | 64 | 8192 | 2048 | 256 |  |
| 64 | 32 | 8192 | 2048 | 256 |  |
| 64 | 64 | 16384 | 4096 | 512 |  |
| 64 | 128 | 32768 | 8192 | 1024 |  |
| 128 | 64 | 32768 | 8192 | 1024 |  |
| 128 | 128 | 65536 | 16384 | 2048 |  |
|  | sum | 192256 | 48064 | 6008 | bit |
|  | total sum |  |  | 246328 | bit |
|  | total sum |  |  | 30 | KB |
|  | num of modes |  |  | 2 |  |
|  | total storage cost |  |  | 60 | KB |

Storage cost for GEO

| width | height | Luma blending mask bits | chroma blending mask bits | MV storage mask bits |  |
| --- | --- | --- | --- | --- | --- |
| 8 | 8 | 256 | 64 | 8 |  |
| 8 | 16 | 512 | 128 | 16 |  |
| 16 | 8 | 512 | 128 | 16 |  |
| 8 | 32 | 1024 | 256 | 32 |  |
| 32 | 8 | 1024 | 256 | 32 |  |
| 8 | 64 | 2048 | 512 | 64 |  |
| 64 | 8 | 2048 | 512 | 64 |  |
| 16 | 16 | 1024 | 256 | 32 |  |
| 16 | 32 | 2048 | 512 | 64 |  |
| 32 | 16 | 2048 | 512 | 64 |  |
| 16 | 64 | 4096 | 1024 | 128 |  |
| 64 | 16 | 4096 | 1024 | 128 |  |
| 32 | 32 | 4096 | 1024 | 128 |  |
| 32 | 64 | 8192 | 2048 | 256 |  |
| 64 | 32 | 8192 | 2048 | 256 |  |
| 64 | 64 | 16384 | 4096 | 512 |  |
| 64 | 128 | 32768 | 8192 | 1024 |  |
| 128 | 64 | 32768 | 8192 | 1024 |  |
| 128 | 128 | 65536 | 16384 | 2048 |  |
|  | sum | 188672 | 47168 | 5896 | bit |
|  | total sum |  |  | 241736 | bit |
|  | total sum |  |  | 30 | KB |
|  | num of modes |  |  | 64 |  |
|  | total storage cost |  |  | 1889 | KB |
|  | num of modes |  |  | 140 |  |
|  | total storage cost |  |  | 4131 | KB |

Additional information of changes that using 64 valid modes (same as CE4-1.2) and independent motion mask calculation process (same as CE 4-1.14) is as below. Note that encoding and decoding time is not accurate.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.06% | -0.31% | 109% | 101% |
| Class A2 | -0.23% | -0.21% | -0.23% | 97% | 93% |
| Class B | -0.21% | -0.18% | -0.34% | 103% | 101% |
| Class C | -0.72% | -0.94% | -1.15% | 100% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.34% | -0.37% | -0.53% | 102% | 100% |
| Class D | -0.50% | -0.65% | -0.67% | 104% | 121% |
| Class F | -0.32% | -0.47% | -0.41% | 87% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.35% | -0.46% | -0.44% | 106% | 105% |
| Class C | -0.81% | -1.15% | -1.27% | 106% | 111% |
| Class E | -1.01% | -0.38% | -0.05% | 93% | 89% |
| Overall | -0.67% | -0.67% | -0.62% | 102% | 103% |
| Class D | -0.71% | -0.72% | -0.58% | 114% | 121% |
| Class F | -0.35% | 0.03% | -0.06% | 98% | 91% |