| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0182-v1 |
| --- | --- |

| Title: | On Reference Picture List and Inter-Layer Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Modifications are proposed related to reference picture list and inter-layer prediction signalling.

Following is proposed:

- Proposal 1: A syntax modification is proposed for reference picture list signalling. The proposed modification needs two syntax elements instead of three syntax elements as needed by VVC WD 6 to specify whether an entry is ILRP/ LTRP/ STRP with positive delta POC/ STRP with negative delta POC. Thus it is asserted to have a lower complexity.

- Proposal 2: It is proposed to conditionally signal inter layer reference picture flag (inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ])  such that it is signalled only when necessary. When not signalled the value for inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is inferred in this case.

- Proposal 3: A bug fix is proposed to the reference picture list derivation of delta POC values for the short-term reference picture entries.

# Introduction

VVC WD [1] includes signalling of reference picture list-based reference picture management. VVC WD also includes support for scalability and inter-layer prediction. Modifications are proposed related to reference picture list and inter-layer prediction signalling.

Following is proposed:

- Proposal 1: A syntax modification is proposed for reference picture list signalling. The proposed modification needs two syntax elements instead of three syntax elements as needed by VVC WD 6 to specify whether an entry is ILRP/ LTRP/ STRP with positive delta POC/ STRP with negative delta POC. Thus, it is asserted to have a lower complexity.

- Proposal 2: It is proposed to conditionally signal inter layer reference picture flag (inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ])  such that it is signalled only when necessary. When not signalled the value for inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is inferred in this case.

- Proposal 3: A bug fix is proposed to the reference picture list derivation of delta POC values for the short-term reference picture entries.

# Proposals

## Proposal 1

A syntax modification is proposed for reference picture list signalling.

It is asserted that in VVC each reference picture list entry has one of the following four states that needs to be signalled:

- An inter-layer reference picture (ILRP) entry

- A long-term reference picture (LTRP) entry

- A short-term reference picture entry with positive delta POC

- A short-term reference picture entry with negative delta POC.

In VVC WD three one bit flag syntax elements (inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ], st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ], strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ]) are conditionally signalled to covey one of the above four states. It is asserted that only 2 bits are sufficient to fully convey the information about one of these four states.

It is thus proposed to use two syntax elements instead of three syntax elements as in VVC WD 6 to specify whether an entry is ILRP/ LTRP/ STRP with positive delta POC/ STRP with negative delta POC. It is asserted that using two syntax elements instead of three results in lower complexity. The proposed modification can save one bit in each reference picture list in the most general case when inter layer reference pictures and long term reference pictures may be present.

The proposed changes are shown highlighted compared with VVC WD. The additions are shown in blue and deletions in red strikeout.

### Reference picture list structure syntax

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( long_term_ref_pics_flag ) |  |
| ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if(inter_layer_ref_pics_present_flag ¦ ¦ long_term_ref_pics_flag) |  |
| ref_pic_idc_bitX[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if(ref_pic_idc_bitX[ listIdx ][ rplsIdx ][ i ] ¦ ¦ (!inter_layer_ref_pics_present_flag && !long_term_ref_pics_flag) ¦ ¦ (inter_layer_ref_pics_present_flag && long_term_ref_pics_flag)) |  |
| ref_pic_idc_bitY[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( inter_layer_ref_pics_present_flag ) |  |
| inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( !inter_layer_ref_pics_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| if( long_term_ref_pics_flag ) |  |
| st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] RefPicIdc[ listIdx ][ rplsIdx ][ i ] == 00 ¦ ¦ RefPicIdc[ listIdx ][ rplsIdx ][ i ] == 01) { |  |
| abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| if( AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] > 0 ) |  |
| strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } else if((RefPicIdc[ listIdx ][ rplsIdx ][ i ] == 11) && !ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] ) |  |
| rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ j++ ] | u(v) |
| } else if(RefPicIdc[ listIdx ][ rplsIdx ][ i ] == 10) |  |
| ilrp_idc[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| } |  |
| } |  |

### Reference picture list structure semantics

The ref_pic_list_struct( listIdx, rplsIdx ) syntax structure may be present in an SPS or in a slice header. Depending on whether the syntax structure is included in a slice header or an SPS, the following applies:

–	If present in a slice header, the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure specifies reference picture list listIdx of the current picture (the picture containing the slice).

–	Otherwise (present in an SPS), the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure specifies a candidate for reference picture list listIdx, and the term "the current picture" in the semantics specified in the remainder of this clause refers to each picture that 1) has one or more slices containing ref_pic_list_idx[ listIdx ] equal to an index into the list of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures included in the SPS, and 2) is in a CVS that refers to the SPS.

num_ref_entries[ listIdx ][ rplsIdx ] specifies the number of entries in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure. The value of num_ref_entries[ listIdx ][ rplsIdx ] shall be in the range of 0 to sps_max_dec_pic_buffering_minus1 + 14, inclusive.

ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] equal to 0 specifies that the POC LSBs of the LTRP entries in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure are present in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure. ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] equal to 1 specifies that the POC LSBs of the LTRP entries in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure are not present in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure.

ref_pic_idc_bitX[ listIdx ][ rplsIdx ][ i ] and ref_pic_idc_bitY[ listIdx ][ rplsIdx ][ i ]  are used for the signaling of information about i-th entry in the reference picture list. When not present ref_pic_idc_bitX[ listIdx ][ rplsIdx ][ i ] is inferred to be equal to 0.

–	If inter_layer_ref_pics_present_flag and  long_term_ref_pics_flag are both equal to 1 or are both equal to 0 following applies: A 2-bit variable RefPicIdc[ listIdx ][ rplsIdx ][ i ] is derived as follows:

RefPicIdc[ listIdx ][ rplsIdx ][ i ] = ref_pic_idc_bitX[ listIdx ][ rplsIdx ][ i ] << 1 | ref_pic_idc_bitY[ listIdx ][ rplsIdx ][ i ]

RefPicIdc[ listIdx ][ rplsIdx ][ i ] equal to:

–	00 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is an STRP entry and has a value greater than or equal to 0,

–	01 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is an STRP entry and has a value less than 0,

–	10  specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is an ILRP entry,

–	11 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is an LTRP entry.

–	Otherwise (i.e. when inter_layer_ref_pics_present_flag is equal to 1 and long_term_ref_pics_flag is equal to 0 or when inter_layer_ref_pics_present_flag is equal to 0 and long_term_ref_pics_flag is equal to 1) ref_pic_idc_bitX[ listIdx ][ rplsIdx ][ i ] equal to 0 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure:

–	is an ILRP entry if inter_layer_ref_pics_present_flag is equal to 1 and  RefPicIdc[ listIdx ][ rplsIdx ][ i ] is derived as 10

–	is a LTRP entry if long_term_ref_pics_flag is equal to 1 and  RefPicIdc[ listIdx ][ rplsIdx ][ i ] is derived as 11.

ref_pic_idc_bitX[ listIdx ][ rplsIdx ][ i ] equal to 1 specifies that:

–	if  ref_pic_idc_bitY[ listIdx ][ rplsIdx ][ i ] is equal to 0 then the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is an STRP entry and has a value greater than or equal to 0,

–	if  ref_pic_idc_bitY[ listIdx ][ rplsIdx ][ i ] is equal to 1 then the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is an STRP entry and has a value less than 0.

inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] equal to 1 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is an ILRP entry. inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] equal to 0 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is not an ILRP entry. When not present, the value of inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is inferred to be equal to 0.

st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] equal to 1 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is an STRP entry. st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] equal to 0 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is an LTRP entry. When inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is equal to 0 and st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is not present, the value of st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is inferred to be equal to 1.

The variable NumLtrpEntries[ listIdx ][ rplsIdx ] is derived as follows:

for( i = 0, NumLtrpEntries[ listIdx ][ rplsIdx ] = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++ )
		if( !inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ]  &&  !st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] RefPicIdc[ listIdx ][ rplsIdx ][ i ] == 11 )	(7-120)
			NumLtrpEntries[ listIdx ][ rplsIdx ]++

abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] specifies the value of the variable AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] as follows:

if( sps_weighted_pred_flag  | |  sps_weighted_bipred_flag )
	AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] = 	abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] 	(7-121)
else
	AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] = 	 abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] + 1

The value of abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] shall be in the range of 0 to 215 − 1, inclusive.

strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] equal to 1 specifies that i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx ) has a value greater than or equal to 0. strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] equal to 0 specifies that the i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx ) has a value less than 0. When not present, the value of strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] is inferred to be equal to 1.

The list DeltaPocValSt[ listIdx ][ rplsIdx ] is derived as follows:

for( i = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++ )
     if( !inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ]  &&  st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] )	(7-122)
		DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] = ( strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] ) ?
				AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] : 0 − AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ]
     if(RefPicIdc[ listIdx ][ rplsIdx ][ i ] == 00)
		   DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] = AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] 
 	 else if(RefPicIdc[ listIdx ][ rplsIdx ][ i ] == 01)
			DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] = 0 − AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ]

rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ i ] specifies the value of the picture order count modulo MaxPicOrderCntLsb of the picture referred to by the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure. The length of the rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ i ] syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits.

ilrp_idc[ listIdx ][ rplsIdx ][ i ] specifies the index, to the list of directly dependent layers, of the ILRP of i-th entry in ref_pic_list_struct( listIdx, rplsIdx ) syntax structure to the list of directly dependent layers. The value of ilrp_idc[ listIdx ][ rplsIdx ][ i ] shall be in the range of 0 to the GeneralLayerIdx[ nuh_layer_id ] − 1, inclusive.

## Proposal 2

In current VVC WD 6 inter layer reference picture flag (inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ]) is signalled for each entry in the reference picture list when inter_layer_ref_pics_present_flag is equal to 1. Thus a total of num_ref_entries[ listIdx ][ rplsIdx ] number of inter layer reference picture flags are sent in this case for each reference picture list.

It is asserted that under certain scenarios the inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i  ] does not need to be signalled as it is unnecessary. Two alternative options are proposed to avoid signaling inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i  ] when it is unnecessary.

Option A: It is proposed to keep track of number of inter-layer reference prediction entries signalled and conditionally signal inter layer reference picture flag (inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ])  such that it is signalled only when the number of signalled inter-layer reference prediction entries in the reference picture list is less than or equal to the number of direct reference layers (or number of direct dependent layers). When not signalled the value for inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is inferred in this case.

Option B: It is proposed to conditionally signal inter layer reference picture flag (inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ])  such that it is signalled only for non-independent layers. When not signalled the value for inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is inferred in this case. This provides bit-savings and avoid signaling unnecessary information.

The proposed modification to VVC WD are shown highlighted below.

## Proposal 2- Option A

### Reference picture list structure syntax

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( long_term_ref_pics_flag ) |  |
| ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, NumILRPEntries=0; j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( inter_layer_ref_pics_present_flag && (NumILRPEntries < NumDirectReferenceLayers[ GeneralLayerIdx[ nuh_layer_id ] ])) |  |
| inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( !inter_layer_ref_pics_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| if( long_term_ref_pics_flag ) |  |
| st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| if( AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] > 0 ) |  |
| strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } else if( !ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] ) |  |
| rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ j++ ] | u(v) |
| } else { |  |
| ilrp_idc[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| NumILRPEntries++ |  |
| } |  |
| } |  |
| } |  |

NumDirectReferenceLayers[ i ] is derived as:

vps_direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. vps_direct_dependency_flag [ i ][ j ] equal to 1 specifies that the layer with index j is a direct reference layer for the layer with index i. When vps_direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, inclusive, it is inferred to be equal to 0.

The variable DirectDependentLayerIdx[ i ][ j ], specifying the j-th direct dependent layer of the i-th layer, is derived as follows:

for( i = 1; i < vps_max_layers_minus1; i− − )
	if( !vps_independent_layer_flag[ i ] )
		for( j = i, k = 0; j  >=  0; j− − )		(7-2)
			if( vps_direct_dependency_flag[ i ][ j ] )
				DirectDependentLayerIdx[ i ][ k++ ] = j
NumDirectReferenceLayers[ i ] = (vps_independent_layer_flag[ i ] ? 0:k)

### Proposal 2- Option B

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( long_term_ref_pics_flag ) |  |
| ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( inter_layer_ref_pics_present_flag && !vps_independent_layer_flag[GeneralLayerIdx[ nuh_layer_id ]] ) |  |
| inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( !inter_layer_ref_pics_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| if( long_term_ref_pics_flag ) |  |
| st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| if( AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] > 0 ) |  |
| strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } else if( !ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] ) |  |
| rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ j++ ] | u(v) |
| } else |  |
| ilrp_idc[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| } |  |
| } |  |

Alternatively, a conformance constraint is proposed on inter_layer_ref_pics_present_flag based on if a layer is independent or non-independent layer.

inter_layer_ref_pics_present_flag equal to 0 specifies that no ILRP is used for inter prediction of any coded picture in the CVS. inter_layer_ref_pics_flag equal to 1 specifies that ILRPs may be used for inter prediction of one or more coded pictures in the CVS. When sps_video_parameter_set_id is equal to 0, the value of inter_layer_ref_pics_present_flag is inferred to be equal to 0.

When vps_independent_layer_flag[GeneralLayerIdx[ nuh_layer_id ]]  is equal to 1, the value of inter_layer_ref_pics_present_flag shall be equal to 0.

## Proposal 3

A bug fix is proposed to the reference picture list derivation of delta POC values for the short-term reference picture entries.

It is asserted that the text specifying delta coding of reference picture list’s abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] values for STRP entries was missed in JVET-O2001 and the corresponding text needs to be updated.

### Reference picture list structure semantics

The ref_pic_list_struct( listIdx, rplsIdx ) syntax structure may be present in an SPS or in a slice header. Depending on whether the syntax structure is included in a slice header or an SPS, the following applies:

–	If present in a slice header, the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure specifies reference picture list listIdx of the current picture (the picture containing the slice).

–	Otherwise (present in an SPS), the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure specifies a candidate for reference picture list listIdx, and the term "the current picture" in the semantics specified in the remainder of this clause refers to each picture that 1) has one or more slices containing ref_pic_list_idx[ listIdx ] equal to an index into the list of the ref_pic_list_struct( listIdx, rplsIdx ) syntax structures included in the SPS, and 2) is in a CVS that refers to the SPS.

num_ref_entries[ listIdx ][ rplsIdx ] specifies the number of entries in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure. The value of num_ref_entries[ listIdx ][ rplsIdx ] shall be in the range of 0 to sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ] + 14 or sps_max_dec_pic_buffering_minus1[ HighestTid ]+14, inclusive.

ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] equal to 0 specifies that the POC LSBs of the LTRP entries in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure are present in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure. ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] equal to 1 specifies that the POC LSBs of the LTRP entries in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure are not present in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure.

inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] equal to 1 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is an ILRP entry. inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] equal to 0 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is not an ILRP entry. When not present, the value of inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is inferred to be equal to 0.

st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] equal to 1 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is an STRP entry. st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] equal to 0 specifies that the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is an LTRP entry. When inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is equal to 0 and st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is not present, the value of st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] is inferred to be equal to 1.

The variable NumLtrpEntries[ listIdx ][ rplsIdx ] is derived as follows:

for( i = 0, NumLtrpEntries[ listIdx ][ rplsIdx ] = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++ )
		if( !inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ]  &&  !st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] )	(7-120)
			NumLtrpEntries[ listIdx ][ rplsIdx ]++

abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] when the i-th entry is the first STRP entry in ref_pic_list_struct( listIdx, rplsIdx ) syntax structure, specifies the delta between the picture order count values of the current picture and the picture referred to by the i-th entry, or, when the i-th entry is an STRP entry but not the first STRP entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure, specifies the delta between the picture order count values of the pictures referred to by the i-th entry and by the previous STRP entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure.

Specifies the value of Tthe variable AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] is derived as follows:

if( sps_weighted_pred_flag  | |  sps_weighted_bipred_flag )
	AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] = 	abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] 	(7-121)
else
	AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] = 	 abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] + 1

The value of abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] shall be in the range of 0 to 215 − 1, inclusive.

strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] equal to 1 specifies that i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx ) has a value greater than or equal to 0. strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] equal to 0 specifies that the i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx ) has a value less than 0. When not present, the value of strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] is inferred to be equal to 1.

The list DeltaPocValSt[ listIdx ][ rplsIdx ] is derived as follows:

for( i = 0, prevSTRPIdx=0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++ )
	if( !inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ]  &&  st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) {	(7-122)
		DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] = ( strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] ) ?
				AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] : 0 − AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] 
	if( i!=0){ 
				DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] = DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] + DeltaPocValSt[ listIdx ][ rplsIdx ][ prevStrpIdx ] 
				prevStrpIdx = i	
	}
}

…

The reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ] are constructed as follows:

for( i = 0; i < 2; i++ ) {
		for( j = 0, k = 0, pocBase = PicOrderCntVal; j < num_ref_entries[ i ][ RplsIdx[ i ] ]; j++) {
			if( !inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
				if( st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
					RefPicPocList[ i ][ j ] = pocBase PicOrderCntVal − DeltaPocValSt[ i ][ RplsIdx[ i ] ][ j ]
					if( there is a reference picture picA in the DPB with the same nuh_layer_id as the current picture
								and 	PicOrderCntVal equal to RefPicPocList[ i ][ j ] )
						RefPicList[ i ][ j ] = picA
					else
						RefPicList[ i ][ j ] = "no reference picture"				(8-5)
					pocBase = RefPicPocList[ i ][ j ]
				} else {
					if( !delta_poc_msb_cycle_lt[ i ][ k ] ) {
						if( there is a reference picA in the DPB with the same nuh_layer_id as the current picture and
									PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ) equal to PocLsbLt[ i ][ k ] )
							RefPicList[ i ][ j ] = picA
						else
							RefPicList[ i ][ j ] = "no reference picture"
						RefPicLtPocList[ i ][ j ] = PocLsbLt[ i ][ k ]
					} else {
						if( there is a reference picA in the DPB with the same nuh_layer_id as the current picture and
									PicOrderCntVal equal to 	FullPocLt[ i ][ k ] )
							RefPicList[ i ][ j ] = picA
						else
							RefPicList[ i ][ j ] = "no reference picture"
						RefPicLtPocList[ i ][ j ] = FullPocLt[ i ][ k ]
				}
					k++
			}
			} else {
				layerIdx = 	DirectReferenceLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ ilrp_idc[ i ][ RplsIdx ][ j ] ]
				refPicLayerId = vps_layer_id[ layerIdx ]
				if( there is a reference picture picA in the DPB with nuh_layer_id equal to refPicLayerId and
							the same PicOrderCntVal as the current picture )
					RefPicList[ i ][ j ] = picA
				else
					RefPicList[ i ][ j ] = "no reference picture"
		}
	}
}

rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ i ] specifies the value of the picture order count modulo MaxPicOrderCntLsb of the picture referred to by the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure. The length of the rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ i ] syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits.

ilrp_idc[ listIdx ][ rplsIdx ][ i ] specifies the index, to the list of directly reference layers, of the ILRP of i-th entry in ref_pic_list_struct( listIdx, rplsIdx ) syntax structure to the list of directly dependent layers. The value of ilrp_idc[ listIdx ][ rplsIdx ][ i ] shall be in the range of 0 to the GeneralLayerIdx[ nuh_layer_id ] − 1, inclusive.

# Conclusion

This document proposes modifications to reference picture list and inter-layer prediction signalling. It is proposed to adopt the proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-O2001-vE, Versatile Video Coding (Draft 6), Gothenburg, July 2019