| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0199-v3 |
| --- | --- |

| Title: | Non-CE3: Align MIP matrix multiplication process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Z. Zhang, K. Andersson, D. Saffar, R. Sjöberg, J. Ström, R. Yu | Tel: Email: | +46761499428 zhi.a.zhang@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In the current version of VVC, the MIP multiplication process is different for a 4 × 16 or 16 × 4 MIP coded block and all other MIP coded blocks. Given a 4 × 16 or 16 × 4 MIP coded block, therefore, a subset of the matrix multiplication output with odd indices is used as the reduced prediction, while for all other MIP coded blocks, the entire matrix multiplication output is used as the reduced prediction.

This contribution proposes to align the MIP matrix multiplication process by defining MipSizeId to be equal to 1 for a 4 × 16 or 16 × 4 MIP coded block. The proposed method eliminates several condition checks which were required to handle different matrix multiplication process.

The proposed method has a luma BDR impact for AI/RA/LDB of 0.01%/0.01%/-0.04% compared to VTM6.0 and decoding time impact for AI/RA/LDB of 100%/100%/99% compared to VTM6.0.

# Introduction

In the current version of VVC, given a 4 × 16 or 16 × 4 Matrix based Intra Prediction (MIP) coded block, the size of the matrix multiplication output is greater than the size of the reduced prediction. Hence, a subset of the matrix multiplication output with odd indices is used as the reduced prediction for the current MIP coded block. In another word, the reduced prediction for a 4 × 16 or 16 × 4 MIP coded block is calculated by:

predred[k]=i=0N-1bdryredi*mWeight2*k+1i,   where k ranges from 0 to M2-1.

The matrix multiplication process for a MIP coded block, which has a dimension other than 4 × 16 or 16 × 4, the reduced prediction is calculated by:

predredk=i=0N-1bdryredi*mWeightki,   where k ranges from 0 to M-1.

Here,

- mWeight specifies the weight matrix, which has a dimension of N × M,

- bdryred specifies the reduced boundary, which has a size of N,

- predred specifies the reduced prediction, which has a size of:

  - (M ∕ 2), if the MIP coded block has a dimension of 4 × 16 or 16 × 4

  - M, if the MIP coded block has a dimension other than 4 × 16 or 16 × 4

The matrix multiplication process of a 4 × 16 or 16 × 4 block is not aligned with the matrix multiplication process of the other block. Therefore, the handling of different matrix multiplication processes requires several condition checks in the MIP prediction process.

# Proposed method

This contribution proposes to align the MIP matrix multiplication process. The proposed method determines the MipSizeId to be equal to 1 for a 4 × 16 or 16 × 4 MIP coded block.

The weight matrix mWeight that is derived from the MIP matrix look-up table by using MipSizeId=1 as the index has a dimension of 8 × 16. The reduced prediction for a 4 × 16 or 16 × 4 MIP coded block has a size of 16 which is equal to the size of the matrix multiplication output.

The reduced prediction for all MIP coded block is calculated by:

predred[k]=i=0N-1bdryredi*mWeightki,   where k ranges from 0 to M-1.

With the proposed method, given a W × H MIP coded block, where W specifies the width of the current coding block, and H specifies the height of the current coding block, the MipSizeId is defined as follows:

- If both W and H are equal to 4, MipSizeId is set equal to 0,

- Otherwise, if (W * H) is less than or equal to 64, MipSizeId is set equal to 1

- Otherwise, MipSizeId is set equal to 2

The proposed method eliminates several condition checks which were required to handle deficient matrix multiplication process. The proposed VVC text change:

8.4    Decoding process for coding units coded in intra prediction mode

8.4.1    General decoding process for coding units coded in intra prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

Output of this process is a modified reconstructed picture before in-loop filtering.

The derivation process for quantization parameters as specified in clause 8.7.1 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight, and the variable treeType as inputs.

When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA, the decoding process for luma samples is specified as follows:

- If pred_mode_plt_flag is equal to 1, the following applies:

- The general decoding process for palette blocks as specified in clause 8.4.5.3 is invoked with the luma location ( xCb, yCb ), the variable startComp set equal to 0, the variable cIdx set to 0, the variable nCbW set equal to cbWidth, the variable nCbH set equal to cbHeight.

- Otherwise (pred_mode_plt_flag is equal to 0), the following applies:

- The variable MipSizeId[ x ][ y ] for x = xCb..xCb + cbWidth − 1 and  y = yCb..yCb + cbHeight − 1 is derived as follows:

- If both cbWidth and cbHeight are equal to 4, MipSizeId[ x ][ y ] is set equal to 0.

- If both cbWidth and cbHeight are less than or equal to 8, MipSizeId[ x ][ y ] is set equal to 1.

- Otherwise, if ( cbWidth * cbHeight ) is less than or equal to 64, MipSizeId[ x ][ y ] is set equal to 1

- Otherwise, MipSizeId[ x ][ y ] is set equal to 2.

- The luma intra prediction mode is derived as follows:

- If intra_mip_flag[ xCb ][ yCb ] is equal to 1, IntraPredModeY[ x ][ y ] with  x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to intra_mip_mode[ xCb ][ yCb ].

- Otherwise, the derivation process for the luma intra prediction mode as specified in clause 8.4.2 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight as input.

- The variable predModeIntra is set equal to IntraPredModeY[ xCb ][ yCb ].

The general decoding process for intra blocks as specified in clause 8.4.5.1 is invoked with the sample location ( xTb0, yTb0 ) set equal to the luma location ( xCb, yCb ), the variable nTbW set equal to cbWidth, the variable nTbH set equal to cbHeight, predModeIntra, and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before in-loop filtering.

8.4.5.2.1    Matrix-based intra sample prediction

Inputs to this process are:

- a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable predModeIntra specifying the intra prediction mode,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

Variables numModes, boundarySize, predW, predH and predC are derived using MipSizeId[ xTbCmp ][ yTbCmp ] as specified in Table 8-4.

Table 8-4 – Specification of number of prediction modes numModes, boundary size boundarySize, and prediction sizes predW, predH and predC using MipSizeId

| MipSizeId | numModes | boundarySize | predW | predH | predC |
| --- | --- | --- | --- | --- | --- |
| 0 | 35 | 2 | 4 | 4 | 4 |
| 1 | 19 | 4 | 4 | 4 | 4 |
| 2 | 11 | 4 | Min( nTbW, 8 ) | Min( nTbH, 8 ) | 8 |
| 2 | 11 | 4 | 8 | 8 | 8 |

The flag isTransposed is derived as follows:

isTransposed = ( predModeIntra > ( numModes / 2 ) )  ?  TRUE  :  FALSE	(8-55)

The variable inSize is derived as follows:

inSize = ( 2 * boundarySize ) − ( MipSizeId[ xTbCmp ][ yTbCmp ]  = =  2 )  ?  1  :  0	(8-56)

The variables mipW and mipH are derived as follows:

mipW = isTransposed  ?  predH  :  predW				(8-57)

mipH = isTransposed  ?  predW  :  predH				(8-58)

For the generation of the reference samples refT[ x ] with x = 0..nTbW − 1 and refL[ y ] with y = 0..nTbH − 1, the following applies:

- The reference sample availability marking process as specified in clause 8.4.5.2.7 is invoked with the sample location ( xTbCmp, yTbCmp ), reference line index equal to 0, the reference sample width nTbW, the reference sample height nTbH, colour component index equal to 0 as inputs, and the reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..nTbH − 1 and x = 0..nTbW − 1, y = −1 as output.

- When at least one sample refUnfilt[ x ][ y ] with x = −1, y = −1.. nTbH − 1 and x = 0.. nTbW − 1, y = −1 is marked as "not available for intra prediction", the reference sample substitution process as specified in clause 8.4.5.2.8 is invoked with reference line index 0, the reference sample width nTbW, the reference sample height nTbH, the reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..nTbH − 1 and x = 0..nTbW − 1, y = −1, and colour component index 0 as inputs, and the modified reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..nTbH − 1 and x = 0..nTbW − 1, y = −1 as output.

- The reference samples refT[ x ] with x = 0..nTbW − 1 and refL[ y ] with y = 0..nTbH − 1 are assigned as follows:

refT[ x ] = refUnfilt[ x ][ −1 ]						(8-59)

refL[ y ] = refUnfilt[ −1 ][ y ]						(8-60)

For the generation of the input samples p[ x ] with x = 0..2 * inSize − 1, the following applies:

- The MIP boundary downsampling process as specified in clause 8.4.5.2.2 is invoked for the top reference samples with the block size nTbW, the reference samples refT[ x ] with x = 0..nTbW − 1, and the boundary size boundarySize as inputs, and reduced boundary samples redT[ x ] with x = 0..boundarySize − 1 as outputs.

- The MIP boundary downsampling process as specified in clause 8.4.5.2.2 is invoked for the left reference samples with the block size nTbH, the reference samples refL[ y ] with y = 0..nTbH − 1, and the boundary size boundarySize as inputs, and reduced boundary samples redL[ x ] with x = 0..boundarySize − 1 as outputs.

- The reduced top and left boundary samples redT and redL are assigned to the boundary sample array pTemp[ x ] with x = 0..2 * boundarySize − 1 as follows:

- If isTransposed is equal to 1, pTemp[ x ] is set equal to redL[ x ] with x = 0..boundarySize − 1 and pTemp[ x + boundarySize ] is set equal to redT[ x ] with x = 0..boundarySize − 1.

- Otherwise, pTemp[ x ] is set equal to redT[ x ] with x = 0..boundarySize − 1 and pTemp[ x + boundarySize ] is set equal to redL[ x ] with x = 0..boundarySize − 1.

- The input values p[ x ] with x = 0.. inSize − 1 are derived as follows:

- If MipSizeId[ xTbCmp ][ yTbCmp ] is equal to 2, the following applies:

p[ x ] = pTemp[ x + 1 ] − pTemp[ 0 ]				(8-61)

- Otherwise (MipSizeId[ xTbCmp ][ yTbCmp ] is less than 2), the following applies:

p[ 0 ] = pTemp[ 0 ] − ( 1 << ( BitDepthY − 1 ) )
p[ x ] = pTemp[ x ] − pTemp[ 0 ]	for x = 1.. inSize − 1	(8-62)

For the intra sample prediction process according to predModeIntra, the following ordered steps apply:

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW predW− 1, y = 0..mipH predH − 1 are derived as follows:

- The variable modeId is derived as follows:

modeId = predModeIntra − ( isTransposed  ?  numModes / 2  :  0 )	(8-63)

- The weight matrix mWeight[ x ][ y ] with x = 0..2 * inSize − 1, y = 0..predC * predC − 1 is derived by invoking the MIP weight matrix derivation process as specified in clause 8.4.5.2.3 with MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as inputs.

- The variable sW is derived using MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as specified in Table 8-5.

- The variable sO is derived using MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as specified in Table 8-6.

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW predW − 1, y = 0..mipH  predH − 1 are derived as follows:

oW = ( 1 << ( sW − 1 ) ) − fO * (i = 0inSize - 1p i )	(8-64)

incW = ( predC > mipW )  ?  2  :  1			(8-65)

incH = ( predC > mipH )  ?  2  :  1			(8-66)

predMip[ x ][ y ] = ( ( ( i = 0inSize - 1mWeight i  y*incH*predC+x*incW *p i  ) + 
	oW ) >> sW ) + pTemp[ 0 ]			(8-67)

predMip[ x ][ y ] = ( ( ( i = 0inSize - 1mWeight i  y*predC+x *p i  ) + 	oW ) >> sW ) + pTemp[ 0 ]			(8-67)

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW predW− 1, y = 0..mipH predH − 1 are clipped as follows:

predMip[ x ][ y ] = Clip1Y( predMip[ x ][ y ] )	(8-68)

- When isTransposed is equal to TRUE, the predH x predW array predMip[ x ][ y ] with x = 0..predH − 1, y = 0..predW − 1 is transposed as follows:

predTemp[ y ][ x ] = predMip[ x ][ y ]	 	(8-69)

predMip =predTemp						(8-70)

- The predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If nTbW is greater than predW or nTbH is greater than predH, the MIP prediction upsampling process as specified in clause 8.4.5.2.4 is invoked with the input block width predW, the input block height predH, matrix-based intra prediction samples predMip[ x ][ y ] with x = 0..predW − 1, y = 0..predH − 1, the transform block width nTbW, the transform block height nTbH, the top reference samples refT[ x ] with x = 0..nTbW − 1, and the left reference samples refL[ y ] with y = 0..nTbH − 1 as inputs, and the output is the predicted sample array predSamples.

Otherwise, predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 is set equal to predMip[ x ][ y ].

# Simulation results

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | -0.01% | 100% | 98% |
| Class A2 | 0.02% | 0.02% | 0.01% | 101% | 100% |
| Class B | 0.01% | -0.01% | 0.00% | 100% | 99% |
| Class C | 0.02% | -0.05% | 0.06% | 100% | 101% |
| Class E | 0.01% | -0.02% | 0.06% | 100% | 100% |
| Overall | 0.01% | -0.01% | 0.02% | 100% | 100% |
| Class D | 0.02% | 0.02% | 0.06% | 100% | 98% |
| Class F | 0.01% | -0.01% | -0.11% | 100% | 100% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.03% | 0.16% | 99% | 101% |
| Class A2 | 0.03% | 0.06% | 0.10% | 100% | 100% |
| Class B | 0.00% | 0.05% | 0.05% | 100% | 99% |
| Class C | 0.03% | 0.00% | -0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.05% | 100% | 100% |
| Class D | -0.07% | -0.23% | 0.19% | 100% | 99% |
| Class F | 0.01% | -0.02% | -0.02% | 100% | 100% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,00% | 0,13% | 0,15% | 101% | 99% |
| Class C | 0,03% | 0,01% | -0,05% | 103% | 101% |
| Class E | -0,19% | 1,01% | 0,70% | 103% | 99% |
| Overall | -0,04% | 0,31% | 0,22% | 102% | 99% |
| Class D | -0,03% | 0,17% | -0,32% | 102% | 99% |
| Class F | -0,08% | -0,34% | 0,66% | 102% | 99% |

# Conclusion

The proposed method aligns the MIP matrix multiplication process. The proposed method eliminates several condition checks which were used to handle the differences between MIP prediction of a 4 × 16 or 16 × 4 MIP coded block and all other MIP coded blocks. The proposed method has very small coding efficiency impact compared to VTM6.0.

We propose to adopt align MIP matrix multiplication process to VVC and VTM.

# References

[1]. Versatile Video Coding (Draft 6)

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).