| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0200-v2 |
| --- | --- |

| Title: | Non-CE4: Modifications on BDOF intermediate parameter derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Du Liu Ruoyang Yu Torshamnsgatan 23, 164 83 Stockholm, Sweden | Email: | du.liu@ericsson.com ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In VTM-6.0, the bi-directional optical flow (BDOF) method is used to improve prediction accuracy for a bi-predicted inter CU. When BDOF is enabled, it calculates a set of intermediate parameters and derives a pair of refinement parameters for each 4x4 subblock inside the inter block. It then derives final prediction samples for each 4x4 subblock using the corresponding refinement parameters. The intermediate parameters include one sample difference block diff[ x ][ y ] and two gradient summation blocks tempH[ x ][ y ] and tempV[ x ][ y ]. It is reported that the shift operation (shift2) of diff[ x ][ y ] depends on the bit depth of luma samples and that the calculation of tempH[ x ][ y ] and tempV[ x ][ y ] requires shift operations for each sample.

This contribution proposes following two methods. Method 1) removes the shift operations for deriving tempH[ x ][ y ] and tempV[ x ][ y ]. Method 2) unifies the shift parameter (shift2) for deriving the diff[ x ][ y ] for all bit depths.

The proposed methods were reportedly tested under VTM-6.0 random access (RA) common test configuration (10-bit). The BD-rate impact for method 1 is reported to be 0.00%. The BD-rate impact for combining method 1 and method 2 is reported to be 0.00%.

# Introduction

In VTM-6.0 BDOF, the horizontal and vertical gradients gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are computed as

gradientHL0[ x ][ y ]  =  ( predSamplesL0[ hx + 1 ][vy] >> shift1 ) − 	(8-803)
	( predSampleL0[ hx − 1 ][ vy ] ) >> shift1 )

gradientVL0[ x ][ y ]  =  ( predSampleL0[ hx ][ vy + 1 ] >> shift1 ) − 	(8-804)
	( predSampleL0[ hx ][vy − 1 ] ) >> shift1 )

gradientHL1[ x ][ y ]  =  ( predSamplesL1[ hx + 1 ][vy] >> shift1 ) − 	(8-805)
	( predSampleL1[ hx − 1 ][ vy ] ) >> shift1 )

gradientVL1[ x ][ y ]  =   ( predSampleL1[ hx ][ vy + 1 ] >> shift1 ) − 	(8-806)
	( predSampleL1[ hx ][vy − 1 ] ) >> shift1 )

where predSamplesL0 and predSamplesL1 are prediction sample values and shift1 = Max( 6, bitDepth − 6 ).

The sample difference block diff[ x ][ y ], gradient summation tempH[ x ][ y ], and gradient summation tempV[ x ][ y ] are derived as

diff[ x ][ y ] = (predSamplesL0[ hx ][ vy ] >> shift2 ) − ( predSamplesL1[ hx ][ vy ] >> shift2 )	(8-807)

tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] ) >> shift3	(8-808)

tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] ) >> shift3	(8-809)

where shift2 = Max( 4, bitDepth − 8 ) and shift3 = Max( 1, bitDepth − 11 ).

The BDOF correlation parameters are calculated as

sGx2 = ij Abs( tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-810) sGy2 = ij Abs( tempV[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-811) sGxGy = ij( Sign( tempV[ xSb + i ][ ySb + j ] ) * tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-812) sGxGym = sGxGy >> 12 (8-813) sGxGys = sGxGy & ( ( 1 << 12 ) − 1 ) (8-814) sGxdI = ij( − Sign( tempH[ xSb + i ][ ySb + j ] ) * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-815) sGydI = ij( − Sign( tempV[ xSb + i ][ ySb + j ] ) * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-816) The horizontal and vertical motion offset of the current subblock are derived as vx = sGx2 > 0 ? Clip3( −mvRefineThres, mvRefineThres, (8-817) −( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) ) : 0 vy = sGy2 > 0 ? Clip3( −mvRefineThres, mvRefineThres, ( ( sGydI << 3 ) − (8-818) ( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGy2 ) ) ) : 0 The BDOF offset and prediction are given as bdofOffset = ( vx * ( gradientHL0[ x + 1 ][ y + 1 ] − gradientHL1[ x + 1 ][ y + 1 ] ) ) >> 1 (8-819) + ( vy * (gradientVL0[ x + 1 ][ y + 1 ] − gradientVL1[ x + 1 ][ y + 1 ] ) ) >> 1 pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset4 + (8-820) predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset ) >> shift4 )

The following table shows the bit-width for each parameter.

| Parameter | bit-width |  |
| --- | --- | --- |
| predSamplesL0, predSamplesL1 | 16 |  |
| gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ], gradientVL1[ x ][ y ] | 17 – shift1 | = 17 – Max(6, bitDepth – 4) |
| diff[ x ][ y ] | 17 – shift2 | = 17 – Max(4, bitDepth – 8) |
| tempH[ x ][ y ], tempV[ x ][ y ] | 18 – shift1 – shift3 | = 18 – Max(6, bitDepth – 4) – Max( 1, bitDepth − 11 ) |
| sGx2, sGy2, sGxGy | 24 – shift1 – shift3 | = 24 – Max(6, bitDepth – 4) – Max( 1, bitDepth − 11 ) |
| sGxdI, sGydI | 23 – shift2 | = 23 – Max(4, bitDepth – 8) |

Table 1. Bit-width for each parameter for BDOF intermediate calculations.

# Proposal

## Method 1: Remove shift3

The method proposes to remove shift3 when calculating tempH[ x ][ y ], tempV[ x ][ y ], i.e.,

tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] )

tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] )

This saves two shifts for each (i,j) within the window Ω. For a subblock of 4x4, the computed window is 6x6 and this saves 6x6x2 = 72 shifts.

Then, the horizontal and vertical motion offset of the current subblock are derived as:

vx = sGx2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres,	
	−( sGxdI << 43 ) >> Floor( Log2( sGx2 ) ) )  :  0

vy = sGy2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres, ( ( sGydI << 43 ) − 	
	( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGy2 ) ) )  :  0

The affected bit-widths of the intermediate parameters are given in Table 2. The modified bit-widths are within 32 bits.

| Parameter | bit-width |
| --- | --- |
| tempH[ x ][ y ], tempV[ x ][ y ] | 18 – Max(6, bitDepth – 4) |
| sGx2, sGy2, sGxGy | 24 – Max(6, bitDepth – 4) |
| sGxdI, sGydI | 23 – Max(4, bitDepth – 8) |

Table 2. Bit-width for BDOF intermediate calculations after modification.

The specification text [1] is modified as follows:

8.5.6 Decoding process for inter blocks 8.5.6.5 Bi-directional optical flow prediction process Inputs to this process are: - two variables nCbW and nCbH specifying the width and the height of the current coding block, - two (nCbW + 2)x(nCbH + 2) luma prediction sample arrays predSamplesL0 and predSamplesL1, - the prediction list utilization flags predFlagL0 and predFlagL1, - the reference indices refIdxL0 and refIdxL1, - the bi-directional optical flow utilization flags bdofUtilizationFlag[ xIdx ][ yIdx ] with xIdx = 0..( nCbW >> 2 ) − 1, yIdx = 0..( nCbH >> 2 ) − 1. Output of this process is the (nCbW)x(nCbH) array pbSamples of luma prediction sample values. Variables bitDepth, shift1, shift2, shift3, shift4, offset4, and mvRefineThres are derived as follows: - The variable bitDepth is set equal to BitDepthY. - The variable shift1 is set to equal to Max( 6, bitDepth − 6 ). - The variable shift2 is set to equal to Max( 4, bitDepth − 8 ). - The variable shift3 is set to equal to Max( 1, bitDepth − 11 ). - The variable shift4 is set equal to Max( 3, 15 − bitDepth ) and the variable offset4 is set equal to 1 << ( shift4 − 1 ). - The variable mvRefineThres is set equal to 1 << Max( 5, bitDepth − 7 ). For xIdx = 0..( nCbW >> 2 ) − 1 and yIdx = 0..( nCbH >> 2 ) − 1, the following applies: - The variable xSb is set equal to ( xIdx << 2) + 1 and ySb is set equal to ( yIdx << 2 ) + 1. - If bdofUtilizationFlag[ xIdx ][ yIdx ] is equal to FALSE, for x = xSb − 1..xSb + 2, y = ySb − 1.. ySb + 2, the prediction sample values of the current subblock are derived as follows: pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset4 + (8-800) predSamplesL1[ x + 1 ][ y + 1 ] ) >> shift4 ) - Otherwise (bdofUtilizationFlag[ xIdx ][ yIdx ] is equal to TRUE), the prediction sample values of the current subblock are derived as follows: - For x =xSb − 1..xSb + 4, y = ySb − 1..ySb + 4, the following ordered steps apply: - The locations ( hx, vy ) for each of the corresponding sample locations ( x, y ) inside the prediction sample arrays are derived as follows: hx = Clip3( 1, nCbW, x ) (8-801) vy = Clip3( 1, nCbH, y ) (8-802) - The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows: gradientHL0[ x ][ y ] = ( predSamplesL0[ hx + 1 ][vy] >> shift1 ) − (8-803) ( predSampleL0[ hx − 1 ][ vy ] ) >> shift1 ) gradientVL0[ x ][ y ] = ( predSampleL0[ hx ][ vy + 1 ] >> shift1 ) − (8-804) ( predSampleL0[ hx ][vy − 1 ] ) >> shift1 ) gradientHL1[ x ][ y ] = ( predSamplesL1[ hx + 1 ][vy] >> shift1 ) − (8-805) ( predSampleL1[ hx − 1 ][ vy ] ) >> shift1 ) gradientVL1[ x ][ y ] = ( predSampleL1[ hx ][ vy + 1 ] >> shift1 ) − (8-806) ( predSampleL1[ hx ][vy − 1 ] ) >> shift1 ) - The variables diff[ x ][ y ], tempH[ x ][ y ] and tempV[ x ][ y ] are derived as follows: diff[ x ][ y ] = (predSamplesL0[ hx ][ vy ] >> shift2 ) − ( predSamplesL1[ hx ][ vy ] >> shift2 ) (8-807) tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] ) >> shift3 (8-808) tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] ) >> shift3 (8-809) - The variables sGx2, sGy2, sGxGy, sGxdI and sGydI are derived as follows: sGx2 = ij Abs( tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-810) sGy2 = ij Abs( tempV[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-811) sGxGy = ij( Sign( tempV[ xSb + i ][ ySb + j ] ) * tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-812) sGxGym = sGxGy >> 12 (8-813) sGxGys = sGxGy & ( ( 1 << 12 ) − 1 ) (8-814) sGxdI = ij( − Sign( tempH[ xSb + i ][ ySb + j ] ) * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-815) sGydI = ij( − Sign( tempV[ xSb + i ][ ySb + j ] ) * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-816) - The horizontal and vertical motion offset of the current subblock are derived as: vx = sGx2 > 0 ? Clip3( −mvRefineThres, mvRefineThres, (8-817) −( sGxdI << 43 ) >> Floor( Log2( sGx2 ) ) ) : 0 vy = sGy2 > 0 ? Clip3( −mvRefineThres, mvRefineThres, ( ( sGydI << 43 ) − (8-818) ( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGy2 ) ) ) : 0 - For x =xSb − 1..xSb + 2, y = ySb − 1..ySb + 2, the prediction sample values of the current sub-block are derived as follows: bdofOffset = ( vx * ( gradientHL0[ x + 1 ][ y + 1 ] − gradientHL1[ x + 1 ][ y + 1 ] ) ) >> 1 (8-819) + ( vy * (gradientVL0[ x + 1 ][ y + 1 ] − gradientVL1[ x + 1 ][ y + 1 ] ) ) >> 1 pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset4 + (8-820) predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset ) >> shift4 )

## Method 2: Unify shift2 for all bit depths

The method proposes to unify shift2 to a fixed value shift2 = 4 for all bit depths. The affected bit-widths of the intermediate parameters are given in Table 3. From Table 3, after fixing the value of shift2 = 4, the modified bit-widths are still within 32 bits. It is asserted that even if the input bit-width exceeds 12 bits, e.g. 14-bit or 16-bit, the BDOF intermediate variables does not exceed 32 bits and diff[ x ][ y ] does not exceed 16 bits.

| Parameter | bit-width |
| --- | --- |
| diff[ x ][ y ] | 17 – shift2 = 13 |
| sGxdI, sGydI | 23 – shift2 = 19 |

Table 3. Bit-width for each parameter for BDOF intermediate calculations with shift2 = 4.

The specification text is modified as follows:

8.5.6 Decoding process for inter blocks 8.5.6. 6 5 Bi-directional optical flow prediction process … Variables bitDepth, shift1, shift2, shift3, shift4, offset4, and mvRefineThres are derived as follows: - The variable bitDepth is set equal to BitDepthY. - The variable shift1 is set to equal to Max( 6, bitDepth − 6 ). - The variable shift2 is set to equal to 4 Max( 4, bitDepth − 8 ). - …

# Results

The proposed method was implemented in VTM-6.0 reference software [2]. The simulations were run using the VTM common test configurations [3].

## Test 1: Results of Method 1 - Remove shift3

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.05% | 100% | 98% |
| Class A2 | 0.01% | 0.01% | 0.04% | 99% | 94% |
| Class B | -0.01% | 0.05% | 0.00% | 101% | 102% |
| Class C | -0.01% | -0.05% | -0.04% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.01% | 101% | 99% |
| Class D | -0.02% | -0.02% | 0.06% | 101% | 94% |
| Class F | -0.01% | 0.00% | 0.02% | 104% | 98% |

## Test 2: Supplementary results of Method 1 when setting the internal bitDepth to 8-bit

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 8-bit |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.00% | 0.03% | -0.04% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.02% | -0.06% | 0.09% | 101% | 100% |
| Class F | -0.01% | 0.05% | 0.05% | 108% | 101% |

## Test 3: Supplementary results of Method 1 when setting the internal bitDepth to 12-bit

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 12-bit |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | -0.02% | 100% | 96% |
| Class A2 | 0.01% | -0.01% | 0.03% | 99% | 100% |
| Class B | -0.01% | 0.00% | 0.03% | 100% | 102% |
| Class C | 0.03% | -0.03% | 0.00% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | 0.01% | 100% | 99% |
| Class D | -0.02% | 0.09% | 0.07% | 100% | 98% |
| Class F | -0.01% | -0.03% | -0.02% | 102% | 92% |

Note that the simulation results of the combination of method 1 and method 2 for all the three tests are the same as using method 1 only.

In v1, the PSNR numbers in the accompanying excel sheets are in 6-decimal accuracy. In v2, the PSNR numbers are corrected to be 4-decimal accuracy.

# Conclusion

This contribution proposes two methods:

(1) Remove the shift operations for tempH[ x ][ y ] and tempV[ x ][ y ] to save the number of shifts per sample. Based on the simulation results, it is asserted that those shifts are unnecessary.

(2) Unify the shift parameter (shift2) for all bit depths. It is asserted that the intermediate variables can still be kept within 32 bits range for all bit depths.

We recommend adopting the method 1 or a combination of method 1 and method 2 into the next VTM software and VVC working draft.

# Reference

[1] B. Bross, J. Chen, S. Liu, "Versatile Video Coding (Draft 6)", Output document approved by JVET, document number JVET-O2001.

[2] VTM-6.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).