| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0207 |
| --- | --- |

| Title: | Mobile based heterogeneous multi-core for a VVC Decoder |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | WEI LIU, Yong Beom Cho Yong Beom Cho | Tel: Email: | +82 10 9888 1423 ybcho@konkuk.ac.kr |
| Source: | Department of Electronic Engineering Konkuk University, VLSI LAB |  |  |

_____________________________

- Abstract

This contribution presents a heterogeneous multi-core of the universal mobile processor platform. Because the most difficult task in a heterogeneous multi-core is to schedule tasks, it keeps the entire multi-core system in balance, but often needs to combine the data types of scheduled tasks. Therefore, a support vector machine prediction method based on new parallel processing method for VVC and mobile platform features is proposed to improve the task allocation accuracy of a VVC heterogeneous multi-core system.

Introduction

Many mobile systems on chip (SoC) manufacturers have recently developed (large core, medium core, small core) based ARM Big.Little to trade off performance and energy consumption. Although this solution solves the contradiction between performance and energy consumption, it also increased load balancing when using all cores simultaneously. If we further use heterogeneous mobile devices, it will be more difficult for load balancing. Therefore, we use the SVM to train the features of mobile performance and the video decompression to predict the number of worker threads to achieve real-time processing VVC for each core.

Parallel processing

Fig. 1 shows the parallel solution based on inter-view and intra-view parallelism. The proposed method consists of three modules: the management thread, worker thread pool, and task queue pool. For the first thread, we use parallel technology based on the thread pool and divide the thread into two levels: the management thread and the worker thread. The management thread includes frame threads, and the number of frame threads corresponds to the number of views. There are two types of task queue pools. Q0 represents the main thread task queue, and Q1 represents the in-frame task queue; these correspond to the management thread and the frame thread, respectively. The number of working threads corresponds to the number of central processing unit (CPU) cores in cluster system, and the worker thread is fixed to each CPU core. The main computing task is to complete the main part of the worker thread. The management thread includes decoding the slice header, reference picture set, and reference picture list while writing images to play video or writing inter-frame YUV. There is no data dependency for slice header decoding between multiple frames. Therefore, multiple slice headers can be decoded, and multiple slice header tasks can be performed simultaneously. In order to avoid thread locks between the reference frame management task and YUV input/output (I/O) task, only one task is allowed in the task queue. The management thread is also responsible for waking up the frame thread, and the frame thread sends the computation tasks to the in-frame queue. The management thread is responsible for not only generating the task queue but also waking up the worker thread in the worker thread pool. The worker thread searches for the task unit that needs to be processed from the task pool. The order in which the worker thread searches for the task is from Q0 to Q3 to ensure that the tasks in the management thread queue are executed first. As long as the number of existing tasks is greater than the number of worker threads, all of the computing resources of the CPU are utilized, which improves the parallelism and thus the decompression speed.

Fig. 1 Overall architecture. MT: management thread, F0–F2: frame threads, WT0–WT (n − 1): work

Block level task allocation

In a heterogeneous multi-core system composed of mobile devices, each coding block is used as a parallel technology to implement VVC decoding. However, implementing a method to send tasks with appropriate size to correspond performance level sub nodes is difficult in heterogeneous multi-core VVC decoder. Fig. 1 shows the quadtree structure of different symbols indicate different number of WT (work thread). This is a pruned quadtree since the unused nodes and leaves are removed accordingly. Compared with full quadtree, it only has limited nodes and leaves. The quadtree structure of number of WT partitions can be modeled as a cascaded classification task, as shown in Fig. 2. The task reveals that the splitting or non-splitting of number of WT can be determined by the Classifiers #0, #1 and #2 illustrated in Fig. 2. With these cascaded classifiers, the full quadtree can be effectively pruned.

Fig. 2 worker thread partition and cascaded classifiers

Patent rights declaration(s)

Konkuk University does not have any current or pending patent rights relating to the technology described in this contribution.