| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0210 |
| --- | --- |

| Title: | Non-CE8: Transform skip for chroma block in the single tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jeeyoon Park (SKKU) Byeungwoo Jeon (SKKU) 2066 Seoburo, Jangangu Suwon, Korea | Tel: Email: | +82-31-290-7186 jiyoonpark@skku.edu bjeon@skku.edu |
| Source: | Sungkyunkwan University (SKKU) |  |  |

_____________________________

# Abstract

This contribution proposes to allow the transform skip for a chroma block coded as the DM mode in a single tree according to the transform skip signal of its corresponding luma block. Two test are conducted: one (test 1) is to use the transform skip signal of the corresponding luma block for a chroma block in the DM mode. The other test (test 2) is to use the transform skip signal of the corresponding luma block only for a chroma block of size 4x4 in the DM mode. Simulation results show for the test 1 (second number is for the class F), an average of Y(0.00%, 0.03%), U(-0.01%, -0.01%) and V(0.02%, -0.04%) BDBR for AI; an average of Y(0.00%, 0.01%), U(-0.01%, -0.11%) and V(0.03%, -0.15%) BDBR for RA; and an average of Y(-0.03%, -0.04%), U(0.05%,-0.33%), and V(-0.10%, -0.67%) BDBR for LD. For the test 2, an average of Y(0.00%, 0.00%), U(-0.01%, -0.05%) and V(0.00%, -0.07%) BDBR for AI; an average of Y(-0.01%, -0.04%), U(0.05%, -0.08%) and V(0.03%, -0.15%) BDBR for RA; and an average of Y(-0.03%, -0.16%), U(-0.21%, -0.23%), and V(-0.22%, 0.13%) BDBR for LD.

# Introduction

Transform skip has been introduced since HEVC as an effective coding tool to preserve edges. Together with the intra block copy (IBC), the palette coding, and the BDPCM (block differential pulse coded modulation), the transform skip is found to be effective especially for screen contents coding (SCC). After the MTS and the transform skip are signaled together in VTM6, the transform skip mode is disallowed for chroma blocks. Therefore, it is noted that for a chroma block in the single tree, the transform skip is inherently inapplicable while IBC is applicable to chroma block. Since both IBC and transform skip are effective tool especially for SCC, it is proposed that the transform skip is allowed to a chroma block in a single tree same as the IBC.

# Proposed modification

Compression efficiency can be improved by applying the transform skip for a chroma block in a single tree. In VTM 6.0, under the dual tree configuration, the transform skip and IBC are allowed only for luma blocks, that is, both of them are disallowed for chroma blocks. In case of the single tree, IBC is applied to a chroma block if its corresponding luma block is in the IBC mode. However, the transform skip is disallowed to the chroma block irrespective of whether the corresponding luma is in the transform skip mode or not. Motivated by the same nature of IBC and the transform skip, two proposals for the single tree follows as below.

Proposal 1: In the single tree, the transform skip is applied to a chroma block encoded in a DM mode when the transform skip is applied to its corresponding luma block.

A slightly modified proposal is to apply the proposal one only for the 4x4 chroma blocks as below.

Proposal 2: In the single tree, the transform skip is applied to a 4x4 chroma block encoded in a DM mode when the transform skip is applied to its corresponding luma block.

Two tests are conducted according to the proposal 1 and 2 as below.

Test 1 (proposal 1): A chroma block shares tu.idx == MTS_SKIP when its intra prediction mode is DM.

Test 2 (proposal 2): A chroma block shares tu.idx == MTS_SKIP when its intra prediction mode is DM && (Width==4, Height==4).

# Experimental Results

Experiment is conducted to evaluate the proposed method under the common test condition [2] as below.

- Anchor: VTM6.0

- Test : VTM-6.0 + SKKU_Scheme (TrSkip_single_chroma_test1, TrSkip_single_chroma_test2)

< Test 1 >

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.03% | 99% | 101% |
| Class A2 | -0.01% | 0.00% | 0.04% | 101% | 100% |
| Class B | 0.00% | 0.00% | -0.02% | 96% | 101% |
| Class C | 0.01% | -0.03% | -0.07% | 97% | 101% |
| Class E | 0.01% | 0.02% | 0.17% | 99% | 100% |
| Overall | 0.00% | -0.01% | 0.02% | 98% | 101% |
| Class D | 0.00% | -0.05% | 0.02% | 99% | 99% |
| Class F | 0.03% | -0.01% | -0.04% | 95% | 101% |
| Class SCC | 0.08% | -0.18% | -0.22% | 99% | 100% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.03% | 0.02% | 97% | 100% |
| Class A2 | 0.01% | 0.02% | -0.04% | 99% | 100% |
| Class B | -0.03% | -0.03% | 0.06% | 97% | 100% |
| Class C | 0.01% | -0.05% | 0.07% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.03% | 98% | 100% |
| Class D | 0.01% | 0.03% | -0.14% | 103% | 100% |
| Class F | 0.01% | -0.11% | -0.15% | 99% | 100% |
| Class SCC | 0.04% | -0.17% | -0.18% | 100% | 100% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.15% | 0.01% | 99% | 100% |
| Class C | -0.02% | 0.15% | -0.01% | 98% | 100% |
| Class E | -0.13% | -0.25% | -0.40% | 98% | 100% |
| Overall | -0.03% | 0.05% | -0.10% | 98% | 100% |
| Class D | -0.02% | -0.08% | -0.30% | 95% | 100% |
| Class F | -0.04% | -0.33% | -0.67% | 97% | 99% |

< Test 2 >

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | -0.01% | 0.03% | 0.02% | 100% | 100% |
| Class B | 0.00% | 0.01% | 0.00% | 99% | 101% |
| Class C | 0.01% | -0.06% | 0.00% | 95% | 100% |
| Class E | 0.00% | -0.04% | 0.01% | 101% | 101% |
| Overall | 0.00% | -0.01% | 0.00% | 99% | 100% |
| Class D | -0.01% | 0.01% | -0.03% | 97% | 100% |
| Class F | 0.00% | -0.05% | -0.07% | 96% | 101% |
| Class SCC | 0.01% | -0.06% | -0.07% | 99% | 100% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | 0.02% | 99% | 100% |
| Class A2 | 0.00% | 0.04% | -0.03% | 100% | 101% |
| Class B | -0.01% | 0.03% | 0.06% | 98% | 100% |
| Class C | -0.01% | 0.07% | 0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.05% | 0.03% | 99% | 100% |
| Class D | 0.00% | -0.21% | -0.02% | 101% | 101% |
| Class F | -0.04% | -0.08% | -0.15% | 100% | 100% |
| Class SCC | 0.00% | -0.01% | 0.00% |  |  |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.21% | -0.21% | 100% | 99% |
| Class C | -0.05% | 0.18% | 0.08% | 99% | 97% |
| Class E | -0.07% | -0.75% | -0.65% | 100% | 99% |
| Overall | -0.03% | -0.21% | -0.22% | 100% | 99% |
| Class D | -0.03% | -0.02% | 0.06% | 95% | 102% |
| Class F | -0.16% | -0.23% | 0.13% | 99% | 101% |

- (test results are in the attached excel file JVET-P0210.xlsm).

# Conclusion

This contribution proposes to allow the transform skip for chroma block coded in DM mode in a single tree in the same way as IBC (proposal 1). The proposal is further confined only to the case of 4x4 chroma blocks (proposal 2). It is suggested to adopt the proposed harmonization with IBC into future VTM7.0.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, 15th Meeting, Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

Sungkyunkwan University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).