| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0212 |
| --- | --- |

| Title: | AHG8/AHG12: Decoding multiple independent layers with single-layer decoding process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela (Nokia) Stephan Wenger Byeongdoo Choi (Tencent) Emmanuel Thomas (TNO) Vadim Seregin, Muhammed Coban (Qualcomm) Yong He (InterDigital) Lulin Chen (MediaTek) | Email: | miska.hannuksela@nokia.com stewe@stewe.org bdchoi@tencent.com emmanuel.thomas@tno.nl vseregin@qti.qualcomm.com mcoban@qti.qualcomm.com Yong.He@InterDigital.com lulin.chen@mediatek.com |
| Source: | Nokia, Tencent, TNO, Qualcomm, InterDigital Communications, Inc., MediaTek Inc. |  |  |

_____________________________

# Abstract

This proposal enables using of a single decoder instance and the single-layer VVC decoding process for decoding multiple independent layers. The decoding is performed as if all NAL units resided in a single layer only.

The proposal can be used with any types of independent layers, such as "independent coded region" layers, and texture or depth layers, e.g. for 3DoF+. The type of a layer has no normative impact in the decoding process.

# Introduction

This contribution updates JVET-O1130-v3 for alignment with VVC Draft 6. JVET-O1130 was presented in the JVET closing plenary under some time pressure and further study was encouraged.

JVET-O1130 contained the results of the offline work requested by the HLS BOG review on JVET-O0334, noted in the BOG minutes (JVET-O1076) as follows:

The proponent and other interested participants were requested to work on completing the layered approach based on O0334, taking into account the comments noted above, possibly taking into account related aspects in O0497 and O0393. Revisit after the output of the offline work is available.

# Overview of the proposal

This proposal enables using of a single decoder instance and the single-layer VVC decoding process for decoding multiple independent layers. The decoding is performed as if all NAL units resided in a single layer only.

The proposal can be used with any types of independent layers, such as "independent coded region" layers, and texture or depth layers, e.g. for 3DoF+. The type of a layer has no normative impact in the decoding process.

The proposal keeps all aspects related to independent layers in VVC Draft 6 unchanged. For example, it is still possible to decode each independent layer with a separate decoder instance.

Encoding is performed so that POC values differ across layers, i.e. at most one picture is present for a given POC value among all layers. Fig.1 shows an example of the sequence structure with independently coded region layers. Any prediction structure within an independently coded region layer can be supported by the RPL signaling, based on the POC delta indication.

Figure 1. An example of a sequence structure with independently coded region layers

An asserted benefit of the proposal is that it enables signalling a single profile-tile-level for decoding all the independent layers with a single decoding process. For example, this enables indicating a single profile-tier-level e.g. for a bitstream containing texture and depth coded as independent layers.

The proposal has the following ingredients:

- A VPS flag (inbl_one_pic_per_poc_flag) to indicate that there is one and only one picture per each POC value among all independent layers and that the independent layers can be decoded as if there were in the same layer.

- "Collective" HRD parameters can be signalled for the bitstream that contains multiple independent layers as follows:

  - When inbl_one_pic_per_poc_flag is equal to 1, the profile-tier-level and the HRD parameters applying collectively to the independent layers are indicated as follows in the VPS:

    - A single profile-tier-level syntax structure that applies to the bitstream (as if there were only a single layer)

    - max_dec_pic_buffering_minus1, max_num_reorder_pics, and max_latency_increase_plus1 that apply collectively to the bitstream (rather than per layer)

    - hrd_parameters( ) that apply to the bitstream

  - Independent layer set nesting SEI message is specified for carrying SEI messages that apply collectively to the independent layers of the bitstream. It can be used to carry buffering period and picture timing SEI messages that apply to the bitstream (rather than per layer).

- Possibility to control by external means that the entire bitstream is decoded as if it were a single-layer bitstream. Otherwise, the current layer-wise operation of VVC is carried out.

- The layout information of independently coded region layers can be signaled in the proposed independently coded region layer SEI message. Mapping information to connect each independently coded region to each layer is also included in the SEI message.

Two options are proposed in this contribution:

  - Fig. 2 shows a simple example of independently coded regions with a layer mapping information, as a first option, which includes the offset value of the left-top position and the coded region size of each layer.

  - A second option, Fig. 3, is to indicate the 4-neighbour information of each layer, top, right, bottom and left which constitutes the layout information.

With any of these options, it is asserted that the granularity of the picture size of a layer is finer than that of the sub-picture in the sub-picture partitioning approach as a single layer.

When the independently coded region layer SEI message is present in the bitstream, the layout information signalled in this SEI message is used in the rendering process for composition. The composed picture for displaying is constructred by merging one or more output pictures of the independently coded region layers.

Figure 2. An example of independently coded region partitioning (Option 1 signaling)

Figure 3. An example of 4 layers connected by 4-neighbor information (Option 2 signaling)

# Proposed specification text

## VPS syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(6) |
| if( vps_max_layers_minus1 > 0 ) |  |
| vps_all_independent_layers_flag | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_layer_id[ i ] | u(6) |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |
| vps_independent_layer_flag[ i ] | u(1) |
| if( !vps_independent_layer_flag[ i ] ) |  |
| for( j = 0; j < i; j++ ) |  |
| vps_direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |
| if( vps_max_layers_minus1 > 0 ) { |  |
| vps_output_layers_mode | u(2) |
| if( vps_output_layers_mode = = 2 ) |  |
| for( i = 0; i < vps_max_layers_minus1; i++ ) |  |
| vps_output_layer_flag[ i ] | u(1) |
| } |  |
| vps_constraint_info_present_flag | u(1) |
| vps_reserved_zero_7bits | u(7) |
| if( vps_constraint_info_present_flag ) |  |
| general_constraint_info( ) |  |
| if( vps_max_layers_minus1 > 0 && vps_all_independent_layers_flag ) { |  |
| inbl_one_pic_per_poc_flag | u(1) |
| if( inbl_one_pic_per_poc_flag ) { |  |
| inbl_hrd_param_present_flag | u(1) |
| if( inbl_hrd_param_present_flag ) { |  |
| inbl_num_units_in_tick | u(32) |
| inbl_time_scale | u(32) |
| } |  |
| inbl_max_sub_layers_minus1 | u(3) |
| profile_tier_level( inbl_max_sub_layers_minus1 ) |  |
| inbl_sub_layer_ordering_present_flag | u(1) |
| for( j = ( inbl_sub_layer_ordering_present_flag ? 0 : inbl_max_sub_layers_minus1 ); j <= inbl_max_sub_layers_minus1; j++ ) { |  |
| inbl_max_dec_pic_buffering_minus1[ j ] | ue(v) |
| inbl_max_num_reorder_pics[ j ] | ue(v) |
| inbl_max_latency_increase_plus1[ j ] | ue(v) |
| } |  |
| if( inbl_hrd_param_present_flag ) |  |
| hrd_parameters( inbl_max_sub_layers_minus1 ) |  |
| } |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Additions to VPS semantics

inbl_one_pic_per_poc_flag equal to 0 specifies that coded pictures at different layers may have the same PicOrderCnt value. ols_one_pic_per_poc_flag equal to 1 specifies that the bitstream consists of independent layers and there is one and only one coded picture with a particular PicOrderCnt value in the same CVS among all layers and the content of all the SPS RBSBs referencing this VPS RBSP is identical except potentially for the content of the profile_tier_level( ), general_hrd_parameters( ) and vui_parameters( ) syntax structures contained in the SPS RBSPs.

inbl_hrd_param_present_flag equal to 1 specifies that the syntax elemnts inbl_num_units_in_tick, inbl_time_scale, and the syntax structure hrd_parameters( ) are present in the VPS RBSP syntax structure. inbl_hrd_param_present_flag equal to 0 specifies that the syntax elemnts inbl_num_units_in_tick, inbl_time_scale, and the syntax structure hrd_parameters( ) applying to all indepndent layers collectively are not present in the VPS RBSP syntax structure.

inbl_num_units_in_tick and inbl_time_scale are specified identically to num_units_in_tick and time_scale, respectively, and apply to the hrd_parameters( ) present in the VPS and to the buffering period and picture timing SEI messages that are nested within independent layer set nesting SEI message.

inbl_max_sub_layers_minus1 specifies the maximum number of temporal sub-layers that may be present.

inbl_sub_layer_ordering_present_flag equal to 1 specifies that inbl_max_dec_pic_buffering_minus1[ j ], inbl_max_num_reorder_pics[ j ], and inbl_max_latency_increase_plus[ j ] are present for inbl_max_sub_layers_minus1 + 1 sub-layers. inbl_sub_layer_ordering_present_flag equal to 0 specifies that the values of inbl_max_dec_pic_buffering_minus1[ inbl_max_sub_layers_minus1 ], inbl_max_num_reorder_pics[ inbl_max_sub_layers_minus1 ], and inbl_max_latency_increase_plus1[ inbl_max_sub_layers_minus1 ] apply to all sub-layers.

inbl_max_dec_pic_buffering_minus1[ j ], inbl_max_num_reorder_pics[ j ], and inbl_max_latency_increase_plus[ j ] are specified identically to sps_max_dec_pic_buffering_minus1[ j ], sps_max_num_reorder_pics[ j ], and sps_max_latency_increase_plus1[ j ], respectively, for a bitstream where all the layers present.

## Additions to the decoding process in clause 8.1.1

…

For each CVS in the bitstream, the variable TargetLayerId, which identifies the target layer to be decoded, and the variable HighestTid, which identifies the highest temporal sub-layer to be decoded, are specified as follows:

–	If some external means, not specified in this Specification, is available to set TargetLayerIdList, TargetLayerIdList is set by the external means.

–	Otherwise, if vps_all_independent_layers_flag is equal to 1 and some external means, not specified in this Specification, is available to indicate that the entire bitstream containing all layers is decoded, DecodeAllLayersAsSingleLayerFlag is set equal to 1.

–	Otherwise, if vps_all_independent_layers_flag is equal to 1, TargetLayerIdList is set to contain only vps_layer_id[ 0 ].

–	Otherwise, TargetLayerIdList is set to contain vps_layer_id[ i ] for i in the range of 0 to vps_max_layers_minus1, inclusive.

–	If some external means, not specified in this Specification, is available to set HighestTid, HighestTid is set by the external means.

–	Otherwise, HighestTid is set equal to sps_max_sub_layers_minus1.

…

If TargetLayerIdList is provided by external means or inferred, the following applies:

–	for each CVS in the bitstream, the sub-bitstream extraction process as specified in clause 10 is applied with the CVS, TargetLayerIdList, and HighestTid as inputs, and the output is assigned to a bitstream referred to as CvsToDecode.

–	After that, the instances of CvsToDecode of all the CVSs are concatenated, in decoding order, and the result is assigned to the bitstream BitstreamToDecode.

–	Clause 8.1.2 is repeatedly invoked for each coded picture in BitstreamToDecode in decoding order.

Otherwise (DecodeAllLayersAsSingleLayerFlag is equal to 1), the following applies:

–	It is a requirement for bitstream conformance that inbl_one_pic_per_poc_flag is equal to 1.

–	Separate instances of variables starting with an upper case letter are derived and kept for each layer. Clause 8.1.2 is repeatedly invoked for each coded picture in the bitstream in decoding order. The upper case variables pertaining to the nuh_layer_id of the coded picture provided as input to the decoding process are used in the decoding process.

## Independent layer set nesting SEI message

The independent layer set nesting SEI message provides a mechanism to associate SEI messages applying collectively to all independent layers of the bitstream. An independent layer set nesting SEI message contains one or more SEI messages.

| inbl_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| nesting_zero_bit /* equal to 0 */ | u(1) |
| do |  |
| sei_message( ) |  |
| while( more_rbsp_data( ) ) |  |
| } |  |

Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Independent layer set nesting SEI | Depending on the nested SEI messages. Each nested SEI message has the same persistence scope as if the SEI message was not nested. |

## Independently coded region layer SEI message

Independently coded region layer SEI message syntax

Option 1

| independent_coded_region_layer_info( payloadSize ) { | Descriptor |
| --- | --- |
| composed_pic_width_in_luma_samples | ue(v) |
| composed_pic_height_in_luma_samples | ue(v) |
| num_independent_regions_in_pic_minus1 | ue(v) |
| layer_id_present_flag | u(1) |
| for( i = 0; i <= num_independent_regions_in_pic_minus1; i++) { |  |
| if( layer_id_present_flag ) |  |
| region_layer_id[ i ] | u(6) |
| if( i > 0 ) { |  |
| layer_region_offset_x_in_luma_samples[ i ] | ue(v) |
| layer_region_offset_y_in_luma_samples[ i ] | ue(v) |
| } |  |
| layer_region_width_in_luma_samples[ i ] | ue(v) |
| layer_region_height_in_luma_samples[ i ] | ue(v) |
| } |  |
| } |  |

Independently coded region layer SEI message semantics

An independently coded region layer SEI message provides the location and size information of a independently coded region covered by a layer.

When the independently coded region layer SEI message is present, each region in a picture is independently coded in a layer, and the location and size information of a region corresponding to a layer is used for composition of a composed output picture.

The composed output picture is constructred by merging one or more output pictures of the independently coded region layers.

num_independent_regions_in_pic_minus1 plus 1 specifies the number of independently coded regions in a picture. The value of num_independent_regions_in_pic_minus1 shall be in the range of 0 to 125, inclusive.

layer_id_present_flag equal to 1 specifies that region_layer_id[ i ] is present in this SEI message. layer_id_present_flag equal to 0 specifies that region_layer_id[ i ] is not present in this SEI message

region_layer_id[ i ] specifies the value of NuhLayerId corresponding to the i-th independently coded region. One or more NAL units with nuh_layer_id equal to region_layer_id[ i ] shall be present in the CVS associated with this SEI message. When not present, the value of region_layer_id[ i ] is inferred to be equal to i.

composed_pic_width_in_luma_samples specifies the width of a picture composed by one or more independently coded regions in units of luma samples. composed_pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

composed_pic_height_in_luma_samples specifies the height of a picture composed by one or more independently coded regions in units of luma samples. composed_pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

layer_region_offset_x_in_luma_samples[ i ] specifies the horizontal offset, in units of luma samples, of the top-left corner luma sample of the i-th independently coded region relative to the top-left corner luma sample of the composed picture. When not present, the value of layer_region_offset_x_in_luma_samples[ i ] is inferred to be equal to 0. layer_region_offset_x_in_luma_samples[ i ] shall be an integer multiple of MinCbSizeY.

layer_region_offset_y_in_luma_samples[ i ] specifies the vertical offset, in units of luma samples, of the top-left corner luma sample of the i-th independently coded region relative to the top-left corner luma sample of the composed picture. When not present, the value of layer_region_offset_y_in_luma_samples[ i ] is inferred to be equal to 0. layer_region_offset_y_in_luma_samples[ i ] shall be an integer multiple of MinCbSizeY.

layer_region_width_in_luma_samples[ i ] specifies the width of the i-th independently coded region in units of luma samples. The value of layer_region_width_in_luma_samples[ i ] shall be equal to the value of PicOutputWidthL of the layer with nuh_layer_id equal to region_layer_id[ i ].

layer_region_height_in_luma_samples[ i ] specifies the height of the i-th independently coded region in units of luma samples. The value of layer_region_height_in_luma_samples[ i ] shall be equal to the value of PicOutputHeightL of the layer with nuh_layer_id equal to region_layer_id[ i ].

Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Independently coded region layer | The access unit containing the SEI message and up to but not including the next access unit, in decoding order, that contains an SEI message of the same type or starts a new CVS. |

Option 2

| independent_coded_region_layer_info( payloadSize ) { | Descriptor |
| --- | --- |
| boundary_identifier_length_minus1 | ue(v) |
| boundary_identifier_north | u(v) |
| boundary_identifier_east | u(v) |
| boundary_identifier_south | u(v) |
| boundary_identifier_west | u(v) |
| } |  |

boundary_ identifier_length_minus1 plus 1 specifies the number of bits used to represent the syntax elements boundary_identifier_north[ i ], boundary_identifier_east[ i ], boundary_identifier_south[ i ] and boundary_identifier_west[ i ]. The value of boundary_id_length_minus1 shall be in the range of 0 to 15, inclusive.

boundary_identifier_north[ i ], boundary_identifier_east[ i ], boundary_identifier_south[ i ], boundary_identifier_west[ i ] specifies the identifier respecitvely at the north, east, south and west boundary of the decoded picture of the i-th layer within the output picture. The length of the boundary_identifier_north[ i ], boundary_identifier_east[ i ], boundary_identifier_south[ i ] and boundary_identifier_west[ i ] syntax elements is boundary_id_length_minus1 + 1 bits.

When the values of the pair boundary_identifier_north[ i ] and boundary_identifier_south[ j ] or the values of the pair boundary_identifier_east[ i ] and boundary_identifier_west[ j ] are equal the decoded picture of the i-th layer and the decoded picture of the j-th layer are adjacent in the output picture and they share a common boundary respectively at the boundary north/south and east/west. Two decoded pictures adjacent by the north/south boundary are aligned on their west boundary in the output picture. Two decoded pictures adjacent by the east/west boundary are aligned on their north boundary in the output picture.

Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Independently coded region layer | The access unit containing the SEI message and up to but not including the next access unit, in decoding order, that contains an SEI message of the same type or starts a new CVS. |

# Patent rights declaration(s)

Nokia, Tencent, TNO, Qualcomm, InterDigital Communications, Inc., and MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).