| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0216 |
| --- | --- |

| Title: | Non-CE6 : Simplified SBT mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jeeyoon Park (SKKU) Byeungwoo Jeon (SKKU) 2066 Seoburo, Jangangu Suwon, Korea | Tel: Email: | +82-31-290-7186 jiyoonpark@skku.edu bjeon@skku.edu |
| Source: | Sungkyunkwan University (SKKU) |  |  |

_____________________________

- Abstract

This contribution proposes context simplification for cu_sbt_pos_flag in VVC 6.0. Since its probability being 0 or 1 is expected to be similar to each other, it is proposed to encode cu_sbt_pos_flag in the bypass-mode. This simplification shows an average of Y(-0.01%), U(0.07%) and V(0.00%) BDBR for RA; and an average of Y(0.01%), U(0.05%) and V(0.08%) BDBR for LD.

Introduction

In VVC 6.0, the sub-block transform (SBT) mode is used to split an INTER coding block into 2 TUs – one is zero-residual part, and the other is non-zero residual part. It is signaled by four flags as below:

cu_sbt_flag equal to 1 specifies that for the current coding unit, subblock transform is used. Its value being 0 specifies that for the current coding unit, subblock transform is not used.

cu_sbt_quad_flag equal to 1 specifies that for the current coding unit, the subblock transform includes a transform unit of 1/4 size of the current coding unit. cu_sbt_quad_flag equal to 0 specifies that for the current coding unit the subblock transform includes a transform unit of 1/2 size of the current coding unit.

cu_sbt_horizontal_flag equal to 1 specifies that the current coding unit is split horizontally into 2 transform units. cu_sbt_horizontal_flag equal to 0 specifies that the current coding unit is split vertically into 2 transform units.

cu_sbt_pos_flag equal to 1 specifies that the tu_cbf_luma, tu_cbf_cb and tu_cbf_cr of the first transform unit in the current coding unit are not present in the bitstream. cu_sbt_pos_flag equal to 0 specifies that the tu_cbf_luma, tu_cbf_cb and tu_cbf_cr of the second transform unit in the current coding unit are not present in the bitstream.

This contribution proposes to code the cu_sbt_pos_flag (whose values of 0 or 1 is expected to be equally probable) in the bypass mode.

Experimental Results

Experiment is conducted to evaluate the proposed method under the common test condition [2] as below.

- Anchor: VTM6.0

- Test : VTM-6.0 + SKKU_Scheme (SBT_Simplification)

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 98% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 104% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 100% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.08% | 0.01% | 100% | 100% |
| Class A2 | 0.02% | 0.06% | 0.03% | 101% | 101% |
| Class B | -0.02% | 0.07% | -0.06% | 99% | 100% |
| Class C | -0.02% | 0.09% | 0.04% | 95% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.07% | 0.00% | 99% | 100% |
| Class D | -0.03% | -0.01% | 0.05% | 92% | 101% |
| Class F | 0.01% | -0.03% | 0.01% | 99% | 100% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01 | -0.01 | -0.13 | 102% | 99% |
| Class C | 0.01% | -0.18% | 0.03% | 101% | 100% |
| Class E | 0.00% | 0.45% | 0.49% | 98% | 102% |
| Overall | 0.01 | 0.05 | 0.08 | 101% | 100% |
| Class D | -0.04% | 0.26% | 0.00% | 100% | 99% |
| Class F | 0.01 | -0.06 | 0.83 | 101% | 99% |

- (test results are in the attached excel file JVET-P0216.xlsm).

Proposed modification to VVC draft 6

The following table shows a suggested modification by this contribution to the coding unit syntax table of VVC draft 6 of JVET-O2001.

7.3.8.5 Coding unit syntax in VVC draft 6 of JVET-O2001

| if( allowSbtVerH ¦ ¦ allowSbtHorH ¦ ¦ allowSbtVerQ ¦ ¦ allowSbtHorQ ) |  |
| --- | --- |
| cu_sbt_flag | ae(v) |
| } |  |
| if( cu_sbt_flag ) { |  |
| if( ( allowSbtVerH ¦ ¦ allowSbtHorH ) && ( allowSbtVerQ ¦ ¦ allowSbtHorQ) ) |  |
| cu_sbt_quad_flag | ae(v) |
| if( ( cu_sbt_quad_flag && allowSbtVerQ && allowSbtHorQ ) ¦ ¦ ( !cu_sbt_quad_flag && allowSbtVerH && allowSbtHorH ) ) |  |
| cu_sbt_horizontal_flag | ae(v) |
| cu_sbt_pos_flag | ae(v)u(1) |
| } |  |

The following table shows a suggested modification to the table 9-82 of VVC draft 6 of JVET-O2001, which specifies the CABAC context assigned to each bin of the VVC bit-stream syntax.

Table 9-82-Assignment of ctxInc to syntax elements with context code bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| … | … | … | … | … | … | … |
| cu_sbt_flag | ( cbWidth * cbHeight < 256 ) ? 1 : 0 | na | na | na | na | na |
| cu_sbt_quad_flag | 0 | na | na | na | na | na |
| cu_sbt_horizontal_flag | ( cbWidth = = cbHeight ) ? 0 : ( cbWidth < cbHeight ) ? 1 : 2 | na | na | na | na | na |
| cu_sbt_pos_flag | 0 Bypass | na | na | na | na | na |

Conclusion

This contribution proposes to simplify the coding of the cu_sbt_pos_flag flag by encoding it in the bypass mode. It is suggested to adopt the proposed simplification into future VTM7.0.

References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, 15th Meeting, Gothenburg, SE, 3–12 July 2019.

- Frank Bossen, Jill Boyce, K. Suehring, X. Li, and Vadim Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

Patent rights declaration(s)

Sungkyunkwan University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).