| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0231 |
| --- | --- |

| Title: | AHG12: On brick information signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bae-Keun Lee (XRis Corp) Dongsan Jun (Kyungnam University) | Tel: Email: | +82-31-620-1200 baekeun.lee@xris.co.kr dsjun9643@kyungnam.ac.kr |
| Source: | XRis Corporation, Kyungnam University |  |  |

_____________________________

- Abstract

This contribution proposes to modify the semantics of brick information signalling such as brick_height_minus1[ i ] and single_tile_in_pic_flag.

Introduction

VVC Draft6 [1] includes the signalling methods for bricks, tiles, and slices. This contribution proposes two modifications in terms of the brick signaling.

Following is applied

- Proposal 1: It is proposed to clarify the semantic of brick_height_minus1[i].

- Proposal 2: It is proposed to disallow the single tile with more than one brick in the picture, when single_tile_in_pic_flag is equal to 1.

Proposal

Proposal 1

It is proposed to clarify the semantics of brick_height_minus1[i], where the index of i specifies the i-th tile.

If brick_height_minus1[i] is present, the value of brick_height_minus1[ i ] shall be in the range of 0 to RowHeight[ i ] – 2.

When not present (for example, i-th tile divides into two bricks and RowHeight[i] is equal to 2), brick_height_minus1[i] is inferred as RowHeight[i]-2. So the sematics of brick_height_minus1[i] in the VVC draft6 [1] is needed to clarify as followings:

brick_height_minus1[ i ] plus 1 specifies the height of the brick rows excluding the bottom brick in the i-th tile in units of CTBs when uniform_brick_spacing_flag[ i ] is equal to 1. When present, the value of brick_height_minus1 shall be in the range of 0 to RowHeight[ i ] − 2, inclusive. When not present, the value of brick_height_minus1[ i ] is inferred to be equal to RowHeight[ i ] − 1. RowHeight[i]-2.

Proposal 2

Aa bitstream conformance constraint is proposed that single tile is disallowed to have multiple bricks.

It means that single tile is prohibited to have multiple bricks in the picture, when single_tile_in_pic_flag is equal to 1.

If a picture has single tile with multiple bricks, it is hypothetically same with multiple tiles with no further brick splitting for each tile.

After adding bitstream constraint as proposal 3, Figure 1 (a) is regarded as Figure 1 (b).

Figure 1

The proposed syntax and semantic are shown highlighted below compared with VVC draft6 [1]

- 7.4.3.4 Picture parameter set RBSP semantics

single_tile_in_pic_flag equal to 1 specifies that there is only one tile in each picture referring to the PPS. single_tile_in_pic_flag equal to 0 specifies that there is more than one tile in each picture referring to the PPS.

NOTE – In absence of further brick splitting within a tile, the whole tile is referred to as a brick. When a picture contains only a single tile without further brick splitting, it is referred to as a single brick.

It is a requirement of bitstream conformance that if single_tile_in_pic_flag is equal to 1, there is no further brick splitting in the tile.

It is a requirement of bitstream conformance that the value of single_tile_in_pic_flag shall be the same for all PPSs that are referred to by coded pictures within a CVS.

References

[1] JVET-O2001-vE, Versatile Video Coding (Draft 6), Gothenburg, July 2019

Patent rights declaration(s)

XRis Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kyungnam University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).