| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0239-v1 |
| --- | --- |

| Title: | AHG17: Picture Header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan, Tim Hellman, Brian Heng | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the addition of a mandatory picture header and proposes to move syntax elements currently in the slice header to this picture header. Two groups of syntax elements are identified in the contribution.  The first group of syntax elements are already constrained to be the same in all slices of a picture.  It is noted that moving these fields to the picture header so they are signalled only once per picture instead of once per slice avoids unnecessary redundant transmission of bits.  The second group of syntax elements are not currently constrained to be the same in all slices of a picture.  It is suggested to assess the anticipated usage of these syntax elements to determine which should be moved into the picture header to simplify the overall VVC design as it is claimed there is a complexity impact to processing a large number of syntax elements in every slice header.

# Introduction

A mandatory picture header concept is proposed to be transmitted once per picture as the first VCL NAL unit of a picture.  It is also proposed to move syntax elements currently in the slice header to this picture header.  Syntax elements that functionally only need to be transmitted once per picture could be moved to the picture header instead of being transmitted multiple times for a given picture, e.g., syntax elements in the slice header are transmitted once per slice.  The authors claim there is a benefit to moving syntax elements from the slice header as the computation required for slice header processing can be a limiting factor to overall throughput.

Two groups of syntax elements are identified in the following sections.  Section 2 lists syntax elements that are already constrained to be the same in all slices of a picture.  Section 3 lists syntax elements that are not currently constrained to be the same in all slices of a picture.

# Moving slice header syntax elements constrained to be the same within a picture

The syntax elements in this section are already constrained to be the same in all slices of a picture.  It is asserted that moving these fields to the picture header so they are signalled only once per picture instead of once per slice avoids unnecessary redundant transmission of bits without any change to the functionality of these syntax elements.

- In section 7.4.7.1 of the current draft [1], there is the following semantic constraint:

When present, the value of each of the slice header syntax elements slice_pic_parameter_set_id, non_reference_picture_flag, colour_plane_id, slice_pic_order_cnt_lsb, recovery_poc_cnt, no_output_of_prior_pics_flag, pic_output_flag, and slice_temporal_mvp_enabled_flag shall be the same in all slice headers of a coded picture.

Thus each of these syntax elements could be moved to the picture header to avoid unnecessary redundant bits. 

The recovery_poc_cnt and no_output_of_prior_pics_flag are not moved to the picture header in this contribution.  Their presence in the slice header is dependent on a conditional check of the slice header nal_unit_type, so they are suggested to be studied if there is a desire to move these syntax elements to the picture header.

- In section 7.4.7.1 of the current draft [1], there are the following semantic constraints:

When present, the value of slice_lmcs_aps_id shall be the same for all slices of a picture.

When present, the value of slice_scaling_list_aps_id shall be the same for all slices of a picture.

Thus each of these syntax elements could be moved to the picture header to avoid unnecessary redundant bits.

# Moving slice header syntax elements not constrained to be the same within a picture

The syntax elements in this section are not currently constrained to be the same in all slices of a picture.  It is suggested to assess the anticipated usage of these syntax elements to determine which can be moved into the picture header to simplify the overall VVC design as it is claimed there is a complexity impact to processing a large number of syntax elements in every slice header.

- The following syntax elements are proposed to be moved to the picture header.  There are currently no restrictions on them having different values for different slices but it is claimed there is no/minimal benefit and coding loss to transmitting them in every slice header as their anticipated usage would change at the picture level:

  - six_minus_max_num_merge_cand

  - five_minus_max_num_subblock_merge_cand

  - slice_fpel_mmvd_enabled_flag

  - slice_disable_bdof_dmvr_flag

  - max_num_merge_cand_minus_max_num_triangle_cand

  - slice_six_minus_max_num_ibc_merge_cand

- The following syntax elements are proposed to be moved to the picture header.  There are currently no restrictions on them having different values for different slices but it is claimed there is no/minimal benefit and coding loss to transmitting them in every slice header as their anticipated usage would change at the picture level:

  - partition_constraints_override_flag

  - slice_log2_diff_min_qt_min_cb_luma

  - slice_max_mtt_hierarchy_depth_luma

  - slice_log2_diff_max_bt_min_qt_luma

  - slice_log2_diff_max_tt_min_qt_luma

  - slice_log2_diff_min_qt_min_cb_chroma

  - slice_max_mtt_hierarchy_depth_chroma

  - slice_log2_diff_max_bt_min_qt_chroma

  - slice_log2_diff_max_tt_min_qt_chroma

The conditional check “slice_type == I” associated with some of these syntax elements has been removed with the move to the picture header.

- The following syntax elements are proposed to be moved to the picture header.  There are currently no restrictions on them having different values for different slices but it is claimed there is no/minimal benefit and coding loss to transmitting them in every slice header as their anticipated usage would change at the picture level:

  - mvd_l1_zero_flag

The conditional check “slice_type == B” associated with some of these syntax elements has been removed with the move to the picture header.

- The following syntax elements are proposed to be moved to the picture header.  There are currently no restrictions on them having different values for different slices but it is claimed there is no/minimal benefit and coding loss to transmitting them in every slice header as their anticipated usage would change at the picture level:

  - dep_quant_enabled_flag

  - sign_data_hiding_enabled_flag

# Proposed syntax changes

The proposed syntax changes in this section provide a conceptual overview of the proposed changes with the detailed syntax and semantic changes compared to the current draft text [1] in an attachment to this contribution.

It is recognized that there could also be editorial changes to rename syntax elements slice_XXXXX to pic_XXXXX but these are not provided in this contribution to facilitate easier understanding of the proposed normative changes.

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| non_reference_picture_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| slice_pic_order_cnt_lsb | u(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | ue(v) |
| if( partition_constraints_override_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_luma | ue(v) |
| slice_max_mtt_hierarchy_depth_luma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_luma != 0 ) { |  |
| slice_log2_diff_max_bt_min_qt_luma | ue(v) |
| slice_log2_diff_max_tt_min_qt_luma | ue(v) |
| } |  |
| if(qtbtt_dual_tree_intra_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_chroma | ue(v) |
| slice_max_mtt_hierarchy_depth_chroma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_chroma != 0 ) { |  |
| slice_log2_diff_max_bt_min_qt_chroma | ue(v) |
| slice_log2_diff_max_tt_min_qt_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( sps_temporal_mvp_enabled_flag && !pps_temporal_mvp_enabled_idc ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if(!pps_mvd_l1_zero_idc ) |  |
| mvd_l1_zero_flag | u(1) |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag && !pps_five_minus_max_num_subblock_merge_cand_plus1 ) [Ed. (YK): There is a syntax element name subsetting issue here.] |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| slice_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_dmvr_slice_present_flag ) |  |
| slice_disable_bdof_dmvr_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 ) [Ed. (YK): There is a syntax element name subsetting issue here.] |  |
| max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| if ( sps_ibc_enabled_flag ) |  |
| slice_six_minus_max_num_ibc_merge_cand | ue(v) |
| if ( !pps_dep_quant_enabled_flag ) [Ed. (YK): There is a syntax element name subsetting issue here.] |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( sps_lmcs_enabled_flag ) { |  |
| slice_lmcs_enabled_flag | u(1) |
| if( slice_lmcs_enabled_flag ) { |  |
| slice_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| slice_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| if( sps_scaling_list_enabled_flag ) { |  |
| slice_scaling_list_present_flag | u(1) |
| if( slice_scaling_list_present_flag ) |  |
| slice_scaling_list_aps_id | u(3) |
| } |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| if( nal_unit_type = = GDR_NUT ) |  |
| recovery_poc_cnt | ue(v) |
| if( nal_unit_type = = IDR_W_RADL ¦ ¦ nal_unit_type = = IDR_N_LP ¦ ¦ nal_unit_type = = CRA_NUT ¦ ¦ NalUnitType = = GDR_NUT ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_sps_flag[ i ] | u(1) |
| if( ref_pic_list_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && |  |
| ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| slice_poc_lsb_lt[ i ][ j ] | u(v) |
| delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( delta_poc_msb_present_flag[ i ][ j ] ) |  |
| delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| if( ( slice_type != I && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( slice_type = = B && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( slice_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| if ( slice_type != I ) { |  |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B && !pps_collocated_from_l0_idc ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && NumRefIdxActive[ 0 ] > 1 ) ¦ ¦ ( !collocated_from_l0_flag && NumRefIdxActive[ 1 ] > 1 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( pps_weighted_pred_flag && slice_type = = P ) ¦ ¦ ( pps_weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( sps_joint_cbcr_enabled_flag ) |  |
| slice_joint_cbcr_sign_flag | u(1) |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| slice_joint_cbcr_qp_offset | se(v) |
| } |  |
| if( sps_sao_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( entry_point_offsets_present_flag && NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

# Conclusion

This contribution proposes a mandatory picture header be added to VVC and moving some syntax elements from the slice header into the picture header to avoid unnecessarily transmitting them for each slice as they are already constrained to be the same in all slices of a picture.  The contribution also identifies other syntax elements in the slice header and suggests assessing the anticipated usage of these syntax elements to identify which can be moved into the picture header to reduce the complexity required for slice header processing.

# References

[1] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, document JVET-O2001, JVET, Gothenburg, Sweden, July 2019.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).