| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0240-v2 |
| --- | --- |

| Title: | AHG17: Signalling of Rectangular Slices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Hellman, Wade Wan, Minhua Zhou, Brian Heng, Peisong Chen | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution proposes that the dimensions and locations of rectangular slices be signalled directly by using tile information rather than bricks. The proposal removes the need to predefine and store brick information at the PPS level in an effort to reduce the computation and storage required to determine rectangular slice dimensions and locations. It is asserted that there is no change to the slice and brick partitioning options available to the encoder, as the proposed changes only affect the signalling while keeping the underlying partitioning concepts intact.

# Version Control

v2:   Updated to properly support entry points by signaling the CTU heights of bricks in the slice header.  In addition, it was requested by the group to implement the text changes proposed in P0096 with the text changes of this updated proposal to enable experts to better understand what a combined solution might look like.

# Background

The processing required to compute the dimensions and locations of rectangular slices based on the current draft text [1] is shown on the left hand side of Figure 1 and involves the following steps:

- Parsing and storage of the tile dimensions and locations.

- Parsing and storage of all brick information in the picture. When brick splitting is present, this information is transmitted by scanning through each tile and sending the height of each brick inside each tile.

- Parsing and storage of the indices for the bottom right brick of each slice. The indices for the top left bricks of each slice must be inferred.

- The dimensions and locations of each rectangular slice are then derived by mapping these brick indices to the associated slices.

The authors of this proposal are concerned about the processing and cost associated with predefining all brick information at the PPS level, noting that the brick information for the entire picture must be stored and available prior to the rectangular slice information derivations. It is also noted that the theoretical worst case number of bricks is quite significant, since each CTU row within a tile could potentially be a brick boundary. At 8K resolution with 64x64 CTUs, each PPS could potentially have up to 8192 bricks defined, and the decoder must store up to 64 PPSs.

Even if the worst case number of bricks is constrained, it is desirable to define this information at a larger granularity to reduce the worst-case complexity and storage associated with this information. This was the motivation behind investigating whether rectangular slices could be specified directly, without changing the current partitioning functionality.

This proposal claims that by specifying rectangular slice information using tiles, rather than bricks, there is no loss in partitioning functionality from the current draft text, with the benefit that such an approach avoids the complexity and cost associated with pre-defining all brick information, as shown on the right hand side of Figure 1.

Figure 1: Overview of Current and Proposed Methods for Signalling Rectangular Slice Information

# Proposal

It is useful to note per section 6.3.1 of the draft text [1]: “A slice either contains a number of tiles of a picture or a number of bricks of a tile.” As shown in Figure 2, this means there are two slice partitioning options:

- OPTION 1: If the slice is larger than a single tile, it must be composed of multiple (complete) tiles.

  - In this case, slice boundaries are always aligned with tile boundaries.

  - It is therefore unnecessary and inefficient to use brick granularity when one can use tile granularity to specify slice information.

  - Thus, it is proposed to define each slice by specifying the number of tiles in each dimension.

- OPTION 2: If the slice is not larger than a single tile, then the tile is composed of multiple (complete) slices.

  - In this case, the height of each slice within the tile is sufficient to convey the slice dimensions and locations, because the dimensions of the tile are already known, and tiles can only be split horizontally.

  - Therefore, it is proposed to send the number of slices and the height (number of CTU rows) for each slice inside such a tile (except the height of the last slice which can be inferred).

Regardless of whether OPTION 1 or OPTION 2 applies, the start of the following slice is aligned to the start of a tile so it is proposed to send the tile index (delta coded) to represent this location.

Figure 2: Slice Partitioning and Relationship to Bricks and Tiles

By defining rectangular slices as proposed above, it is no longer necessary to pre-define the entire brick structure of the picture ahead of time. Additional brick splitting can be supported “in-line” by using an end_of_brick syntax element to indicate whether each CTU row is the end of the current brick.

In raster scan slice mode, tiles are not allowed to be split further into smaller bricks. So, the removal of brick information from the PPS has no effect on raster scan slices.

# Proposed Specification Changes

Detailed specification text changes are provided as an attachment to this contribution. The main changes are summarized here.

PPS syntax changes are shown below:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| output_flag_present_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| log2_pps_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| loop_filter_across_bricks_enabled_flag | u(1) |
| if( loop_filter_across_bricks_enabled_flag ) |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] == 0 && slice_height_in_tiles_minus1[ i ] == 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ] |  |
| for( j = 0; j < numSlicesInTileMinus1; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| entropy_coding_sync_enabled_flag | u(1) |
| if( !single_tile_in_pic_flag ¦ ¦ entropy_coding_sync_enabled_flag ) |  |
| entry_point_offsets_present_flag | u(1) |
| cabac_init_present_flag | u(1) |
| for( i = 0; i < 2; i++ ) |  |
| num_ref_idx_default_active_minus1[ i ] | ue(v) |
| rpl1_idx_present_flag | u(1) |
| init_qp_minus26 | se(v) |
| if( sps_transform_skip_enabled_flag ) |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| cu_qp_delta_enabled_flag | u(1) |
| if( cu_qp_delta_enabled_flag ) |  |
| cu_qp_delta_subdiv | ue(v) |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| if( cu_chroma_qp_offset_enabled_flag ) { |  |
| cu_chroma_qp_offset_subdiv | ue(v) |
| chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_offset_list_len_minus1; i++ ) { |  |
| cb_qp_offset_list[ i ] | se(v) |
| cr_qp_offset_list[ i ] | se(v) |
| joint_cbcr_qp_offset_list[ i ] | se(v) |
| } |  |
| } |  |
| pps_weighted_pred_flag | u(1) |
| pps_weighted_bipred_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| constant_slice_header_params_enabled_flag | u(1) |
| if( constant_slice_header_params_enabled_flag ) { |  |
| pps_dep_quant_enabled_idc | u(2) |
| for( i = 0; i < 2; i++ ) |  |
| pps_ref_pic_list_sps_idc[ i ] | u(2) |
| pps_temporal_mvp_enabled_idc | u(2) |
| pps_mvd_l1_zero_idc | u(2) |
| pps_collocated_from_l0_idc | u(2) |
| pps_six_minus_max_num_merge_cand_plus1 | ue(v) |
| pps_five_minus_max_num_subblock_merge_cand_plus1 | ue(v) |
| pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 | ue(v) |
| } |  |
| pps_loop_filter_across_virtual_boundaries_disabled_flag | u(1) |
| if( pps_loop_filter_across_virtual_boundaries_disabled_flag ) { |  |
| pps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_ver_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_x[ i ] | u(13) |
| pps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_hor_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_y[ i ] | u(13) |
| } |  |
| slice_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

A summarized version of the semantics for these new syntax elements is as follows:

num_slices_in_pic_minus1 plus 1 specifies the number of rectangular slices in each picture referring to the PPS.

slice_width_in_tiles_minus1[ i ] plus 1 specifies the width of the i-th rectangular slice in units of tile columns.

slice_height_in_tiles_minus1[ i ] plus 1 specifies the height of the i-th rectangular slice in units of tile rows.

num_slices_in_tile_minus1[ i ] plus 1 specifies the number of slices in the current tile for the case where the i-th slice contains a subset of bricks from a single tile.

slice_height_in_ctu_minus1[ i ] plus 1 specifies the height of the i-th rectangular slice in units of CTU rows for the case where the i-th slice contains a subset of bricks from a single tile.

tile_idx_delta[ i ] specifies the difference in tile index between the i-th rectangular slice and the (i+1)-th rectangular slice. The value of tile_idx_delta[ i ] shall not be equal to 0.

The slice_header syntax changes are shown below.  Note that the main change between v1 and v2 is the introduction of the num_bricks_in_slice_minus1 and brick_height_in_ctu_minus1[i] syntax elements which enable appropriate entry points for bricks to be derived.

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| non_reference_picture_flag | u(1) |
| slice_type | ue(v) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| slice_pic_order_cnt_lsb | u(v) |
| if( nal_unit_type = = GDR_NUT ) |  |
| recovery_poc_cnt | ue(v) |
| if( nal_unit_type = = IDR_W_RADL ¦ ¦ nal_unit_type = = IDR_N_LP ¦ ¦ nal_unit_type = = CRA_NUT ¦ ¦ NalUnitType = = GDR_NUT ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_sps_flag[ i ] | u(1) |
| if( ref_pic_list_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && |  |
| ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| ref_pic_list_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| slice_poc_lsb_lt[ i ][ j ] | u(v) |
| delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( delta_poc_msb_present_flag[ i ][ j ] ) |  |
| delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| if( ( slice_type != I && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( slice_type = = B && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( slice_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | ue(v) |
| if( partition_constraints_override_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_luma | ue(v) |
| slice_max_mtt_hierarchy_depth_luma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_luma != 0 ) |  |
| slice_log2_diff_max_bt_min_qt_luma | ue(v) |
| slice_log2_diff_max_tt_min_qt_luma | ue(v) |
| } |  |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag ) { |  |
| slice_log2_diff_min_qt_min_cb_chroma | ue(v) |
| slice_max_mtt_hierarchy_depth_chroma | ue(v) |
| if( slice_max_mtt_hierarchy_depth_chroma != 0 ) |  |
| slice_log2_diff_max_bt_min_qt_chroma | ue(v) |
| slice_log2_diff_max_tt_min_qt_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if ( slice_type != I ) { |  |
| if( sps_temporal_mvp_enabled_flag && !pps_temporal_mvp_enabled_idc ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if( slice_type = = B && !pps_mvd_l1_zero_idc ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B && !pps_collocated_from_l0_idc ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && NumRefIdxActive[ 0 ] > 1 ) ¦ ¦ ( !collocated_from_l0_flag && NumRefIdxActive[ 1 ] > 1 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( pps_weighted_pred_flag && slice_type = = P ) ¦ ¦ ( pps_weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag && !pps_five_minus_max_num_subblock_merge_cand_plus1 ) [Ed. (YK): There is a syntax element name subsetting issue here.] |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| slice_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_dmvr_slice_present_flag ) |  |
| slice_disable_bdof_dmvr_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 ) [Ed. (YK): There is a syntax element name subsetting issue here.] |  |
| max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| } |  |
| if ( sps_ibc_enabled_flag ) |  |
| slice_six_minus_max_num_ibc_merge_cand | ue(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| slice_joint_cbcr_sign_flag | u(1) |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| slice_joint_cbcr_qp_offset | se(v) |
| } |  |
| if( sps_sao_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| if ( !pps_dep_quant_enabled_flag ) [Ed. (YK): There is a syntax element name subsetting issue here.] |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( sps_lmcs_enabled_flag ) { |  |
| slice_lmcs_enabled_flag | u(1) |
| if( slice_lmcs_enabled_flag ) { |  |
| slice_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| slice_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| if( sps_scaling_list_enabled_flag ) { |  |
| slice_scaling_list_present_flag | u(1) |
| if( slice_scaling_list_present_flag ) |  |
| slice_scaling_list_aps_id | u(3) |
| } |  |
| num_bricks_in_slice_minus1 | ue(v) |
| for( i = 0; i < num_bricks_in_slice_minus1; i++ ) |  |
| brick_height_in_ctu_minus1[ i ] | ue(v) |
| if( entry_point_offsets_present_flag && NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

The slice_data syntax changes are shown below.

| slice_data( ) { | Descriptor |
| --- | --- |
| FirstCtbRowInBrick = 1 |  |
| for( i = 0; i < NumCtuInCurrSlice; i++ ) { |  |
| CtbAddrInRs = CtbAddrInCurrSlice[ i ] |  |
| CtbAddrX = ( CtbAddrInRs % PicWidthInCtbsY ) |  |
| CtbAddrY = ( CtbAddrInRs / PicWidthInCtbsY ) |  |
| if( CtbAddrX = = CtbToTileColBd[ CtbAddrX ] ) { |  |
| NumHmvpCand = 0 |  |
| NumHmvpIbcCand = 0 |  |
| ResetIbcBuf = 1 |  |
| } |  |
| coding_tree_unit( ) |  |
| if( i = = NumCtuInCurrSlice – 1 ) { |  |
| end_of_slice_one_bit /* equal to 1 */ | ae(v) |
| else if( CtbAddrX = = CtbToTileColBd[ CtbAddrX + 1 ] – 1 ) { |  |
| FirstCtbRowInBrick = 0 |  |
| if( CtbBrickIdxInCurrSlice[ i ] != CtbBrickIdxInCurrSlice[ i + 1 ] ) { |  |
| end_of_brick_one_bit /* equal to 1 */ | ae(v) |
| FirstCtbRowInBrick = 1 |  |
| byte_alignment( ) |  |
| } else if( entropy_coding_sync_enabled_flag ) { |  |
| end_of_subset_one_bit /* equal to 1 */ | ae(v) |
| byte_alignment( ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Example for discussion

Figure 3 shows an example to illustrate how the proposed solution would represent the partitioning shown. This picture has 12 tiles (enumerated starting from tile 0) with tile boundaries shown by red dotted lines and 8 slices (enumerated starting from slice 0) with slice boundaries shown by black solid lines.

Figure 3: Example for Discussion

Using the naming convention shown in Figure 2, one can start from the top left corner and then loop through the picture partitioning as follows:

- At Tile 0, encoding Slice 0 and Slice 1 (Slice 0 is not larger than a single tile means OPTION 2)

  - slice_width_in_tiles_minus1[0] = 0		// to signal OPTION 2

  - slice_height_in_tiles_minus1[0] = 0

  - num_slices_in_tile_minus1[0] = 1 		// 2 slices in this tile

  - slice_height_in_ctu_minus1[0] = 1 		// number CTU rows in first slice of tile

  - Height of final slice in tile (Slice 1) is inferred

  - tile_idx_delta[1] = 4					//delta tile index for next slice

- At Tile 4, encoding Slice 2 and Slice 3 (Slice 2 not larger than a single tile means OPTION 2)

  - slice_width_in_tiles_minus1[2] = 0		// to signal OPTION 2

  - slice_height_in_tiles_minus1[2] = 0

  - num_slices_in_tile_minus1[2] = 1 		// 2 slices in this tile

  - slice_height_in_ctu_minus1[2] = 2 		// number CTU rows in first slice of tile

  - Height of final slice in tile (Slice 3) is inferred

  - tile_idx_delta[3] = -3					// delta tile index for next slice

- At Tile 1, encoding Slice 4 (Slice 4 larger than a single tile means OPTION 1)

  - slice_width_in_tiles_minus1[4] = 1

  - slice_height_in_tiles_minus1[4] = 1

  - tile_idx_delta[4] = 2					// delta tile index for next slice

- At Tile 3, encoding Slice 5 (Slice 5 larger than a single tile means OPTION 1)

  - slice_width_in_tiles_minus1[5] = 0

  - slice_height_in_tiles_minus1[5] = 1

  - tile_idx_delta[5] = 5					// delta tile index for next slice

- At Tile 8, encoding Slice 6 (Slice 6 larger than a single tile means OPTION 1)

  - slice_width_in_tiles_minus1[6] = 2

  - slice_height_in_tiles_minus1[6] = 0

  - tile_idx_delta[6] = 3					// delta tile index for next slice

- At Tile 11, encoding Slice 7

  - Attributes of final slice are inferred.

# Summary of Text Simplifications

The attached proposed text changes reduce the overall number of lines of text by a delta of approximately 86 lines, and reduce the number of brick/slice related syntax elements in the PPS from 13 down to 7. Additionally, a significant number of lookup tables required for brick structure processing are no longer necessary. Specifically:

BrickColBd[ ], BrickRowBd[ ], BrickWidth[ ], BrickHeight[ ]

CtbAddrRsToBs[ ], CtbAddrBsToRs[ ], BrickId[ ]

NumCtusInBrick[ ], FirstCtbAddrBs[ ], BricksToSliceMap[ ], BottomRighBrickIdx[ ]

TopLeftBrickIdx[ ], NumBrickInSlice[ ]

# Conclusion

In this document, rectangular slice dimensions and locations are directly specified using tile-based information in an effort to reduce the processing complexity and storage requirements associated with deriving the same information from brick level information. With this change, it is no longer necessary to predefine all brick information for a picture in the PPS. It is proposed to adopt the proposed changes into the VVC draft standard.

# References

[1] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, document JVET-O2001, JVET, Gothenburg, Sweden, July 2019.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).