| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0242-v1 |
| --- | --- |

| Title: | AHG12/AHG16/AHG17: Subpicture Support |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan, Tim Hellman, Minhua Zhou, Brian Heng, Peisong Chen | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

A subpicture grid based approach to signal subpicture boundaries was adopted into the draft text at the Gothenburg July 2019 meeting.  This contribution presents an analysis of the subpicture concept as described in the current draft text where the authors express concern that the current definition has potential implications for decoder implementations (potential interop issues as well as storage and processing requirements).  The authors emailed slides to the JVET reflector on Aug. 26, 2019 in an effort to:

- Confirm understanding of the adoption and design philosophy

- Provide feedback on the implementation implications

- Motivate development of alternative approaches to reduce the implementation impact while still satisfying the general subpicture functionality

There was some feedback on the reflector and this contribution was submitted to continue discussions in this area.  The authors understand that the current draft may not reflect what will be in the final draft and/or what is expected in a final implementation but believe the information is useful to correct aspects in the current design and/or avoid similar issues in new proposals/adoptions.

# Introduction

The approach documented in JVET-O1143 [1] was adopted into the draft text [2] as a starting point for signalling subpicture boundaries.  The authors analyzed the current draft text and posted slides to the JVET reflector on Aug. 26, 2019 with their analysis in an effort to improve the implications for decoder implementations as they suggest there are potential interop issues as well as high storage and processing requirements with the current scheme.

The slides posted to the JVET reflector are provided as an attachment to this contribution.

# Discussion

## Overview of Current Subpicture Signalling

The subpicture signalling scheme defined in the current draft defines a brand new partitioning scheme using the SPS syntax elements subpic_grid_col_height and subpic_grid_col_width to define a grid. Each “block” in the grid has a transmitted subpic_grid_idx which must be “grouped together” to determine the extent/boundaries of each subpicture.

## Concerns on Checking Conformance & Interop

There are semantic to constrain the possible values in these elements but it is claimed it is not difficult to provide illegal indexing values and hard to check conformance which may result in potential interop problems.  More specifically, there are no syntactical constraints which prevent non-rectangular indexing, grouping non-connected regions and/or subpicture to not be aligned with slice boundaries

## Storage costs

The current definition of the subpicture grid is allowed to be as small as 4 sample resolution resulting in the following worst case memory requirement (per SPS) shown in the following table:

| Video resolution | Worst case memory requirement per SPS |
| --- | --- |
| 4K (4096x2160): | 8 bits for grid_idx 552,960 elements 553 kB |
| 8K (8192x4320) | 8 bits for grid_idx 2,211,840 elements 2.21 MB |

It is understood by the authors that in many scenarios the intent would be to use a different/higher sample resolution, however, the worst case resolution would need to be supported by every decoder and this amount of memory would be costly.

It is also unclear to the authors why the grid resolution is not based on CTU size rather than a fixed size since subpictures are not allowed to “split” a CTU.

## Complexity with determining subpicture boundaries

A decoder needs to determine subpicture boundaries because these boundaries potentially can affect:

- Motion Compensation

- Clip TMVP positions

- Disable deblocking

There appear to be multiple mapping tables required to map each slice to a subpicture ID.  It would be preferable to directly map slices to subpictures without intermediate mapping operations to reduce the processing complexity.

## Design considerations

It was confirmed by an earlier email on the reflector that the design intent is for a subpicture to be composed of one or more complete slices.  Therefore, the authors wonder why a complex grid based approach was used to define subpicture boundaries.  It seems better to directly match subpicture definitions to the existing bricks/tiles/slices structures which are already defined.

The following alternative approaches are suggested to be considered in an effort to reduce the implementational complexity while still satisfying the general subpicture functionality:

- Define a subpicture index for each slice.  This could be a fixed length code (FLC) at the top of the slice header for easy bit stream manipulation

- Associate slices to subpictures in one of the loops across the slices of a picture in the PPS

# Conclusion

The authors recognize that there are pros/cons with different signalling approaches and some aspects may not be required but simply to make things easier for subpicture use.  They would caution against adding elements to optimize the general framework just for subpicture use as it is unclear how widespread subpictures will be used in practice especially since the option of running multiple video decode instances without using the subpicture concept is currently available and used in industry.  Therefore, the authors believe the design intent should be to enable the basic functionality provided by subpicture use with minimal additional burden compared to a profile definition that does not include subpictures.

# References

[1] Y.-K. Wang et al., “AHG12: Text for subpicture agreements integration”, document JVET-O1143, JVET, Gothenburg, Sweden, July 2019.

[2] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, document JVET-O2001, JVET, Gothenburg, Sweden, July 2019.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).