| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0244-v1 |
| --- | --- |

| Title: | AHG17: Miscellaneous SPS HLS corrections |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan, Tim Hellman | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution proposes five changes to syntax elements in the SPS.  The first three proposed changes are to change the parsing of syntax elements from Exp-Golomb coding to fixed length coding.  The remaining two proposed changes suggest specifying valid conformance ranges for syntax elements that currently do not have an explicitly bounded range.

# Proposal

## sps_seq_parameter_set_id

- This syntax element has a range of 0 to 15, inclusive and is currently coded with ue(v).

- It is proposed to code this syntax element with u(4) to be consistent with other similar syntax elements, e.g., the IDs for the DPS and VPS are coded in the SPS with u(4).

## chroma_format_idc

- This syntax element has a range of 0 to 3, inclusive and is currently coded with ue(v).

- Using Exp-Golomb coding is efficient if one expects the values to be biased toward small values in the range and inefficient if they are not.

- Note that chroma_format_idc equal to zero represents monochrome which is not expected to be the most commonly used chroma format.

- Therefore, it is proposed to code this syntax element with u(2).

- It was recognized that there is a clause in the draft text stating “Other values of chroma_format_idc, SubWidthC and SubHeightC may be specified in the future by ITU T | ISO/IEC.”  If future extensibility is desired, it is recommended to change to u(n) and define an explicit range with reserved values rather than keep the current ue(v) coding.

## log2_max_pic_order_cnt_lsb_minus4

- This syntax element has a range of 0 to 12, inclusive and is currently coded with ue(v).

- Using Exp-Golomb coding is efficient if one expects the values to be biased toward small values in the range and inefficient if they are not.

- This syntax element is not expected to be biased towards small values.

- It is proposed to code this syntax element with u(4).

## min_qp_prime_ts_minus4

- This syntax element is currently coded with ue(v) but does not have a specified range which is undesirable for defining conformance.

- The VTM6.0 encoder currently sets this value as follows (with no option to configure it manually):

(6 * (m_bitDepth[channelType] - m_inputBitDepth[channelType]))

- Assuming a maximum coded bit depth of 16 and a minimum input bit depth of 8 results in a maximum value of 48.

- It is proposed to specify the conformant range of this syntax element to be 0 to 48, inclusive.

## log2_min_luma_coding_block_size_minus2

- This syntax element is currently coded with ue(v) but does not have a specified range which is undesirable for defining conformance.

- The value of log2_min_luma_coding_block_size must not be larger than log2_ctu_size.

- It is proposed to specify the conformant range of this syntax element to be 0 to (log2_ctu_size_minus5 + 3), inclusive.

Detailed specification changes relative to the current draft text [1] are provided as an attachment to this contribution.

# References

[1] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, document JVET-O2001, JVET,

Gothenburg, Sweden, July 2019.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).