| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0250 |
| --- | --- |

| Title: | CE4-related: On simplification for GEO weight derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yucheng Sun Fangdong Chen Li Wang | Tel: Email: | +86 18105755698 sunyucheng@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This contribution presents simplification based on GEO mode [1] proposed in JVET-P0068. This proposal simplify GEO weight matrix derivation process which only use the tan value to calculate distance from partition line instead of cos or sin values, which reduced multiplication, addition and shift process compare to current GEO scheme. This proposal reuse intra angular prediction table to derive GEO weight matrix. With this modification, 4 table can be removed from the spec and the weight calculation in intra prediction process can help reduce the cycles used in GEO modes and keep majority coding gain in GEO mode.

GEO with 56 modes: 8 intra modes + 7 steps

•	RA: -0.31% (Y), -0.37% (U), -0.41% (V)

•	LD: -0.64% (Y), -0.59% (U), -0.42% (V)

GEO with 48 modes: 8 intra modes + 6 steps

•	RA: -0.30% (Y), -0.34% (U), -0.44% (V)

•	LD: -0.61% (Y), -0.56% (U), -0.50% (V)

GEO with 40 modes: 8 intra modes + 5 steps

•	RA: -0.28% (Y), -0.31% (U), -0.42% (V)

•	LD: -0.55% (Y), -0.48% (U), -0.56% (V)

Proposed method

In this proposal, after setting proper reference sample value with weight array, intra prediction process will be invoked to derive GEO weight matrix. As shown in Figure 1, after setting reference sample value, the GEO weight matrix can be derived after invoking intra prediction.

| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 4 | 6 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 1 | 2 | 4 | 6 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 2 | 4 | 6 | 7 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 1 | 3 | 5 | 7 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 1 | 2 | 4 | 6 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 1 | 3 | 5 | 7 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 1 | 3 | 5 | 7 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 4 | 6 | 7 | 8 | 8 | 8 | 8 | 8 | 8 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 3 | 5 | 7 | 8 | 8 | 8 | 8 | 8 | 8 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 4 | 6 | 8 | 8 | 8 | 8 | 8 | 8 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 4 | 6 | 7 | 8 | 8 | 8 | 8 | 8 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 3 | 5 | 7 | 8 | 8 | 8 | 8 | 8 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 4 | 6 | 8 | 8 | 8 | 8 | 8 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 3 | 5 | 7 | 8 | 8 | 8 | 8 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 3 | 5 | 7 | 8 | 8 | 8 | 8 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 4 | 6 | 7 | 8 | 8 | 8 |
|  |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 3 | 5 | 7 | 8 | 8 | 8 |

Figure 1 Proposed default intra mode based on splitting mode

Reference sample calculation

In this contribution, instead of pick some special angles and remove some redundant steps for GEO modes, this contribution simply pick (2+8*k) intra modes with k = 1,2...,7 and enable all steps among selected angle, 3 tests are tested:

- Test 1 include 56 GEO modes (8 angles and 7 steps) shown in Figure 2;

- Test 2 include 48 GEO modes (8 angles and 6 steps) shown in Figure 3;

- Test 3 include 40 GEO modes (8 angles and 5 steps) shown in Figure 4.

Figure 2 Proposed Test 1 configuration

Figure 3 Proposed Test 2 configuration

Figure 4 Proposed Test 3 configuration

The reference sample value can be set as follows:

- If predModeIntra is greater than or equal to 34

  - usefulsize = (nTbW - 1) << 5) + (abs(intraPredAngle) * (nTbH - 1)

  - If intraPredAngle is less than 0, the following applies:

    - usefulcenter = ((32 - abs(intraPredAngle) * nTbH) << 1) + usefulsize – 64

    - ref[ x ] = Clip3(0, 8, f ( x )), with x = - nTbH..nTbW + 1

  - Otherwise,

    - usefulcenter = (((32 + abs(intraPredAngle)) << 1) + usefulsize) – 64

    - ref[ x ] = Clip3(0, 8, (f ( x )), with x = 0.. nTbW << 1

- Otherwise (predModeIntra is less than 34)

  - usefulsize = (nTbH - 1) << 5) + (abs(intraPredAngle) * (nTbW - 1)

  - If intraPredAngle is less than 0, the following applies:

    - usefulcenter = ((32 - abs(intraPredAngle) * nTbW) << 1) + usefulsize - 64)

    - ref[ x ] = Clip3(0, 8, f ( x ) ), with x = - nTbW..nTbH + 1

  - Otherwise,

    - usefulcenter = (((32 + abs(intraPredAngle)) << 1) + usefulsize) - 64)

    - ref[ x ] = Clip3(0, 8, f ( x ) ), with x = 0.. nTbH << 1

The value in above format is shown in Figure 5 and Figure 6.

Figure 5 format schematic diagram (predModeIntra ≥ 34)

Figure 6 format schematic diagram (predModeIntra < 34)

The function f( x ) is defined as follows:

- For Test 1,

f ( x ) = 2x - ((((step * (usefulsize - 64)) + (usefulcenter << 2) + 64) >> 7))

- For Test 2,

f ( x ) = 2x - ((((step * usefulsize * 2) + usefulsize + (usefulcenter << 3) +128) >> 8))

- For Test 3,

f ( x ) = 2x - ((((step * (usefulsize - 64) * 3) + (usefulcenter << 3) + 128) >> 8))

Harmonized with other tools

Harmonized with intra prediction

As shown in Figure 7, when compared with intra prediction, the only difference is replace neighbouring reference sample fetching with GEO reference weight derivation.

Figure 7 Harmonized with intra prediction

Harmonized with TPM

As shown in Figure 8, when considering TPM mode, the diagonal angle for each block size is already exist in intra angle.

Figure 8 Harmonized with TPM

- Performance results

The performance result is summarised in Table 1 ~ Table 6. The tests were performed using the JVET common test conditions [2]with parallel encoding/decoding used for RA. The software modifications were made on top of VTM 6.0[3]. The encoding/decoding time is not reliable because of churn of network.

Table 1 - Performance of Test 1 under RA

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.15% | -0.11% | -0.22% | 102% | 102% |
| Class A2 | -0.18% | -0.31% | -0.19% | 103% | 102% |
| Class B | -0.22% | -0.16% | -0.26% | 103% | 102% |
| Class C | -0.65% | -0.89% | -0.90% | 103% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | -0.37% | -0.41% | 103% | 103% |
| Class D | -0.48% | -0.73% | -0.64% | 103% | 101% |
| Class F | -0.30% | -0.42% | -0.37% | 101% | 102% |

Table 2 - Performance of Test 1 under LD

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.37% | -0.38% | -0.58% | 104% | 102% |
| Class C | -0.76% | -0.93% | -0.75% | 104% | 104% |
| Class E | -0.91% | -0.49% | 0.27% | 102% | 102% |
| Overall | -0.64% | -0.59% | -0.42% | 104% | 103% |
| Class D | -0.72% | -0.17% | -0.34% | 105% | 105% |
| Class F | -0.37% | -0.36% | 0.30% | 103% | 105% |

Table 3 - Performance of Test 2 under RA

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.13% | -0.06% | -0.30% | 102% | 102% |
| Class A2 | -0.21% | -0.19% | -0.21% | 103% | 102% |
| Class B | -0.19% | -0.11% | -0.30% | 103% | 103% |
| Class C | -0.61% | -0.94% | -0.92% | 103% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.30% | -0.34% | -0.44% | 103% | 103% |
| Class D | -0.46% | -0.74% | -0.50% | 103% | 101% |
| Class F | -0.28% | -0.41% | -0.32% | 101% | 102% |

Table 4 - Performance of Test 2 under LD

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.37% | -0.36% | -0.36% | 104% | 102% |
| Class C | -0.78% | -0.94% | -0.78% | 104% | 105% |
| Class E | -0.78% | -0.39% | -0.35% | 102% | 103% |
| Overall | -0.61% | -0.56% | -0.50% | 103% | 103% |
| Class D | -0.68% | -0.79% | -0.53% | 104% | 106% |
| Class F | -0.43% | -0.54% | -0.32% | 103% | 104% |

Table 5 - Performance of Test 3 under RA

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.12% | -0.17% | -0.29% | 102% | 102% |
| Class A2 | -0.15% | -0.17% | -0.19% | 103% | 102% |
| Class B | -0.20% | -0.11% | -0.26% | 103% | 103% |
| Class C | -0.59% | -0.77% | -0.87% | 102% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.28% | -0.31% | -0.42% | 102% | 103% |
| Class D | -0.43% | -0.67% | -0.55% | 103% | 101% |
| Class F | -0.30% | -0.45% | -0.43% | 101% | 103% |

Table 6 - Performance of Test 3 under LD

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.32% | -0.41% | -0.62% | 104% | 103% |
| Class C | -0.70% | -0.95% | -0.81% | 104% | 104% |
| Class E | -0.73% | 0.03% | -0.12% | 103% | 102% |
| Overall | -0.55% | -0.48% | -0.56% | 103% | 103% |
| Class D | -0.57% | -0.76% | -0.54% | 104% | 105% |
| Class F | -0.37% | -0.47% | -0.31% | 103% | 105% |

- Conclusion

In this contribution, intra prediction is used to derive weight matrix for GEO modes. The test results reportedly show that with this modification majority coding performance of GEO is kept. And with this modification, it’s easy to harmonize with other coding tools, like intra prediction and TPM. When combined with JVET-P0264, multiplication process can be fully removed from calculating weights for each pixel position. In order to seek the best weight matrix derivation and further optimal parameter sets for both TPM and GEO, it’s recommended to further study in CE.

- References

- Han Gao, Semih Esenlik, Elena Alshina, Anand Meher Kotra, Biao Wang (Huawei), Max Bläser, Johannes Sauer (RWTH Aachen), “CE4: CE4-1.1, CE4-1.2 and CE4-1.14: Geometric Merge Mode (GEO) “, JVET-P0068-v1, Geneva, October 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, March 2019.

- VTM-6.0 software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git, February 2019

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).