| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0255-v2 |
| --- | --- |

| Title: | CE5-related: Bilateral filter with SAO band offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiyofumi Abe Tadamasa Toma 3-1-1 Yagumo-naka-machi, Moriguchi, Osaka, Japan | Tel: Email: | +81-80-3083-1171 abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

This contribution is related to CE5-3.1 (JVET-P0073). CE5-3.1 proposes the combination of bilateral filter and SAO. Instead of that, this contribution proposes to remove SAO edge offset but keep SAO band offset on top of bilateral filter. The effectiveness of SAO edge offset and bilateral filter is particularly　overlapped. On the other hand, SAO band offset has valuable benefit for subjective improving and there is no tool which has same functionality. This proposal achieves similar performance to CE5-3.1 with remarkable logic reduction. Simulation results reportedly show that the performance of proposed method is -0.40%, -0.39%, -0.28% BD-rate gain for AI, RA and LDB on VTM-6.0.

Introduction

The combination of bilateral filter and SAO is proposed by CE5-3.1 (JVET-P0073 [1]). In this proposal, both bilateral filter and SAO (including band offset and edge offset) are performed in parallel, and the filtered samples are generated using the sum of both output offsets as shown in Figure1.

IOUT=Clip3(IIN+BifOffset+SaoOffset)

Figure1: The combination of bilateral filter and SAO (CE5-3.1)

Table1 shows the comparison of SAO and bilateral filter (BIF). Although there are some differences, SAO edge offset and BIF have similar functionality. We have the following understandings.

- SAO BO is useful for subjective improving, and there is no tool which has same functionality.

- The effectiveness of SAO EO and BIF is particularly overlapped.

- SAO EO for chroma has not enough benefit for subjective and objective performance.

Table1: Comparison of SAO and BIF

|  | SAO (Band Offset) | SAO (Edge Offset) | BIF |
| --- | --- | --- | --- |
| apply to | Luma + Chroma | Luma + Chroma | Luma |
| on / off | CTU | CTU | CTU |
| sample reference | 1 pixel | 3x3 pixel | 3x3 pixel |
| padding | no | no | yes |
| offset derivation | signaling 4 offsets, select one using band of sample value | signaling 4 offsets, select one using 2 neighboring differences | no signaling, derive using LUT and 8 neighboring differences |

Proposal

This contribution proposes to remove SAO edge offset but keep SAO band offset on top of bilateral filter.

Figure2 shows the diagram of this proposal. It is similar to CE5-3.1 but SAO EO is removed. The filtered samples are generated using the sum of both output offsets same as CE5-3.1. But SaoOffset is derived only from SAO BO processing. It is noted, that BIF is applied to Luma only, so the filtered samples of chroma are generated using only SAO BO.

IOUT=Clip3(IIN+BifOffset+SaoOffset)

Figure2: The diagram of proposed method

According to the above modification, the semantics of sao_type_idx_luma and sao_type_idx_chroma are changed as shown in Table2. Originally, these syntaxes have three values (none/SAO BO/SAO EO). But, in this proposal, they have only two values (none/SAO BO). Wherein, bif_ctu_flag will be switched on/off regardless of sao_type.

Table2: BIF and SAO switching flag

| coding_tree_unit( ) { |  |
| --- | --- |
| …. |  |
| if( pps_bilateral_filter_flag ) |  |
| bif_ctu_flag | 0: none, 1: BIF |
| if( slice_sao_enabled_flag) { |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( cIdx == 0 ) |  |
| sao_type_idx_luma | 0: none, 1: SAO BO |
| else if( cIdx == 1 ) |  |
| sao_type_idx_chroma | 0: none, 1: SAO BO |
| …. |  |
| } |  |

Simulation results

Table3 shows the simulation result of the proposed method. The simulation is conducted using VTM-6.0 with the test condition described in JVET-N1010 [2]. This result includes the gain of bilateral filter and the loss of disabling SAO band offset.

In addition, Table4 shows the simulation results of CE5-3.1 and CE5-3.1 disabling SAO (both BO and EO) for reference. The result of CE5-3.1 disabling SAO shows some loss compared to original CE5-3.1 especially for Class F and TGM. On the other hand, the result of proposed method shows similar gain to original CE5-3.1. It indicates that SAO EO brings small gain on top of BIF, but SAO BO provides enough benefit of coding performance especially for Class F and TGM.

Table3: Simulation results of the proposed method

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | 0.20% | -0.10% | 106% | 104% |
| Class A2 | -0.36% | 0.14% | 0.11% | 106% | 102% |
| Class B | -0.34% | 0.26% | 0.26% | 107% | 104% |
| Class C | -0.52% | 0.06% | 0.23% | 106% | 102% |
| Class E | -0.43% | 0.24% | 0.17% | 106% | 104% |
| Overall | -0.40% | 0.18% | 0.15% | 107% | 103% |
| Class D | -0.48% | 0.11% | 0.12% | 105% | 103% |
| Class F | -0.55% | -0.03% | -0.12% | 105% | 104% |
| TGM | -0.44% | -0.04% | -0.02% | 104% | 105% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | 0.17% | 0.09% | 103% | 103% |
| Class A2 | -0.33% | 0.21% | 0.21% | 103% | 101% |
| Class B | -0.37% | 0.41% | 0.30% | 104% | 102% |
| Class C | -0.52% | 0.40% | 0.34% | 104% | 101% |
| Class E | -0.52% | 0.40% | 0.34% | 104% | 101% |
| Overall | -0.39% | 0.32% | 0.25% | 104% | 102% |
| Class D | -0.68% | 0.06% | 0.37% | 103% | 101% |
| Class F | -0.48% | 0.04% | 0.03% | 103% | 102% |
| TGM | -0.32% | -0.03% | -0.05% | 103% | 103% |

|  | Low delay Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.17% | 0.82% | 0.71% | 104% | 103% |
| Class C | -0.47% | 0.49% | 0.37% | 104% | 101% |
| Class E | -0.21% | 0.63% | 1.10% | 101% | 101% |
| Overall | -0.28% | 0.66% | 0.70% | 103% | 102% |
| Class D | -0.82% | 0.85% | 0.96% | 104% | 99% |
| Class F | -0.25% | 0.11% | 0.30% | 103% | 101% |
| TGM | -0.44% | -0.14% | -0.12% | 103% | 104% |

Table4: Simulation results of CE5-3.1 and CE5-3.1 disabling SAO (for reference)

|  | All Intra Main 10 (CE5-3.1) |  |  |  |  | All Intra Main 10 (CE5-3.1 disable SAO) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | 0.27% | 0.22% | 105% | 105% | -0.34% | 0.49% | 0.35% | 108% | 103% |
| Class A2 | -0.39% | 0.18% | 0.12% | 108% | 103% | -0.38% | 0.50% | 0.32% | 109% | 101% |
| Class B | -0.34% | 0.24% | 0.23% | 108% | 105% | -0.32% | 0.32% | 0.42% | 109% | 103% |
| Class C | -0.53% | 0.07% | 0.23% | 104% | 102% | -0.56% | 0.39% | 0.68% | 111% | 109% |
| Class E | -0.44% | 0.26% | 0.16% | 103% | 103% | -0.42% | 0.23% | 0.16% | 108% | 110% |
| Overall | -0.40% | 0.20% | 0.20% | 106% | 104% | -0.40% | 0.38% | 0.41% | 109% | 105% |
| Class D | -0.47% | 0.08% | 0.06% | 104% | 102% | -0.48% | 0.19% | 0.20% | 109% | 109% |
| Class F | -0.55% | -0.03% | -0.07% | 106% | 104% | -0.25% | 0.33% | 0.45% | 106% | 105% |
| TGM | -0.38% | -0.02% | 0.00% | 104% | 105% | 1.51% | 0.53% | 0.61% | 104% | 103% |

|  | Random Access Main 10 (CE5-3.1) |  |  |  |  | Random Access Main 10 (CE5-3.1 disable SAO) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | 0.11% | 0.00% | 102% | 107% | -0.31% | 0.37% | 0.23% | 102% | 109% |
| Class A2 | -0.42% | 0.13% | 0.21% | 102% | 99% | -0.31% | 0.55% | 0.46% | 103% | 101% |
| Class B | -0.43% | 0.43% | 0.28% | 104% | 102% | -0.34% | 0.63% | 0.73% | 105% | 106% |
| Class C | -0.53% | 0.38% | 0.35% | 103% | 101% | -0.53% | 0.64% | 0.65% | 104% | 110% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.44% | 0.29% | 0.23% | 103% | 102% | -0.38% | 0.56% | 0.55% | 103% | 106% |
| Class D | -0.61% | 0.12% | 0.26% | 103% | 101% | -0.74% | 0.13% | 0.23% | 103% | 114% |
| Class F | -0.51% | 0.05% | 0.06% | 104% | 101% | -0.10% | 0.46% | 0.51% | 105% | 110% |
| TGM | -0.30% | -0.04% | -0.03% | 103% | 103% | 1.61% | 0.52% | 0.53% | 104% | 104% |

|  | Low delay B Main 10 (CE5-3.1) |  |  |  |  | Low delay B Main 10 (CE5-3.1 disable SAO) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.19% | 0.62% | 0.46% | 103% | 102% | -0.05% | 1.45% | 2.08% | 105% | 107% |
| Class C | -0.50% | 0.63% | 0.68% | 101% | 102% | -0.47% | 1.07% | 1.52% | 105% | 106% |
| Class E | -0.28% | 0.81% | 1.15% | 102% | 102% | -0.18% | 0.29% | 1.03% | 105% | 108% |
| Overall | -0.32% | 0.67% | 0.71% | 102% | 102% | -0.22% | 1.03% | 1.63% | 105% | 107% |
| Class D | -0.75% | 0.93% | 1.15% | 103% | 100% | -0.79% | 1.32% | 1.60% | 106% | 106% |
| Class F | -0.33% | 0.65% | 0.86% | 103% | 100% | 0.29% | 0.55% | 1.38% | 105% | 105% |
| TGM | -0.43% | -0.02% | -0.11% | 102% | 101% | 1.87% | 1.05% | 1.16% | 103% | 100% |

Conclusion

This contribution is related to CE5-3.1 (JVET-P0073). CE5-3.1 proposes the combination of bilateral filter and SAO. Instead of that, this contribution proposes to remove SAO edge offset but keep SAO band offset on top of bilateral filter. The effectiveness of SAO edge offset and bilateral filter is particularly　overlapped. On the other hand, SAO band offset has valuable benefit for subjective improving and there is no tool which has same functionality. This proposal achieves similar performance to CE5-3.1 with remarkable logic reduction. Simulation results reportedly show that the performance of proposed method is -0.40%, -0.39%, -0.28% BD-rate gain for AI, RA and LDB on VTM-6.0. We propose to adopt it into VVC.

References

[1] J. Ström, P. Wennersten, J. Enhorn, D. Liu, K. Andersson, L. Litwic, D. Saffar, C. Hollmann, R. Yu, R. Sjöberg, “CE5-3.1 Combination of bilateral filter and SAO”, JVET-P0073, JVET 16th Meeting: Geneva, CH, 1–11 October 2019.

[2]	F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, JVET 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).