| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0259 |
| --- | --- |

| Title: | Non-CE6: On LFNST reduced kernels |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Gan Chris Rosewarne Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 jonathan.gan@canon.com.au chris.rosewarne@canon.com.au |
| Source: | Canon Australia Pty Ltd. |  |  |

_____________________________

# Abstract

In the current design, the Low-frequency Non-separable Transform (LFNST) requires 8KB of memory to store the transform matrices. This contribution proposes to reduce the LFNST 16x48 matrices to 16x36 by removing primary transform coefficients that correspond to columns of low magnitude weights in the 16x48 matrices. It is reported that the proposed method achieves 18.75% table size reduction with 0.04% loss in All Intra configuration in CTC.

# Introduction

LFNST [1] has been adopted in VVC-5. In the current design, there are eight 16x48 matrices which are used in transform blocks of size 8xN and Nx8 for N ≥ 8. The rows of each matrix (each row 48 weights in length) are the basis vectors, and each weight indicates the relative importance of the corresponding primary transform coefficient to that basis vector.

It is asserted that particular columns, corresponding to primary transform coefficient positions, are identified as consisting of weights which are relatively low in magnitude. Furthermore, it is asserted that these columns consist of weights which are low in magnitude consistently for each of the eight matrices.

# Proposed method

This contribution proposes to replace the 16x48 matrices with smaller 16x36 matrices to achieve reduction in table size and computation. The 16x36 matrices are derived from the 16x48 matrices by removing primary coefficient positions corresponding to the columns with weights of lowest magnitude. No retraining was performed. The coefficient positions of the proposed 16x36 matrices are shown in Figure 1.

Figure 1: 36 coefficients pattern (predModeIntra<=34)

The largest magnitude of the weights in the matrix columns corresponding to the removed coefficient positions is 24.

It is asserted that the orthogonality of the proposed 16x36 matrices is not significantly degraded relative to the LFNST 16x48 matrices.

# Experimental results

The simulations were conducted in the condition of ‘All Intra’, ‘Random Access’ defined in JVET-N1010 [2]. Table 1 shows the results (CTC). Table 2 shows Low QP (2, 7, 12, and 17) results.

Table 1: Result of proposed method over CTC

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.05% | 0.15% | 100% | 100% |
| Class A2 | 0.03% | 0.05% | 0.01% | 100% | 100% |
| Class B | 0.03% | 0.01% | -0.04% | 100% | 100% |
| Class C | 0.01% | -0.12% | 0.08% | 100% | 100% |
| Class E | 0.04% | 0.04% | -0.04% | 100% | 100% |
| Overall | 0.04% | 0.00% | 0.03% | 100% | 100% |
| Class D | 0.01% | -0.03% | 0.01% | 99% | 100% |
| Class F | 0.00% | 0.08% | 0.03% | 100% | 101% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.16% | 0.12% | 100% | 99% |
| Class A2 | 0.01% | 0.13% | 0.05% | 100% | 100% |
| Class B | 0.00% | 0.15% | -0.09% | 100% | 100% |
| Class C | 0.00% | 0.07% | -0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.13% | -0.02% | 100% | 100% |
| Class D | -0.01% | 0.04% | -0.27% | 100% | 100% |
| Class F | -0.03% | -0.07% | -0.08% | 100% | 101% |

Table 2: Result of proposed method over low QPs

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.00% | 100% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | 0.00% | 0.01% | 100% | 99% |
| Class C | 0.00% | 0.01% | 0.02% | 100% | 99% |
| Class E | 0.01% | -0.02% | 0.01% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class D | -0.01% | -0.01% | 0.03% | 100% | 99% |
| Class F | 0.00% | 0.01% | 0.02% | 100% | 100% |

# On orthogonality (informative)

A matrix M is orthogonal if the matrix product M*MT results in the identity matrix. Orthogonality is desirable as the forward LFNST and inverse LFNST may be implemented with the same matrix weights. Due to integerisation, the LFNST matrices are not perfectly orthogonal. It is asserted that the degree of orthogonality may be assessed by calculating the matrix product

A=M*MT*2^(-14)

Where M is an 8-bit integerised LFNST matrix.

The matrix product A was calculated for each of the eight matrices of the current 16x48 LFNST matrices. Across all eight matrix products, the largest deviation of the diagonal terms from unity was 0.0117. Across all eight matrix products, the largest deviation of the non-diagonal terms from zero was 0.0112.

The matrix product A was calculated for each of the eight matrices of the proposed 16x36 matrices. Across all eight matrix products, the largest deviation of the diagonal terms from unity was 0.0455. Across all eight matrix products, the largest deviation of the non-diagonal terms from zero was 0.0270.

The orthogonality is also assessed by calculation of a “C2” measure as defined in [3].

|  |  | Mode=0 |  | Mode=1 |  | Mode=2 |  | Mode=3 |  | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | lfnstIdx=1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 |  |
| C2 (16x16) | Proposed method | 0.06 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.04 | 0.05 | 0.05 |
|  | VTM-6.0 | 0.04 | 0.04 | 0.04 | 0.05 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |

# Additional results (informative)

## Reduction to 16x44 and 16x40 matrices

Additional results are provided with different amount of matrix reduction. Firstly, 16x44 matrices are derived from the 16x48 matrices. The largest magnitude of weight in the removed coefficient positions is 7. The coefficient positions of the 16x44 matrices are shown in Figure 2. All Intra results performed on CTC are shown in Table 3.

Figure 2: 44 coefficients pattern

Table 3: Result of 44 coefficients pattern over CTC

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.03% | 0.01% | 100% | 100% |
| Class A2 | 0.02% | 0.01% | -0.01% | 100% | 100% |
| Class B | 0.00% | -0.04% | -0.02% | 100% | 100% |
| Class C | 0.01% | -0.06% | 0.07% | 101% | 100% |
| Class E | 0.01% | -0.05% | 0.02% | 100% | 100% |
| Overall | 0.01% | -0.04% | 0.01% | 100% | 100% |
| Class D | 0.01% | -0.04% | -0.11% | 100% | 101% |
| Class F | 0.02% | 0.02% | -0.21% | 100% | 101% |

16x40 matrices are derived from the 16x48 matrices. The largest magnitude of weight in the removed coefficient positions is 11. The coefficient positions of the 16x40 matrices are shown in Figure 3. All Intra results performed on CTC are shown in Table 4.

Figure 3: 40 coefficients pattern

Table 4: Result of 40 coefficients pattern over CTC

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.06% | 0.08% | 100% | 99% |
| Class A2 | 0.02% | -0.02% | 0.00% | 100% | 100% |
| Class B | 0.02% | 0.00% | -0.01% | 100% | 100% |
| Class C | 0.01% | -0.11% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.06% | 0.07% | 100% | 99% |
| Overall | 0.02% | -0.01% | 0.02% | 100% | 100% |
| Class D | 0.00% | 0.06% | -0.07% | 100% | 101% |
| Class F | -0.01% | 0.04% | -0.02% | 100% | 100% |

## Reduction from the 16x64 RST to 16x48 and 16x40

The CE6-3.1b test of [1] proposed eight 16x64 matrices to be used for transform blocks of size 8xN or Nx8 for N ≥ 8. In the same method as described above, 16x48 matrices are derived from the 16x64 matrices by removing primary coefficient positions corresponding to the columns with weights of lowest magnitude. The largest magnitude of weight across the removed coefficients is 17. The coefficient positions of the 16x48 matrices are shown in Figure 4. All Intra results performed on CTC are shown in Table 5.

Figure 4: 48 coefficients pattern derived from 16x64 RST kernel

Table 5: Result of 64 reduced to 48 coefficients over CTC

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.09% | -0.13% | 100% | 99% |
| Class A2 | -0.02% | -0.02% | -0.05% | 100% | 99% |
| Class B | -0.02% | -0.15% | -0.16% | 101% | 99% |
| Class C | -0.09% | -0.21% | 0.00% | 100% | 100% |
| Class E | -0.06% | -0.16% | -0.12% | 100% | 100% |
| Overall | -0.04% | -0.13% | -0.10% | 100% | 99% |
| Class D | -0.04% | -0.04% | -0.11% | 100% | 100% |
| Class F | -0.09% | -0.18% | -0.22% | 100% | 100% |

16x40 matrices are also derived from the 16x64 matrices by removing primary coefficient positions corresponding to the columns with weights of lowest magnitude. The largest magnitude of weight across the removed coefficients is 27. The coefficient positions of the 16x40 matrices are shown in Figure 5. All Intra results performed on CTC are shown in Table 6.

Figure 5: 40 coefficients pattern derived from 16x64 RST kernel

Table 6: Result of 64 reduced to 40 coefficients over CTC

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -0.02% | -0.04% | 100% | 100% |
| Class A2 | 0.00% | -0.13% | -0.08% | 100% | 99% |
| Class B | 0.00% | -0.12% | -0.21% | 100% | 99% |
| Class C | -0.07% | -0.16% | -0.04% | 100% | 99% |
| Class E | -0.04% | -0.16% | -0.01% | 100% | 99% |
| Overall | 0.00% | -0.12% | -0.09% | 100% | 99% |
| Class D | -0.02% | -0.15% | -0.20% | 100% | 99% |
| Class F | -0.07% | -0.13% | -0.10% | 100% | 100% |

## On adaptive patterns

The CE6-2.3 test of [2] proposes an adaptive pattern, with dependency on the lfnstIdx. CE6-2.3 proposes a symmetrical pattern when lfnstIdx is 1, and an asymmetrical pattern when lfnstIdx is 2.

Each of the eight 16x48 LFNST matrices correspond to an lfnstTrSetIdx and lfnstIdx combination. Each of the eight matrices was analysed separately for coefficient positions to remove, resulting in eight different patterns. It is reported that a strong preference for symmetrical patterns was exhibited for lfnstTrSetIdx 0, which corresponds to Intra DC and Intra Planar prediction modes.

If an adaptive pattern is of interest, this contribution recommends adaptivity dependent on lfnstTrSetIdx, with patterns as shown in Figure 6. All Intra results performed on CTC are shown in Table 7.

lfnstTrSetIdx == 0                                                 lfnstTrSetIdx != 0

Figure 6: 36 coefficients pattern adaptive on lfnstTrSetIdx

Table 7: Result of 36 coefficient pattern dependent on lfnstTrSetIdx

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.06% | 0.08% | 100% | 99% |
| Class A2 | 0.03% | 0.03% | -0.02% | 100% | 100% |
| Class B | 0.02% | 0.03% | -0.04% | 100% | 100% |
| Class C | 0.02% | -0.11% | 0.09% | 100% | 99% |
| Class E | 0.02% | -0.11% | 0.00% | 100% | 100% |
| Overall | 0.03% | -0.02% | 0.02% | 100% | 100% |
| Class D | 0.01% | 0.00% | -0.02% | 100% | 99% |
| Class F | 0.01% | 0.11% | -0.06% | 100% | 100% |

# References

- M. Koo, M. Salehifar, J. Lim, S. Kim, “CE6: Reduced Secondary Transform (RST) (CE6-3.1)”, JVET-N0193, Geneva, CH, March 2019.

- H. Egilmez, M. Salehifar, X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signaling”, JVET-O2026, Gothenburg, SE, July 2019.

- T. Zhou, T. Hashimoto, T. Ikai, “CE6-2.3a: Simplification with new LFNST transform basis”, JVET-P0065, Geneva, CH, October 2019.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).