| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0260_v2 |
| --- | --- |

| Title: | CE4-related: On inheritance of half-pel interpolation filter in merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Suhong Wang1, Xiaozhen Zheng2, Xuewei Meng1, Shanshe Wang1, Siwei Ma1 Peking University1, DJI2 | Email: | suhong.wang@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd., Peking University |  |  |

_____________________________

- Abstract

This contribution proposes to simplify the process when getting the information which interpolation filter is applied for half-pel position in skip/merge mode. In this contribution, three tests has been conducted.

- Test (a): Always set luma half-pel interpolation filter for pairwise candidates as 8-tap filter:

0.01% and -0.03% for RA and LDB, respectively

- Test (b): Always set luma half-pel interpolation filter for pairwise candidates as 6-tap filter:

0.11% and -0.02% for RA and LDB, respectively

- Test (c): Remove propagation of selected half-pel filter in merge mode:

0.15% and 0.09% for RA and LDB, respectively

Introduction

At the Gothenburg meeting, JVET-O0057[1] present the results of different aspects of the switchable interpolation filters for half-sample (half-pel) positions. It is adopted to allow signaling of motion vector differences at half-pel accuracy and use an alternative half-pel interpolation filter(Gauss filter, 6-tap) for non-affine and non-merge inter-coded CU which uses half-pel motion vector accuracy.

In case of skip/merge mode using a spatial merging candidate, the information which interpolation filter is applied for the half-pel position is inherited from the neighbouring block. In current design, when selecting half-pel interpolation filter for pairwise average merging candidate, there is a two-step process:

Step 1: Compare the half-pel interpolation filter for the first merge candidate and second merge candidate in each list. If they are the same, go to step 2.1; otherwise, go to step 2.2;

Step 2.1: The interpolation filter for this pairwise merging candidate is set identical to which of the first merge candidate.

Step 2.2: The interpolation filter for this pairwise merging candidate is set to the 8-tap filter(HEVC half-pel filter).

Pairwise average merging candidates are generated by averaging predefined pair in the current merge candidate list, and the predefined pair is defined as (0,1) in current design, where the numbers denote the merge indices to the merge candidate list. When selecting half-pel interpolation filter for pairwise merging candidate, the decision is based on whether or not the half-pel interpolation filter of the two merge candidates are the same. However, after averaging these two merge candidates, the features of this constructed motion vector predictor might not be the same as this pair of merge candidates.

Fig. 1 Process of selecting half-pel IF for pairwise candidate in current design

Proposed Method

From Fig.1, it can be seen that the process for selecting the half-pel interpolation filter for pairwise average merging candidate is complex, which makes the timing of small blocks even tight, since the construction of merge list is quite complicated in current design. Also, as mentioned before, this complex selection process does not show any evidence that it will be benefit for choosing a better interpolation filter, considering the features of the constructed pairwise candidate is not the same as the first two candidates in the merge list.

In order to make this process more simple and remove unnecessary operations during the construction of merge list, it is proposed to set a fix half-pel interpolation filter for pairwise average merging candidates. Three cases have been tested over the construction of merge list in current design.

Always set luma half-pel interpolation filter for pairwise candidates as 8-tap filter(Method 1)

In this test, the process of selecting half-pel interpolation filter for pairwise average merging candidate has been simplified. Different from current design, the half-pel interpolation filter for pairwise candidate is always set as 8-tap filter, coefficients of which is shown as Table. 1. The process of the proposed method is illustrated in Fig. 2. From the flowchart, it can be seen that additional checks have been saved in the proposed method.

Table 1 Coefficients of half-pel IF for pairwise candidate(8-tap)

| Filter | Coefficients h[i] |
| --- | --- |
| 8-tap(HEVC) | [-1 4 -11 40 40 -11 4 -1 ] |

Fig. 2 Process of selecting half-pel IF for pairwise candidate in proposed method(test a)

Always set luma half-pel interpolation filter for pairwise candidates as 6-tap filter(Method 2)

In this test, the half-pel interpolation filter for pairwise average merging candidate is always set as 6-tap filter, which is shown in Table 2. Compared to test(a), this 6-tap interpolation filter is more smoother than the 8-tap filter.

Table 2 Coefficients of half-pel IF for pairwise candidate(6-tap)

| Filter | Coefficients h[i] |
| --- | --- |
| 6-tap(Gauss) | [ 3 9 20 20 9 3 ] |

Remove propagation of selected half-pel filter in merge mode (Method 3)

In this test, there’s only one fix interpolation filter(8-tap filter) for half-pel position in merge mode, including spatial merge candidates, hmvp merge candidates, pairwise merge candidates and MMVD candidates. Thus, for these merge candidates, there is no need to inherit the information of half-pel interpolation filter from neighboring blocks.

Experimental Results

The experiments were performed under JVET common test conditions [2].

BD-Rate results of the tests on top of the VTM-6.1 compared to the anchor with RA and LDB configurations are shown in following tables. The full results are provided in the attached excel sheets. Due to the unstable performance of the computer resource, the coding time is not reliable.

Coding performance and complexity of Test (a)

Table 3. Test results of Test (a)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.16% | -0.03% | 101% | 100% |
| Class A2 | 0.03% | 0.08% | 0.02% | 110% | 102% |
| Class B | 0.00% | 0.06% | -0.01% | 101% | 103% |
| Class C | -0.01% | 0.00% | -0.03% | 103% | 111% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.07% | -0.01% | 103% | 104% |
| Class D | 0.00% | 0.04% | 0.05% | 101% | 101% |
| Class F | 0.00% | 0.01% | 0.01% | 99% | 105% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.02% | -0.01% | 98% | 97% |
| Class C | -0.05% | 0.02% | -0.16% | 99% | 99% |
| Class E | -0.05% | 0.11% | -0.11% | 97% | 95% |
| Overall | -0.03% | 0.03% | -0.09% | 98% | 97% |
| Class D | 0.03% | 0.05% | 0.03% | 100% | 99% |
| Class F | 0.07% | -0.18% | 0.32% | 88% | 94% |

Coding performance and complexity of Test (b)

Table 4. Test results of Test (b)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.13% | 0.01% | 101% | 99% |
| Class A2 | 0.20% | 0.19% | 0.09% | 129% | 113% |
| Class B | 0.09% | 0.16% | 0.04% | 105% | 105% |
| Class C | 0.07% | 0.09% | 0.02% | 107% | 111% |
| Class E |  |  |  |  |  |
| Overall | 0.11% | 0.14% | 0.04% | 109% | 107% |
| Class D | 0.07% | -0.07% | -0.03% | 101% | 101% |
| Class F | 0.03% | -0.04% | -0.06% | 100% | 111% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.07% | -0.25% | 99% | 97% |
| Class C | -0.01% | 0.02% | 0.11% | 98% | 99% |
| Class E | -0.05% | 0.50% | 0.45% | 96% | 93% |
| Overall | -0.02% | 0.11% | 0.04% | 98% | 97% |
| Class D | 0.00% | 0.03% | 0.34% | 101% | 101% |
| Class F | 0.04% | 0.01% | 0.17% | 87% | 93% |

Coding performance and complexity of Test (c)

Table 5. Test results of Test (c)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.14% | 0.14% | 0.15% | 105% | 99% |
| Class A2 | 0.12% | 0.35% | 0.19% | 104% | 98% |
| Class B | 0.22% | 0.35% | 0.40% | 101% | 101% |
| Class C | 0.08% | 0.10% | -0.02% | 102% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.15% | 0.24% | 0.20% | 103% | 101% |
| Class D | 0.01% | 0.00% | 0.09% | 102% | 104% |
| Class F | -0.01% | 0.04% | 0.03% | 102% | 113% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11% | 0.15% | 0.06% | 101% | 98% |
| Class C | 0.03% | 0.14% | 0.27% | 99% | 99% |
| Class E | 0.12% | 0.48% | 0.59% | 98% | 101% |
| Overall | 0.09% | 0.23% | 0.26% | 99% | 99% |
| Class D | 0.02% | 0.24% | 0.45% | 101% | 100% |
| Class F | 0.00% | 0.03% | 0.21% | 91% | 105% |

Changes of text (test a)

- 8.5.2.2	Derivation process for luma motion vectors for merge mode

…

The motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1 and the prediction utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are derived by the following ordered steps:

…

8.	When numCurrMergeCand is less than MaxNumMergeCand and greater than 1, the following applies:

–	The derivation process for pairwise average merging candidate specified in clause 8.5.2.4 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N, and the half sample interpolation filter index hpelIfIdxN of every candidate N in mergeCandList, and numCurrMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0avgCand and refIdxL1avgCand, the prediction list utilization flags predFlagL0avgCand and predFlagL1avgCand and the motion vectors mvL0avgCand and mvL1avgCand of candidate avgCand being added into mergeCandList. The bi-prediction weight index bcwIdx and the half sample interpolation filter index hpelIfIdx of candidate avgCand being added into mergeCandList is set equal to 0.

–	numOrigMergeCand is set equal to numCurrMergeCand.

…

- 8.5.2.4	Derivation process for pairwise average merging candidate

Inputs to this process are:

- a merging candidate list mergeCandList,

- the reference indices refIdxL0N and refIdxL1N of every candidate N in mergeCandList,

- the prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N in mergeCandList,

- the motion vectors in 1/16 fractional-sample accuracy mvL0N and mvL1N of every candidate N in mergeCandList,

- the half sample interpolation filter index hpelIfIdxN of every candidate N in mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList.

Outputs of this process are:

- the merging candidate list mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the reference indices refIdxL0avgCand and refIdxL1avgCand of candidate avgCand added into mergeCandList during the invocation of this process,

- the prediction list utilization flags predFlagL0avgCand and predFlagL1avgCand of candidate avgCand added into mergeCandList during the invocation of this process,

- the motion vectors in 1/16 fractional-sample accuracy mvL0avgCand and mvL1avgCand of candidate avgCand added into mergeCandList during the invocation of this process,

- the half sample interpolation filter index hpelIfIdxavgCand of every candidate avgCand added into mergeCandList during the invocation of this process.

…

- The half sample interpolation filter index hpelIfIdxavgCand is derived as follows:

- If hpelIfIdxp0Cand is equal to hpelIfIdxp1Cand, hpelIfIdxavgCand is set equal to hpelIfIdxp0Cand.

- Otherwise, hpelIfIdxavgCand is set equal to 0.

Conclusion

This proposal suggests to simplify the process of selecting half-pel interpolation filters for merge candidates. Under the CTC test conditions, the average luma BD-rate change of three tests are: Test a: 0.01% and -0.03% for RA and LDB, respectively, Test b: 0.11 % and -0.02 % for RA and LDB, respectively, and Test c: 0.15% and 0.09% for RA and LDB, respectively.

References

- [1]	A. Henkel, I. Zupancic, B. Bross, M. Winken, H. Schwarz, D. Marpe, T. Wiegand (HHI), “CE4: Switchable interpolation filter”, JVET-O0057, July, 2019.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March, 2019.

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).