| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0264-v2 |
| --- | --- |

| Title: | CE4-related: Simplification of GEO using angles with power-of-two tangents |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Kevin Reuzé1, Chun-Chi Chen2, Han Huang3, Wei-Jung Chien, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +1-858-845-4257 1kreuz@qti.qualcomm.com 2chunchic@qti.qualcomm.com 3hanhuang@qti.qualcomm.com |
| Source: | Qualcomm Inc |  |  |

_____________________________

# Abstract

A geometric merge mode (GEO) was introduced in JVET-P0068. It is described as an extension of the Triangle Partition Mode (TPM) and introduces 140 ways to partition a CU, from 32 different angles and 5 possible displacements, into two parts. The angles used for GEO in JVET-P0068 are evenly distributed, with steps of 11.25°. It is asserted that by using angles which have a tangent equal to a power of two, one multiplication per sample and one multiplication per CU can be replaced with a shift. The proposed method provides 0.23%/0.73%/0.54% gains for Class B/C/D in RA configuration and 0.84%/0.67%/1.02% gains for Class C/D/E in LDB configuration.

# Introduction

GEO as introduced in JVET-P0068 supports 140 different partitioning manners, differentiated by 32 angles (quantized between 0 and 360° with 11.25° of equal separation) and 5 edges relative to the center of a CU, in addition to TPM to split a CU into two parts. Figure 1.a shows that the 5 edges are distributed uniformly along the direction of normal vector within a CU, starting from Edge0 that passes through the CU center. Each partition mode (i.e., a pair of an angle index and an edge index) in GEO is assigned with a pixel-adaptive weight table to blend samples on the two partitioned parts, where the weight value of a sampled ranges from 0 to 8 and is determined by the L2 distance from the center position of a pixel to the edge. Basically, unit-gain constraint is followed when weight values are assigned, that is, when a small weight value is assigned to a GEO partition, a large complementary one is assigned to the other partition, summing up to 8.

The computation of the weight value of each pixel is two-fold: (a) computing the displacement from a pixel position to a given edge and (c) mapping the computed displacement to a weight value through a pre-defined look-up table. The way to compute the displacement from a pixel position (x, y) to a given edge Edgei is actually the same as computing the displacement from (x, y) to Edge0 and subtract this displacement by the distance ρ between Edge0 and Edgei. Figure 1.b illustrates the geometric relations among (x, y) and edges. Specifically, the displacement from (x, y) to Edgei can be formulated as follows:

| PB | =PA- ρ |  |
| --- | --- | --- |
|  | =PC+CD*sin⁡(α)- ρ |  |
|  | =H2-y+x-W2*cot⁡(α)*sin⁡(α)- ρ |  |
|  | =x-W2*cosα-y-H2*sin(α)- ρ |  |
|  | =x*cosα-y*sin(α)-(ρ+W2*cosα-H2*sinα) |  |
|  | =x*cosα+y*cos(α+π2)-(ρ+W2*cosα+H2*cosα+π2). |  |

| (a) | (b) |
| --- | --- |

- (a) Edges supported in GEO. (b) Geometric relations between a given pixel position (x,y) and two edges.

The value of ρ is a function of the maximum length (denoted by ρmax) of the normal vector and edge index i, that is:

| ρ | =i*(ρmax-1)/N |  |
| --- | --- | --- |
|  | =i*((H2*tan(α)+W2)*cos⁡(α)-1)/N, |  |

where N is the number of edges supported by GEO and the “1” is to prevent the last edge EdgeN-1 from falling too close to a CU corner for some angle indices. Substituting Eq. (8) from (6), we can compute the displacement from each pixel (x,y) to a given Edgei. In short, we denote PB as wIdx(x,y). The computation of ρ is needed once per CU, and the computation of wIdx(x,y) is needed once per sample, in which multiplications are involved.

# Proposal

It is proposed to replace the angles used by GEO in JVET-P0068 by using the angles featuring with powers of 2 as tangent and harmonize the weight-value derivation of TPM by using the same one as for GEO.

## GEO using angles having power of 2 as tangent

It is proposed to replace the angles used by GEO in JVET-P0068 with angles having powers of 2 as tangent. The proposed angles are shown in Figure 2. As there are 16 different angles, times 2 as each angle can be used with +180°, which means that the number of allowed GEO partitions is still 140, and no change is made to the signalling of GEO.

- Proposed angles for GEO along with their corresponding width:height ratio.

Thanks to the characteristics that the tangent of the proposed angles is a power-of-2 number, most of the multiplications required in Eq. (6) for wIdx(x,y) can be replaced by using bit-shifting.

| wIdx(x,y) | =x*cosα+y*cos(α+π2)-(ρ+W2*cosα+H2*cosα+π2) |  |
| --- | --- | --- |
|  | =x*cosα-y*sin(α)-(ρ+W2*cosα-H2*sinα) |  |
|  | =(x-W2)*cosα-(y-H2)*sin(α)-ρ |  |
|  | =cosα*(x-W2-y-H2*tanα)-ρ. |  |

Using those angles that having powers of 2 as tangent, we can replace one multiplication per sample by using bit-shifting operations in implementations where GEO weight values would be computed ‘on the fly’. As triangle functions may be implemented as integer-precision operation, reduction of one multiplication helps supress rounding error.

In some implementations, it can be preferred to store masks with the weights for each CU size and each partition to reduce the number of operations needed by GEO, because weight values repeat row-by-row with phase shift. This behavior can be formulated by using Eq. (12), as follows:

| wIdx(x,y) | =cosα*(x-W2-y-H2*tanα)-ρ+cosα*(tanα-tanα) |  |
| --- | --- | --- |
|  | =cosα*(x-tanα-W2-y-1-H2*tanα)-ρ |  |
|  | =wIdx(x-tanα,y-1). |  |

As can be seen, wIdx(x,y) can be interpreted as inferring one of the weight value assigned to immediately previous row, and thus one row of weight values is the only thing required for mask storage.

In some cases, the value of tan(α) may not be integer, cot(α) is used as replacement and thus the weight values repeat column-by-column with phase shift as in Eq. (18). As can be seen, wIdx(x,y) can be interpreted as inferring one of the weight value assigned to immediately left row, and thus one column of weight values is the only thing required for mask storage.

| wIdx(x,y) | =cosα*(x-W2-y-H2*tanα)-ρ+cosα*(1-1) |  |
| --- | --- | --- |
|  | =cosα*(x-1-W2-y-cotα-H2*tanα)-ρ |  |
|  | =wIdx(x-1,y-cotα). |  |

In JVET-P0068, most of the angles have non-integer values for tan(α), meaning that (x-tan(α), y-1) and (x-1, y-cot(α)) does not fall on the center of a pixel, and this property cannot be used.

## Appling GEO weight calculation to TPM

It is proposed to align the derivation process of TPM to GEO and thus TPM mode can share the same function as GEO has to derive its weight values. With this, TPM mode becomes exactly the same as a particular GEO mode with diagonal (or anti-diagonal) split and an edge index equal to zero. In the other word, GEO can produce exactly the same prediction signals as TPM does, which is redundant in syntax design. Accordingly, for each CU size with an angle other than 0o, 90o, 180o and 270o, diagonal and anti-diagonal angles are removed from GEO.

# Experimental Results

The experiments for GEO with the proposed methods were conducted on top of the VTM-6.0 software with the common test conditions (JVET-N1010) to collect BD-rate numbers, encoding-time ratio and decoding-time ratio. The VTM anchor and testing results were generated using the VTM configurations with --PrintHexPSNR=1 and --SIMD=SSE42 for encoder and --SIMD=SSE42 for decoder. Macro settings as specified in the VTM-6.0 software are not changed. Parallel encoding and decoding were applied to Random Access case for both anchors and tests.

Six tests were conducted. The respective performance results are summarized in the table below.

(Test A1) CE4-1.14 + Replace GEO angles.

(Test A2) Test A1 + Apply shift-based simplification to wIdx.

(Test A3) Test A2 + Apply GEO weight calculation to TPM.

(Test B1) CE4-1.2 with 64 modes + Replace GEO angles.

(Test B2) Test B1 + Apply shift-based simplification to wIdx.

(Test B3) Test B2 + Apply GEO weight calculation to TPM.

- Performance results of Test A1.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  | x | x | x | x | x |
| Class A2 |  |  |  |  |  | x | x | x | x | x |
| Class B | -0.23% | -0.16% | -0.27% | 104% | 99% |  |  |  |  |  |
| Class C | -0.73% | -1.10% | -1.09% | 107% | 101% | -0.84% | -1.09% | -1.07% | 110% | 101% |
| Class E | x | x | x | x | x | -1.02% | -0.79% | -0.61% | 109% | 104% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.54% | -0.82% | -0.78% | 105% | 98% | -0.67% | -1.42% | -0.57% | 109% | 108% |
| Class F |  |  |  |  |  |  |  |  |  |  |

- Performance results of Test A2.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  | x | x | x | x | x |
| Class A2 |  |  |  |  |  | x | x | x | x | x |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.71% | -1.03% | -1.06% | 108% | 102% | -0.79% | -1.12% | -1.11% | 112% | 103% |
| Class E | x | x | x | x | x | -1.10% | -0.19% | -0.23% | 108% | 104% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.48% | -0.77% | -0.83% | 107% | 100% | -0.69% | -0.65% | -0.84% | 111% | 110% |
| Class F |  |  |  |  |  |  |  |  |  |  |

- Performance results of Test A3.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  | x | x | x | x | x |
| Class A2 |  |  |  |  |  | x | x | x | x | x |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E | x | x | x | x | x |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

- Performance results of Test B1.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  | x | x | x | x | x |
| Class A2 |  |  |  |  |  | x | x | x | x | x |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E | x | x | x | x | x |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

- Performance results of Test B2.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  | x | x | x | x | x |
| Class A2 |  |  |  |  |  | x | x | x | x | x |
| Class B | -0.23% | -0.20% | -0.28% | 104% | 101% |  |  |  |  |  |
| Class C | -0.72% | -1.05% | -1.11% | 104% | 100% |  |  |  |  |  |
| Class E | x | x | x | x | x | -0.96% | -0.08% | -0.02% | 104% | 100% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.48% | -0.92% | -0.64% | 103% | 101% | -0.69% | -0.63% | -0.35% | 108% | 106% |
| Class F | -0.33% | -0.51% | -0.42% | 102% | 101% |  |  |  |  |  |

- Performance results of Test B3.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  | x | x | x | x | x |
| Class A2 |  |  |  |  |  | x | x | x | x | x |
| Class B | -0.18% | -0.07% | -0.22% | 109% | 101% |  |  |  |  |  |
| Class C | -0.64% | -1.06% | -1.02% | 109% | 101% |  |  |  |  |  |
| Class E | x | x | x | x | x |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.52% | -0.83% | -0.54% | 109% | 102% | -0.79% | -0.69% | -0.56% | 112% | 107% |
| Class F | -0.29% | -0.46% | -0.39% | 106% | 102% |  |  |  |  |  |

# Conclusion

The proposed modification can induce either a complexity reduction or a storage reduction compared to the original GEO as studied in CE4-1 and shows extremely similar BD-rate gains compared to VTM6.0. It is recommended to adopt this proposal for the next version of VVC.

# Patent rights declaration(s)

Qualcomm Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).