| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0265 |
| --- | --- |

| Title: | CE3-related: CCLM with unified filter shape |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dae-Yeon Kim 11/12/13F, V&S Tower, 26 Samseong-ro 85-gil, Gangnam-gu, Seoul, 06194, KOREA Sung-Chang Lim Jinho Lee Jungwon Kang Hahyun Lee 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-10-3060-3361 brad.kim@chipsnmedia.com sclim@etri.re.kr jinosoul@etri.re.kr jungwon@etri.re.kr hanilee@etri.re.kr |
| Source: | Chips&Media and Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

- Abstract

At the previous Gothenburg meeting, JVET-O0343 proposed a CCLM simplification method in both H/W and S/W implementation-wise. In JVET-O0343, unavailable neighbouring samples are substituted by available sample values using reference sample construction process, so that it is able to reduce conditional branches and unify down-sampling filter shapes. In this contribution, the complexity analysis on the additional operation by the reference sample construction process and the number of down-sampling filter shapes to be used are provided to compare the current CCLM with the proposed CCLM. The proposed CCLM is implemented on top of VTM-6.0 and shows 0.01%/0.03%/0.06% and x.xx%/x.xx%/x.xx% BD-rate impact on Y/Cb/Cr components for AI and RA cases, respectively.

Problem Statement

The current CCLM process has many conditional branches to use only available neighbouring samples as in VVC CD [1]. Thus, it is not desirable in terms of hardware and software implementations. More specifically, the neighbouring sample position derivation process is dependent on the availabilities of top neighbouring samples and left neighbouring samples. And, for the luma down-sampling process of the luma samples, the current CCLM defines various luma down-sampling filter shapes which are selected depending on the position of samples and its neighbouring sample availabilities. Furthermore, both the number and positions of neighbouring samples to be used in the linear model parameter derivation are not fixed because they are also dependent on their availabilities. Therefore, the current CCLM design would be difficult to allow parallel processing such as SIMD in software implementation and consumes a lot of cycles and logics in hardware implementation.

Proposed method

In JVET-O0343 [2], the proposed method reuses reference sample availability marking process in VVC CD without any change to generate the chroma neighbouring sample availability information. And, to make all of the luma and chroma neighbouring samples available, the neighbour samples of the current chroma block and that of the corresponding luma block are generated using the chroma neighbouring sample availability. By adding the reference sample construction process, maximum three luma neighbouring sample lines in both left and top sides are generated to remove the availability checking operations in the CCLM process. To generate multiple luma neighbouring sample lines, the reference sample construction process is also employed as defined for the generation of multiple luma neighbouring sample lines for multi-reference line intra prediction (MRL). Fig 1 shows an example of performing the reference sample construction using the chroma neighbouring sample availability in case of 4x4 block with INTRA_LT_CCLM mode. As shown Fig. 1, by using performing the reference sample construction, all neighbouring samples can be available, so that the luma down-sampling filter shape can be unified and many conditional branches to select the filter shape can be removed.

Fig. 1. An example of performing the reference sample construction by the proposed method in case of INTRA_LT_CCLM mode

At the previous Gothenburg meeting, the complexity issue was brought up because the reference sample construction process is needed by the proposed method.

Table 1 shows the comparison of the reference sample availability marking and padding process in VVC CD and the proposed method in case of 8x8 block with INTRA_LT_CCLM mode when both above and left samples are available. As shown in Table 1, the current CCLM checks the neighbour availability of only two positions, up (0, -1) and left (-1, 0) in 4x4 unit if both up and left are available, because the current CCLM has an individual neighbour availability operation which is simplified by assuming that it is not needed to consider a constrained intra prediction. On the other hands, the proposed method checks the neighbour availability of five positions, (0, -1), (0, -2), (-1, 0), (-2, 0), and (-1, -1) in 4x4 unit if the reference sample availability checking process is used without change. However, the number of neighbour availability check can be reduced by 2 if the constrained intra prediction is not considered like the current CCLM.

Table 1. Comparison of the reference sample availability marking and padding process in VVC CD and the proposed method in case of 8x8 block with INTRA_LT_CCLM mode

|  | VVC CD | Proposed method |
| --- | --- | --- |
| Number of reference sample availability check | VTM S/W: 4 Draft text: 2 | Proposed VTM S/W: 5 Proposed draft text: 5 (By non-normative optimization, it can be reduced by 2) |
| Reference sample padding | No needed | Needed |

Table 2 shows the comparison of the number of down-sampling filter shape to be used for luma reconstructed samples in VVC CD and the proposed method in case of 8x8 block with INTRA_LT_CCLM mode. The current CCLM uses one to three filters depending on the position of sample to be down-sampled, whereas the proposed method uses only one unified filter shape regardless of the sample position.

Table 2. Comparison of the number of down-sampling filter shape to be used for luma reconstructed samples in VVC CD and the proposed method in case of 8x8 block with INTRA_LT_CCLM mode

- The number of down-sampling filter shapes in VVC CD

|  | Sample position to be down-sampled | sps_cclm_colocated_ chroma_flag =0 | sps_cclm_colocated_ chroma_flag =1 |
| --- | --- | --- | --- |
| Outer sample | Above | 3 | 3 |
|  | Left | 1 | 2 |
|  | Total # of filter shapes | 3 | 3 |
| Inner sample | (0, 0) | 2 | 3 |
|  | (x > 0, 0) | 1 or 2 | 2 |
|  | (0, y > 0) | 2 | 2 |
|  | Otherwise, | 1 | 1 |
|  | Total # of filter shapes | 2 | 3 |

- The number of down-sampling filter shapes in the proposed method

|  | Sample position to be down-sampled | sps_cclm_colocated_ chroma_flag =0 | sps_cclm_colocated_ chroma_flag =1 |
| --- | --- | --- | --- |
| Outer sample | Above | 1 | 1 |
|  | Left | 1 | 1 |
|  | # of filter shapes | 1 | 1 |
| Inner sample | All positions | 1 | 1 |

Table 3 shows the comparison of the positions of four samples to derive linear parameter in VVC CD and the proposed method in case of 8x8 chroma block. As shown Table 3, in case of the current CCLM, the position of four samples can be different depending on neighbour availability. On the other hands, the positions can be fixed by the proposed method.

Table 3. Comparison of the positions of four samples to be used for the linear parameter derivation in VVC CD and proposed method in case of 8x8 chroma block.

|  |  | VVC CD | Proposed method |
| --- | --- | --- | --- |
| INTRA_LT_CCLM |  | (2, -1), (7, -1), (-1, 2), (-1, 7), (1, -1), (3, -1), (5, -1), (7, -1), or (-1, 1), (-1, 3), (-1, 5), (-1, 7) | (2, -1), (7, -1), (-1, 2), (-1, 7) |
| INTRA_T_CCLM | Top | (2, -1), (6, -1), (10, -1), (14, -1), (1, -1), (4, -1), (7, -1), (10, -1), or (1, -1), (3, -1), (5, -1), (7, -1) | (2, -1), (6, -1), (10, -1), (14, -1) |
| INTRA_L_CCLM | Left | (-1, 2), (-1, 6), (-1, 10), (-1, 14), (-1, 1), (-1, 4), (-1, 7), (-1, 10), or (-1, 1), (-1, 3), (-1, 5), (-1, 7) | (-1, 2), (-1, 6), (-1, 10), (-1, 14) |

Experimental results

The experiments of the proposed methods were performed based on VTM-6.0 software [3] with VTM configuration files under JVET common test condition [4]. And, the experiments were conducted on the platform of Intel Xeon E5-2690 v2 (3.0GHz) and 64GB RAM with GCC 7.3.1 compiler under CentOS Linux.

The experimental results of the proposed methods compared to VTM-6.0 anchor are summarized in Table 4. The proposed method shows 0.01%/0.03%/0.06% and x.xx%/x.xx%/x.xx% BD-rate impact on Y/Cb/Cr components for AI and RA cases, respectively. And, the detailed results for tests can be found in accompanied Excel files.

Table 4. Experimental results of the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.14% | 0.28% | 101% | 100% |
| Class A2 | 0.01% | 0.10% | 0.02% | 100% | 100% |
| Class B | 0.01% | 0.01% | 0.02% | 101% | 100% |
| Class C | 0.02% | -0.04% | -0.03% | 101% | 100% |
| Class E | 0.00% | -0.05% | 0.07% | 101% | 100% |
| Overall | 0.01% | 0.03% | 0.06% | 101% | 100% |
| Class D | 0.00% | -0.02% | -0.15% | 101% | 100% |
| Class F | 0.04% | 0.04% | 0.14% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.19% | 0.26% | 100% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.02% | 0.14% | 0.13% | 100% | 100% |
| Class C | 0.01% | 0.04% | 0.08% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.06% | -0.10% | 0.01% | 100% | 100% |
| Class F | -0.01% | 0.06% | 0.09% | 100% | 100% |

Conclusion

In this contribution, a CCLM simplification method is proposed to reduce conditional branches and unify down-sampling filter shapes. And, the complexity analysis on the additional operation by the reference sample construction process and the number of down-sampling filter shapes to be used are provided to compare the current CCLM with the proposed CCLM. From the experimental results, it is observed that the proposed method has almost no impact on BD-rate. Therefore, it is recommended to adopt this proposal to clean up the current CCLM design.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6)”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-O2001, July. 2019

- D.-Y. Kim, J.-O. Lee, S.-C. Lim, J. Lee, J. Kang, “CE3-related: Simplification on CCLM process”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-O0343, July. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

Patent rights declaration(s)

Chips&Media may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).