| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0270 |
| --- | --- |

| Title: | Non-CE7: Modified signaling method of cu_cbf and tu_cbf_luma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dong-Jae Won Jae-Min Ha Joo-Hee Moon | Tel: Email: | +82-2-3408-3829 +82-2-3408-3194 bigdj2002@sju.ac.kr hajama0123@sju.ac.kr jhmoon@sejong.ac.kr |
| Source: | Sejong University |  |  |

_____________________________

- Abstract

This contribution proposes a modified signaling method for cu_cbf and tu_cbf_luma syntax when the current tree type is equal to dual tree and the current coding unit is coded by MODE_IBC. In MODE_IBC of the current VVC draft 6, the cbf information for luma is encoded by cu_cbf in the dual tree. while in the other coding modes, the cbf information for luma is encoded by tu_cbf_luma in Transform unit syntax. It is inconsistent in terms of the semantics of encoding the syntax. This contribution presents a signaling method in which cu_cbf is signaled only when the current tree type is single tree and the cbf information for luma of the current block coded by MODE_IBC in dual tree is informed by tu_cbf_luma. And an additional context for tu_cbf_luma is added to minimize a coding loss.

In Test 1, only syntax changes on cu_cbf and tu_cbf_luma are applied. In Test 2, additional context is added for tu_cbf_luma based on Test 1. Under the AI CTC configurations with IBC being on, the average (Y, U, V) BD rate performances of the proposed are reported with negligible encoding and decoding time change as follows:

Test 1 - VTM6.0-AI: {(Y, U, V)-BD-rate = (0.00%, 0.00%,0.00%), EncT = 100%, DecT = 100%}

Test 2 - VTM6.0-AI: {(Y, U, V)-BD-rate = (0.00%, 0.00%,0.00%), EncT = 100%, DecT = 100%}

Introduction

Table 1 shows how to encode cu_cbf in current Coding unit syntax in VVC draft 6 [1].

Table 1. 7.3.8.5 Coding unit syntax in VVC draft 6

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && !pred_mode_plt_flag && general_merge_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |

In dual tree, cu_cbf is encoded only when current Coding unit is coded by MODE_IBC and then it is inferred to the cbf information for luma. But, the cbf information for luma in other coding tools is encoded by tu_cbf_luma in Transform unit syntax. This proposal proposes a reasonable alternative of signaling to solve the above mentioned inconsistency.

Proposed Method

It is proposed to encode tu_cbf_luma instead of signaling cu_cbf as the cbf information for luma of current block when current coding unit is coded by MODE_IBC in dual tree. Table 2 and 3 show the proposed Coding unit and Transform unit syntax, and the changes are highlighted in yellow color.

Table 2. Proposed Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && !pred_mode_plt_flag && general_merge_flag[ x0 ][ y0 ] = = 0 && treeType = = SINGLE_TREE ) |  |
| cu_cbf | ae(v) |

Table 3. Proposed Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) && ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC && treeType = = DUAL_TREE_LUMA ) ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ¦ ¦ CbWidth[ chType ][ x0 ][ y0 ] > MaxTbSizeY ¦ ¦ CbHeight[ chType ][ x0 ][ y0 ] > MaxTbSizeY ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex < NumIntraSubPartitions − 1 ¦ ¦ !InferTuCbfLuma ) ) ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| if (IntraSubPartitionsSplitType != ISP_NO_SPLIT ) |  |
| InferTuCbfLuma = InferTuCbfLuma && !tu_cbf_luma[ x0 ][ y0 ] |  |
| } |  |

One additional context is introduced for tu_cbf_luma to minimize a coding loss as following Table 3 with the change highlighted in yellow color.

Table 4. CtxInc derivation process for tu_cbf_luma

| Conditions | ctxInc |
| --- | --- |
| If intra_bdpcm_flag is equal to 1, ctxInc is set equal to | 1 |
| Otherwise, if current coding unit is coded by MODE_IBC, ctxInc is set equal to | 2 |
| Otherwise, if IntraSubpartitionsSplitType is equal to ISP_NO_SPLIT , ctxInc is set equal to | 0 |
| Otherwise, if the current transform unit is the first one to be parsed in a coding unit or the value of tu_cbf_luma of the previous luma transform unit in the current coding unit is equal to 0, ctxInc is set equal to | 23 |
| Otherwise, ctxInc is set equal to | 34 |

Experimental Results

The proposed tests have been implemented on top of VTM6.0 with IBC being on. The following tests are conducted to check the performance of the proposed signaling:

Test 1: only syntax changes are applied based on Table 2 and Table 3

Test 2: Test 1 + additional context is added for tu_cbf_luma based on Table 4

Table 5 and Table 6 show the BD-rate impact of the proposed tests for the AI CTC configuration [2] respectively.

Table 5-1. AI_Test 1 result

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.00% | -0.05% | -0.01% | 100% | 99% |
| Class C | -0.01% | 0.05% | 0.09% | 101% | 99% |
| Class E | 0.00% | 0.07% | 0.13% | 101% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.00% | 0.01% | 0.12% | 100% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 6. AI_Test 2 result

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | 0.01% | 0.05% | 101% | 99% |
| Class E | -0.01% | -0.02% | 0.02% | 100% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.00% | -0.03% | -0.01% | 100% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusions

This proposal proposes simple modifications to make the signaling cu_cbf and tu_cbf_luma syntax more consistent when current tree type is equal to dual tree and current coding unit is coded by MODE_IBC. It is recommended to adopt proposed methods into the next version of VVC.

References

[1] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)” Document of Joint Video Experts Team, JVET-O2001.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

Patent rights declaration(s)

Sejong university may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).