| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0278 |
| --- | --- |

| Title: | Non-CE4: HMVP buffer update for TPM block |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In VVC, construction of initial merge candidate list between regular merge and triangle prediction mode are the same, and it includes spatial, temporal, HMVP, pairwise and zero candidates. However, triangle prediction mode and regular merge mode requires different operation in the HMVP buffer update process. Specifically, motion vector of the triangle prediction mode is not updated in HMVP buffer. In this proposal, for consistency in design, motion vector of the TPM is also considered for HMVP buffer update as regular merge. Following two tests are introduced to update HMVP buffer as follows:

- TEST1: motion vector on center position is updated

- TEST2: motion vector on triangle direction dependent center position is updated.

The experimental results for TEST1 reportedly show 0.xx% (0.xx%) BD-rate changes with xxx% (xxx%) and xxx% (xxx%) encoding and decoding run-time compared to VTM6.0 as anchor in RA(LB) configurations. And the experimental results for TEST2 reportedly show 0.xx% (0.xx%) BD-rate changes with xxx% (xxx%) and xxx% (xxx%) encoding and decoding run-time compared to VTM6.0 as anchor in RA(LB) configurations.

Introduction

The candidate list construction process for triangle prediction mode was adopted in [1]. As a results, TPM and regular merge mode has same candidate list construction process. However, motion vector of the TPM is not used in HMVP buffer update process, on the other hands, motion vector of the regular merge mode is updated in HMVP buffer.

Proposed method

For consistency in design, motion vector of the TPM is updated in HMVP buffer. Using the proposed method, the merge candidate list construction process and HMVP buffer updating process are shared to both TPM and regular merge mode.

After the storing process for triangle merge mode, the representative motion vector of the TPM is updated for HMVP candidate. As the representative motion vector, each motion vector of the 1) center position and 2) triangle direction dependent center position are tested in this proposal.

TEST1. Center position

TEST2. Triangle direction dependent center position

Experimental results

The proposed method has been implemented on top of the VTM6.0 with default setting and tested using JVET SDR common test condition [2]. The experimental results for TEST1 reportedly show 0.xx% (0.xx%) BD-rate changes with xxx% (xxx%) and xxx% (xxx%) encoding and decoding run-time compared to VTM6.0 as anchor in RA(LB) configurations. And the experimental results for TEST2 reportedly show 0.xx% (0.xx%) BD-rate changes with xxx% (xxx%) and xxx% (xxx%) encoding and decoding run-time compared to VTM6.0 as anchor in RA(LB) configurations.

TEST1: motion vector on center position is updated

|  | Over VTM 6.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.06% | 0.04% | 100% | 101% |  |  |  |  |  |
| Class A2 | 0.06% | 0.07% | 0.05% | 101% | 102% |  |  |  |  |  |
| Class B | 0.01% | 0.08% | 0.04% | 100% | 101% | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class C | 0.02% | 0.02% | -0.05% | 99% | 101% | -0.04% | 0.16% | 0.02% |  |  |
| Class E |  |  |  |  |  | -0.03% | 0.35% | 0.84% |  |  |
| Overall | 0.02% | 0.03% | 0.02% | 100% | 101% | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -0.05% | 0.04% | 0.07% | 101% | 103% | 0.00% | 0.40% | 0.52% |  |  |
| Class F | -0.02% | 0.03% | 0.04% | 101% | 101% | #VALUE! | #VALUE! | #VALUE! |  |  |

TEST2: motion vector on triangle direction dependent center position is updated

|  | Over VTM 6.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | 0.01% |  |  |  |  |  |  |  |
| Class A2 | 0.04% | 0.05% | -0.02% |  |  |  |  |  |  |  |
| Class B | -0.02% | 0.05% | 0.06% |  |  |  |  |  |  |  |
| Class C | -0.02% | 0.09% | 0.03% |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.01% | 0.04% | 0.03% |  |  |  |  |  |  |  |
| Class D | -0.06% | -0.03% | 0.04% |  |  |  |  |  |  |  |
| Class F | -0.01% | 0.05% | 0.02% |  |  |  |  |  |  |  |

Conclusion

In this contribution, HMVP buffer updating methods for TPM are proposed. The experimental results reportedly show negligible changes in BD-rate and encoding/decoding run-time. It is recommended to consider this method in the next VTM.

Reference

- X. Wang, et al., “CE4-related: An improved method for triangle merge list construction”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N0340, 14th Meeting: Geneva, CH, 19–27 March 2019.

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).