| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0281 |
| --- | --- |

| Title: | Non-CE4: Corrections on parameter calculation for PROF and BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In current VTM6.0, prediction refinement with optical flow (PROF) and bi-directional optical flow (BDOF) are used to refine the subblock based prediction in the aspect of each sample based on optical flow concept. Current PROF and BDOF have been found to have 16-bit overflow issue, unnecessary dependency with input bit-depth when deriving parameters, such as gradient and motion vector difference. In this contribution, following corrections and clean-ups are proposed for both PROF and BDOF.

Method I. Remove of bit-depth factor on parameter calculation: In current PROF and BDOF design, the bit-width of factors on parameter calculation depends on the input bit-depth. However, bit-width of the motion vector difference and some parameters are decided according to the input bit-depth even though it has no correlation with bit-depth, and gradient has fixed bit-width even though it has high correlation. Therefore, bit-depth factors on parameter calculation are removed in this proposal.

Method II. Minimize the truncation error: The gradient and motion vector difference are used to derive the refinement offset. But, right-shift operation for normalization of the refinement offset is performed just for gradient. This operation makes gradient parameter to have the truncation error. Therefore, methods to reduce truncation error are introduced.

Method III. Prevent the 16-bit overflow: The range of the prediction sample is within 16- bit range, but the range of the refinement offset exceeds the 16-bit. In this proposal, range of the refinement offset is restricted to prevent the 16-bit overflow.

|  | PROF |  |  | CE |  | BDOF |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | METHOD I | METHOD II | METHOD III | CE4-2.1 | CE4-2.2 | METHOD I | METHOD II | METHOD III |
| TEST1 | O |  |  |  |  | O |  |  |
| TEST2 |  | O |  |  |  |  |  |  |
| TEST3 |  |  | O |  |  |  |  |  |
| TEST4 |  |  | O* |  |  |  |  |  |
| TEST5 | O | O | O |  |  | O |  |  |
| TEST6 | O | O | O | O |  | O |  |  |
| TEST7 | O | O | O |  | O | O |  |  |
| TEST8 |  |  |  |  |  |  | O |  |
| TEST9 |  |  |  |  |  |  |  | O |

The simulation tests are performed as above table. The experimental results reportedly show overall BD-rate, encoding time and decoding time changes as follows in RA/LB configurations, respectively.

TEST 1: {0.xx%, 0.00%, 0.00%, 100%, 100%}, {0.xx%, 0.00%, 0.00%, 100%, 100%}

TEST 2: {0.xx%, 0.00%, 0.00%, 100%, 100%}, {0.xx%, 0.00%, 0.00%, 100%, 100%}

TEST 3: {0.xx%, 0.00%, 0.00%, 100%, 100%}, {0.xx%, 0.00%, 0.00%, 100%, 100%}

TEST 4: {0.xx%, 0.00%, 0.00%, 100%, 100%}, {0.xx%, 0.00%, 0.00%, 100%, 100%}

TEST 5: {0.xx%, 0.00%, 0.00%, 100%, 100%}, {0.xx%, 0.00%, 0.00%, 100%, 100%}

TEST 6: {0.xx%, 0.00%, 0.00%, 100%, 100%}, {0.xx%, 0.00%, 0.00%, 100%, 100%}

TEST 7: {0.xx%, 0.00%, 0.00%, 100%, 100%}, {0.xx%, 0.00%, 0.00%, 100%, 100%}

TEST 8: {0.xx%, 0.00%, 0.00%, 100%, 100%}, {0.xx%, 0.00%, 0.00%, 100%, 100%}

TEST 9: {0.xx%, 0.00%, 0.00%, 100%, 100%}, {0.xx%, 0.00%, 0.00%, 100%, 100%}

Introduction

At 15th JVET meeting, prediction refinement with optical flow (PROF) for affine mode was adopted [1]. According to the [1], in the PROF derivation process, horizontal / vertical gradient (gx, gy) at each sample position are derived as follows.

gxi, j=Ii+1, j-Ii-1, j

gyi, j=Ii, j+1-Ii, j-1

Also, the motion vector difference (Δvx, Δvy) for each sample location in 4x4 affine sub-block is calculated. Using the derived gradient and motion vector difference, refinement offset (ΔI(i,j)) is calculated by the optical flow equation.

ΔI(i,j)= gxi,j*Δvx(i,j)+gyi,j*Δvy(i,j)

Based on the VTM6.0 and VVC 6.0 Draft [2], the bit-width of each parameters for PROF are analyzed as follows according to the input bit-depth 8 ~ 16.

Table 1 Bit-width analysis of the PROF

| input bit-depth | predSample | diffMv | gradientH/V | ΔI(i,j) |
| --- | --- | --- | --- | --- |
| 8 | 16 | 7 | 11 | 18 |
| 10 | 16 | 7 | 11 | 18 |
| 12 | 16 | 7 | 11 | 18 |
| 14 | 18 | 9 | 11 | 20 |
| 16 | 20 | 11 | 11 | 22 |

where bit-width of the prediction sample, predSample can be calculated by following equation in spec text. In here, the bit-width of the predSample is in 16 ~ 20 according to the input bit-depth because shift1 is set equal to Min( 4, BitDepthY − 8 ) and shift2 is set equal to 6,

- if xFracL is not equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows: - The sample array temp[ n ] with n = 0..7, is derived as follows: temp[ n ] = i=07fL[ xFracL ][ i ] * refPicLXL[ xInti ][ yIntn ] >> shift1 (8-780) - The predicted luma sample value predSampleLXL is derived as follows: predSampleLXL = i=07fL[ yFracL ][ i ] * temp[ i ] >> shift2 (8-781)

To derive the PROF refinement offset, motion vector difference and gradient are derived as follows. For motion vector difference, diffMV can be calculated as follows, where dmvLimit is defined as 1 << Max ( 6, BitDepthY − 6 ).

- The value of diffMv[ x ][ y ][ i ] is clipped as follows: diffMv[ x ][ y ][ i ] = Clip3( −dmvLimit, dmvLimit − 1, diffMv[ x ][ y ][ i ]) (8-736)

Also, horizontal/vertical gradient can be calculated by following equation in spec text. Using the motion vector difference and gradient, refinement offset, dI can be calculated.

Variable shift1 is set equal to Max( 6, BitDepthY − 6 ). For x =0..sbWidth − 1, y =0..sbHeight − 1, the following ordered steps apply: - The variables gradientH[ x ][ y ] and gradientV[ x ][ y ] are derived as follows: gradientH[ x ][ y ] = ( predSamples[ x + 2 ][ y ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 ) (8-796) gradientV[ x ][ y ] = ( predSamples[ x ][ y + 2 ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 ) (8-797) - The variable dI is derived as follows: dI = gradientH[ x ][ y ] * diffMv[ x ][ y ][ 0 ] + gradientV[ x ][ y ] * diffMv[ x ][ y ][ 1 ] (8-798) - Prediction sample value at location ( x, y ) in the subblock is derived as follows: pbSamples[ x ][ y ] = predSamples[ x + 1 ][ y + 1 ] + ( ( dI + 1 ) >> 1 ) (8-799)

Proposed method

In this proposal, following factors are corrected on PROF and BDOF.

- Remove of bit-depth factor on parameter calculation

- Minimize the truncation error

- Prevent the 16-bit overflow

Proposed correction on parameter calculation for PROF

Method I: Remove bit-depth factor on parameter calculation

In current PROF design, the bit-width of the prediction sample, motion vector difference and gradient depends on the input bit-depth. However, bit-width of the motion vector difference is decided according to the input bit-depth even though it has no correlation with bit-depth, and gradient has fixed bit-width even though it has high correlation.

The bit-width of the motion vector difference, diffMV is decided by dmvLimit, where dmvLimit is set to 1 << Max( 6, BitDepthY − 6 ).

diffMv[ x ][ y ][ i ] = Clip3( −dmvLimit, dmvLimit − 1, diffMv[ x ][ y ][ i ])

Also, the bit-width of the gradient is derived using shift1, where shift1 is set equal to Max( 6, BitDepthY − 6 ).

gradientH[ x ][ y ]  =  ( predSamples[ x + 2 ][ y ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 )

gradientV[ x ][ y ]  =  ( predSamples[ x ][ y + 2 ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 )

In this proposal, to correct the parameter calculation, definition of the dmvLimit for diffMV and shift1 for gradient are modified as follows:

dmvLimit  =  1  <<  Max( 6, BitDepthY − 6 )  dmvLimit  =  1  <<  6

Variable shift1 is set equal to Max( 6, BitDepthY − 6 )  Variable shift1 is set equal to 6

By applying the modified definition, considering the input bit-depth is removed and bit-width of the each factors at Table 1 are modified as Table 2.

Table 2 Modified bit-width of the PROF

| input bit-depth | predSample | diffMv | gradientH/V | ΔI(i,j) |
| --- | --- | --- | --- | --- |
| 8 | 16 | 7 | 11 | 18 |
| 10 | 16 | 7 | 11 | 18 |
| 12 | 16 | 7 | 11 | 18 |
| 14 | 18 | 9 7 | 11 13 | 20 |
| 16 | 20 | 11 7 | 11 15 | 22 |

Method II: Minimize the truncation error

The gradient and motion vector difference are used to derive the refinement offset, dI. In the derivation process, gradient is calculated and then dI is calculated as the multiplication of the gradient and motion vector difference. But, right-shift operation for normalization of the dI is performed just for gradient. This operation make gradient parameter to have the truncation error.

In this proposal, right-shift operation is performed at dI calculation stage to minimize the truncation error. Also, the shift operation at final offset derivation stage is moved to dI calculation stage. Moreover, clipping operation is performed at gradient calculation stage to prevent the 16-bit overflow at intermediate calculation stage.

Using the proposed scheme, the shift-operation at each stage is changed as Table 3.

Table 3. Modified shift operation

|  | Current design | Proposed design |
| --- | --- | --- |
| gradientH/V | >> 6 | Clipping |
| dI | X | >> 7 |
| Final offset | >> 1 | X |

Following table shows the proposed draft changes. If the proposed method I is applied, the variable shift1 is set equal to fixed value, 7. Otherwise, the variable shift1 is set equal to Max( 7, BitDepthY − 5 ).

Variable shift1 is set equal to Max( 6 7, BitDepthY – 6 5 ). Variable gradLimit is set equal to 1 << Max( 15, BitDepth + 3 ) For x =0..sbWidth − 1, y =0..sbHeight − 1, the following ordered steps apply: - The variables gradientH[ x ][ y ] and gradientV[ x ][ y ] are derived as follows: gradientH[ x ][ y ] = Clip3 ( -gradLimit , gradLimit – 1, ( predSamples[ x + 2 ][ y ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 ) ) (8-796) gradientV[ x ][ y ] = Clip3 ( -gradLimit , gradLimit – 1, ( predSamples[ x ][ y + 2 ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 ) ) (8-797) - The variable dI is derived as follows: dI = (gradientH[ x ][ y ] * diffMv[ x ][ y ][ 0 ]) >> shift1 + (gradientV[ x ][ y ] * diffMv[ x ][ y ][ 1 ]) >> shift1 (8-798) - Prediction sample value at location ( x, y ) in the subblock is derived as follows: pbSamples[ x ][ y ] = predSamples[ x + 1 ][ y + 1 ] + ( ( dI + 1 ) >> 1 ) (8-799)

Also, by applying the modified definition, bit-width of the each factors at Table 1 are modified as Table 4.

Table 4 Modified bit-width of the PROF

| input bit-depth | predSample | diffMv | gradientH/V | ΔI(i,j) |
| --- | --- | --- | --- | --- |
| 8 | 16 | 7 | 11 16 | 18 |
| 10 | 16 | 7 | 11 16 | 18 |
| 12 | 16 | 7 | 11 16 | 18 |
| 14 | 18 | 9 7 | 11 18 | 20 |
| 16 | 20 | 11 7 | 11 20 | 22 |

Method III: Prevent the 16-bit overflow

The range of the prediction sample, predSample is within [-22258, 19317], when input bit-depth is 10-bit (for affine block, 6-tap interpolation filter is used). Also, the range of the refinement offset, ΔI(i,j) is within [-41471, 40824]. Eventually, the range of the final prediction sample is within [-22258 - 41471, 19317 + 40824] and it exceed the 16-bit range [-32768, 32767].

In this proposal, to prevent the 16-bit overflow, range of the refinement offset is restricted to 14-bit range, [-8192, 8191]. Therefore, the range of the final prediction sample is within [-32768, 32767] as [-22258 - 8192, 19317 + 8191]. Applying the proposed clipping operation, spec text can be modified as follows.

dILimit is set equal to 1 << Max(13, BitDepthY + 1 ). Variable shift1 is set equal to Max( 6, BitDepthY − 6 ). For x =0..sbWidth − 1, y =0..sbHeight − 1, the following ordered steps apply: - The variables gradientH[ x ][ y ] and gradientV[ x ][ y ] are derived as follows: gradientH[ x ][ y ] = ( predSamples[ x + 2 ][ y ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 ) (8-796) gradientV[ x ][ y ] = ( predSamples[ x ][ y + 2 ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 ) (8-797) - The variable dI is derived as follows: dI = gradientH[ x ][ y ] * diffMv[ x ][ y ][ 0 ] + gradientV[ x ][ y ] * diffMv[ x ][ y ][ 1 ] (8-798) - Prediction sample value at location ( x, y ) in the subblock is derived as follows: pbSamples[ x ][ y ] = predSamples[ x + 1 ][ y + 1 ] + Clip3( -dILimit, dILimit - 1 ( ( dI + 1 ) >> 1 )) (8-799)

Note. The minimum value and maximum value in range of the prediction sample is simply calculated as (80 * (-22) * 1023) + (80 * (-22) * 1023)) >> 8) and (80 * 80 * 1023) + ((-22) * (-22) * 1023)) >> 8), respectively. Also, according above equations, min/max value of the gradient, diffMV, dI, and final offset are calculated as Table 5.

Table 5 min/max value for PROF

|  | predSample(SW) |  | Gradient |  | diffMV |  | dI |  | Final offset |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BitDepth | Min | Max | Min | Max | Min | Max | Min | Max | Min | Max |
| 8 | -25022 | 24958 | -779 | 779 | -64 | 63 | -99712 | 98154 | -49855 | 49077 |
| 10 | -25071 | 25055 | -782 | 782 | -64 | 63 | -100096 | 98532 | -50047 | 49266 |
| 12 | -25083 | 25079 | -782 | 782 | -64 | 63 | -100096 | 98532 | -50047 | 49266 |

Considering the CE4-2.1 and CE4-2.2

In CE4-2.1, the clipping range of the motion vector difference for PROF is restricted to [-32, 31] with 1/32-pel precision. In CE4-2.2, the clipping range of the motion vector difference for PROF is restricted to [-32, 31] with 1/64-pel precision.

The proposed method I, II and III can be applied on top of CE4-2.1 without changes except following two definition. The dmvLimit is changed because the clipping range of the motion vector difference is restricted in [-32, 31] not [-64, 63] with 1/32-pel precision.

dmvLimit  =  1  <<   6    dmvLimit  =  1  <<  5

Variable shift1 is set equal to 7.   Variable shift1 is set equal to 6.

Variable shift1 is set equal to Max( 6, BitDepthY − 6 ) 6. Variable gradLimit is set equal to 1 << Max( 15, BitDepth + 3 ) Variable dILimit is set equal to 1 << Max(13, BitDepthY + 1 ). For x =0..sbWidth − 1, y =0..sbHeight − 1, the following ordered steps apply: - The variables gradientH[ x ][ y ] and gradientV[ x ][ y ] are derived as follows: gradientH[ x ][ y ] = Clip3 ( -gradLimit , gradLimit – 1, ( predSamples[ x + 2 ][ y ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 ) ) (8-796) gradientV[ x ][ y ] = Clip3 ( -gradLimit , gradLimit – 1, ( predSamples[ x ][ y + 2 ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 ) ) (8-797) - The variable dI is derived as follows: dI = (gradientH[ x ][ y ] * diffMv[ x ][ y ][ 0 ]) >> shift1 + (gradientV[ x ][ y ] * diffMv[ x ][ y ][ 1 ]) >> shift1 (8-798) - Prediction sample value at location ( x, y ) in the subblock is derived as follows: pbSamples[ x ][ y ] = predSamples[ x + 1 ][ y + 1 ] + Clip3( -dILimit, dILimit - 1 ( ( dI + 1 ) >> 1 )) (8-799)

Also, the proposed method I, II and III can be applied on top of CE4-2.2 without changes except following one definition. The dmvLimit is changed because the clipping range of the motion vector difference is restricted in [-32, 31] not [-64, 63] with 1/64-pel precision.

dmvLimit  =  1  <<   6    dmvLimit  =  1  <<  5

Variable shift1 is set equal to Max( 6, BitDepthY − 6 ) 7. Variable gradLimit is set equal to 1 << Max( 15, BitDepth + 3 ) Variable dILimit is set equal to 1 << Max(13, BitDepthY + 1 ). For x =0..sbWidth − 1, y =0..sbHeight − 1, the following ordered steps apply: - The variables gradientH[ x ][ y ] and gradientV[ x ][ y ] are derived as follows: gradientH[ x ][ y ] = Clip3 ( -gradLimit , gradLimit – 1, ( predSamples[ x + 2 ][ y ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 ) ) (8-796) gradientV[ x ][ y ] = Clip3 ( -gradLimit , gradLimit – 1, ( predSamples[ x ][ y + 2 ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 ) ) (8-797) - The variable dI is derived as follows: dI = (gradientH[ x ][ y ] * diffMv[ x ][ y ][ 0 ]) >> shift1 + (gradientV[ x ][ y ] * diffMv[ x ][ y ][ 1 ]) >> shift1 (8-798) - Prediction sample value at location ( x, y ) in the subblock is derived as follows: pbSamples[ x ][ y ] = predSamples[ x + 1 ][ y + 1 ] + Clip3( -dILimit, dILimit - 1 ( ( dI + 1 ) >> 1 )) (8-799)

Extension to BDOF

Method I: Remove bit-depth factor on parameter calculation

In current BDOF design, the bit-width of the prediction sample, motion vector difference and gradient depends on the input bit-depth. However, bit-width of the motion vector difference and the parameters (i.e., diff, tempH, tempV) are calculated by the input bit-depth even though it have no correlation with bit-depth. On the other hand, the gradient derived from prediction sample value finally have the fixed bit-width without considering the input bit-depth because shift value (shift1) is calculated using the bit-depth.

In current VVC6.0 draft, shift1 ~ shift3 and mvRefineThres are used to derive the parameters for the motion vector difference and gradient as follows.

- For x =xSb − 1..xSb + 4, y = ySb − 1..ySb + 4, the following ordered steps apply: - The locations ( hx, vy ) for each of the corresponding sample locations ( x, y ) inside the prediction sample arrays are derived as follows: hx = Clip3( 1, nCbW, x ) (8-801) vy = Clip3( 1, nCbH, y ) (8-802) - The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows: gradientHL0[ x ][ y ] = ( predSamplesL0[ hx + 1 ][vy] >> shift1 ) − (8-803) ( predSampleL0[ hx − 1 ][ vy ] ) >> shift1 ) gradientVL0[ x ][ y ] = ( predSampleL0[ hx ][ vy + 1 ] >> shift1 ) − (8-804) ( predSampleL0[ hx ][vy − 1 ] ) >> shift1 ) gradientHL1[ x ][ y ] = ( predSamplesL1[ hx + 1 ][vy] >> shift1 ) − (8-805) ( predSampleL1[ hx − 1 ][ vy ] ) >> shift1 ) gradientVL1[ x ][ y ] = ( predSampleL1[ hx ][ vy + 1 ] >> shift1 ) − (8-806) ( predSampleL1[ hx ][vy − 1 ] ) >> shift1 ) - The variables diff[ x ][ y ], tempH[ x ][ y ] and tempV[ x ][ y ] are derived as follows: diff[ x ][ y ] = (predSamplesL0[ hx ][ vy ] >> shift2 ) − ( predSamplesL1[ hx ][ vy ] >> shift2 ) (8-807) tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] ) >> shift3 (8-808) tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] ) >> shift3 (8-809) - The variables sGx2, sGy2, sGxGy, sGxdI and sGydI are derived as follows: sGx2 = ij Abs( tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-810) sGy2 = ij Abs( tempV[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-811) sGxGy = ij( Sign( tempV[ xSb + i ][ ySb + j ] ) * tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-812) sGxGym = sGxGy >> 12 (8-813) sGxGys = sGxGy & ( ( 1 << 12 ) − 1 ) (8-814) sGxdI = ij( − Sign( tempH[ xSb + i ][ ySb + j ] ) * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-815) sGydI = ij( − Sign( tempV[ xSb + i ][ ySb + j ] ) * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-816) - The horizontal and vertical motion offset of the current subblock are derived as: vx = sGx2 > 0 ? Clip3( −mvRefineThres, mvRefineThres, (8-817) −( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) ) : 0 vy = sGy2 > 0 ? Clip3( −mvRefineThres, mvRefineThres, ( ( sGydI << 3 ) − (8-818) ( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGy2 ) ) ) : 0 - For x =xSb − 1..xSb + 2, y = ySb − 1..ySb + 2, the prediction sample values of the current sub-block are derived as follows: bdofOffset = ( vx * ( gradientHL0[ x + 1 ][ y + 1 ] − gradientHL1[ x + 1 ][ y + 1 ] ) ) >> 1 (8-819) + ( vy * (gradientVL0[ x + 1 ][ y + 1 ] − gradientVL1[ x + 1 ][ y + 1 ] ) ) >> 1 pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset4 + (8-820) predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset ) >> shift4 )

In this proposal, definition of the variables for diffMV and gradient are modified to correct the parameter calculation as follows:

Variables bitDepth, shift1, shift2, shift3, shift4, offset4, and mvRefineThres are derived as follows: - The variable bitDepth is set equal to BitDepthY. - The variable shift1 is set to equal to Max( 6, bitDepth − 6 ) 6. - The variable shift2 is set to equal to Max( 4, bitDepth − 8 ) 4. - The variable shift3 is set to equal to Max( 1, bitDepth − 11 ) 1. - The variable shift4 is set equal to Max( 3, 15 − bitDepth ) and the variable offset4 is set equal to 1 << ( shift4 − 1 ). - The variable mvRefineThres is set equal to 1 << Max( 5, bitDepth − 7 ) 5.

By applying the modified definition, considering the input bit-depth is removed and bit-width of the each factors modified as Table 6.

Table 6 Modified bit-width of the BDOF

| input bit-depth | predSample | diffMv | gradientH/V | ΔI(i,j) |
| --- | --- | --- | --- | --- |
| 8 | 16 | 6 | 11 | 18 |
| 10 | 16 | 6 | 11 | 18 |
| 12 | 16 | 6 | 11 | 18 |
| 14 | 18 | 8 6 | 11 13 | 20 |
| 16 | 20 | 10 6 | 11 15 | 22 |

Method II: Minimize the truncation error

The gradient and motion vector difference are used to derive the refinement offset, bdofOffset. In the derivation process, gradient is calculated and then bdofOffset is calculated as the multiplication of the gradient and motion vector difference. But, right-shift operation for normalization of the bdofOffset is performed just for gradient. This operation make gradient parameter to have the truncation error.

In this proposal, right-shift operation is performed at bdofOffset calculation stage to minimize the truncation error. Also, clipping operation is performed at gradient calculation stage to prevent the 16-bit overflow at intermediate calculation stage.

According to the bit-width of the gradient is extended, intermediate values in tempH, tempV calculation step exceed the 16-bit range. Therefore, the shift value, shift3 is applied in each gradient values in the tempH, tempV calculation step.

Using the proposed scheme, the shift-operation at each stage is changed as Table 3.

Table 7. Modified shift operation

|  | Current design | Proposed design |
| --- | --- | --- |
| gradientHL/V (shift1) | >> 6 | Clipping |
| tempH/V (shift3) | >> 1 | >> 6 |
| bdofOffset | >> 1 | >> 7 |

Following table show the proposed draft changes. In here, variable gradLimit is set equal to 1 << Max( 15, BitDepth + 3 ).

- For x =xSb − 1..xSb + 4, y = ySb − 1..ySb + 4, the following ordered steps apply: - The locations ( hx, vy ) for each of the corresponding sample locations ( x, y ) inside the prediction sample arrays are derived as follows: hx = Clip3( 1, nCbW, x ) (8-801) vy = Clip3( 1, nCbH, y ) (8-802) - The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows: gradientHL0[ x ][ y ] = Clip3 ( -gradLimit , gradLimit – 1 , ( predSamplesL0[ hx + 1 ][vy] >> shift1) − ( predSampleL0[ hx − 1 ][ vy ] ) >> shift1)) (8-803) gradientVL0[ x ][ y ] = Clip3 ( -gradLimit , gradLimit – 1 , ( predSampleL0[ hx ][ vy + 1 ] >> shift1) − ( predSampleL0[ hx ][vy − 1 ] ) >> shift1)) (8-804) gradientHL1[ x ][ y ] = Clip3 ( -gradLimit , gradLimit – 1 , ( predSamplesL1[ hx + 1 ][vy] >> shift1) − ( predSampleL1[ hx − 1 ][ vy ] ) >> shift1)) (8-805) gradientVL1[ x ][ y ] = Clip3 ( -gradLimit , gradLimit – 1 , ( predSampleL1[ hx ][ vy + 1 ] >> shift1) − ( predSampleL1[ hx ][vy − 1 ] ) >> shift1)) (8-806) - The variables diff[ x ][ y ], tempH[ x ][ y ] and tempV[ x ][ y ] are derived as follows: diff[ x ][ y ] = (predSamplesL0[ hx ][ vy ] >> shift2 ) − ( predSamplesL1[ hx ][ vy ] >> shift2 ) (8-807) tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] ) >> shift3 (8-808) tempH[ x ][ y ] = (gradientHL0[ x ][ y ] >> shift3)+ (gradientHL1[ x ][ y ] >> shift3) (8-808) tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] ) >> shift3 (8-809) tempV[ x ][ y ] = (gradientVL0[ x ][ y ] >> shift3) + (gradientVL1[ x ][ y ] >> shift3) (8-809) - The variables sGx2, sGy2, sGxGy, sGxdI and sGydI are derived as follows: sGx2 = ij Abs( tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-810) sGy2 = ij Abs( tempV[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-811) sGxGy = ij( Sign( tempV[ xSb + i ][ ySb + j ] ) * tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-812) sGxGym = sGxGy >> 12 (8-813) sGxGys = sGxGy & ( ( 1 << 12 ) − 1 ) (8-814) sGxdI = ij( − Sign( tempH[ xSb + i ][ ySb + j ] ) * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-815) sGydI = ij( − Sign( tempV[ xSb + i ][ ySb + j ] ) * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4 (8-816) - The horizontal and vertical motion offset of the current subblock are derived as: vx = sGx2 > 0 ? Clip3( −mvRefineThres, mvRefineThres, (8-817) −( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) ) : 0 vy = sGy2 > 0 ? Clip3( −mvRefineThres, mvRefineThres, ( ( sGydI << 3 ) − (8-818) ( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGy2 ) ) ) : 0 - For x =xSb − 1..xSb + 2, y = ySb − 1..ySb + 2, the prediction sample values of the current sub-block are derived as follows: bdofOffset = ( vx * ( gradientHL0[ x + 1 ][ y + 1 ] − gradientHL1[ x + 1 ][ y + 1 ] ) ) >> 1 shift1 (8-819) + ( vy * (gradientVL0[ x + 1 ][ y + 1 ] − gradientVL1[ x + 1 ][ y + 1 ] ) ) >> 1 shift1 pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset4 + (8-820) predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset ) >> shift4 )

B applying the modified definition, bit-width of the each factors are modified as Table 8.

Table 8 Modified bit-width of the BDOF

| input bit-depth | predSample | diffMv | gradientH/V | ΔI(i,j) |
| --- | --- | --- | --- | --- |
| 8 | 16 | 6 | 11 16 | 18 |
| 10 | 16 | 6 | 11 16 | 18 |
| 12 | 16 | 6 | 11 16 | 18 |
| 14 | 18 | 8 6 | 11 18 | 20 |
| 16 | 20 | 10 6 | 11 20 | 22 |

Method III: Prevent the 16bit overflow

The range of the prediction sample, predSample is within [-25071, 25055], when input bit-depth is 10-bit. Also, the range of the refinement offset, ΔI(i,j) is within [-50048, 48484]. Eventually, the range of the final prediction sample is within [-25071 - 50048, 25055 + 48484] and it exceed the 16-bit range [-32768, 32767].

In this proposal, to prevent the 16-bit overflow, range of the refinement offset is restricted to 13-bit range, [-4096, 4095]. Therefore, the range of the final prediction sample is within [-32768, 32767] as [-25071 - 4096, 25055 + 4095]. Applying the proposed clipping operation, spec text can be modified as follows.

Variable OffsetLimit is set equal to 1 << Max(12, BitDepthY) pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset4 + (8-820) predSamplesL1[ x + 1 ][ y + 1 ] + Clip3( -OffsetLimit, OffsetLimit - 1, bdofOffset) ) >> shift4 )

Note. The minimum value and maximum value in range of the prediction sample is simply calculated as (88 * (-24) * 1023) + (88 * (-24) * 1023)) >> 8) and (88 * 88 * 1023) + ((-24) * (-24) * 1023)) >> 8), respectively. Also, according above equations, min/max value of the gradient, diffMV, bdofOffset are calculated as Table 9.

Table 9 min/max value for BDOF

|  | predSample(SW) |  | Gradient |  | diffMV |  | bdofOffset |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BitDepth | Min | Max | Min | Max | Min | Max | Min | Max |
| 8 | -25022 | 24958 | -779 | 779 | -32 | 31 | -49856 | 48298 |
| 10 | -25071 | 25055 | -782 | 782 | -32 | 31 | -50048 | 48484 |
| 12 | -25083 | 25079 | -782 | 782 | -32 | 31 | -50048 | 48484 |

Additionally, the range of the refinement offset for PROF can be aligned to the range of the refinement offset for BDOF because the offset range between PROF and BDOF is different. Therefore, limit value, dILimit in Method III for PROF is adjusted as follows.

dILimit is set equal to 1 << Max(12, BitDepthY ).

Experimental results

In this proposal, three aspects are introduced. First one is removing the bit-depth factor on parameter calculation and second one is to reduce the truncation error and last one is to prevent the 16-bit overflow. These aspects are applied to PROF and BDOF. The proposed methods have been implemented on top of the VTM6.0 with default setting and tested using JVET SDR common test condition [3].

The simulation tests are performed as following Table 10. In TEST 4, * mark means that the clipping range of the refinement offset for PROF is [-4096, 4095], which is aligned to clipping range of BDOF.

Table 10 Test sets

|  | PROF |  |  | CE |  | BDOF |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | METHOD I | METHOD II | METHOD III | CE4-2.1 | CE4-2.2 | METHOD I | METHOD II | METHOD III |
| TEST1 | O |  |  |  |  | O |  |  |
| TEST2 |  | O |  |  |  |  |  |  |
| TEST3 |  |  | O |  |  |  |  |  |
| TEST4 |  |  | O* |  |  |  |  |  |
| TEST5 | O | O | O |  |  | O |  |  |
| TEST6 | O | O | O | O |  | O |  |  |
| TEST7 | O | O | O |  | O | O |  |  |
| TEST8 |  |  |  |  |  |  | O |  |
| TEST9 |  |  |  |  |  |  |  | O |

TEST 1

In TEST 1, Method I is applied both PROF and BDOF. There is no changes in BD-rate and encoding time and decoding is slightly changed as follows in RA/LB configurations, respectively.

Table 11 TEST 1 experimental results on VTM6.0 RA/LB

|  | Over VTM 6.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 101% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 98% | 102% | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 96% | 96% | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 99% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 100% | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 94% | 91% | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 96% | 98% | 0.00% | 0.00% | 0.00% | 99% | 103% |

TEST 2, TEST 3 and TEST 4

In TEST2 and TEST3, proposed Method II and Method III for PROF is tested. In TEST 4, clipping range of the refinement offset for PROF in Method III is [-4096, 4095], which is aligned to clipping range of BDOF. The experimental results reportedly show overall BD-rate, encoding time and decoding time changes as follows in RA/LB configurations, respectively.

Table 12 TEST 2 experimental results on VTM6.0 RA/LB

|  | Over VTM 6.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |  |  |  |  |
| Class A2 | 0.01% | 0.04% | 0.07% | 99% | 100% |  |  |  |  |  |
| Class B | -0.01% | 0.11% | -0.02% | 97% | 100% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.01% | 0.03% | 0.02% | 95% | 96% | -0.03% | 0.09% | 0.03% | 98% | 99% |
| Class E |  |  |  |  |  | -0.05% | 0.09% | 0.14% | 98% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | -0.05% | -0.05% | 94% | 91% | -0.04% | 0.62% | 0.35% | 98% | 100% |
| Class F | 0.00% | 0.03% | -0.02% | 96% | 97% | 0.09% | 0.24% | 0.30% | 98% | 100% |

Table 13 TEST 3 experimental results on VTM6.0 RA/LB

|  | Over VTM 6.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |  |  |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | 0.00% | 0.00% | 96% | 96% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 94% | 92% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 14 TEST 4 experimental results on VTM6.0 RA/LB

|  | Over VTM 6.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |  |  |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |  |  |  |  |
| Class B | -0.01% | -0.01% | -0.01% | 98% | 99% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | -0.01% | -0.03% | 96% | 96% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  | 0.02% | 0.11% | 0.18% | 99% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.02% | 0.01% | 94% | 91% | 0.05% | 0.08% | 0.07% | 98% | 98% |
| Class F | 0.01% | -0.02% | 0.01% | 96% | 97% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

TEST 5, TEST6 and TEST 6

In TEST 5, combination of the proposed Methods I, II and III is tested. And, in TEST 6 and TEST 7, proposed Methods on top of CE4-2.1 and CE4-2.2 is tested, respectively. The experimental results reportedly show overall BD-rate, encoding time and decoding time changes as follows in RA/LB configurations, respectively.

Table 15 TEST 5 experimental results on VTM6.0 RA/LB

|  | Over VTM 6.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |  |  |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.01% | 0.03% | 0.02% | 96% | 95% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  | -0.05% | 0.09% | 0.14% | 99% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -0.04% | 0.62% | 0.35% | 99% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 16 TEST 6 experimental results on VTM6.0 RA/LB

|  | Over VTM 6.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

Table 17 TEST 7 experimental results on VTM6.0 RA/LB

|  | Over VTM 6.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

TEST 8 and TEST 9

In TEST 8 and TEST 9, proposed Method II and Method III on BDOF are tested, respectively. The experimental results reportedly show overall BD-rate, encoding time and decoding time changes as follows in RA configurations.

Table 18 TEST 8 experimental results on VTM6.0 RA

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 19 TEST 9 experimental results on VTM6.0 RA

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Conclusion

In this proposal, three aspects are introduced. First one is removing the bit-depth factor on parameter calculation and second one is to minimize the truncation error and last one is to prevent the 16-bit overflow. These aspects are applied to PROF and BDOF. The experimental results reportedly show minor changes in coding performance and run-time compared to VTM6.0 as anchor in CTC [2]. It is recommended to consider this method in the next VTM.

Reference

- J. Luo, et al., “CE4-2.1: Prediction refinement with optical flow for affine mode”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-O0070, 15th Meeting: Gothenburg, SE, 3–12 July. 2019.

- B. Bross, et al., “Versatile Video Coding (Draft 6)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-MO2001, 15th Meeting: Gothenburg, SE, 3–12 July. 2019.

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).