| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0300-v2 |
| --- | --- |

| Title: | High throughput CABAC mode for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Heiner Kirchhoffer, Detlev Marpe, Benjamin Bross, Tung Nguyen, Christian Rudat, Heiko Schwarz, Thomas Wiegand | Tel: Email: | +49 30/31002-184 heiner.kirchhoffer@ hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

A high throughput mode for VVC is proposed where the context model update is disabled for all context models on a stream level and the arithmetic coding engine uses the generalized bypass mode as proposed in JVET-O0554. The high throughput mode can be implemented with only a few minor changes to the current design of VVC. For three generalized bypass probability values (1/4, 1/8, and 1/16) and QPs 22-37, the BD-rates are 3.68% for AI, 4.49% for RA, 3.09% for LB, and 3.15% for LP. For QPs 2-17, the BD rates are 3.89% for AI, x.xx % for RA, 4.09% for LB, and 4.23% for LP.

# Introduction

This proposal was inspired by the CABAC skip mode of JVET-O0308 and the generalized bypass mode of JVET-O0554. Key aspects of these two proposals are:

- JVET-O0308 proposes a high throughput mode where all bins are directly written to the bitstream instead of passing them through the coding engine. This avoids the derivation and update of associated context models.

- JVET-O0554 proposes a generalized bypass mode where the coding interval subdivision in CABAC employs a simple bit shift instead of a multiplication.

# Proposed method

It is proposed to add a high throughput mode to VVC that encodes regular bins with the generalized bypass mode of JVET-O0554. Context models are initialized as usual and the resulting initial state is then mapped to a generalized bypass mode that is used for all bins of the context model.

## Impact on the implementation complexity

The proposed high throughput mode has the following effects:

| Aspect | Regular mode (VTM-6.0) | Proposed high throughput mode |
| --- | --- | --- |
| Context model update | After each bin | Never |
| RAM per context model | 24 bit | 3 bit |
| Coding interval subdivision | 4x5 bit Multiplier | Bit shift |

## Detailed description of the method

### Context model initialization

The context model initialization for the high throughput uses the existing variable preCtxState to derive a new flag valMpsGB and a new variable pShiftGB as follows:

valMpsGB = preCtxState >> 6
pLps = valMpsGB ? 128 - preCtxState : preCtxState
logShift = Ceil( Log2( pLps + ( pLps >> 2 ) ) )
pShiftGB = Min( 3, 6 – logShift )

Note that the Min() operation controls the number of generalized bypass shifts which is here set to 3, but different values may also be of interest.

### Coding interval subdivision (once per regular bin)

The derivation of the LPS employs a simple bit shift only as follows:

valMps = valMpsGB
ivlLpsRange = ivlCurrRange >> pShiftGB

Note that only 2 bits are required to store pShiftGB.

## Context init value computation

New initialization values have jointly been trained for VTM-6.0 and the proposed method so that no additional initialization values are needed. The initialization value computation method of JVET-N1021 was used with VTM-6.0 bitstreams produced under common test conditions as well as the low QP setting with QPs 2, 7, 12, and 17 with 100 frames. For all bitstreams, the normalized rate d() of JVET-N1021 has been calculated for the VTM-6.0 initialization and entropy coding method as well as for the proposed initialization and entropy coding method. Normalized rates of the VTM-6.0 methods have been multiplied with 80 while normalized rates of the proposed method have been multiplied by 20 before proceeding with the procedure of JVET-N1021.

In this way, normalized rates of VTM-6.0 have a five times higher importance in the selection process of the initialization values than the ones for the proposed method.

## Variant 1: Fixed LPS width coding interval subdivision

A small change to the proposed method is presented in this section, which allows the coding interval to be implemented with 6 instead of 9 bit in the high throughput mode.

For pShiftGB > 1, the coding interval subdivision is modified from

ivlLpsRange = ivlCurrRange >> pShiftGB

to

ivlLpsRange = 384 >> pShiftGB

and for pShiftGB = 1, the bypass mode is used.

Furthermore, the CABAC engine is initialized with

ivlCurrRange = 480

This modifications ensure that the three LSBs of the coding interval variable ‘ivlCurrRange’ are always zero. Consequently, if an implementation needs only to support the high throughput mode, ‘ivlCurrRange’ can be implemented as 6 bit variable. The terminating bit is not changed as it only used for terminating the CABAC stream.

# Experimental results

This section shows the effect of using the new context initialization values and of the proposed high throughput mode.

## VTM-6.0 with new init values over VTM-6.0

As can be seen in this section, the effect of training context init values jointly for VTM-6.0 and the proposed method has a negligible effect on the BD rates for the VTM-6.0 configuration.

Corresponding Excel file names start with ‘JVET-P0300_VTM-6.0_newInit.’

Runtimes are not accurate due to varying simulation conditions.

### QP 22 – 37

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP22-37) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | 0,08% | -0,05% | 94% | 95% |
| Class A2 | 0,03% | 0,23% | -0,06% | 96% | 96% |
| Class B | 0,04% | 0,02% | -0,06% | 100% | 98% |
| Class C | 0,07% | -0,13% | -0,08% | 98% | 98% |
| Class E | 0,11% | 0,09% | -0,02% | 97% | 99% |
| Overall | 0,05% | 0,04% | -0,06% | 97% | 97% |
| Class D | 0,10% | 0,03% | -0,12% | 99% | 97% |
| Class F | 0,02% | -0,17% | -0,22% | 97% | 100% |

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP22-37) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | 0,58% | 0,25% | 90% | 93% |
| Class A2 | 0,02% | 0,73% | 0,02% | 91% | 95% |
| Class B | -0,01% | 0,54% | 0,21% | 93% | 95% |
| Class C | 0,01% | 0,28% | 0,46% | 93% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0,00% | 0,52% | 0,25% | 92% | 95% |
| Class D | -0,01% | 0,05% | 0,37% | 95% | 98% |
| Class F | 0,00% | -0,02% | 0,04% | 94% | 95% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP22-37) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,04% | 0,53% | 0,45% | 91% | 91% |
| Class C | -0,03% | 0,07% | 0,66% | 92% | 92% |
| Class E | 0,01% | 0,14% | 0,69% | 88% | 90% |
| Overall | -0,03% | 0,28% | 0,58% | 90% | 91% |
| Class D | -0,06% | -0,47% | 2,08% | 94% | 96% |
| Class F | -0,09% | -0,53% | 0,43% | 91% | 91% |

|  |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP22-37) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,04% | 0,46% | 0,80% | 91% | 90% |
| Class C | 0,00% | 0,39% | 1,00% | 93% | 93% |
| Class E | 0,05% | 0,22% | 1,21% | 87% | 87% |
| Overall | -0,01% | 0,38% | 0,97% | 91% | 90% |
| Class D | -0,01% | -0,03% | 2,22% | 93% | 95% |
| Class F | -0,11% | 0,49% | -0,01% | 91% | 89% |

### QP 2 – 17, 100 frames

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP2-17) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | -0,03% | -0,03% | 100% | 98% |
| Class A2 | 0,01% | -0,03% | -0,03% | 100% | 95% |
| Class B | -0,01% | -0,05% | -0,01% | 98% | 96% |
| Class C | -0,01% | -0,06% | -0,03% | 99% | 97% |
| Class E | -0,02% | -0,11% | -0,06% | 97% | 98% |
| Overall | -0,01% | -0,05% | -0,03% | 99% | 97% |
| Class D | -0,01% | -0,11% | -0,07% | 102% | 97% |
| Class F | -0,07% | -0,08% | -0,08% | 99% | 97% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP2-17) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | 0,03% | -0,11% | 97% | 96% |
| Class A2 | 0,01% | -0,03% | -0,04% | 95% | 96% |
| Class B | -0,01% | -0,11% | -0,08% | 97% | 95% |
| Class C | -0,05% | -0,18% | -0,09% | 97% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0,02% | -0,08% | -0,08% | 96% | 96% |
| Class D | -0,14% | -0,45% | -0,35% | 96% | 95% |
| Class F | -0,17% | -0,27% | -0,20% | 96% | 98% |

|  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP2-17) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,02% | -0,11% | -0,09% | 96% | 94% |
| Class C | -0,04% | -0,17% | -0,12% | 97% | 97% |
| Class E | -0,01% | -0,25% | -0,20% | 94% | 93% |
| Overall | -0,03% | -0,17% | -0,13% | 96% | 95% |
| Class D | -0,15% | -0,37% | -0,27% | 97% | 97% |
| Class F | -0,17% | -0,29% | -0,17% | 94% | 94% |

|  |  |  | Low Delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP2-17) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,02% | -0,09% | -0,09% | 99% | 97% |
| Class C | -0,04% | -0,19% | -0,13% | 99% | 98% |
| Class E | 0,00% | -0,20% | -0,22% | 96% | 96% |
| Overall | -0,02% | -0,15% | -0,14% | 98% | 97% |
| Class D | -0,10% | -0,33% | -0,23% | 97% | 95% |
| Class F | -0,07% | -0,21% | -0,08% | 95% | 96% |

## Proposed method (with new init values) over VTM-6.0

Three generalized bypass modes are used.

Corresponding Excel file names start with ‘JVET-P0300_numGB3-newInit.’ Encoder runtimes are not accurate due to varying simulation conditions. Decoder runtimes have been produced on an isolated computer with hyperthreading and turbo mode disabled and should be accurate.

### QP 22 – 37

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP22-37) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 5,98% | 3,10% | 3,07% | 98% | 95% |
| Class A2 | 4,93% | 2,92% | 4,71% | 93% | 93% |
| Class B | 3,01% | 2,58% | 2,06% | 96% | 94% |
| Class C | 2,62% | 2,70% | 2,70% | 91% | 95% |
| Class E | 2,66% | 3,11% | 2,37% | 91% | 97% |
| Overall | 3,68% | 2,84% | 2,86% | 94% | 95% |
| Class D | 2,01% | 2,31% | 2,38% | 90% | 95% |
| Class F | 4,53% | 4,25% | 4,21% | 95% | 95% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP22-37) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 6,56% | 6,77% | 7,90% | 87% | 95% |
| Class A2 | 4,37% | 2,74% | 5,79% | 86% | 97% |
| Class B | 4,26% | 2,17% | 2,41% | 88% | 97% |
| Class C | 3,30% | 2,71% | 2,96% | 89% | 98% |
| Class E |  |  |  |  |  |
| Overall | 4,49% | 3,35% | 4,33% | 88% | 97% |
| Class D | 2,80% | 1,95% | 2,89% | 91% | 98% |
| Class F | 5,22% | 5,43% | 5,65% | 92% | 99% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP22-37) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 3,92% | -0,01% | -0,79% | 86% | 97% |
| Class C | 2,57% | 1,03% | 1,42% | 87% | 98% |
| Class E | 2,42% | 1,18% | 0,60% | 84% | 97% |
| Overall | 3,09% | 0,63% | 0,30% | 86% | 97% |
| Class D | 2,85% | -0,82% | 1,46% | 89% | 97% |
| Class F | 5,81% | 6,29% | 7,05% | 87% | 99% |

|  |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP22-37) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 3,84% | -0,28% | -0,82% | 87% | 96% |
| Class C | 2,69% | 1,18% | 1,15% | 89% | 97% |
| Class E | 2,60% | 0,89% | 0,61% | 83% | 96% |
| Overall | 3,15% | 0,50% | 0,20% | 87% | 97% |
| Class D | 2,77% | -1,79% | 1,15% | 91% | 97% |
| Class F | 5,77% | 6,28% | 5,98% | 88% | 98% |

### QP 2 – 17, 100 frames

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP2-17) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 5,07% | 4,19% | 4,58% | 102% | 89% |
| Class A2 | 4,21% | 4,16% | 4,23% | 93% | 87% |
| Class B | 4,49% | 4,60% | 5,08% | 97% | 90% |
| Class C | 2,98% | 2,76% | 2,75% | 95% | 90% |
| Class E | 2,64% | 2,97% | 3,37% | 93% | 89% |
| Overall | 3,89% | 3,78% | 4,05% | 96% | 89% |
| Class D | 2,70% | 2,23% | 2,39% | 97% | 91% |
| Class F | 4,68% | 3,53% | 3,60% | 96% | 93% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP2-17) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #VALUE! |
| Class B | 5,46% | 4,99% | 5,24% | 101% | 89% |
| Class C | 3,51% | 3,85% | 3,32% | 91% | 91% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #VALUE! |
| Class D | 3,04% | 2,79% | 2,71% | 92% | 93% |
| Class F | 6,12% | 4,14% | 4,07% | 93% | 95% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP2-17) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 5,12% | 4,83% | 5,09% | 104% | 89% |
| Class C | 3,32% | 3,68% | 3,11% | 92% | 91% |
| Class E | 3,42% | 1,19% | 2,43% | 92% | 87% |
| Overall | 4,09% | 3,54% | 3,77% | 97% | 89% |
| Class D | 2,82% | 2,49% | 2,75% | 90% | 93% |
| Class F | 5,39% | 4,67% | 4,58% | 93% | 96% |

|  |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP2-17) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 5,18% | 5,05% | 5,10% | 98% | 89% |
| Class C | 3,53% | 3,69% | 2,81% | 91% | 90% |
| Class E | 3,58% | 1,62% | 2,40% | 91% | 85% |
| Overall | 4,23% | 3,74% | 3,66% | 94% | 88% |
| Class D | 2,95% | 2,53% | 2,51% | 89% | 91% |
| Class F | 5,40% | 4,54% | 4,17% | 92% | 95% |

## Variant 1 of proposed method (with new init values) over VTM-6.0

Three generalized bypass modes are used.

Corresponding Excel file names start with ‘JVET-P0300_numGB3fixed-newInit.’

Runtimes are not accurate due to varying simulation conditions.

### QP 22 – 37

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (22-37) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 6,43% | 3,51% | 3,47% | 98% | 89% |
| Class A2 | 5,38% | 3,34% | 5,14% | 92% | 89% |
| Class B | 3,49% | 3,05% | 2,52% | 96% | 91% |
| Class C | 2,97% | 3,05% | 3,05% | 91% | 90% |
| Class E | 3,00% | 3,45% | 2,70% | 91% | 93% |
| Overall | 4,10% | 3,24% | 3,26% | 94% | 91% |
| Class D | 2,37% | 2,69% | 2,75% | 90% | 91% |
| Class F | 4,83% | 4,56% | 4,52% | 95% | 94% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP22-37) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 6,90% | 7,41% | 8,15% | 86% | 89% |
| Class A2 | 4,72% | 3,07% | 6,11% | 85% | 91% |
| Class B | 4,70% | 2,62% | 2,86% | 87% | 92% |
| Class C | 3,61% | 3,00% | 3,30% | 88% | 94% |
| Class E |  |  |  |  |  |
| Overall | 4,85% | 3,77% | 4,68% | 87% | 92% |
| Class D | 3,14% | 2,24% | 3,18% | 91% | 95% |
| Class F | 5,53% | 5,73% | 5,94% | 92% | 94% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (22-37) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 3,05% | 1,65% | 1,83% | 87% | 90% |
| Class E | 3,13% | 1,87% | 1,25% | 83% | 87% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 3,35% | -1,02% | 2,16% | 90% | 93% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (22-37) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 3,15% | 1,60% | 1,61% | 89% | 91% |
| Class E | 3,31% | 1,65% | 1,39% | 83% | 86% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 3,36% | -1,05% | 0,98% | 91% | 94% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

### QP 2 – 17, 100 frames

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP2-17) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 5,45% | 4,57% | 4,95% | 100% | 84% |
| Class A2 | 4,54% | 4,50% | 4,57% | 92% | 82% |
| Class B | 4,75% | 4,87% | 5,33% | 97% | 86% |
| Class C | 3,32% | 3,09% | 3,08% | 95% | 85% |
| Class E | 3,09% | 3,42% | 3,82% | 93% | 86% |
| Overall | 4,24% | 4,12% | 4,39% | 96% | 85% |
| Class D | 3,01% | 2,54% | 2,69% | 97% | 86% |
| Class F | 5,01% | 3,84% | 3,91% | 96% | 89% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP2-17) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 3,97% | 4,22% | 3,69% | 91% | 88% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 3,44% | 3,07% | 3,11% | 91% | 88% |
| Class F | 6,45% | 4,43% | 4,33% | 93% | 90% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP2-17) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 3,75% | 4,08% | 3,51% | 91% | 87% |
| Class E | 4,09% | 1,82% | 3,05% | 93% | 81% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 3,20% | 2,83% | 3,13% | 92% | 89% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (QP2-17) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 5,53% | 5,36% | 5,41% | 97% | 86% |
| Class C | 4,01% | 3,89% | 3,20% | 90% | 88% |
| Class E | 4,23% | 2,25% | 3,00% | 91% | 80% |
| Overall | 4,70% | 4,09% | 4,07% | 93% | 85% |
| Class D | 3,36% | 2,86% | 2,94% | 90% | 87% |
| Class F | 5,66% | 4,68% | 4,54% | 91% | 90% |

## Effect of the number of generalized bypass probabilities

In the following, the effect of using different numbers of generalized bypass probabilities and of recalculating context initialization values is discussed. Different counts of generalized bypass probabilities are realized by modifying the following statement in the context model initialization procedure from

pShiftGB = Min( 3, 6 – logShift )

to

pShiftGB = Min( numGB, 6 – logShift )

where numGB is the number of generalized bypass probabilities.

The following tables show the overall BD rates for different numbers of generalized bypass probabilities and it compares the effect of recalculating context initialization values. Due to the large number of simulations, the number of frames was set to 17 for these simulations.

It can be seen that the new init values give a small improvement for QPs 22 – 37 and a large improvement for QPs 2 – 17.

### QP 22 – 37, 17 frames

numGB 0 – 4 uses the original VTM-6.0 init values while the last row (‘3 newInit’) uses the new init values. The corresponding excel files are in folder ‘JVET-P0300_numGB-comparison’.

|  | AI, Overall results |  |  |
| --- | --- | --- | --- |
|  | Over VTM-6.0, QP22-37, 17 frames |  |  |
| numGB | Y | U | V |
| 0 | 19,51% | 13,72% | 13,70% |
| 1 | 7,26% | 6,64% | 6,26% |
| 2 | 4,60% | 3,77% | 4,56% |
| 3 | 4,15% | 2,76% | 3,50% |
| 4 | 4,00% | 2,25% | 2,80% |
| 3 newInit | 3,80% | 2,90% | 3,21% |

|  | RA, Overall results |  |  |
| --- | --- | --- | --- |
|  | Over VTM-6.0, QP22-37, 17 frames |  |  |
| numGB | Y | U | V |
| 0 | 20,75% | 9,94% | 10,02% |
| 1 | 8,14% | 5,17% | 4,48% |
| 2 | 5,27% | 5,11% | 4,60% |
| 3 | 4,72% | 5,27% | 5,12% |
| 4 | 4,52% | 5,13% | 5,22% |
| 3 newInit | 4,30% | 3,49% | 4,81% |

|  | LB, Overall results |  |  |
| --- | --- | --- | --- |
|  | Over VTM-6.0, QP22-37, 17 frames |  |  |
| numGB | Y | U | V |
| 0 | 19,33% | 9,72% | 7,72% |
| 1 | 6,87% | 4,69% | 2,17% |
| 2 | 3,96% | 3,99% | 2,20% |
| 3 | 3,37% | 3,73% | 2,62% |
| 4 | 3,20% | 3,61% | 2,33% |
| 3 newInit | 3,07% | 2,45% | 2,21% |

|  | LP, Overall results |  |  |
| --- | --- | --- | --- |
|  | Over VTM-6.0, QP22-37, 17 frames |  |  |
| numGB | Y | U | V |
| 0 | 19,27% | 9,47% | 7,26% |
| 1 | 6,66% | 4,28% | 1,70% |
| 2 | 3,76% | 3,89% | 1,66% |
| 3 | 3,25% | 3,57% | 1,94% |
| 4 | 3,16% | 3,24% | 1,13% |
| 3 newInit | 2,97% | 2,33% | 1,96% |

### QP 2 – 17, 17 frames

numGB 0 – 4 uses the original VTM-6.0 init values while the last row (‘3 newInit’) uses the new init values. The corresponding excel files are in folder ‘JVET-P0300_numGB-comparison’.

|  | AI, Overall results |  |  |
| --- | --- | --- | --- |
|  | Over VTM-6.0, QP2-17, 17 frames |  |  |
| numGB | Y | U | V |
| 0 | 18,27% | 19,89% | 20,31% |
| 1 | 7,67% | 8,28% | 8,65% |
| 2 | 6,03% | 6,00% | 6,44% |
| 3 | 5,95% | 5,66% | 6,12% |
| 4 | 6,05% | 5,56% | 6,03% |
| 3 newInit | 3,96% | 3,82% | 4,10% |

|  | RA, Overall results |  |  |
| --- | --- | --- | --- |
|  | Over VTM-6.0, QP2-17, 17 frames |  |  |
| numGB | Y | U | V |
| 0 | 23,88% | 22,63% | 22,06% |
| 1 | 11,85% | 10,75% | 10,83% |
| 2 | 8,12% | 9,79% | 9,08% |
| 3 | 7,90% | 9,33% | 8,72% |
| 4 | 7,97% | 9,17% | 8,61% |
| 3 newInit | 4,94% | 4,14% | 4,35% |

|  | LB, Overall results |  |  |
| --- | --- | --- | --- |
|  | Over VTM-6.0, QP2-17, 17 frames |  |  |
| numGB | Y | U | V |
| 0 | 23,90% | 24,94% | 23,94% |
| 1 | 9,92% | 12,53% | 12,06% |
| 2 | 7,75% | 10,07% | 9,18% |
| 3 | 7,50% | 9,41% | 8,55% |
| 4 | 7,60% | 9,12% | 8,32% |
| 3 newInit | 4,21% | 3,71% | 3,86% |

|  | LP, Overall results |  |  |
| --- | --- | --- | --- |
|  | Over VTM-6.0, QP2-17, 17 frames |  |  |
| numGB | Y | U | V |
| 0 | 23,66% | 24,13% | 23,00% |
| 1 | 9,44% | 12,83% | 11,47% |
| 2 | 7,08% | 10,20% | 8,63% |
| 3 | 6,93% | 9,73% | 8,13% |
| 4 | 6,99% | 9,49% | 7,95% |
| 3 newInit | 4,31% | 3,89% | 3,74% |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).