| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0303 |
| --- | --- |

| Title: | Non-CE3: Removal of leaving out operation for 4×16 and 16×4 MIP blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xinwei Li, Ru-Ling Liao, Yamei Chen, Jie Chen, Jiancong Luo, Yan Ye Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | ruling.lrl@alibaba-inc.com jiechen.cj@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

In VVC draft 6, to predict 4x16 and 16x4 blocks coded using matrix weighted intra prediction (MIP) mode, a leaving out operation that reduces half of rows of matrices is performed. In this contribution, it is proposed to remove the leaving out operation by modifying the block classification in MIP. Specifically, 4x16 and 16x4 blocks are classified into the same set as the 8x8 blocks. It is reported that, under the CTC, the overall performance impact for {Y, U, V} is {0.03%, -0.02%, 0.00%} for AI and {0.01%, 0.08%, -0.01%} for RA.

Introduction

In VVC draft 6, blocks predicted using MIP mode are classified into three sets according to the block size and block shape as follows:

- S0 : for blocks of size 4×4

- S1 : for blocks of sizes 4×8, 8×4 and 8×8

- S2 : for all other blocks

Each set contains a different number of matrices and a different matrix size as shown in Table 1.

Table 1. Differences between sets

| Set | The number of matrices | The size of matrices |
| --- | --- | --- |
| S0 | 18 | 16 rows and 4 columns |
| S1 | 11 | 16 rows and 8 columns |
| S2 | 6 | 64 rows and 7 columns |

To predict a block coded in MIP mode, the reconstructed neighboring samples are firstly averaged to generate a vector for a matrix-vector multiplication process. In the process, the vector is multiplied with a selected matrix, and then a reduced prediction signal can be obtained. Finally, a bilinear interpolation is applied to the reduced prediction signal to generate the prediction samples. An illustration of MIP prediction process is shown in Figure 1.

Figure 1. Matrix weighted intra prediction

During the matrix-vector multiplication process, a leaving out operation is performed only for 4x16 and 16x4 blocks. Since the size of the reduced prediction signal for S2 is 64 samples and is arranged into 8x8, the leaving out operation is performed to guarantee that the size of the reduced prediction signal does not exceed the short side of the blocks. For 4×16 blocks with mode<6 and 16×4 blocks with mode≥6, the second row for every two rows is left out of the matrix and the reduced prediction signal is arranged into 4×8. On the other hands, for 16×4 blocks with mode<6 and 4×16 blocks with mode≥6, the last 8 rows of every 16 rows are left out of the matrix and the reduced prediction signal is arranged into 8×4.

The leaving out operation creates some discrepancy in the prediction process for 16×4 and 4×16 blocks only. It is asserted that such discrepancy unnecessarily complicates the decoder design.

Proposed methods

To remove the leaving out operation, it is proposed to modify the classification rules of MIP blocks. Specifically, the 4×16 and 16×4 blocks are categorized as set S1 instead of set S2.

- S0 : for blocks of size 4×4

- S1 : for blocks of sizes 4×8, 8×4 , 8×8 , 4×16 and 16×4

- S2 : for all other blocks

With this modification, the size of the reduced prediction signal of 4x16 and 16x4 blocks are 16 samples and are arranged into 4x4, which no longer exceeds the short side of the blocks. Thus, the leaving out operation is no longer needed.

Simulation results

The proposed modification is implemented on top of VTM-6.0 and is tested under common test condition. According to the simulation results, it shows that there is negligible impact on coding performance in all intra and random access conditions. Besides, there is no impact on both the encoding time and decoding time.

Table 2. Simulation results of the proposed method

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.08% | -0.03% | 100% | 100% |
| Class A2 | 0.02% | 0.02% | -0.02% | 101% | 100% |
| Class B | 0.02% | 0.00% | -0.09% | 101% | 100% |
| Class C | 0.04% | -0.09% | 0.05% | 100% | 100% |
| Class E | 0.03% | 0.06% | 0.10% | 100% | 99% |
| Overall | 0.03% | -0.02% | 0.00% | 100% | 100% |
| Class D | 0.04% | -0.01% | -0.16% | 101% | 100% |
| Class F | 0.02% | -0.01% | -0.05% | 100% | 100% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.00% | 100% | 100% |
| Class A2 | 0.01% | 0.14% | 0.06% | 100% | 100% |
| Class B | 0.00% | 0.06% | -0.06% | 100% | 99% |
| Class C | 0.01% | 0.12% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.08% | -0.01% | 100% | 100% |
| Class D | -0.02% | -0.08% | 0.03% | 100% | 101% |
| Class F | 0.04% | -0.01% | 0.01% | 100% | 100% |

Conclusion

In this contribution, the leaving out operation in the matrix-vector multiplication process is removed by modifying the block classification rules for MIP mode. Compared to VTM-6.0, this modification has negligible impact on coding performance, encoding time and decoding time. It is suggested to adopt the proposed method to the next version of VVC draft.

Modification of the spec

The proposed text change is highlighted in yellow.

- 8.4.1 General decoding process for coding units coded in intra prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

Output of this process is a modified reconstructed picture before in-loop filtering.

The derivation process for quantization parameters as specified in clause 8.7.1 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight, and the variable treeType as inputs.

When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA, the decoding process for luma samples is specified as follows:

- If pred_mode_plt_flag is equal to 1, the following applies:

- The general decoding process for palette blocks as specified in clause 8.4.5.3 is invoked with the luma location ( xCb, yCb ), the variable startComp set equal to 0, the variable cIdx set to 0, the variable nCbW set equal to cbWidth, the variable nCbH set equal to cbHeight.

- Otherwise (pred_mode_plt_flag is equal to 0), the following applies:

- The variable MipSizeId[ x ][ y ] for x = xCb..xCb + cbWidth − 1 and  y = yCb..yCb + cbHeight − 1 is derived as follows:

- If both cbWidth and cbHeight are equal to 4, MipSizeId[ x ][ y ] is set equal to 0.

- Otherwise, if both cbWidth and cbHeight are less than or equal to 8cbWidth * cbHeight is less than or equal to 64, MipSizeId[ x ][ y ] is set equal to 1.

- Otherwise, MipSizeId[ x ][ y ] is set equal to 2.

- The luma intra prediction mode is derived as follows:

- If intra_mip_flag[ xCb ][ yCb ] is equal to 1, IntraPredModeY[ x ][ y ] with  x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to intra_mip_mode[ xCb ][ yCb ].

- Otherwise, the derivation process for the luma intra prediction mode as specified in clause 8.4.2 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight as input.

- The variable predModeIntra is set equal to IntraPredModeY[ xCb ][ yCb ].

- The general decoding process for intra blocks as specified in clause 8.4.5.1 is invoked with the sample location ( xTb0, yTb0 ) set equal to the luma location ( xCb, yCb ), the variable nTbW set equal to cbWidth, the variable nTbH set equal to cbHeight, predModeIntra, and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before in-loop filtering.

When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA, and when ChromaArrayType is not equal to 0, the decoding process for chroma samples is specified as follows:

- If pred_mode_plt_flag is equal to 1, the following applies:

- The general decoding process for palette blocks as specified in clause 8.4.5.3 is invoked with the luma location ( xCb, yCb ), the variable startComp set equal to 0, the variable cIdx set to 1, the variable nCbW set equal to ( cbWidth / SubWidthC ), the variable nCbH set equal to ( cbHeight / SubHeightC ).

- The general decoding process for palette blocks as specified in clause 8.4.5.3 is invoked with the luma location ( xCb, yCb ), the variable startComp set equal to 0, the variable cIdx set to 2, the variable nCbW set equal to ( cbWidth / SubWidthC ), the variable nCbH set equal to ( cbHeight / SubHeightC ).

- Otherwise (pred_mode_plt_flag is equal to 0), the following applies:

- The derivation process for the chroma intra prediction mode as specified in clause 8.4.3 is invoked with the luma location ( xCb, yCb ) , the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight as input.

- The general decoding process for intra blocks as specified in clause 8.4.5.1 is invoked with the sample location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / SubWidthC , yCb / SubHeightC  ), the variable nTbW set equal to ( cbWidth / SubWidthC  ), the variable nTbH set equal to ( cbHeight / SubHeightC ), the variable predModeIntra set equal to IntraPredModeC[ xCb ][ yCb ], and the variable cIdx set equal to 1, and the output is a modified reconstructed picture before in-loop filtering.

- The general decoding process for intra blocks as specified in clause 8.4.5.1 is invoked with the sample location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / SubWidthC  , yCb / SubHeightC ), the variable nTbW set equal to ( cbWidth / SubWidthC  ), the variable nTbH set equal to ( cbHeight / SubHeightC ), the variable predModeIntra set equal to IntraPredModeC[ xCb ][ yCb ], and the variable cIdx set equal to 2, and the output is a modified reconstructed picture before in-loop filtering.

- 8.4.5 Decoding process for intra blocks

- 8.4.5.2 Intra sample prediction

- 8.4.5.2.1 Matrix-based intra sample prediction

Inputs to this process are:

- a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable predModeIntra specifying the intra prediction mode,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

Variables numModes, boundarySize, predW, predH and predC are derived using MipSizeId[ xTbCmp ][ yTbCmp ] as specified in Table 8-4.

Table 8-4 – Specification of number of prediction modes numModes, boundary size boundarySize, and prediction sizes predW, predH and predC using MipSizeId

| MipSizeId | numModes | boundarySize | predW | predH | predC |
| --- | --- | --- | --- | --- | --- |
| 0 | 35 | 2 | 4 | 4 | 4 |
| 1 | 19 | 4 | 4 | 4 | 4 |
| 2 | 11 | 4 | Min( nTbW, 8 )8 | Min( nTbH, 8 )8 | 8 |

The flag isTransposed is derived as follows:

isTransposed = ( predModeIntra > ( numModes / 2 ) )  ?  TRUE  :  FALSE	(8-55)

The variable inSize is derived as follows:

inSize = ( 2 * boundarySize ) − ( MipSizeId[ xTbCmp ][ yTbCmp ]  = =  2 )  ?  1  :  0	(8-56)

The variables mipW and mipH are derived as follows:

mipW = isTransposed  ?  predH  :  predW		(8-57)

mipH = isTransposed  ?  predW  :  predH		(8-58)

For the generation of the reference samples refT[ x ] with x = 0..nTbW − 1 and refL[ y ] with y = 0..nTbH − 1, the following applies:

- The reference sample availability marking process as specified in clause 8.4.5.2.7 is invoked with the sample location ( xTbCmp, yTbCmp ), reference line index equal to 0, the reference sample width nTbW, the reference sample height nTbH, colour component index equal to 0 as inputs, and the reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..nTbH − 1 and x = 0..nTbW − 1, y = −1 as output.

- When at least one sample refUnfilt[ x ][ y ] with x = −1, y = −1.. nTbH − 1 and x = 0.. nTbW − 1, y = −1 is marked as "not available for intra prediction", the reference sample substitution process as specified in clause 8.4.5.2.8 is invoked with reference line index 0, the reference sample width nTbW, the reference sample height nTbH, the reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..nTbH − 1 and x = 0..nTbW − 1, y = −1, and colour component index 0 as inputs, and the modified reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..nTbH − 1 and x = 0..nTbW − 1, y = −1 as output.

- The reference samples refT[ x ] with x = 0..nTbW − 1 and refL[ y ] with y = 0..nTbH − 1 are assigned as follows:

refT[ x ] = refUnfilt[ x ][ −1 ]		(8-59)

refL[ y ] = refUnfilt[ −1 ][ y ]		(8-60)

For the generation of the input samples p[ x ] with x = 0..2 * inSize − 1, the following applies:

- The MIP boundary downsampling process as specified in clause 8.4.5.2.2 is invoked for the top reference samples with the block size nTbW, the reference samples refT[ x ] with x = 0..nTbW − 1, and the boundary size boundarySize as inputs, and reduced boundary samples redT[ x ] with x = 0..boundarySize − 1 as outputs.

- The MIP boundary downsampling process as specified in clause 8.4.5.2.2 is invoked for the left reference samples with the block size nTbH, the reference samples refL[ y ] with y = 0..nTbH − 1, and the boundary size boundarySize as inputs, and reduced boundary samples redL[ x ] with x = 0..boundarySize − 1 as outputs.

- The reduced top and left boundary samples redT and redL are assigned to the boundary sample array pTemp[ x ] with x = 0..2 * boundarySize − 1 as follows:

- If isTransposed is equal to 1, pTemp[ x ] is set equal to redL[ x ] with x = 0..boundarySize − 1 and pTemp[ x + boundarySize ] is set equal to redT[ x ] with x = 0..boundarySize − 1.

- Otherwise, pTemp[ x ] is set equal to redT[ x ] with x = 0..boundarySize − 1 and pTemp[ x + boundarySize ] is set equal to redL[ x ] with x = 0..boundarySize − 1.

- The input values p[ x ] with x = 0.. inSize − 1 are derived as follows:

- If MipSizeId[ xTbCmp ][ yTbCmp ] is equal to 2, the following applies:

p[ x ] = pTemp[ x + 1 ] − pTemp[ 0 ]		(8-61)

- Otherwise (MipSizeId[ xTbCmp ][ yTbCmp ] is less than 2), the following applies:

p[ 0 ] = pTemp[ 0 ] − ( 1 << ( BitDepthY − 1 ) )
p[ x ] = pTemp[ x ] − pTemp[ 0 ]	for x = 1.. inSize − 1	(8-62)

For the intra sample prediction process according to predModeIntra, the following ordered steps apply:

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW − 1, y = 0..mipH − 1 are derived as follows:

- The variable modeId is derived as follows:

modeId = predModeIntra − ( isTransposed  ?  numModes / 2  :  0 )	(8-63)

- The weight matrix mWeight[ x ][ y ] with x = 0..2 * inSize − 1, y = 0..predC * predC − 1 is derived by invoking the MIP weight matrix derivation process as specified in clause 8.4.5.2.3 with MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as inputs.

- The variable sW is derived using MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as specified in Table 8-5.

- The variable sO is derived using MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as specified in Table 8-6.

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW − 1, y = 0..mipH − 1 are derived as follows:

oW = ( 1 << ( sW − 1 ) ) − fO * (i = 0inSize - 1p i )	(8-64)

incW = ( predC > mipW )  ?  2  :  1	(8-65)

incH = ( predC > mipH )  ?   2  :  1	(8-66)

predMip[ x ][ y ] = ( ( ( i = 0inSize - 1mWeight i  y*incH*predC+x*incW *p i  ) + oW ) >> sW ) + pTemp[ 0 ]	(8-67)

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW − 1, y = 0..mipH − 1 are clipped as follows:

predMip[ x ][ y ] = Clip1Y( predMip[ x ][ y ] )	(8-68)

- When isTransposed is equal to TRUE, the predH x predW array predMip[ x ][ y ] with x = 0..predH − 1, y = 0..predW − 1 is transposed as follows:

predTemp[ y ][ x ] = predMip[ x ][ y ]	 	(8-69)

predMip =predTemp			(8-70)

- The predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If nTbW is greater than predW or nTbH is greater than predH, the MIP prediction upsampling process as specified in clause 8.4.5.2.4 is invoked with the input block width predW, the input block height predH, matrix-based intra prediction samples predMip[ x ][ y ] with x = 0..predW − 1, y = 0..predH − 1, the transform block width nTbW, the transform block height nTbH, the top reference samples refT[ x ] with x = 0..nTbW − 1, and the left reference samples refL[ y ] with y = 0..nTbH − 1 as inputs, and the output is the predicted sample array predSamples.

- Otherwise, predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 is set equal to predMip[ x ][ y ].

Table 8-5 – Specification of weight shift sW depending on MipSizeId and modeId

|  | modeId |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MipSizeId | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| 0 | 6 | 6 | 6 | 5 | 6 | 5 | 5 | 6 | 5 | 6 | 6 | 6 | 6 | 6 | 5 | 6 | 5 | 5 |
| 1 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 7 |  |  |  |  |  |  |  |  |
| 2 | 7 | 5 | 6 | 6 | 6 | 6 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8-6 – Specification of offset factor sO depending on MipSizeId and modeId

|  | modeId |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MipSizeId | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| 0 | 34 | 21 | 7 | 27 | 27 | 28 | 56 | 13 | 47 | 15 | 40 | 21 | 16 | 7 | 45 | 66 | 21 | 32 |
| 1 | 17 | 20 | 11 | 45 | 17 | 11 | 23 | 10 | 21 | 11 |  |  |  |  |  |  |  |  |
| 2 | 8 | 46 | 16 | 10 | 13 | 11 |  |  |  |  |  |  |  |  |  |  |  |  |

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).